#############################################################################

#                             Makeuls V 1.01		                    #

#                      Written by K. Terpstra - PA3GNP                      #

#                   BBS list converter for TstHost V 1.41                   #

#############################################################################

#  This program and source contains no Copyright and may used, changed ,    #

#                    updated etc. as much as you like                       #

#############################################################################

The Objective of the program.

-----------------------------

Sometime's our local FBB Mailbox on 2 Mtr went of the air caused by natural or

technical problems, and since on 2 meter only the unproto list was broadcast,

we have to find a way to add the titles in our unproto list already on disk.

Our local BBS is also active on 70 cm, but without unproto broadcast's.

The main question was "how do we get our list?"

The main answer was simple, read the following lines to examine how simple

all is done.

1). Connect your local BBS

2). If connected, type in the CMD mode RECORD <filename>

3). Return to the CONV mode and type 'L' to get the list

4). Now you will receive the BBS list, wait until ready.

5). When your BBS prompt returns, go to the CMD mode

and type RECORD OFF.

6). Return to the CONV mode and type 'B' to disconnect.

Now the only thing wat you must do is to run Makeuls.exe <filename> <filename>

The two parameters may be the same since Makeuls adds the extention .MUL to

the converted file.

Example: 	Makeuls PI8VNW.TXT PI8VNW

Now the files PI8VNW.TXT will be read into memory, status will be adjusted

so that only the P or B status remain in the destination file. If all was

run ok, the program will save the contents into the name PI8VNW and will

be added the extension .MUL to the file, so that deletion of already

existing files can not take place! The contents of the file will be sorted

too and written to disk.

The only thing you have to do now, is to run Tsthost. When Tsthost is ready

to receive data, you can type the following:

Ulist Mergelist PI8VNW.MUL

Now the file will be read into memory and Tsthost will merge the contains

from that file to the existing file. That's all !!

NOTE: I have tested several way's to get the list merged to the existing

list. Even without using this program it seems that somtimes Tsthost

will add the unprocessed list to the existing list, but sometime's

the file i like to add to the list, was NOT added to the existing

list !!

So for 100% FB list's keep use this or other program's to convert

the BBS list into an unproto list, otherwise strange things can

happen !!

For any addition, comment or POSITIVE reactions only HI, you can reach me

on the following packet address: PA3GNP @ PI8VNW.#ZH2.NLD.EU

Best 73 and happy BBS'ing!