AUTOBPQ V1.0

-------------------------------------------------------------------------------

AUTOBPQ is a program designed to operate under the G8BPQ node software

and is primarily designed for the BPQ node-op.

The program will not function unless BPQ is loaded and configured properly.

The main function of the program is to automatically log on to your home

bbs and issue the "L" List command to the bbs and then log off.

The text received from your home bbs is appended to a file

titled AUTOBPQ.LOG for viewing with your favorite text editor or the one

supplied with the program. (LIST.COM)

(The DOS TYPE command will not work with this file)

The program is "hard coded" to log on once every hour and issue the List

command and log off.  I may change this in a later version to be

configurable by the node-op.

-------------------------------------------------------------------------------

REQUIREMENTS

#1.

First, BPQ must be loaded and configured properly of course.

The BPQHTNC2 program included with BPQ must be loaded after BPQCODE.

At the present time AUTOBPQ is "hard coded" to communicate with BPQ using

the virtual com port address of COM8.

In BPQCFG.TXT you MUST define COM=8 as a virtual port in order for the

program to work.

Example of parameter to be included in your BPQCFG.TXT file:

TNCPORT

COM=8

TYPE=TNC2

APPLMASK=$02

APPLFLAGS=$06

ENDPORT

#2.

Next, you must edit the file titled AUTOBPQ.DAT with your text editor.

If the file does not exist then create it.

The file must be created using a standard ASCII text editor.

There are two parameters that MUST be included within this file.

The first is the CALLSIGN or ALIAS Callsign of the bbs in which you wish the

program to connect to.

The second parameter is your home bbs's bbs prompt.

THIS MUST MATCH EXACTLY!

The bbs prompt usually contains the callsign of the bbs and the ">" (right

arrow or more than symbol).

ie; N8MLU BBS>

AUTOBPQ will wait until it see's your bbs's prompt and at which point will

send the "L" List command to the bbs.

After the list of new messages has been sent by your bbs the bbs will once

again send its bbs prompt.

When AUTOBPQ see's the bbs prompt the second time, it will then issue the

"B" Bye command to the bbs and commence logging off.

I must stress once again the importance of the bbs prompt in your

AUTOBPQ.DAT file.

*** IT MUST MATCH YOUR BBS PROMPT EXACTLY!!! ***

AUTOBPQ also recognizes the Busy, Failure, Invalid, etc node messages.

#3.

After configuring BPQCFG.TXT and AUTOBPQ.DAT you are ready to begin.

Load BPQCODE first.

Load BPQHTNC2 second.

Load AUTOBPQ.EXE last.

AUTOBPQ will log onto the bbs in exactly one hour upon execution of the

program and every hour there after.

Or you may force the program to log on at any time by pressing the F1 key.

You may also use the program as a terminal between connect times.

However, during the auto connect sequence to the bbs the keyboard is locked.

The terminal portion of the program contains no bells and whistles.

No capture buffer or scroll back buffer in this version for the terminal.

-------------------------------------------------------------------------------

Testing of AUTOBPQ was performed on an PC XT 8088 640K clone with monochrome

graphics and a 386 AT 3Mb clone with VGA graphics.

DOS 5.0

The tnc's in use were MFJ-1270 and a PK-88.

I have only tested the program in conjunction with the F6FBB bbs software.

It should work with any bbs program. Just remember to disable PAGING or

the CONTINUE/MORE prompt at your home bbs.

_______________________________________________________________________________

Thank You's are directed to Gary N8GTC for the use of his bbs during the

testing of the program.

and

Roy N5RG and Mike K8LG for the use of some of their routines and

programming expertise.

-------------------------------------------------------------------------------

If you have any questions, suggestions, etc, concerning the program please

send them to me (the author) at the packet address listed below.

I'd like to hear from anyone using the program on a steady basis.

Frank N8MLU @ N8MLU.#CIN.OH.USA.NOAM

-------------------------------------------------------------------------------

AUTOBPQ is FREEWARE.

I claim no responsibilty nor make any guarantees regarding the operation

of the program.

-------------------------------------------------------------------------------

LIST.COM copyright by Vernon D. Buerg

G8BPQ software copyright by John Wiseman G8BPQ