To:

Karl Medcalf, Esq,

Kantronics, Inc.

From:

John Wiseman,

76, Haywood Road,

Mapperley,

Nottingham.

NG3 6AE.

28/6/90

Hello Karl,

I have modified the following in line with our conversation yesterday - I

hope I followed what you wanted!.

If, as I gather you wanted, you expect all frames to have leading and trailing

FENDS, and all data between the FENDS to have the transparency rules applied,

it leaves a couple of problems.

The first applies even to the current KISS spec - how would you encode a 'set

persistence' frame with a value of 192?  The code that comes with the KA9Q

package expects FEND 02 FEND, where the second FEND is actually the value 192.

In fact none of the 'parameter setting' messages require a terminating

FEND, or use transparency.

The other concerns the command byte combinations which match the reserved

KISS controls - ie a data frame to channel 12 (c0 hex = FEND), or an 'ack

required' frame to channel 13 (dc hex = TFEND). I seem to remember that your

host mode spec gets round a similar problem by only applying the transparency

rules to the data portion of frames. I don't really mind which way we go,

but it is obviously important that we both follow the same rules.

1. CWID active in KISS mode.

I havn't mentioned this before, but our licencing authority is getting

rather nasty about stations which are not sending CWIDs as required

by our licence.

2. Confirmation that a frame has been sent.

If a frame has a control byte of 'xC', where x is the channnel number,

then it is a data frame which requirs a response to be sent to the

host when the frame has been completely transmitted. The first two

bytes of the frame should not be transmitted on air, but returned to

the host as part of ACK. The ACK consists of the first 3 bytes (including

control byte) of the received frame, followed by FEND. If either of the

two ID bytes has the value FEND or FESC, the normal transparency processing

should be applied.

ie  If the following is received from the host

FEND 0C 12 23 45 56 67 .... FEND

Then  the message 45 56 67 ....   should be transmitted on air,

and then the message   FEND 0C 12 34 FEND should be returned to the host.

Note that the normal data frame (control byte 0) will still be used

when confirmation of transmission is not required.

3. Checksum.

For all data frames (original or 'confirmation required'), a simple

checksum should be appended, preceeding the closing FEND. This should

be the XOR of all byte in the frame, including the two FENDS and the

control byte. The checksum should be calculated on the bytes actually

sent over the async link, ie after any imbedded FEND or FESC chars have

been replaced by the corresponding escape pair. This mode is not

compatible with 'standard' KISS, so will need to be selected by some

sort of configuration option.

4. Configurable channel number.

The channel number (upper 4 bits of control byte) should be user

configurable. This is primarily for use with Polled Mode (see next

item).

5. Polled Mode.

When Polled Mode is selected, the TNC should only transmit to the host

on receipt of a poll, of format 'FEND xE FEND' , where x is the channel

number. If the TNC has no data to send, it should return a 'no data'

frame, of the same format as the received poll.

73,

John