Round Table for the G8BPQ switch.

by

H. N. Oredson, W0RLI

# Table of Contents

Table of Contents

What is RoundTable?

Getting started:

The file RT.INI

G8BPQ Switch Application area

Nodes, Topics and Users

User and Link Time-out

Links

Key Definitions

Window Specifications

Example Window Specification:

Sounds and Events

ROUND TABLE COMMANDS

Sysop Commands

Common Commands

# What is RoundTable?

RoundTable is a G8BPQ node application that permits people to chat in a ”Round Table” discussion. Copies of RoundTable running on a group of nodes may be linked into a single chat session, available at each node. Chats can be grouped by topic.

# Getting started:

First, configure the G8BPQ switch for RoundTable. In the file BPQCFG.TXT, add the new application RT. It must be the fourth or later application.

Example: APPLICATIONS=BBS,,,RT

RoundTable running at different nodes can be linked together, to form a single RoundTable. If you wish to do this, add the application *RTL in your BPQCFG.TXT file.

Example: APPLICATIONS=BBS,,,RT,*RTL

When RoundTable is in use, it will automatically link to the RoundTable servers running at the nodes you specify. RoundTable must be running on both systems for this to work.

Run BPQCFG and re-boot your system.

Configure RoundTable. See the file RT.INI and the information below.

The file RT.INI must be put in the directory specified by environment variable RLICFG, if defined, otherwise in directory \BBS.

Startup RoundTable in an 85K DESQview window.

Now anyone that wishes to use the RoundTable can connect to your node and issue the RT command. You can log in to and out of RoundTable locally with the F3 key. The F4 key shuts RoundTable down.

RoundTable keeps track of the name and qth of each user in the file RT.USR. RoundTable keeps this file in the directory specified by environment variable RLIDB, if defined, otherwise in directory \BBS.

# The file RT.INI

This file consists of areas separated by blank lines. Each area defines a specific aspects of RoundTable and how it will function.

### G8BPQ Switch Application area

RTAPPL: 5 (The G8BPQ application number as defined in BPQCFG.TXT file for RoundTable. It must be g at least 4.)

RTLinkAPPL: 6 (The BPQ application number as defined in BPQCFG.TXT file for *RTL. It must be at least 4.)

### Buffers

Buffer: 2000 [Size of the buffer to use for each connection. Use a larger size if the network is slow and you have a large number of links to other RT systems.]

### Nodes, Topics and Users

MaxLocal: 11 (The number of users that can simultaneously use RoundTable from this node.)

MaxTopics: 10 (The number of potential topics that can run simultaneously during an RoundTable session.)

MaxNodes: 30 (The maximum number of potential nodes that can be linked in the Linked Round Table Session.)

MaxUsers: 64 (The maximum number of users that can be connected to the Linked Round Table Session for all nodes.)

### User and Link Time-out

Timeout: 600 (Time-out in seconds on non-activity before a user or link to another node is automatically disconnected from an RoundTable session. This number should be set to a sufficiently high number to allow for all links to be included. Generally with a small system of 1 or 2 nodes with a good path 240 seconds might be OK. But with a large number of nodes 600 seconds might be more appropriate.)

InfoMsg: YES Displays message to the sysop when links to other systems occur. Shows possible routing loops and multiple connects. This is toggle. NO will turn it off.

### Links

Links: Callsign Callsign etc. (Defines which nodes will be included in the Linked RoundTable session. Aliases may be used here instead of or in addition to callsigns.

Examples:

Links: breeze bparc kalama:wa7sjn-1 rdland sandy

In the case you wish to link you RoundTable to a distant node running RoundTable, and the distant node does not appear in your local node list, you can specify the full path required to connect to the distant node.

Links: breeze cedar lyons/#kf2/john1/john2/kreg/n6iya-1:mrybay

In this example, the link to node mrybay will be made by first connecting to lyons, then to #kf2, etc.

### Key Definitions

KeyShut: F4 Defines control key for shutdown.

KeyLogin: F3 Defines control key to login from console.

KeyLogout: F3 Defines control key to logout from console.

### Window Specifications

Syntax: <Window Name:> <Foreground> <Background> <Cursor>

Window names:

WinTop

WinMain

WinBottom

Foreground Colors: All colors in the table below.

Background Colors: Colors from the first column of the table below.

| BLACK | DARKGREY |
| --- | --- |
| BLUE | LIGHTBLUE |
| GREEN | LIGHTGREEN |
| CYAN | LIGHTCYAN |
| RED | LIGHTRED |
| MAGENTA | LIGHTMAGENTA |
| BROWN | YELLOW |
| WHITE | LIGHTGREY |

Cursor Types:

NORMAL

BLANK

INSERT

Text Blink Characteristics

BLINK

NOBLINK

### Example Window Specification:

WinTop: White Red Blank

WinMain: White Blue Normal

WinBottom: Black White Insert

### Sounds and Events

You can attach sounds to various events that happen during a RoundTable session. These sounds are in the form of tunes that can be written by the sysop using a 4 octave scale, setting tempos, note duration and rests. The musical selection is defined in the RT.INI file as follows:

Tune: Name of the tune

Tempo: The tempo that the tune will play at. The lower the number the faster the tune will play.

Play: defines the notes in <duration> <note> pairs

There is a maximum of 64 notes per tune.

The scale contains 4 octaves. It starts at C0 and runs through G4 and includes sharp notes. The note names are as follows:

C0 C0S D0 D0S E0 F0 F0S G0 G0S A0 A0S B0

C1 C1S D1 D1S E1 F1 F1S G1 G1S A1 A1S B1

C2 C2S D2 D2S E2 F2 F2S G2 G2S A2 A2S B2

C3 C3S D3 D3S E3 F3 F3S G3 G3S A3 A3S B3

C4 C4S D4 D4S E4 F4 F4S G4

Sample Tune:

Tune: Green

Tempo: 3

Play: 2 E1 4 G1 2 A1 3 B1 1 C2S 2 B1 4 A1 2 F1S 4 D1

Events

There are 4 events that may occur during a RoundTable session. They are Join, Leave, Link and Unlink. A tune can be attached to each of these events to notify the sysop of the occurrence of that event.

To attach tunes to the events use the syntax <Event name:> <Tune name>

For example:

Join: TingTong

Leave: BingBong

Link: Alarm2

Unlink: Clonk

Note: These tunes are included in the example RT.INI file.

Note: The user does not hear any of these tunes, they are only heard at the computer running RoundTable.

### ROUND TABLE COMMANDS

There are 2 classes of commands. These are the sysop only commands and the commands common to both users and the sysop. All commands are in the syntax </><COMMAND LETTER> [QUALIFIER] except for sysop join and shut down of RoundTable. These are the F3 and F4 keys respectively which are immediate actions.

### Sysop Commands

/D <n> - Disconnect stream n from the Round Table Session.

/F - Force all links to be made as defined in the Link list in RT.INI.

/XT <Tune> - Play a tune from the tune list in RT.INI.

/XA <Window Name> <Foreground> <Background> <Cursor> Change window attributes.

### Common Commands

/H - Show the command list.

/U - Show users.

/N <name> - Enter your name.

/Q <qth> - Enter your location.

/T - Show topics during current session.

/T <n> - Join a topic where <n> is the topic name.

/T <name> - Create a new topic in the current session.

/P - Show streams and links.

/E - Toggle echo on and off. Stops the echo of your text on screen. Defaults to on.

/B - Leave Round Table and Return to node.

/S <call> <Text> - Send text to specific call only.