--  KISSMODE  --

version  1.0

by

OZ4ABL

Jesper "Jones" Hansen

Disclaimer:

-----------

This program (and it's source) is destributed as blse-ware, which means

that i don't care what you do with it. You man change my name and claim

that you have codet it, i won't care a bit because i codet it for my own

use, but i have mailed it because there are maybe other OM's (and YL's)

that can use this program.

I can't in any way be responsible in any way if this program blows of you'r

house, you'r dog dies of cancer two days after you got this program, or

your computer melts right down through you house.

But if such a thing should happend i will be werry glad to hear about it.

Description:

------------

KISSMODE is a program to switch you'r TNC2C modem in and out of KISS mode.

I know there are other utilities that can do this, BUT they reset the

TNC to default values upon exiting KISS mode. This is also ok for most

users of KISS mode, but if you have some speciel values that not are burned

into the ROM, reseting to the defailt values can be pretty anoing.

An exaplme: you have just bought you'r TNC from a friend and his call is

still in the ROM because you don't know anybody with a ROM programmer, you

must then remember to change the call every time you exit from KISS mode.

Why do i want to swap in and out of KISS mode ??

1) some programs can only work in KISS mode.

2) Some programs can only work in HOST mode, but not all TNC's have a host

mode, so therefor you must first load a special driver (such as TFPCR).

But programs like TFPCR requires the TNC to be in KISS mode before you

start them.

The later was my problem: i have just gotten GP (Graphical Packet), but it

required a HOST mode TNC. There wasn't a HOST mode i ny TNC2C so therefor

i had to load TFPCR before starting GP. Ofcause i could have my TNC in

KISS mode all the time but in the night and when my computer was off i

would like to use the PMS bould into it, and that diddent work in KISS mode.

How KISSMODE operates:

----------------------

When you want to enter KISS mode, the program first sends a "DISPLAY" command,

and records the walues in a file. After this it will send the "KISS ON" and a

"RESTART" command. Now you'r TNC is in KISS MODE!

When the modem must leave KISS mode KISSMODE sends the KISS-off string:

0xD0 0xFF 0xD0, witch will reset the modem. Now all values are default and

the program sends back those that was recorded when kiss mode was entered.

Usage:

------

KISSMODE [-caibfh] mode

where:

-c<port>     = com port number (default:1)

-a<address>  = Hexadecimal address of comport (default:3F8)

-i<irq>      = interrupt vector number (default:4)

-b<baud>     = baud-rate (default:19200)

-f<filename> = file name to backup/restore (default:TNCPARMS.OLD

-h           = a help text.

mode         = can be either ON or OFF, ON to turn KISS mode on,

and OFF to turn KISS mode off!

c:	If you have you'r TNC hooked up to you'r computer via another port

than COM1 you can use this option to tell the program.

a & i:	If you don't use COM1 or COM2 you will have to specify the base

address and the irq number this by using this and the I options.

b:	Use this to tell the program wich speed you'r modem is uses to

communicate with the computer.

f:	If you want to configure you'r modem with another file than the

one KISSMODE have made you can use this option.

h:	Here you can get a little help.

mode:	This ain't a switch but a parameter you will have to specifye.

There are two possible values:

ON - Will switch you'r TNC into KISS mode after it have recorded

the current settings.

OFF - Will drop you'r TNC out of KISS mode and then send a

reconfigure-file to it.

Some exaples:

KISSMODE on - will use COM1 with a speed of 19200bps to enter KISS mode.

KISSMODE -c2 -b9600 off - will use COM2 and 9600bps to leave KISS mode.

KISSMODE -a3f8 -i3 -fDEFAULT1.TNC OFF - will use the port at address 0x3f8

(normaly COM1) and interrupt 3 to leave kiss mode. Then it will send the

file DEFAULT1.TNC to the TNC.

Known bugs:

-----------

There is a bug in some TNC's witch will cut down long strings in some

special occaions. This can normaly be solved by removing the extra spaces

between the command and the value. For example:

BTEXT   OZ4ABL - Jesper, QTH: DK-Soednerborg, JO44VW

^^^

When KISSMODE sends such a string it will automaticly remove the two extra

spaces and there will be no broblem.

Unknown bugs?

-------------

If you use this program and find some bugs i will be glad to hear from you.

Last words:

----------

This is the first program i have posted over the HAMNET, but it will

certanly not be the last.

All spelling-errors are (C) Copyrighted by me and may not be changed!

+---------------------------------+---------------------------------+

| There are no such thing as bugs | Vy 73 de OZ4ABL - Jesper Hansen |

|   only unrecognizes features!   |  "Jones/Funky Buddhas/Rednex"   |

+---------------------------------+---------------------------------+

|      --==>>      OZ4ABL @ OZ6BOX.kru.jys.dnk.eu.      <<==--      |

+-------------------------------------------------------------------+