�������������������������������������������������Ŀ

� ���������������������������������������������Ŀ �

� �  J N O S    I N S T A L L    P R O G R A M  � �

� ����������������������������������������������� �

���������������������������������������������������

by David   ZL3AI @ ZL3AC.#80.NZL.OC

25 September 1995

for configuring JNOS versions 1.10h, i, j and k.

����������͸

� CONTENTS �

����������;

1.0   INTRODUCTION

1.1     Why bother to run JNOS at all

1.2     What you need to run JNOS

1.3     Why INSTALL was written

1.4     Where to read more about TCP/IP and JNOS

2.0   INSTALLATION

2.1     What does INSTALL do

2.2     What essential things do YOU need to do

2.3     Not-quite-so-essential things to do

3.0   IP ADDRESSES

3.1     IP address scheme

4.0   FAULTFINDING

4.1     Program doesn't seem to run

4.2     Error message: "Autoexec.nos file not found"

4.3     No received data appears on screen

4.4     TNC does not go into KISS mode

4.5     Unable to transmit

4.6     Unable to ping yourself

4.7     Unable to ping others

4.8     Error messages appearing on received packets

4.9     Can't send mail

4.10    No colour in monitored packets, left pointing arrows instead

4.11    Large numbers of error messages on start-up

4.12    JNOS crashes when shelling

5.0   WHEN THE DUST SETTLES

5.1     Feedback request

5.2     JNOS versions

�������������������͸

� 1.0  INTRODUCTION �

�������������������;

1.1  Why bother to run JNOS at all?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

(This list is very brief and simplified, but here it is for openers..)

JNOS enables you to:

* Do ordinary packet connects to and from other stations

* Run a full service ax25 BBS with FBB style compressed forwarding

* Be a Netrom node

* Send and receive mail to/from ax25 BBSes

* Send and receive mail to/from TCP/IP systems

* Read and write mail

* Send and receive text and binary files

* Read, copy, delete, rename, etc, files on your own computer

* Intelligently route packets throughout a network of stations

* Use more than one RF port

* (And more.....)

* Do ALL of the above SIMULTANEOUSLY!

1.2  What you need to run JNOS.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

You need to have a sneaking suspicion that there might be more to

packet radio than using your high powered PC like a teleprinter.

As far as equipment goes, you need an IBM or compatible PC with 640k

of RAM.  If you want to run a modem you will need a 286 or better.  If

you will be using a TNC it must be capable of KISS mode (the

configuration files will automatically get the TNC in and out of KISS

mode).  No fancy operating systems, plain old DOS is ideal.  My JNOS

system takes up about 330k of my 286's 20 meg hard disk space, and I

know of some systems that run on floppy disk based machines.  If you

are using a 286 or better and MS-DOS 5 or better, ensure COMMAND.COM

is loaded into high memory to give you the most free memory in the

640k area.  Check this with the "mem /c" DOS command.  Also be aware

that Double-space and similar TSRs take up valuable room in the 640k

area.

If you REALLY want to raise some dust, you need Ethernet linked

computers in each room of the house (including the toilet, so that you

can reply to mail at your convenience), and HF, VHF and UHF radios and

their TNCs or modems hooked up to the dusty old 286 running JNOS, with

dedicated access to Internet to cap it all off.  And then with faster

packet links you might start thinking about running graphical

interfaces and.....

1.3  Why INSTALL was written.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The primary objective is to make it very easy to get JNOS installed

and running, because it is an excellent program which suffers from

being notoriously difficult to install and configure.

My real motive is that because JNOS is far more enjoyable when there

are a lot or people using it, I end up having more fun!

1.4  Where to read more about TCP/IP and JNOS.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

There is an excellent series of articles in the NZART magazine

'Break-In' between June and November of 1992 written by Max ZL2MAX

which is well worth reading.

�������������������͸

� 2.0  INSTALLATION �

�������������������;

2.1  What does INSTALL do?

~~~~~~~~~~~~~~~~~~~~~~~~~~

This installation program for JNOS will create the appropriate

directories and some rather basic configuration files, hopefully

enough to get you going.  The configuration files will automatically

put your TNC in and out of KISS mode, or if you are using a BayCom

modem it will set things up for use with the AX25.COM driver.

The following configuration files will be created:

AUTOEXEC.NOS, ONEXIT.NOS, FTPUSERS, NOS.CFG, GO.BAT, SPOOL\MOTD.TXT,

SPOOL\FORWARD.BBS, SPOOL\REWRITE, SPOOL\AREAS, SPOOL\HELP\INFO.HLP.

2.2  What essential things do YOU need to do?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

It is a good idea to do one or two dummy runs first, say on a floppy

disk, to get a feel for how the INSTALL program operates and the

questions it will ask you.

Create the main JNOS directory on your hard disk, eg C:\JNOS.

Copy INSTALL.COM into the main JNOS directory it, and run it.

This will result in a number of sub-directories and configuration

files being created.

You will need to obtain and place in the main JNOS directory:

* DOMAIN.TXT

* The JNOS executable file - for example JNOS110K.EXE

And if you will be using a modem:

* AX25.COM   (modem driver, if you will be using a BayCom style modem)

* TERMIN.COM (for removing the AX25.COM TSR modem driver from memory)

Now run GO.BAT and see if you can connect to anyone with a command

"c 2m zl3abc" or similar.....

2.3  Not-quite-so-essential things to do.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Install in appropriate directories:

* Files for the \HELP directory  (contained in CMDSHLP.ZIP for JNOS

1.10g, CMDSHLP1.ZIP for 1.10h, CMDSHLP2.ZIP for 1.10i, CMDSHLP3.ZIP

for 1.10j, CMDSHLP4.ZIP for 1.10k)

* Files for the \SPOOL\HELP directory

SPOOL\HELP\INFO.HLP is sent to users logged into your system who send

the "I" command, so you will need to edit this accordingly.  The

INSTALL program does actually write some pretty basic stuff into this

file anyway.

SPOOL\MOTD.TXT is the file which is displayed to users who log on, so

you will no doubt wish to edit this to suit your circumstances.

The TXDELAY parameter will probably benefit from being fine-tuned to

suit your equipment.  If you are using a TNC the parameter is set in

the file AUTOEXEC.NOS, if you are using a Modem the parameter is set

in the file GO.BAT .

Routing is a matter of local preferences.  A common technique is to

put such information in a file called ADDRESS.LST which is invoked

towards the end of AUTOEXEC.NOS, and it is incorporated in this

installation scheme.  It is up to you.  Follow local custom.

For example purposes ONLY a copy of my own ADDRESS.LST file is

included - note that it is unlikely to be any use directly, only to

show you how things are set up on my system, which is not very typical

as I live in a rural area and do virtually everything through a

hilltop TheNet X1J2 node (ZL3CHC-1) 45 miles away.

The bbs 'P' command lists the port(s) on your system, with a

description which is specified with the "ifconfig 2m descr" command,

so you may wish to edit this line in AUTOEXEC.NOS accordingly.

������������������͸

� 3.0  IP ADRESSES �

������������������;

3.1  IP addresses.

~~~~~~~~~~~~~~~~~~

If you decide to run JNOS on a regular basis, you will need a

"permanent" IP address, which is allocated by:

Roger Harrington  ZL1TK    Auckland  (NZ National Co-ordinator)

Doug Faulkner     ZL4FS    Hamilton

Steve Davis       ZL2UCX   Christchurch

Ian Drage         ZL4TIL   Invercargill

Peter Hallgarten  VK3AVE   VK3 Co-ordinator

For example, my own IP address is 44.147.200.64 which breaks down like

so:-

44  = Amateur radio

147 = New Zealand

200 = Canterbury

64  = Assigned individually

To figure out an appropriate TEMPORARY address, select the first 3

groups of digits according to your location like similar stations

listed in the DOMAIN.TXT file, and choose a 4th group to be in the 250

to 254 number area (0 and 255 are illegal).

�������������������͸

� 4.0  FAULTFINDING �

�������������������;

4.1  Symptom: nothing happens, program doesn't seem to run.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

(You should get a lot of disk activity, followed by a screen display.)

Check:

(a) That you are running GO.BAT (or equivalent) to make it work.

(b) Is the main JNOS program actually there? (Look for a .EXE file

about 200k or 500k in size called NOS.EXE or JNOS110J.EXE, etc.)

(c) Is the main JNOS program specifically named in the GO.BAT file?

(GO.BAT may not have the correct JNOS program filename in it.)

4.2  Symptom: error message - "autoexec.nos not found."

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Either AUTOEXEC.NOS

(a) doesn't exist at all, or

(b) is not in the correct directory.

It should exist, and be in the same directory as the JNOS program.

The INSTALL program should have created it.

4.3  Symptom: no received data appears on screen.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

(When there is normal packet activity on the channel you should see

it.)  Possible causes:

(a) Wiring - does "normal" packet work OK?

(b) TNC not in KISS mode.  Sometimes some TNCs seem to need the "KISS

ON" line repeated, so try duplicating the line.

(c) "attach" line in AUTOEXEC.NOS is incorrect.  Consult documentation.

4.4  Symptom: TNC does not go into KISS mode.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

(a) Sometimes some TNCs seem to need the "KISS ON" line repeated, so

try duplicating the line.

(b) Are you sure the TNC and computer are running at the same speed?

(c) Maybe your TNC needs different commands to go into KISS than what

the INSTALL program has available.  Consult your TNC's manual.

If it is not handled by INSTALL, let me know all the gory details

so that I can incorporate them into INSTALL.

4.5  Symptom: unable to transmit.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

To initiate a connection to another station, type the letter "C"

followed by the name of the interface, typically "2M" or "AX0" or

something like that, followed by the callsign of the station you wish

to connect to. For example: "c 2m zl3abc". This should cause a packet

to be sent, as seen on TNC, modem or radio LEDs. If there is a

transmission but no response from the other station, it may be that

your TXDELAY parameter needs to be set to a longer time.

4.6  Symptom: unable to ping yourself.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Type "ping <callsign>", where you are the callsign.  e.g. "ping

zl3abc". This should result in a short flicker of disk activity

followed by a number appearing on the screen.

If instead you get something like "unknown host", it implies that

either:

(a) the file DOMAIN.TXT does not exist in the main JNOS directory, or

(b) if DOMAIN.TXT is present, your hostname (i.e. callsign) is not in

it.

4.7  Symptom: unable to ping others.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Type "ping <someone>" where <someone> is some other TCP/IP station,

for example "ping zl3abc".  After a few seconds you should see a

number appear.

If you get an error message saying "unknown host" then they are not in

your DOMAIN.TXT file (so manually write it in, or get an up-to-date

one).

If they are in simplex range, it should work.

If they are not in simplex range, then you need to set up routing

information so that JNOS will know how to talk to them.  This varies

from place to place, my personal preference is to have a seperate file

which is called ADDRESS.LST which contains routing information, and

which is invoked by a line at the end of AUTOEXEC.NOS which is "source

address.lst".  It is probably best to talk to local TCP/IP users to

see what scheme(s) are in use.

4.8  Symptom: error messages appearing on received packets.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Try lowering the speed of the RS232 connection between the Computer

and TNC. Experience indicates that 286's seem to be OK at 2400 baud

but can have problems at higher speeds.  386's seem to be OK at 9600

baud.  This may vary somewhat from machine to machine, and the

information offered here is a crude guide to give you some sort of

idea.

4.9  Symptom: can't send mail.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Mail addressed to "zl3abc@zl3def" will automatically be sent by SMTP

(or attempted) if ZL3DEF appears in your DOMAIN.TXT file.

Otherwise, mail will remain on your system unless the FORWARD.BBS file

is set up to handle it.

The above is a very simple way of looking at it all - the REWRITE and

ALIAS files can modify this behaviour.

4.10  Symptom: no colour in monitored packets, left pointing arrows instead.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In this case there will be left pointing arrows occur at the start of

lines on the screen.  To display colour, JNOS relies on ANSI.SYS being

installed, which intercepts the "arrows" and following code

characters, and generates colours.  Amend your CONFIG.SYS file to

include ANSI.SYS (refer to DOS manual) and re-boot.

The documentation inside GO.BAT has more details about colour and mono

displays and command line options relevant to them.

4.11  Symptom: large numbers of error messages on start-up.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Some JNOS compiles do not have some options available, and the

INSTALL program writes a "general purpose" AUTOEXEC.NOS file which

assumes everything is there.  Many custom compiles omit CONVERS and/or

NETROM to save space for incorporating other more desirable features,

however it results in large numbers of errors relating to these

functions. Simply place a # in front of the offending entries in

AUTOEXEC.NOS to disable them.

4.12  Symptom: JNOS crashes when shelling.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

While it is nice to be able to shell, it is a plain fact of life that

JNOS is prone to crash when shelled.  To minimise hassles, ensure that

the mailbox is not in use when you shell because if it crashes, the

mail files can get corrupted, generating incredibly massive grief and

woe.

����������������������������͸

� 5.0  WHEN THE DUST SETTLES �

����������������������������;

5.1  Feedback.

~~~~~~~~~~~~~~

I would appreciate feedback on your experiences with this program, as

well as with JNOS itself.  Hopefully I will keep this program up to

date as new JNOS versions are released, and my own knowledge grows.

Feedback will help improve things.

5.2  JNOS versions.

~~~~~~~~~~~~~~~~~~~

As at September 1995 the latest version of JNOS is 1.10k and work is

under way on 1.10L.

The standard distribution version (JNOS110K.EXE) has LZW compression

for TCP/IP, as well as POP3, along with the usual goodies.

FBB compression for BBS forwarding is a compilable option for version

1.10k, and such versions being very successfully used by some

amateurs.

--------//--------