Using PZTBBS with Ethernet

===========================

If you have an Ethernet card, PZT BBS can be linked with any system

capable of TCP/IP over Ethernet, such as PZT router, Linux, xNOS, BPQ,

and Microsoft Windows.

There are two options, both of which use the EXTERNAL interface, but

using different encapsulation settings.  The first option is somewhat

complicated and limited in capability.

Option 1 - Use BPQ's ODIDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

PZTBBS will interface directly to BPQ's ODIDRV program, allowing

single-protocol (AX25-only) links between PZTBBS and any number of

PZT routers and BPQ nodes.

For this option, the EXTERNAL interface (in PORTS.SYS) should be set

to HDLC mode, and the MTU should be set at 340 since ODIDRV can only

handle small frames. Set the software interrupt to suit your needs.

Example:

<ifacetype> <intnum> <mode> <mtu> <label>

IFACE  EXTERNAL     125     hdlc   340   eth0

You will need to obtain BPQ's ODIDRV.EXE and the appropriate bits of

Novell Netware, and have a reasonable working knowledge of how to

set it all up!

Follow the instructions in BPQ's DRIVERS.DOC for setting up the ODI

system.  PZTBBS currently uses a fixed PID of 0x8FF, which is the

default value for BPQ, and frames are addressed to the multicast

address FF:FF:FF:FF:FF:FF, so the only change you need to make to

BPQ's example is to change ETH_ADDR to that value.

The load order is as follows:

(first)

LSL.COM		(Part of Novell Netware)

NE1000		(Ditto - adjust to suit your Ethernet card)

ODIDRV 125	(BPQ program.  Number must match INTNUM)

PZTBBS

(last)

If you wish to communicate with BPQ nodes via Ethernet, the PID and

multicast addresses must match.

Because ODIDRV only supports HDLC protocol, you won't be able to use

TCP/IP on the link unless it is first encapsulated in unconnected-mode

AX25 frames. This should happen automatically if you try to route IP

via the ethernet interface, but obviously this encapsulated IP cannot

talk directly with TCP/IP systems which don't understand AX25 (such

as Windows).

Option 2 - Use PZT's ETHDRV.EXE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This option does not need any Novell Netware components, is relatively

easy to set up, and supports simultaneous IP, ARP and HDLC traffic.

The software components you require are:  ETHDRV.EXE (supplied with

PZTBBS), and an Ethernet card driver to match your card e.g. NE1000.COM

(You can get these free from Simtel CD-roms or mirror sites)

In PORTS.SYS you should define an "external" interface with mode ETHER,

an MTU up to 1500 and an interrupt number in the range 60 - 127 (125

is used as an example).

If you are not familiar with Ethernet card drivers (aka "packet" or

"Crynwyr" drivers), 3 arguments are usually specified when starting,

e.g...

NE2000 0x60 5 0x300

Where:	NE2000	is the driver matching your card.

0x60	is the software interrupt used to talk to the card,

specified in hex (0x60 is decimal 96, 0x61 is 97 etc.)

5	is the IRQ used by the card.

0x300	is the I/O address of the card.

The I/O address and IRQ are often settable by jumpers on the card,

especially on older cards, but some cards have fixed settings.

0x60 is the default software interrupt used by ETHDRV.EXE to communicate

with the card driver.  If you specify an alternative number (e.g. if you

have several cards), you must specify that number when starting ETHDRV.

ETHDRV.EXE normally requires a single argument, namely the interrupt

number which should match the one specified in the IFACE line.  It

optionally takes a second argument, if you wish to use an interrupt

other than 0x60 to communicate with the card driver.

e.g.	ETHDRV 125		(intnum=125, software int=0x60)

ETHDRV 125 0x63		(Using a different card interrupt)

ETHDRV	125 -U		(To unload the driver)

My startup sequence is as follows:

NE2000 0x60 5 0x300

ETHDRV 125		(Interface intnum=125)

PZT.EXE

Attaching CHANNELs and PORTs

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

You should attach one CHANNEL to the ethernet interface as follows:

; channel <name> <iface> <stream> <mtu>  <freq>   [ip_address]

; -------------------------------------------------------------

channel   tcp3   eth0      0    1500   Ethernet  44.131.91.3

;

The optional IP address can be used to override the main BBS IP address

for that channel.

You can then attach one or more PORTs to this channel:

; PORT <port#> <type> <t/o> <channel> <initstring>

; -----------------------------------------------

port    1     tcp    300   tcp3

port    2     tcp    300   tcp3

Sharing the Ethernet Card

~~~~~~~~~~~~~~~~~~~~~~~~~

ETHDRV.EXE will not currently share an Ethernet card, so if you are

using it on a Microsoft Windows system you MUST prevent Windows from

accessing the card.  You probably know more about how to do this than

I do!  If you simultaneously need Ethernet to communicate with other

computers on a Windows LAN, you will have to install a second card,

using one for Windows, the other for ETHDRV.

----------------------------------------

If you have any questions or feedback PLEASE let me know, don't leave it

to others because they probably won't bother!  I've probably omitted some

vital information in the rush to get this written, so don't hesitate to

chastise me ;-)

73, Paula  g8pzt@gb7pzt.#24.gbr.eu or g8pzt@cableinet.co.uk

10th April 2001