Using PZTNODE with DesqView

===========================

PZTNODE is a FOREGROUND program, designed to be the sole application

on a dedicated machine.  However, even though suitable PC's are ten a

penny, some people still insist on wanting to run everything on one

machine.  This can be achieved within the multitasking environment

provided by Windows95 or Desqview, and the following document attempts

to show how to do it using the latter.

If PZTNODE is run within Desqview (or Windows), it is able to support

most applications which are designed for use on the BPQ HOST

interface, such as PAC4, EasyTerm, PEARL, PZTBBS, and others.

It is assumed that you already have a copy of Desqview properly

installed on your machine, and are conversant with its use.

Configuring PZTNODE to run in Desqview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

STOP!   First of all, set up PZTNODE.CFG to your requirements and make

sure the router runs properly in plain DOS.  If it doesn't work in

DOS, it won't work in Desqview!  If you intend to run applications,

configure the APPL sections and set HOSTINTERRUPT to your preferred

interrupt number.  You must set DESQVIEW=1 otherwise it won't work

properly in DesqView.

Make sure that there are no conflicts between PZTNODE and other

applications for access to hardware such as COM ports or SCC cards.

PZTNODE writes directly to screen memory because the PC's BIOS

routines are not usually fast enough to support multi-port monitoring

at high data rates.  A consequence of this within Desqview is that the

console display may "bleed through" into any Desqview window which

overlays it.

This may or may not be a problem to you, depending on the other

applications you are running alongside the router.  If you set

NUMCONSOLES=0 in PZTNODE.CFG, the router will run without a screen

display, thus avoiding bleed-through, and saving some memory.  Of

course, in this mode you will only be able to control the router by

connecting to it, either from an application or via one of the ports,

much like the TSR version of BPQ.

Configuring Desqview For PZTNODE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Start Desqview and choose Open -> Add Program, then use the

following parameters as a guide....

Program Name:                      PZTNODE

Keys to use on open menu:          PN

Memory size in K:                  300 (see below)

Program:                           PZTNODE.EXE

Parameters:

Directory:                         C:\PZTNODE

Options

-------

Writes directly to screen:         Y

Displays graphic information:      N

Virtualise text/graphics:          N

Uses serial ports:                 N

Requires floppy diskette:          N

Advanced Options

----------------

System memory in K:                10

Maximum program memory size:

Script buffer size:                1000

Max. expanded memory size:

Text pages:                        2

Graphics pages:                    0

Initial mode:

Interrupts:                        00 to FF

Max height:                        25

Max width:                         80

Close on exit:                     Y

Allow close window command:        N

Uses own colours:                  Y

Runs in background:                Y

Uses math coprocessor:             Y

Keyboard conflict:                 0 or 1

Share CPU when foreground:         Y

Share EGA when foreground:         Y

Can be swapped out:                N

Protection level:                  1

You should preferably reserve enough memory to allow at least 30k of

free memory shown on the console top status bar (or use the Stats

command if you have console disabled), but the actual figure may

depend on how much Desqview will let you have, and how much you need

for other applications.  Optimising Desqview is beyond the scope of

this document.

Starting PZTNODE within Desqview

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1)   If you wish to run BPQ-compatible applications, you must start

PZTHOST before you start DesqView itself.  PZTHOST.EXE should be

located in the same directory at PZTNODE.EXE and PZTNODE.CFG.

Simply change to the pztnode directory and type "PZTHOST", or

arrange for this to happen in your autoexec.bat. (Tip: if you

ever need to unload PZTHOST you can do do using "PZTHOST -U.")

2)   Start DesqView.

3)   Choose Open -> Start Program -> PZTNODE or set up a startup

script to do so.  The router should now be running.  If not, and

assuming you made sure it ran OK in DOS, you may need to use a

higher value for "Memory Size".

4)   Start your other applications.

Notes

~~~~~

Some BPQ applications need to read configuration information

(usually HOSTINTERRUPT) from BPQCFG.BIN, so you may have to create

a dummy BPQCFG.BIN to keep them happy.

Because each program only gets a short interval of processor time,

interspersed with periods of inactivity, you may find that some of the

timings on SCC cards (e.g. TXDELAY and the CWID dot spacing) are a

little erratic.  You may need to "tune" the system to minimise the

jitter by setting the foreground and background time slices to 1 tick

each.

Please inform me if you can improve on the suggested settings or offer

any other tips.

Happy Experimenting!

Paula Dowie G8PZT 6/2/01