G8PZT AX25 / IP ROUTER PROGRAM

==============================

SYSOP'S MANUAL

==============

by Paula Dowie G8PZT

--------------------

Revision 1.6 - 28th January 2000

--------------------------------

---------------------------------------------------------------------

PREFACE

=======

This manual is far from complete, and the information is probably not

in the most logical order.  However, I thought you'd rather have some

documentation than nothing at all!  Getting the document perfect would

have needlessly delayed the release of the software (and has in fact

already done so), so I have chosen to refine the documentation after

release.  You may have to search for what you want, and in some cases

you may not find it.

The document's format is plain ascii text, which is fine for reading

off a computer screen with a simple text viewer.  I have not included

page breaks because your printer may not necessarily be using the same

settings as mine.  I will address this in later versions, and will

also create HTML and PDF versions.

Writing documentation is very time consuming, and prevents me from

programming, so I am trying to balance both activities.

If you don't find what you want, or you think something could be

better explained or positioned, PLEASE let me know, don't leave it to

others - they are doing the same!  As a programmer, I am not the best

person to write the documentation - I have tried to include what I

think you need to know, but with intimate knowledge of the code I tend

to make assumptions without realising it.

SECTION 1 - INTRODUCTION

========================

PZTNODE is an AX25 and IP packet router for the amateur packet radio

network, using a standard PC and DOS.  It is intended to be a direct

alternative to G8BPQ's SWITCH.EXE program (*not* BPQCODE.EXE), and

looks very similar both to sysops and users, but provides a lot more

than BPQ.

Despite the filename, I call it a "router" rather than a "node"

because its primary purpose is to route data packets.  Node is a

general-purpose term meaning "junction", and can be an end user.  This

software simply routes packets.  Whether those packets contain ax25,

Netrom or IP is irrelevant.  It is not the job of a router to provide

FTP, SMTP, NNTP, or BBS facilities, so they aren't provided.

The program was developed because G8BPQ node, although excellent, was

unreliable in certain situations, it didn't provide the required

facilities, and its author *appeared* to have lost interest in it.

It was designed to be familiar to sysops who currently use BPQ, thus

allowing easy transition between the two programs.  It fixes some of

the BPQ problems (such as the unsafe command line parsing), properly

integrates IP routing into the system, and adds many facilities which

BPQ did not provide, such as an integral chat server, and the TCP

layer which allows TELNET, RLOGIN etc. (see below for full list of

features)

PZTNODE is a long way from finished and has not been tested under all

conditions.  I am not claiming that it is *better* than BPQ.  It is

not intended to replace BPQ, merely to provide an alternative.

Although some of the commands and configuration options may look

familiar to NOS users, this is largely coincidental.  Some NOS

commands were derived from UNIX, as were some of mine.  Sometimes

there is only one logical way to do things, and both NOS and PZT chose

the same way.  However, even if the commands look familiar, the syntax

may not necessarily be identical, reflecting the fact that this system

has different design criteria.

This program is not a variant of NOS and is not derived in any way

from NOS.  The latter has a reputation for being flaky, cumbersome and

un-intuitive.  This program is aimed at busy sysops who want to get

the job done, not play around with software.  It was designed from

scratch (the console display was the first component), not "cut down"

from someone else's software.

Whereas NOS is a TCP/IP program with rudimentary Netrom capability,

this is an AX25 program, with some TCP/IP capability.  It is not,

repeat NOT a "TCP/IP program".  It does not have, and never will have

"hub" facilities - if you want a hub, use NOS.

PZTNODE is not a TSR, and there will probably never be a TSR version

because it would be too big to allow anything to run on top!

It was designed as a standalone router running as the sole application

on a PC dedicated to that purpose.  I don't like the idea of running

part of the Packet network as a TSR underneath a BBS program for the

following reasons:

a) The node gets a limited amount of processor time because it is

running on clock interrupts.  I have seen considerable delays in

traffic through such systems.

b) The BBS gets less processor time because the node is stealing

it.  Every single byte that passes through the system generates an

interrupt, so a busy node steals a lot of processor time, and the

BBS runs more slowly.

c) If the node crashes, the BBS is taken out of action too, and

the system has to be rebooted.  This may not be important in your

case, but my BBS can also be accessed via telephone, and wire

links, and I don't want it disrupted every time the AX25 side

crashes.

d) If the BBS crashes (which some are particularly prone to!) the

Packet network is disrupted.  This also happens if the sysop keeps

taking the BBS off line to mess about with it.  A NetRom network

does not take kindly to outages, and the effects can last for

hours so nodes need to be left alone, not rebooted every few

hours.

There didn't seem much point making a TSR version, since that would

remove most of the advanced features.  If you need a TSR node to run

under a BBS on a DOS system, you'll have to run BPQ.

However, if you run PZTNODE within a Windows (or Desqview?)

environment, along with the PZTHOST TSR, you may run applications

which conform to G8BPQ's 16 bit HOST application interface spec.

I make no apologies for creating yet another "old fashioned" DOS

program.  DOS is fast, compact, ultra-reliable, boots quickly, is

relatively uncomplicated, can run from floppy, and runs on the

humblest machine.  Therefore I believe it is still the best choice for

a standalone packet router, especially at a remote site.

Linux devotees already have their LinuxNode, and Windows users have

BPQ32.  For DOS users however, apart from BPQ there are no other

English nodes.  Flexnet and the PC version of TheNet suffer the usual

problems with unfamiliar commands, translated documents etc., and are

not popular with either sysops or users in the UK.

I am not ruling out the possibility of a 32 bit Windows version in the

future, but it is not high on my priority list because the Windows

market is already well catered for by BPQ32, with AGW being popular

for non-routing applications.

FEATURES AND LIMITATIONS

========================

The following is a list of some of the features and limitations of PZT

router software.  It is by no means an exhaustive list, nor is it in

any logical order. (* = not provided by BPQ)

- Modest machine requirements: DOS, 640k, XT->Pentium.

- Can run within Windows environment.

- Configuration file familiar to BPQ sysops.

* No pre-compiled configuration file.

* Virtually unlimited nodes, routes, circuits and sessions.

- Compatible with BPQNODES recovery file for easy transition.

- Interfaces: ASYNC, EXTERNAL, BAYCOM USCC, RLC100

- ASYNC interface speeds up to 115,200 bauds

* ASYNC Protocols: KISS, SLIP and ASCII (TTY).

* Configurable software watchdog

* Hardware watchdog driver.

* Year 2000 compatible.

* Budlist and Validcalls lists for each port.

* No 256-character limit on callsign lists.

- Digipeating on/off/UI-only from any port to any port.

* Digicasting

* Frame piping from port to port

- KISS modes: normal, polled, checksum, and bpq slave.

- Can use BPQ polled kiss TNC's.

* No 511 character limit on CTEXT and INFOMSG.

* Very comprehensive Stats.

* MHeard list sizes can be customised for each channel.

* MHeard includes date, time, callsign, frame count and frame type.

* MHeard can display APRS position, distance and heading.

* Integral multi-channel chat server available to all users.

* Proper session flow control.

* No 130 buffer limit, or running out of buffers.

* No crashing on frames bigger than 256 bytes.

* No crashing on "certain" command line characters.

* No crashing due to "too many commands at once".

* No "command can't be split across packets".

* Configurable mtu.

- Can TX on a different port to RX.

* Callsign validation in nodes broadcasts.

* Supports BPQHOST compatible applications (in Windows only)

- 32 bit Windows application support coming soon...

TCP/IP

~~~~~~

* Built in IP router, with full ARP & ICMP implementations.

* IP commands fully accessible on normal console, and to all users.

* Built in TCP, giving TELNET, ECHO, DISCARD, FINGER, CHAT, RLOGIN, FTP

* Domain name to ip address resolution.

- IP datagram, virtual circuit and netrom modes.

Command Interface

~~~~~~~~~~~~~~~~~

- BPQ compatible user commands: bye, connect, info, help, links, mheard,

nodes, ports, routes, stats, users.

- Up to 8 command aliases can be defined.

* Additional user commands: PING, ARP, TELNET, CHAT etc.

* Built in syntax help for most commands.

* Aliased commands can be "hidden".

* Graduated help system gives help in a progressive way.

* Extendible Info system for sysop to set up as s/he wishes.

Console Interface

~~~~~~~~~~~~~~~~~

* Screen saver - automatic and manual.

* Shell to DOS facility.

* Editable command line.

* Fully configurable display colours.

* Up to 5 independent "Virtual" consoles, fully multitasking.

* Each virtual console has a configurable scrollback buffer

* Each virtual console can display ANSI colour.

* Each virtual console can run a separate session.

Local And Remote Sysop

~~~~~~~~~~~~~~~~~~~~~~

- Most parameters configurable while on line.

* Tx for any port may be disabled/enabled by sysop command.

* Time and date settable without rebooting.

* Multitasking DOS commands: cd copy del dir mkdir, ren, rmdir, move type

* Line editor allows any text file to be edited.

* IP routing adjustable without reboot.

* Transaction logging.

* Simple programs, MSDOS commands or batch files run from command line.

* Online sysop's manual.

* Each protocol layer independently traceable.

- Each port independently traceable.

* Trace o/p can be captured to file.

Additional Features For Remote System Maintenance

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

- Secure password system for remote sysops.

- May be remotely rebooted.

* Allows separate passwords for each sysop.

* Remote operation via RS232 wired links. (dialup coming soon).

* Secure FTP server, allowing remote upgrading

* Trace display can be viewed by remote sysop.

COPYING, USE AND DISTRIBUTION

=============================

You must read the following text carefully.  It is not designed to

take away your rights, but rather to protect mine.  The software took

a long time to write, at considerable cost to my health, and I don't

see why others should make a profit from my hard work.

The terms "the code" and "the software" refer to PZT router and all

components thereof, and all documentation relating to it.

This software is "conscienceware".  You may freely use and share it

for any non-commercial purpose, and you are *not* required to pay a

registration fee.  You may however ease your conscience and aid the

continuing development and support of the software by sending a small

donation to the author if you wish ;-)

You are not permitted to make any charge for, or gain any pecuniary

advantage directly or indirectly from, the copying, distribution or

use of this software, or any component thereof, or any driver or

utility written for use with the software, unless by prior written

agreement with the author.  For example, you may not place the

software on the Internet with the intention of attracting more

visitors to a site.

You must not patch, decompile, disassemble, reverse engineer or in any

way modify the software, or cause a modified version or part thereof,

or any portion of the source code to be distributed.

Legal action will be taken against anyone who breaks the terms

outlined above.

SOURCE CODE

===========

After due consideration of the pros and cons of "open source"

projects, I have decided that the source code will not be made

available, and this is not negotiable.

FEEDBACK

========

Because the terms and conditions outlined above prohibit patching, and

the source is not available, the only way the bugs will be fixed and

the software improved is for me to do it.  I am therefore relying on

you to report any bugs promptly, and to make known your ideas for

future improvements.  You are my beta testers.  I promise to correct

all reported bugs promptly where possible, and to carefully consider

all suggestions.

Please also report any errors or omissions in the documentation.  If

you don't like the layout or feel something isn't properly explained,

tell me.  You can make a difference.

If you want to send bulletins regarding this software I suggest the

use of "PZT" as the To field.

You can contact me personally by the following means:

Packet: G8PZT@GB7PZT.#24.GBR.EU

Email:  g8pzt@cableinet.co.uk

Snail:  Ms P J Dowie,

19 Brooklands Drive,

Wolverley Park,

Kidderminster,

Worcestershire.

DY11 5EB

I'm sorry I can't provide support by telephone, as it would take up

too much of my time.

CONVENTIONS USED IN THIS MANUAL

===============================

Certain conventions are used in this document to avoid unnecessary

repetition.  They are as follows:

|               ("or") Separates alternative options or arguments.

<>              Encloses a mandatory argument

[]              Encloses one or more optional arguments.

callsign        A valid amateur radio callsign, e.g. G8PZT

dir             Directory

dirname         Directory name

file

filename        A unique file name without directory

filespec        Specifies one or more files, with or without directory

he              Represents both "he" and "she"

path            A fully qualified directory path

pathname        A full directory plus filename.

string          A contiguous sequence of non-whitespace characters.

text            One or more strings separated by whitespace.

whitespace      Spaces, tabs or newlines

Examples:  DIR [ filespec ]  - DIR command has one optional argument,

which may be a file or file group

description, with or without directory.

In cases where a pathname is required, if only a filename is given,

the program's working directory is assumed.

======================== END OF SECTION 1 ============================

SECTION 2 - INSTALLATION & CONFIGURATION

========================================

I won't bother providing detailed instructions, since every

installation will be different and you won't read this anyway, unless

you're feeling really bored ;-)

MACHINE REQUIREMENTS

~~~~~~~~~~~~~~~~~~~~

There are no hard rules about this; it depends how many ports you

want, how heavily the system will be loaded, what facilities you

require etc.  I have tried to make it as flexible as possible.

The router *will* run on a 512k floppy drive XT with monochrome

display, but for best results anything above and including a 640k AT-

class machine with EGA colour display is ideal.  A hard drive isn't

essential, but is recommended.  My system happily runs 12 ports on a

12MHz '286 with 20Mb hard drive, but if you have lots of fast ports

(i.e. better than 9k6 *average* throughput) you may need something

faster.

Processor:     8088/80x86/Pentium

Clock:         4.77 and above

Memory:        384k minimum.

Video:         MONO/CGA/EGA/VGA

Drives:        At least a floppy. Hard drive recommended.

Ports:         At least one COM port or Ethernet card is required.

The program does not use expanded/extended memory, but you may wish to

use it for a ram drive if you don't have a hard drive.  This will

speed things up, because help, info, manual, and domain lookup files

are all accessed from disk.  You would arrange for autoexec to create

a ramdrive, copy all the router files to it, then run the router from

the ramdrive.

OPERATING SYSTEM

~~~~~~~~~~~~~~~~

PZTNODE requires MSDOS v3.2 and above.  It will also run on DRDOS.

PZTNODE will run in a win95 DOS window, providing no other window uses

com ports sharing the same IRQ.  It has not been tested with later

versions of Windows.  If you wish to use the application interface,

you must run PZTNODE within a Windows (or Desqview) environment.

I realise that there is an ever increasing number of people who have

only ever used Windows95 and know nothing about DOS.  They may have

unwanted machines lying around with Windows95 installed, and may think

that they can't use them to run PZTNODE, whereas in fact they can.

All they need to do is use a text editor such as Notepad to edit the

file C:\MSDOS.SYS, changing the line "BootGui=1", to "BootGui=0".

This file sometimes has the attributes "Hidden", "Read-only" and

"System", so it may be necessary to edit the file attributes first, to

allow Notepad to edit it.  The machine will subsequently boot to DOS 7

which is fine for PZTNODE.

DIRECTORY STRUCTURE

~~~~~~~~~~~~~~~~~~~

All files and directories required by the system are located within a

single directory which can be located anywhere and have any name.  I

suggest you name it PZTNODE.

The directory should contain at least PZTNODE.EXE and PZTNODE.CFG.

All other files are optional and the system will run without them.

If you want to grant access to remote sysops you will need to add

PASSWORD.SYS.  If you want to store IP routing info, you will need

IPROUTE.SYS.  If you want domain lookup, add DOMAIN.SYS.

If you want to import existing nodes and routes information from a BPQ

system, place a copy of the BPQNODES file (produced by BPQ's SAVENODES

command) in the directory and rename it PZTNODES.

If you want the full help system, create a sub-directory called HELP

and place all the .HLP files into it.  Note that the help for the CHAT

server goes in a sub-directory of help called CHAT, and the help for

the FTP server goes in HELP/FTP

If you want an INFO system, create the INFO directory.  I have

included some sample .INF files to give you some ideas.

If you want the online manual, create the MAN directory and put the

*.MAN files into it.

SETTING UP

~~~~~~~~~~

The software will be supplied in ZIP format.  Within the zip file, the

system, help and manual files will be in separate zip files to make it

easier for you to select the components you require.  You may create

the directory structure manually and unzip the components into it, or

you may use PKUNZIP -d to create the sub-directories.

Once the required files are installed in the correct places, your next

job is to edit the configuration file PZTNODE.CFG, to suit your

requirements.  The file is heavily commented, so it should be obvious

what needs changing, especially if you have previously used BPQ node.

If you have installed PASSWORD.SYS, IPROUTE.SYS or DOMAIN.SYS they

will also need editing to suit your requirements.  Again, they are all

self-documenting, but are also detailed elsewhere in this document.

RUNNING THE PROGRAM

~~~~~~~~~~~~~~~~~~~

If you have defined any EXTERNAL type drivers in PZTNODE.CFG, they

must be loaded first.  If you forget to load them, the machine will

crash, so it is advisable to start the node from a batch file in this

case, to ensure things are loaded in the correct sequence.

Once the drivers are loaded, start PZTNODE.EXE.

The program will read its configuration from PZTNODE.CFG, and will

then look for IP routing information from IPROUTE.SYS and saved

nodes/routes information from file PZTNODES.

If you've got everything correct you should then get the console

display.

STOPPING THE PROGRAM

~~~~~~~~~~~~~~~~~~~~

Leesen vairy carefoolee, I shall say zis ernly wernce....

Alt-X stops the program, saving the nodes and routes tables to file

PZTNODES just before it exits back to DOS.  It is protected by an

irritating "are you sure" challenge.

WINDOW LAYOUTS

~~~~~~~~~~~~~~

The screen will display a single "virtual console".  Each virtual

console has the following format:

There is a single-line "status bar" at the top of the screen and a

single-line "menu bar" at the bottom.  Immediately above the menu bar

is a single "command entry" line.  The remaining 22 line central

section is where monitored and received information is displayed, and

this section is ANSI compatible.  The text and background colours of

the various sections are fully configurable.

The status bar shows the date and time, the virtual console number,

the version number, the router's callsign and alias, and the amount of

free memory.  The menu bar shows the most important keys.

CONSOLE KEYS

~~~~~~~~~~~~

Here's a summary of the keys - they are fully explained later.

<F1>                Display console help window.

<F2>                Toggle monitoring on/off.

<F3>                Select port(s) to monitor. (MPORT)

<F4>                Select monitored protocols. (MMASK)

<F5>                Toggle disk capture on/off.

<ESC>               Cancel operation / Disconnect.

L/R arrow           Select previous / next console.

U/D arrow           Scroll back/forward by one line.

<PgUp>/<PgDn>       Scroll back/forward by one page.

<End>               End scrollback.

<Alt-F9>            DOS shell

<Alt-B>             Invoke screen blanker

<Alt-X>             Exit program.

Help

~~~~

Pressing <F1> pops up a window with the above information on it.  The

<ESC> key closes that window when you've finished with it.  The

contents of the window are context-sensitive, i.e. they will vary

according to what you were doing when you press the <F1> key.

Monitoring

~~~~~~~~~~

The <F2> key provides a quick way to turn monitoring on and off, i.e.

it has the same function as BPQ's <F1> key.  Note that I didn't follow

BPQ here because *most* software uses <F1> for help.

Monitor port

~~~~~~~~~~~~

The F3 key selects the ports to be monitored.  At the moment only the

first 16 ports can be monitored, but this will be corrected in a later

version.  Upon pressing this key you will be invited to enter a

hexadecimal number representing the combination of ports to monitor.

If you press <F1> at this point a pop-up window will show you the

following information:

To select port(s) to monitor, add together the following hex values:

Port = HEX     Port = HEX     Port = HEX     Port = HEX

1 = 0001       5 = 0010       9 = 0100      13 = 1000

2 = 0002       6 = 0020      10 = 0200      14 = 2000

3 = 0004       7 = 0040      11 = 0400      15 = 4000

4 = 0008       8 = 0080      12 = 0800      16 = 8000

Example: 004F = monitor ports 1, 2, 3, 4 and 7.

This is (I think) identical to BPQ.  You may cancel the operation by

pressing the <ESC> KEY.

Monitor Mask

~~~~~~~~~~~~

The <F4> key invites you to enter the monitor mask, i.e. the protocols

you wish to monitor.  Pressing <F1> at this point brings up the

following information:

Add together the following HEX values:

0001 - Incoming frames          0100 - ICMP

0002 - Outgoing frames          0200 - TCP

0004 - AX25 layer 2             0400 - KISS

0008 - AX25 info frames         0800 - SLIP

0010 - AX25 layer 3             1000 - PASSALL

0020 - AX25 layer 4             2000 - Hex Dump

0040 - IP frames

0080 - ARP frames

For example, a setting of 0201 will display incoming TCP frames only,

while 0241 will display the underlying IP frames as well.  The default

setting is 03FF, which displays all incoming and outgoing traffic from

AX25 layer 2 upwards.

Disk capture

~~~~~~~~~~~~

The <F5> key toggles disk capture on and off.  The captured text goes

to file PZTNODE.CAP.  When capture is on, everything which appears on

the central 22 line section of the console is sent to file.  You can

make it more specific using MMASK and MPORT.  (Note this is a

completely separate operation to the transaction logging enabled by

setting LOG=1 in the configuration file.)  Make sure your disk is fast

enough and has enough space, otherwise the router's performance will

be impaired.

Escape

~~~~~~

The <ESC> key "backtracks" if you have pressed <F1>, <F3> or <F4>, or

disconnects a console session as in BPQ.

Switching consoles

~~~~~~~~~~~~~~~~~~

The left and right "arrow" keys will switch between the virtual

consoles if you have enabled more than one.  The console number is

shown on the top status bar.  Note that when you reach the last

console, you wrap back to the first and vice versa.

Review mode

~~~~~~~~~~~

The up and down "arrow" keys control scrollback, along with the

<PgUp>, <PgDn> and <End> keys.  You must enable the scrollback buffer

for this to have any effect.  Scrollback or "review" mode enables you

to look at something which went off the top of the screen.  The

console won't receive anything else until you exit review mode.

DOS shell

~~~~~~~~~

<Alt-F9> temporarily suspends the program and loads a DOS shell,

enabling you to do things without taking the router off line with the

consequent loss of all links and tables.  For instance you may wish to

format a floppy, use a text editor, or do something which you couldn't

otherwise do using the router's built-in PZTDOS.  (Note: I will be

building a text editor into future versions of the program)

You shouldn't stay "shelled out" for too long, otherwise any active

links will time out, and you run the risk of being rebooted by the

watchdog.

On some machines, use of the dos shell *may* cause the system to crash

- I think I've fixed this, but would appreciate any reports.  If it

does crash, it may bear some relationship to the order in which

certain programs are loaded.

Screen Blanker

~~~~~~~~~~~~~~

The SAVER keyword in PZTNODE.CFG specifies the screen saver timeout,

i.e. how long the screen will remain visible after any keyboard

activity.  A zero value disables the screen saver, but you may use

<Alt-B> to blank the screen at any time.  Pressing any key will re-

enable the screen.

MISCELLANEOUS TOPICS

====================

These are just random notes which I haven't yet had time to organise

properly....  One day I'll write a proper manual!!

Nodes/routes table saving

~~~~~~~~~~~~~~~~~~~~~~~~~

The Nodes & routes tables are saved to file PZTNODES 1 minute after

starting the program, and at NODESINTERVAL (usually 1 hour) intervals

thereafter.  The SAVENODES command dumps the node table at any time,

and the tables are automatically saved when you exit the program using

<ALT-X>.  Thus if you exit and restart you will not lose the tables,

only the active links.

You can specify an alternative filename when using the SAVENODES

command.  For example, you might want to make a periodic backup of the

nodes table, from which things can be restored if the table gets empty.

(a radio failure will cause nodes to expire from the table, and you

may wish to get it restarted quickly after fixing the radio)

The LOADNODES command enables the table to be loaded from any file, at

any time.

Remote Tracing

~~~~~~~~~~~~~~

Remote sysops may invoke tracing using the MONITOR command, but this

feature should be used with caution.  Tracing generates a lot of data,

and the link should be capable of handling it.  It is mainly intended

for use on TTY (wire) links, but can be used over any connection.

A remote sysop is prevented from tracing any activity on the port upon

which he is uplinked.

Only one remote sysop may receive tracing at any one time.

ANSI

~~~~

The console will respond to ANSI, so you can have full colour terminal

sessions with ANSI-capable systems such as PZT BBS.  ANSI sequences

are not (yet) generated from the keyboard.

AXROUTES

========

The AXROUTES command displays information about active and recently

active AX25 level 2 connections, allowing the sysop to monitor the

performance of the links between the router and its users.

Entries are purged 24 hours after they were last active, unless they

are "locked" (indicated by "!" after the callsign), i.e. in use by

level 3.

A typical display looks like this:

Por Callsign   M Pac  F RTT   Sent Resent  Rty   Rcvd   Lost  Rol Date/Time

>5  G4FPV    ! 5 160  7   5  24217   1265   5%  24010    984   4% 28/01 09:46

>7  GB7PZT   ! 7 240  5   1 100884      0   0% 129788      0   0% 28/01 09:46

>8  GB7WV-12 ! 1 120  4   2  14456   3348  23%  14218    753   5% 28/01 09:46

>9  GB7GH    ! 4 120  7   3  16591    511   3%  14965      4   0% 28/01 09:43

10 GB7CL    ! 3 240  7   2   1122     55   4%   1009      5   0% 27/01 23:32

>11 GB7IPT-7 ! 2 120  7   5 104483  15436  14%  46589   8344  17% 28/01 09:46

12 GB7BM    ! 4 120  7   4  13562   1598  11%   5626     57   1% 28/01 09:36

8  WV12-1     1 120  7   2   3362    999  29%    918     38   4% 28/01 08:07

12 BRUM-1     4 120  7   5  10730    593   5%   1503     52   3% 28/01 09:45

1  G1ZRN      2 160  7   2   3693    164   4%    949     14   1% 28/01 02:24

4  G3MRC      2 120  7   5   4937    160   3%     35      0   0% 27/01 23:38

1  G0NXB      2 160  7   3   1485     89   5%     96      0   0% 27/01 19:37

1  G6YAK-2    2 160  7   3     26      2   7%     23      0   0% 27/01 22:46

1  G0EWH      2 160  7   4   1687    325  19%    213      2   0% 27/01 16:46

1  G4SPZ      2 160  7   2    222     10   4%     83      8   9% 27/01 20:43

1  G4MTG-2    2 160  7   3     10      0   0%      5      0   0% 27/01 11:17

1  G0HTF      2 160  7   4    332     13   3%     63     11  17% 28/01 09:45

6  G3PWJ      2 160  7   3     18      0   0%      3      0   0% 27/01 23:56

1  G7UBX      2 160  7   2     27      1   3%      6      0   0% 28/01 08:46

1  M0ALH      2 160  7   4      8      1  12%      5      2  40% 28/01 09:24

(End of list)

The display shows the port number and callsign, with a chevron ">" in

the leftmost column in the link is currently active. "M", "Pac" and

"F" are the Maxframe, Paclen and Frack last used on that link.  They

may be the same as the port or global defaults, or they may have been

modified by the router to suit link conditions

"RTT" is the Round Trip Time in seconds, i.e the average time from

sending a packet to receiving an ack for that packet.  This is

dependant on many variables, such as the packet size, the quality of

the link, the data rate, the amount of queued data, the TXdelay and

Resptime intervals at each end etc...

"Sent" is the total no. of L2 frames sent to that staion, and "Resent"

is the number of frames which had to be retried. "RTY%" is the ratio

of these expressed as a percentage.

"Rcvd" is the total no. of L2 frames received from that station and

"Lost" is the number which were lost and had to be re-sent from the

other end.  "Rol" is "Rate Of Loss", which is effectively the retry

rate which would be seen by the other station.

Finally, Date/Time shows when the station was last heard.

Line Editor

~~~~~~~~~~~~~~~

The line editor, invoked from DOS mode with the EDIT command, allows

any text file to be viewed and edited by local or remote sysops.

There is no fixed limit on the size of file which can be edited - it

depends on available memory.

When the line editor is invoked, the filename, size and number of

lines is displayed, followed by the editor prompt.  If the editor

fails to operate, you may be short of memory.  Lines of up to 255

characters in length may be accomodated.  Longer lines will be split

into separate 255 character sections.

The file contents are viewed using the "List" command.  Each line

is numbered, so that you may use the Append, Insert, Delete, Move, and

Copy functions to manipulate the text.

Because the file is loaded into, and edited in memory, you may abort

the edit at any time without affecting the original file.  The modifed

text is only written to disk by a "save" or "write" command.  You may

optionally specify a different filename to save into.

All filenames are relative to the "current working directory" you were

logged into when the editor was invoked, but you may specify full

pathnames if you wish.

Line editor commands are detailed in the command reference section.

HELP SYSTEM

===========

Basic syntax help for most commands is built into the router, and is

available using the query (?) command, e.g. "? N" will display the

syntax of the NODES command.

More detailed help is implemented as separate files in the HELP

directory, allowing you to customise it and add extra help topics as

desired.  Each topic occupies a separate file, with the filename the

same as the topic name.  The "H *" command displays a sorted directory

of all the files ending in .HLP.

Within the .hlp files, lines beginning with a semicolon ';' are not

displayed to users, so you can use them for comments, such as file

modification details.

You may choose to omit some or all of the help files if you are

running a floppy-only system.

In addition to the HELP system, sysops will find more information in

the online sysop manual using the MAN command.

INFO SYSTEM

===========

When the user issues the "I" command without any arguments, the text

defined by INFOMSG is sent to him.

However, this command may also form the entry point for a much larger

and more comprehensive information system which the sysop may

implement how he wishes.  For example you may wish to include details

of the local packet network and clubs, and maybe a set of tutorials on

general packet topics...

In order to do this, the INFO directory must exist and must contain

the desired info in the form of text files with the extension .INF.

The user will then, after using "I" by itself, be invited to enter "I

*" to list the available info topics.  If the user then enters that

command, the filenames (minus the .INF) will be displayed in

alphabetical order, and the user will be invited to read the files

using the "I <topic>" form of the command.

The filename should be relevant to the contents, within the

constraints of 8 DOS characters.  You should try to keep each file

concise in order to save air time, preferably breaking complex

subjects into a series of small files with "see also" references to

link them.  It is very frustrating for users to begin reading a file

only to discover it's not of interest, and then having to wait a long

time for it to finish!  You may use ANSI or HTML in the files, but not

pure binary.

Within the .INF files, lines beginning with a semicolon ';' are not

displayed to users, so you can use them for comments, such as file

modification details.

IP ROUTING

==========

IP routing is an integral part of PZTNODE, and is fairly simple to set

up.  You may not see the point, but others will thank you for doing

so!

In order to enable this function you must have an IP address.  If you

don't already have one, contact your local IP co-ordinator who should

be only to pleased to assign you one.  You should be able to find your

local co-ordinator via the UKIP web site or a packet message.

Put your IP address in the IPADDRESS= line in PZTNODE.CFG.

The router normally uses a single IP address for all ports, but if you

need different addresses on different ports you may specify an

additional address for any port, by including an IPADDRESS= line in

the port definition.

Example: IPADDRESS=44.131.91.245

The other thing you need to do is define the routing in IPROUTE.SYS -

see the explanation in the section which relates to that file.  If you

don't know what routes to enter, your local co-ordinator should be

able to help.

Having to enter a full IP address when using the PING and TELNET

commands is tedious, so you should put some entries in DOMAIN.SYS to

perform the translations between host names and IP addresses.  For

example, I have CNAME entries which translate the local node aliases

into IP addresses.

Finally, you should set the router's host name for TCP operations,

using the "HOSTNAME=" keyword in PZTNODE.CFG.  This is optional, and

if you omit it, it will default to "NODEALIAS:NODECALL".

Example: HOSTNAME=g8pzt.ampr.org

TCP/IP SERVICES

===============

ECHO Server

~~~~~~~~~~~

If a user TELNETs to TCP port 7 on the router, anything he sends will

be echoed back to him.  This is a useful tool for testing TCP/IP

systems.  It requires no setting up, other than the IP routing.

DISCARD Server

~~~~~~~~~~~~~~

This uses TCP port 9, and merely dumps anything sent to it.  It is

another useful tool when developing and testing TCP/IP systems.  It

requires no setting up other than the IP routing.

FINGER Server

~~~~~~~~~~~~~

This allows users to find information about other users.  It uses TCP

port 79 and reads text files for local users from the FINGER sub-

directory.  It may also be accessed using the FINGER command at the

main prompt.  If the user isn't local, it attempts to establish

communication with the Finger server on the specified host.

This server is only partly developed at present, and future versions

will give more information.  For the moment, if you wish to activate

this feature, create a FINGER sub-directory within the PZTNODE

directory, then simply create a text file for each user, using the

user's callsign or any other preferred "hostname" as the filename.

The files can contain anything you like, typically the user's name,

location, station details, QSL information etc.

For example the file finger/g8pzt might contain the text:

Name: Paula

Qth:  Kidderminster, Worc's IO82VJ

Age:  (withheld ;-)

Other: Sysop G8PZT:KIDDER router, Sysop GB7PZT BBS, Fourpak

Secretary, Unemployed software author with special interest in

comms software.  Author of: G8PZT AX25/IP BBS, G8PZT Router,

Uk White pages system, PEARL Off-line reader for Packet

Radio...

RLOGIN

~~~~~~

The secure password challenge/response mechanism, using the "@"

command, is inconvenient for frequent use, and unnecessary in cases

where the remote sysop can access the router via a "secure" link such

as a wire link between two systems.  Therefore a plain text login

system is provided for these cases.  This uses a TELNET connection to

TCP port 513.  The user is prompted to enter his callsign, and is then

asked for his password.  If the two match with an entry in

PASSWORD.SYS, the user is granted access with sysop status.  Needless

to say, this facility should *NEVER* be used over a radio link,

otherwise someone will see the password.

FTP server

~~~~~~~~~~

An FTP server is included within the program, allowing remote sysops to

upload, download, list, rename and delete files, create and remove

directories etc., which is useful when the router is located somewhere

inaccessible, such as a remote hilltop.  For example, new

configuration and program files may be uploaded, and the system can

then be restarted to perform a remote upgrade.

The FTP server is not available to non-sysops.  It is protected by a

password grid, and is only accessible to those who have a password

registered in PASSWORD.SYS.

Access to all drives and directories is unrestricted, because the FTP

server is intended only for system maintenance, not as a public file

repository.

The server uses standard FTP commands, with the exception of the USER

and PASS login sequence, which are necessarily tailored for use on a

publicly visible channel.  Unfortunately, this means that FTP clients

which perform automatic logon, such as WS_FTP, CuteFTP etc., cannot be

used.  However, there is an exception to this rule...  If the user

accesses on a port which has the SYSOP=1 option (usually reserved for

secure links) normal plain text passwords may be used.

In order for the FTP server's HELP command to work, you must place the

ftp help files in the HELP/FTP directory.

The FTP server commands are described in detail in the sysop command

reference section.

MHEARD

======

The MHEARD facility lists recently heard stations, and may be enabled

or disabled for any port.  It records callsigns in order of reception,

with the most recent at the top of the list, along with other useful

information, such as the date/time and the number of frames heard.

This is useful for users to discover who else the router can hear, to

aid the search for suitable digipeaters, and to diagnose problems.

Even on linking-only ports, where there is only usually one partner,

it provides a useful indication when the frequency is being

encroached, either by deliberate squatting, unauthorised attempts to

link, or lift conditions.  I recommend that you enable MHEARD on all

ports.

If you have included an APRS-style position report in your ID beacon,

the router will know its own position and will display position,

distance and bearing for any stations which broadcast APRS positions.

This is a great aid to network mapping, and it would be nice if

everyone were to make use of APRS.

If the station was heard via a digipeater, the digipeater call is also

shown.  If the heard station is a node, the word "NODE" is displayed,

and if it is sending IP traffic, "IP" is shown.

Within PZTNODE.CFG, the MHEARD= keyword is used in each port

definition block to enable or disable the MHEARD facility and to set

the size of the list.  For example "MHEARD=50", enables it and sets

the table size to record a maximum of 50 callsigns.  If the keyword is

omitted, the default is 15 callsigns.

You can control what sort of callsigns are recorded in the MH list

using the MHFLAGS= keyword.  The default is 255 (show everything), and

the number is formed by adding the following values:

1    Show directly heard stations

2    Show directly heard digipeaters

4    Show digipeated stations

For example "MHFLAGS=3" would show directly heard stations and

digipeaters, but not the stations heard via digipeaters.

The MHEARD list is available to sysops and users alike, using the MH

command (see command reference section).

WATCHDOG

========

PZT router has an inbuilt software watchdog.  If enabled, this will

attempt to reboot the computer if the router "locks up".

A lock-up may be caused for example by faulty external driver or

third-party server software, by certain hardware errors, or by "soft"

errors due to RF break-through, supply spikes or ionising radiation.

Additionally, a DOS "shell" session of excessive duration is treated

as a lock-up because the operator may have abandoned the console

without terminating the shell, and the shell may be running a process

which cannot be safely terminated.  Therefore a complete reboot is the

safest option.

The software watchdog is enabled by entering a non-zero setting in the

"watchdog interval" parameter in INIT.SYS (see elsewhere).  You should

not use too low a setting, otherwise the system may reboot if an

external process takes too long to run, or if you shell out to DOS

using the F9 key.  120 sec (2 minutes) is a suggested figure.

The software watchdog cannot work miracles, because sometimes the

crash is too severe, and sometimes the computer will not respond to a

"warm" reboot.  The use of an additional "hardware" watchdog is

recommended.

Therefore, in addition to the software watchdog, the system will

supply pulses via a spare parallel port, to drive an external hardware

watchdog.  This would for example consist of an opto-isolator driving

a monostable circuit such that if the pulses ceased for more than 120

secs (must be longer than the boot-up time!) the mains supply would be

interrupted for long enough to cause a "cold" reboot (say 5 secs).

The hardware watchdog pulses are enabled by specifying a suitable

parallel port address for the WATCHADDR parameter.  The D0 pin will be

toggled at 18 Hz, D1 at 9Hz, D2 at 4.5 Hz etc.  You can drive an

optoisolator from one of these pins.

A hardware watchdog should detect the presence or absence of an AC

signal from the parallel port.  A sustained high or low value

indicates a fatal lock-up.  Your circuit should include a delay long

enough to allow the system to reboot, including all programs (e.g.

chkdsk) which are run at boot up.  This is to prevent it from

triggering again before the pulses are established, causing an endless

reboot cycle!

Warning:  If you run PZTNODE in a Windows95 environment, the frequency

of the hardware watchdog pulses will be greatly reduced (by a factor

of 9) if the window is backgrounded.  The software watchdog interval

is similarly inflated, so a 2 minute timeout will become 18 minutes.

I have yet to find a solution to this problem, which is due to Windows

not properly emulating certain BIOS functions.  However, I feel it is

unlikely that anyone would want to run a remote system (and thus

require watchdogs) on a Windows system, due to stability

considerations!

STATS

=====

The following section attempts to explain some of the figures produced

by the "stats" command.

Entering S alone will produce a single page thus:

G8PZT:KIDDER} Stats:

Time active (mins):              3940

Memory: Max/Cur/Min/Out        309832    278056    273720         0

Known nodes:                      139

Total bytes sent/rcvd:       12685081   9695736

Max ccts L2/L3/L4/TCP:             10        22        12         0

IP Heard/Reasm/Rcvd/Routed:         7         0         5         2

IP Bad Hdr/Chksum/Version:          0         0         0

IP Sent/Frag/Unsent/Total:         11         0         0        13

L4 Connects Tried/Made/Rcvd:      683       667       728

L4 total frames Sent/Rcvd:      43347     41880

L4 Sent/Rcvd/Resent/Reseq:      28431     23972       895      1831

L4 Chokes Sent/Rcvd:             1750       168

L4 Timeouts/Failures:             857        26

Use STATS * to display full stats

"Time active" is the number of minutes since the router was last

restarted.

"Memory" shows various stats about the program's RAM usage.  "Max" is

the maximum available free memory after the program is loaded, "Cur"

is the current free memory, "Min" is the minimum that has been reached

during operation, and "Out" is the number of times a memory request

has failed.

"Known nodes" is the number of nodes heard about whose quality is

greater than the minqual setting.

"Total bytes sent/rcvd" are the total bytes sent and received by all

the ports.  They include all ax25 overhead.

"Max ccts" shows the highest number of simultaneous circuits that have

been active at any time.  Separate figures are shown for Ax25 levels

2,3 and 4, and TCP/IP.

"IP Heard/Reasm/Rcvd/Routed" shows the total number of IP frames heard

(i.e. addressed to us and to others), reassembled from fragments,

received (i.e. addressed to us), and routed to others.

"IP Bad Hdr/Chksum/Version" shows the number of IP frames ignored due

to corrupt IP headers (e.g. too short to be a legal IP frame),

checksum mismatch, and incompatible IP version.

"IP Sent" is the number of IP datagrams originated by the router, i.e.

not routed from somewhere else.  "Frag" is the number of datagrams

which had to be fragmented to fit a link.  "Unsent" is (if I remember

correctly) the number of datagrams which couldn't be sent due to low

memory or no route, and "Total" is the total number of datagrams or

fragments thereof which were transmitted.

"L4 Connects Tried/Made/Rcvd" shows the total number of outgoing and

incoming AX25 level 4 connections.  "Tried" is the number of requests

initiated, "Made" is the number which were successful, and "Rcvd" is

the number of incoming connects.

"L4 total frames Sent/Rcvd" shows the total number of AX25 level 4

frames of all types sent and received by the router.

"L4 Sent/Rcvd/Resent/Reseq" shows the totals for AX25 level 4

information-bearing frames.  "Sent" is the number of frames originated

by us.  "Rcvd" is the number addressed to us. "Resent" shows how many

were re-transmitted because no ack was received.  "Reseq" is the

number of frames that re-sequenced, i.e. were received out of sequence

and subsequently used when the missing frames arrived.

"L4 Chokes Sent/Rcvd" counts the number of ax25 level 4 choke frames

sent and received by the router.  A sent choke indicates that we are

receiving L4 data faster than we can route it, and instructs the other

end to back off for a while.  A received choke indicates that we are

sending data too fast for the other end to handle.  Note that these

figures do not necessarily indicate that there is something wrong with

the router's links, as they apply to the "virtual circuit" from one

application to another, which may span many intervening nodes.

"L4 Timeouts" shows the number of times the ax25 level 4 T1 timer

timed out while waiting for an ack, causing re-transmission of a

frame.  "Failures" shows the number of level 4 circuits which were

abandoned due to excessive retries.

-------------------------------

Entering s * will produce the above followed by:

Interface: 1      2      3

RX Overruns:          0      0      0

TX Underruns:         0      0      0

CRC/Framing Errors:   0      0      0

Break/Abort Errors:   0      0      0

Rx Overflow err:      0      0      0

Tx Overflow err:      0      0      0

Misc. errors:         0      0      0

Tx/Rx Active %       0/0    0/0    0/0

L3 Frames Heard       0      0      0      0  12681      0  31364   6736

L3 Frames Rcvd        0      0      0      0   3904      0  23721   1698

L3 Frames Sent        0      0      0      0  13978      0  25438   7418

L3 Frames Relayed     0      0      0      0   8036      0   7643   5038

L2 Frames heard    4896      0     21   2171  25594    440  42977  28402

L2 Frames rcvd     2288      0      0    582  24816    378  42902  27976

L2 Resequenced        0      0      0      0      0      0      0      0

L2 Reassembled        0      0      0      0      0      0      0      0

L2 Info Received    593      0      0    104      0     63      0      0

L2 T1 Timeouts      134      0      0     18   1086     40   4453   3407

L2 Digipeated         0      0      0      0      0      0      0      0

L2 Info Sent       2301      0      0    755  15580    381  25436  12170

L2 Info re-sent     362      0      0     20   1507     38      4   3426

L2 Fragmented         0      0      0      0      0      0      0      0

L2 Frames Sent     4100   1991    263   3096  32154   2051  52573  32568

L2 REJ Received     177      0      0     10    732     27      0    691

L2 Rx out of seq     31      0      0      3    247     12      0   2764

L2 Frames Corrupt     2      0     56     13      0      2      0      0

L2 FRMRs Sent         0      0      0      0      0      0      0      0

L2 FRMRs Rcvd         0      0      0      0      0      0      0      3

L2 Bytes Rcvd     44107  31204   7209 302045  4253K  22043  1235K  7634K

L2 Bytes Sent    100133  10517   3112 317645  3105K  44132  1001K  6512K

Poll Timeouts         3 246969      6      7      9      5      0      7

The first section shows a set of counters for each interface.

"RX Overruns" counts the number of times a second byte was received

before a first was read by the CPU.  A high number indicates that the

PC is too slow for the selected serial port or HDLC card data rate.  A

16550-based serial card may help if the port doesn't already use one.

Failing that, you will have to reduce the baud rate.

"TX Underruns" is used only by HDLC cards, and counts the number of

times the TX went empty while waiting for another frame byte to be

loaded.  A significant figure indicates the computer is too slow for

the baud rate.  Each underrun causes an aborted frame.

"CRC/Framing Errors" counts either the number of bytes received

without proper stop bits (ASYNC interfaces), or the number of received

frames which corrupt (HDLC cards).  For ASYNC interfaces, a

significant count here can indicate a hardware problem, such as a

faulty line driver, serious RS232 line noise or distortion, or

significant baud rate mismatch.  For HDLC cards it indicates a level 1

problem, such as distortion in the TX/RX RF or audio paths, or simply

a lot of packet collisions.

"Break/Abort Errors" counts the number of times a line "space" condition

longer than 1 word interval was received.  For serial ports this can

indicate a faulty line driver, a faulty diode matrix on a multikiss

system, or even (as recently happened to me) a malfunctioning TNC

eprom.  On HDLC cards it can result from insufficient audio drive from

the RX, a mismatched baud rate, squelch tails (I hope to prevent this

in a later version), or genuine ABORT sequences transmitted by the

other end of the link.

"CRC Errors" shows the number of frames abandoned due to CRC or

checksum error.  For KISS interfaces this is only maintained if the

CHECKSUM kissoption is enabled.

"Rx Overflow err" shows the number of times a frame was abandoned

because it was too large to fit into the receive buffer.

"Tx Overflow err" counts the number of times that the TX couldn't

accept a character (serial devices) or a frame (block devices) because

the TX buffer was full.  If you persistently get a high value, it

indicates that the device is too slow for the data throughput.

"Misc. errors" counts all sorts of miscellaneous errors and the

meaning is different for each type of interface.  For example, on KISS

interfaces it counts KISS framing errors.  It is mainly for my

benefit.

Following the interface stats is the ax25 level 3 counters, one for

each port.  Note that on a system with more than 7 ports the display

may wrap.  I will be addressing this in a later version.

"L3 Frames Heard" is the total number of ax25 level 3 frames heard, no

matter who they are addressed to.

"L3 Frames Rcvd" is the number of ax25 level 3 frames which were

addressed to the router.

"L3 Frames Sent" is the number of ax25 level 3 frames which originated

at the router.

"L3 Frames Relayed" is the number of ax25 level 3 frames which were

routed through our system by other systems.

After the level 3 stats, there are the ax25 level 2 counters, one per

port.

"L2 Frames heard" is the total number of ax25 level 2 frames heard,

whether addressed to us or not.

"L2 Frames rcvd" is the number of ax25 level 2 frames received, which

were addressed to the router.

"L2 Resequenced" is the number of ax25 level 2 frames received out of

sequence and subsequently used when the missing frames arrived.

"L2 Reassembled" is the number of ax25 level 2 frames successfully

reassembled from fragments.

"L2 Info Received" is the number of ax25 level 2 information-bearing

frames received, i.e. addressed to the router.

"L2 T1 Timeouts" counts the number of times that the ax25 level 2 T1

(frack) timer timed out, causing transmission of a poll frame.

"L2 Digipeated" is the number of ax25 level 2 frames digipeated by the

port.  Note that if digiport isn't zero they may actually have been

re-transmitted by another port, but are recorded on the "receiving"

port only.

"L2 Info Sent" is the total number of ax25 level 2 information frames

sent by the router.

"L2 Info re-sent" records how many ax25 level 2 information frames

were re-sent due to frame loss.  A high figure here, in proportion to

the "info sent" figure, indicates a problem with the RF link, the L2

settings, or the other end's system (e.g. desense, or running out of

buffers).

"L2 Fragmented" is the number of ax25 level 2 information frames which

were fragmented to fit the outgoing link.

"L2 Frames Sent" is the total number of ax25 level 2 frames, of any

type, sent by the router, i.e. it includes all info, supervisory, and

digipeated frames.

"L2 REJ Received" is the number of ax25 level 2 "reject" frames

received, which indicate that the other end of the link didn't receive

some of our frames.  There are many possible reasons for this, some of

which are mentioned in the next paragraph.

"L2 Rx out of seq" shows how many ax25 level 2 frames were received

out of sequence, and indicates that some incoming frames are getting

lost or trashed.  A few of the possible causes might be: signal too

weak, fading, other signals on channel, natural or man made

interference, desense or key clicks from adjacent transmitters, poor

rx audio response, low received audio, over-deviation, RF frequency

mismatch, badly aligned rx, TNC hardware problems, synthesised rig

taking too long to stabilise on RX after TX, other end's synthesised

rig taking too long to stabilise on TX, hum, noise, distortion or

disturbances on modulated audio... the list is endless.

"L2 Frames Corrupt" is the number of frames which were dumped because

they were too short to be legal ax25 level 2 frames, or were in some

way invalid.  It is sometimes possible for a KISS TNC, especially if

running "open squelch", to send garbage to the router,  or the frame

may be trashed by bit errors on the serial link between TNC and

router, and in either case these frames are dumped if the error can be

detected.

"L2 FRMRs Sent/rcvd" shows how many ax25 level 2 "Frame Reject" frames

were sent by the router, or received by it, indicating serious

protocol errors or deliberate interference.

"L2 Bytes Sent" and "L2 Bytes Rcvd" simply provide a port by port

breakdown of the total bytes sent/rcvd figure.

"Poll Timeouts" counts the number of times a BPQKISS TNC was polled

with no response being received from it.  A large figure might

indicate a crashed, disconnected or unpowered TNC, or data loss on the

serial link.

FRAME PIPE

==========

An optional facility allows frames received (and optionally sent) on

one port to be copied to another port.  A typical use would be to

allow a PMS connected to one port to see the traffic on another port,

e.g. UNPROTO headers from a local BBS.  Note that this is *not* the

same as digipeating.

The facility is enabled by including the PIPE keyword within a port

definition, e.g. PIPE=4 would pipe frames to port 4.  If PIPE=0, or

the keyword is omitted altogether, no piping takes place.

A pipe only allows traffic in one direction - in order to have two way

traffic you must set up a reverse pipe on the partner port, e.g.

(on port 3)    PIPE=4  ; Pipe frames to port 4

(on port 4)    PIPE=3  ; Pipe frames to port 3

You may pipe several ports to a single destination port, but you

cannot "broadcast" from one port to several others - you can only have

one *outgoing* pipe from any port.

This feature is capable of generating an immense amount of traffic, so

use it with care - your target port MUST be capable of dealing with

the traffic load, and you should avoid setting up a network of pipes

which could cause an endless loop!

The types of frame to be piped can be controlled using the optional

PIPEFLAG keyword, which is ignored unless piping is active.

If PIPEFLAG is not specified, the default value is 3.  The value is

made up by adding together the following numbers:

1       - UI frames *not* addressed to nodecall/alias.

2       - Non-UI frames *not* addressed to nodecall/alias.

4       - UI frames addressed to nodecall/alias.

8       - Non-UI frames addressed to nodecall/alias.

16      - UI frames transmitted by node.

32      - Non-UI frames transmitted by node.

64      - Allow budlisted users to be piped.

128     - Netrom frames

256     - IP / ARP frames

e.g. PIPEFLAG=5 will pipe all received UI frames, but not those which

are originated by the node itself.

You are *STRONGLY* recommended to use the default value unless you

specifically need to see additional traffic.

BROADCASTING

============

PZTNODE has the ability to "re-broadcast" a received frame on several

ports at once.  This can be thought of as a "one-to-many" pipe but

there are subtle differences.

Whereas pipes conduct all frames of the selected type(s) virtually

regardless of source and destination addresses, the broadcast function

acts only upon UI frames, and only if the source and destination

addresses match those specified by the sysop.

Another difference is that frames may be broadcast to different groups

of ports depending on the destination address.  For example, FBB

unproto headers from a BBS may be rebroadcast on certain user-access

ports only, while mail beacons may be distributed to a different,

possibly overlapping, set of ports.

You may alternatively use this feature as a "filtered pipe" between

two ports, allowing only frames with acceptable source and destination

calls to pass through.

Note that this feature does not require a "connection" to the router,

it is purely an unconnected mode.  I devised it to allow BBS's to

distribute mail beacons and unproto mail headers.  You may use it (or

not) how you wish.

You should be aware that broadcast traffic takes precedence over all

other frames, so an excessively high level of broadcast activity may

cause other outbound traffic on the destination port to be delayed.

However, it is unlikely that anyone will notice this effect unless

they are seriously overloading the channel.

Broadcasting is controlled by two keywords in each PORT section of the

PZTNODE.CFG file.  The first keyword is BCAST, and is used to specify

the destination addresses which will be accepted for broadcast.

For example, BCAST=MAIL,FBB will re-broadcast received non-digipeater

UI frames addressed either to FBB or MAIL.  The frames addressed to

FBB will be broadcast on all ports which have FBB in their BCAST list,

and those addressed to MAIL will be broadcast on all ports which have

MAIL in the BCAST list.  If no matching ports are found, the frame is

broadcast only on the port upon which it is received.  If you don't

wish to enable the broadcast function, simply omit (or comment out)

the BCAST keyword.

The second keyword controlling broadcasting is BCFROM, which is used

to specify a list of approved broadcasters.  You may use this to

restrict the broadcast facility to certain senders only.  e.g.

BCFROM=GB7PZT,GB7MAX says that only GB7MAX and GB7PZT will be allowed

to broadcast.  If the keyword is omitted, the broadcast facility is

unrestricted.

For both keywords, the list of calls should be separated by commas,

and should include no spaces.

CHAT SERVER

===========

G8PZT router has a built-in chat server, which is a device which

allows groups of users to hold live multi-way conversations without

the usual problems of having to manage several streams at once.  It is

available to all callers, and is accessed by using the CHAT command at

the main prompt, or by connecting directly to its callsign or alias.

TCP/IP users can additionally reach it by TELNETting to port 3600.

The chat server has 32768 "channels", each of which can support an

unlimited number of users, so it is possible for groups to have their

own "private" channel, or to reserve certain channels for specific

topics.  Channel 0 (zero) is the default at log-on and is used as a

sort of "calling channel", from where users may QSY as required.

Users may "join" as many channels as they wish, so they may take part

in several separate conversations at once.

Channels 0 to 255 are "local" to each chat server, and the remaining

channels 256-32767 are "global", i.e. they are linked with all other

servers, providing the sysop sets up at least one link to another

server.  Only Netrom links are allowed at present, so the partner must

be in your node table.

Once logged onto a channel, anything sent by the user is copied to all

other users of that channel, except lines beginning with a forward

slash (/), which are interpreted as chat server commands.  The copied

text is preceded by the channel number and the sender's callsign and

name, to allow the recipients to identify who sent it.

All chat server commands begin with a forward slash (/), and most may

be abbreviated to the initial letter.  The /? command will show the

available commands and syntax, while /HELP will give more details.

The /NAME command is used to enter the user's first name, and serves

as a "login".  Users are not permitted to join any channels until they

have supplied a name.  TCP/IP users must additionally supply a

callsign with the /USER command.  /CHANNEL, /JOIN and /LEAVE are used

to select the desired channel, /WHO shows the active channels and who

is using them, and /QUIT terminates the chat session.  The full

command set is shown in more detail in the command reference section.

If transaction logging has been enabled in PZTNODE.CFG, chat server

activity will be logged in file CHATSERV.LOG.

The chat server is fully automatic and requires minimal setting up. It

has its own callsign and alias defined in PZTNODE.CFG, plus a list of

link partners.  The alias (6 chars max) should preferably end with

"CHT" and begin with something geographically relevant, e.g. BHMCHT

for Birmingham, LDSCHT for Leeds etc., so they can be identified

easily in node tables.  The only other setting up required is to

install the chat help files in the directory help/chat.

As with *all* chat servers, you should be aware that the throughput on

each channel is determined by the slowest link on that channel,

therefore "dxing" into distant chat servers is not encouraged.  The

same applies to partner servers - if the link to your partner isn't up

to the job the throughput for everyone will be compromised

APPLICATION INTERFACE

=====================

Because PZTNODE is not a TSR, you cannot run other applications "on

top" of the router in a DOS environment.  However, you may do so if

you run it in a Windows (or possibly Desqview?) environment.

In order to do this, you must load an additional component,

PZTHOST.EXE, *before* you start Windows.  This is a TSR which occupies

64k of base memory.  After PZTHOST, start Windows itself, then run

PZTNODE in a window and the applications in other windows.  You cannot

run PZTHOST in a window.

The application interface conforms almost exactly to G8BPQ's interface

specification, detailed in BPQHOST.DOC, which is usually included with

the BPQ program. For the moment, you are referred to that document if

you require details of how to write programs for the interface.

I say "almost" exactly, because the "BBS" application is not fixed as

the first application.  Thus in function 6, CX=0 uses the APPLICATION

callsign, not the "BBS" callsign, as there is no specific "BBSCALL" in

PZTNODE.

The application interface has been tested with PZT BBS, PAC4, Easyterm

and PEARL so far.  If you find it works (or doesn't) with other

applications, please let me know.  It will NOT work (yet) with 32 bit

applications such as Winpack, but I'm working on that.

Defining Applications

~~~~~~~~~~~~~~~~~~~~~

Applications must be specified in PZTNODE.CFG, in the following

manner:

Each application definition block must begin with APPL=<number> and

end with ENDAPPL.  There should be a separate block for each

application.  Applications which use more than one stream, need only a

single definition.  The application block should contain one or more

of the following keywords:

APPLNAME       The name by which the application is accessed from

the router's command line. e.g. "PMS".  If a user

types this name, they will be connected to the

application.

APPLCALL       Callsign which the application will use.

If a callsign is specified, the application will

accept level 2 connects, subject to the setting of

APPLMASK (see below).

APPLALIAS      Alias for use by the application.

As APPLCALL.

APPLQUAL       Netrom quality to broadcast.

If a non-zero value is specified here, the

APPLCALL will be included in nodes broadcasts and

the application will be connectible at level 4.

The application number must be between 1 and 8, and corresponds to the

application's position on a BPQ "APPLICATIONS=" line.  Some

applications have fixed application numbers, e.g. BBS's and PMS's must

usually be the first application and Host programs such as PAC4 are

usually the second.

All fields within an application definition block are optional - you

may have for instance choose to have an applname but no applcall, and

the application would only be reached by typing the applname at the

command line.  Or you could have an applcall but no applname, in which

case the application would be directly connectible, but wouldn't be

reachable from a command line shortcut.

Example:

APPL=1

APPLNAME=PMS

APPLCALL=G8PZT-2

APPLALIAS=PZTPMS

APPLQUAL=50

ENDAPPL

If you want an application to be directly connectible on a particular

port, it (the application) must have a callsign, an alias or both, and

the corresponding bit in that port's APPLMASK must be set.

APPLMASK specifies which applications will be directly connectible on

a given port.  The default is 255, which allows all applications.  The

value is made up by adding together the following numbers:

1       - Enable Application 1

2       - Enable Application 2

4       - Enable Application 3

8       - Enable Application 4

16      - Enable Application 5

32      - Enable Application 6

64      - Enable Application 7

128     - Enable Application 8

For example, if a port's definition contains "APPLMASK=9", it will

allow direct connections to applications 1 and 4 on that port,

providing those applications have either an APPLCALL or an APPLALIAS.

Setting The Host Interrupt Number

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In addition to the above, PZTNODE.CFG must contain the HOSTINTERRUPT=

keyword, which specifies the software interrupt number the

applications will use to communicate with the router.  The value is in

decimal, and 127 is the value usually chosen.  The default, if you

omit the keyword, is 0 which disables the host interface.  Do NOT go

below 96, as the operating system may be using the interrupt.

Note that some applications may specify the host interrupt in

Hexadecimal, e.g. 7F is 127 decimal.

FILE FORMATS

============

All PZTNODE configuration files are commented to aid setting up, but I

will document them in more detail here too, just in case you damage a

file or wish to study a paper copy in the garden!  They are presented

in alphabetical order....

DOMAIN.SYS

~~~~~~~~~~

This optional file is used by the router to resolve host names such as

"lgsbbs" and "g8jvm.ampr.org" into their corresponding IP addresses.

If not present, you will have to enter the full IP address of any

target host when using the PING, TELNET, TTY, and FINGER commands.  It

is read every time an address needs resolving, so any changes you make

will take effect immediately.

Name resolution using an external DNS (Domain Name Server) is not yet

complete, so you should add entries to this file for every host you

may wish to contact.  (Even when DNS is complete, this file will

always read first, so it will speed things up if it is kept up to

date.)

Each host is listed on a separate line.  Each field should be

separated by one or more spaces or tabs.  Comments, spaces, tabs and

blank lines are permissible to aid clarity.  The records are case-

insensitive, but most people use lower case for hostnames.  The format

of each record is as follows:

<hostname> IN A <ip-address>

<hostname> IN MX <hostname>

The first form maps a hostname to an IP address, and the second form

maps an alternative hostname to a host already defined.  For example,

the IP address for the GB7PZT mailbox is 44.131.91.2 so it would have

an Internet Address (IN A) record like so:

gb7pzt  IN   A    44.131.91.2

But gb7pzt is also known locally as "pztbbs", and could also be

referred to by its fully qualified domain name "gb7pzt.ampr.org".

There is nothing to stop you adding further IN A records for gb7pzt,

one for each version of the domain name, but you could instead use the

second form shown above, the MX or "Mail Exchange" record.  These are

usually used for defining alternate names for mail servers, but as PZT

router is not concerned with mail it uses them for any host.  The

format of the additional records would be:

pztbbs            IN   MX   gb7pzt

gb7pzt.ampr.org   IN   MX   gb7pzt

Thus if the user types "TEL pztbbs" or "TEL gb7pzt.ampr.org", the

gb7pzt record is used.  This removes the need to keep repeating the IP

address in multiple "A" records, and makes it easier if the IP address

is changed.

IPROUTE.SYS

~~~~~~~~~~~

This optional file defines how IP datagrams should be routed, and is

not required if you don't intend to route IP traffic.  It is read only

at boot-up, so if you make changes to it, they won't have any effect

until the next reboot.  If present, it saves you having to enter all

the IP routing by hand.  In order to keep the number of files down,

this file also contains the permanent ARP (Address Resolution

Protocol) entries.

If you are familiar with IP you can skip this paragraph....  All IP

addresses consist of a 32 bit binary number, which is composed of four

8 bit binary numbers.  For clarity they are usually expressed as four

decimal numbers separated by dots, the so called "dotted quad" form,

for example 44.131.91.2 is the address for gb7pzt.  Each of the

numbers which make up the quad can range from 0 to 255, i.e. 256 in

all.  0, 128 and 255 are usually reserved for special purposes.  The

most significant number identifies the "network" within the whole

Internet.  44 is allocated to Amateur Packet Radio, or "ampr.org".

Within ampr, the second number identifies the country, and in our

example 131 is the code for UK.  The third number identifies the

"region", and in our example region 91 is North Worcestershire.  The

last number identifies up to 256 separate users within the region.

The addresses within a region are sometimes allocated on a first come

first served" basis, or sometimes in groups to allow further

subdivision of a region.  Unlike NetRom, IP routing has to be

explicitly defined by the sysop, in our case using entries in

IPROUTE.SYS.

Each record should be on a separate line, and there should be one or

more spaces or tabs between each field.  Records are not case

sensitive.  Comments are allowed in the file, providing they are on a

line beginning with a semicolon.

All IP routing records begin with the word "route", and there are two

types.  A "Route default" entry defines how datagrams should be routed

if no other route can be found.  "Route add" entries add routes to the

table (what a surprise! ;-)

The general form of a route default entry is:

ROUTE DEFAULT <portnum> [<gateway> [<d|v|n>] ]

<portnum>  is the port number on which to route the datagram.

<gateway>  is the IP address of the neighbouring router to whom

datagrams should be routed if there is no other applicable

route.  If this address is not specified, the destination

will be tried directly on the default port.

<d|v|n>    Mode: (d)atagram, (v)irtual circuit or (n)etrom.  If

omitted, the default is "Datagram".  Datagram mode should

be used on links with low loss rates.  Better performance

can be obtained on lossy links by using Virtual Circuit

mode.  Netrom mode is inefficient, but can "tunnel"

datagrams across non-ip parts of the network.

At least a port number must be specified. A typical entry would look

like:

route default 11 44.131.90.6 d

which means "route all unknown destinations via neighbouring router

44.131.90.6, on port 11, using datagram mode".

The general form of the route add entry is:

ROUTE ADD <host/bits> <gateway> <portnum> <d|v|n>

<host>     is the target host IP address in dotted quad form.

<bits>     is the number of bits to be matched.

If the <gateway> address is 0.0.0.0 or just "*", the destination will

be tried direct on the specified port.

When making routing decisions, only the specified number of bits (0-

32) of the address are compared, the remainder being ignored.  This

allows a group of IP addresses to be matched by a single entry.  For

instance, a setting of 24 would cause only the most significant 24

bits of the address to be compared, allowing a whole region (256

addresses) to be routed to a single gateway.

A typical routing entry would look like this:

route add 44.131.93.0/24   44.131.93.240   5  d

which would route all region 93 traffic (44.131.93.0 - 44.131.93.255)

to the gateway 44.131.93.240 on port 5 using datagram mode.

You may specify more than one route to the same group of addresses

using different bit masks.  For each address within the group, the

router will choose the route with the highest number of matching bits.

ARP entries are responsible for mapping gateway IP addresses to

"hardware" (i.e. AX25 or Ethernet) addresses.  In order to send an IP

datagram over an AX25 or Ethernet network, it must be "wrapped" in an

AX25 or Ethernet packet, and that packet will need a destination

address appropriate to that network.  For example, if I want to route

a datagram to 44.131.91.2 it must be wrapped in an AX25 packet

addressed to GB7PZT-5.

The system *will* sometimes work without any ARP entries, due to the

process of "ARP resolution", whereby a router can make a broadcast

asking adjacent systems if they know the hardware address for a given

IP address, but this process takes time and the adjacent routers may

not know the answer.  Thus it is advisable at least to have one ARP

entry for each of your direct RF neighbours.  The general form of an

ARP entry is:

ARP ADD <host> <ax25 | ether> <callsign | ether_address>

For example:

arp add 44.131.90.6      ax25    GB7IPT-9

PASSWORD.SYS

~~~~~~~~~~~~

This optional file contains the passwords for remote sysop access.  If

not present, remote sysop access will not be possible.

There should be one entry for each remote sysop.  Each entry should be

on a separate line and consists of two fields separated by one or more

spaces or tabs.  The first field is the remote sysop's callsign

(without an SSID) and the second field is the corresponding password

string.  For example:

G8PZT   thisispaulaspassword

G1LOA   thisisrobertspassword

The fields are not case sensitive, and the passwords can be anything

you like, with a couple of limitations, namely PASSWORDS MUST NOT

CONTAIN SPACES, and the TOTAL LINE LENGTH MUST NOT EXCEED 80

CHARACTERS.  You should try to choose a password that you can easily

remember, but that others would find hard to guess.  It should

preferably be longer than 5 characters, to reduce the chances of

someone guessing it.  The longer you make it, the more secure it will

be.  A random string is the most secure, because even if someone works

out part of it, they won't be able to guess the remainder.

You may have a different password for each sysop, or you might choose

to give everyone the same password.  Obviously the former is the most

secure, but it's completely up to you.

PZTNODE.CFG

~~~~~~~~~~~

This file is mandatory, and contains most of the configuration info

for the router.  It is read only when the program starts.

It consists of entries of the general form <keyword>=<value>, each on

a separate line.  Keywords are not case sensitive, and in general they

may be in any order, but interfaces MUST be defined before ports.

Blank lines are allowed, and comment lines must begin with a semicolon

(;) in the leftmost column.  Lines must not exceed 255 characters.

Note that some timings are in millisecs, some are secs.

The following example is for illustration purposes only, and I have

"borrowed" a few callsigns for use in the EXCLUDE, ROUTES and

VALIDCALLS fields.  This does not represent my setup.

; PZTNODE.CFG  Configuration file for G8PZT Packet Router version 1.59e

;

;

;       Interrupt number to use for the BPQ-compatible host interface.

;       The host interface may only be used in a Windows 95 or Desqview

;       environment. To use it you must load pzthost.exe before windows

;       then run pztnode.exe and each application within separate dos

;       windows.

;       If you are not using host interface, comment this parameter out,

;       or set it to 0.

;

; HOSTINTERRUPT=127

;

;       Station Identification:

;       ~~~~~~~~~~~~~~~~~~~~~~~

;

;       Node callsign: Up to 6 chars plus optional SSID between 1 and 15

;       This is the callsign used for all L3/4 operations, and the default

;       for L2 operations on each port.

;

NODECALL=G8PZT-6

;

;       Node alias: Up to 6 characters.

;       Aliases beginning with "#" are not displayed in node lists, and

;       are typically used for "linking only" nodes.

;       You should preferably choose an alias which is geographically

;       relevant beyond your own local area, for example BRSTOL, LEEDS,

;       or BRUM are good, because users can recognise them in node tables,

;       whereas GAB1 and WBA are bad - where on earth are they?

;

NODEALIAS=KDRMIN

;

;       Callsign for console operations.  You can set this independantly

;       of NODECALL or you may set them the same.  You may at any time

;       override this callsign using the "linked as" command.

;

CONSOLECALL=G8PZT

;

;       IP address for IP routing.  Your local IP co-ordinator should

;       be able to assign you one.  If you aren't routing IP (shame on

;       you!) comment this out or set it to 0.0.0.0

;       The router normally uses a single IP address for all ports, but

;       you may specify additional addresses for each port.

;       (If you are routing, remember to define routes and hostnames in

;       IPROUTE.SYS and DOMAIN.SYS respectively)

;

IPADDRESS=44.131.91.4

;

;       Host name for TCP (optional).  If you omit this, it will default

;       to "NODEALIAS:NODECALL".

;

HOSTNAME=g8pzt.ampr.org

;

;       Chat Server parameters:

;       ~~~~~~~~~~~~~~~~~~~~~~~

;       Note - the chat server is an integral part of the system and

;       cannot be disabled.  You may prevent it from being directly

;       connectible (if you wish to deprive users of facilities) by

;       setting CHATCALL and CHATALIAS the same as NODECALL and

;       NODEALIAS, but it will still be available to all users via

;       the "chat" command.

;

;       Chat server callsign. This may be the same as the nodecall, but

;       must use a different SSID, otherwise the chat server will not

;       be directly conectible.

;

CHATCALL=G8PZT-8

;

;       Alias for chat server (6 chars max).  I suggest this should end

;       with "CHT" and begin with something geographically relevant,

;       e.g. BHMCHT for Birmingham, LDSCHT for Leeds etc., so it can be

;       easily identified in node tables. If you specify the NODEALIAS

;       here, the chat server will not be directly connectible.

;

CHATALIAS=KDRCHT

;

;       List of chat servers to which this server will link.  You must

;       supply the *callsigns* not the aliases.  Unilateral links are not

;       allowed - each partner in this list must place your CHATCALL in

;       their CHATLINKS list.  You may only use partners who are in your

;       node tables.  PZT chat servers may be interconnected with each

;       other, but *NOT* with "Ping-Pong Converse" servers.  Don't link

;       with distant servers - if the links are too slow your users will

;       get poor service. You may disable all outgoing and incoming

;       linking by omitting this line (or commenting it out).

;

CHATLINKS=G9XOT-8,G7DTY-8

;

;       Chatcall quality to broadcast.  This can be used to limit the

;       visibility of your server to a reasonable geographical area,

;       and discourage chat server "dxing". Default is 255, i.e. chat

;       call is visible over same distance as node call.

;

CHATQUAL=150

;

;       Optional Software Watchdog.  If you set a non-zero value here,

;       the node will attempt to reboot if the code goes into an

;       endless loop.  The value is in seconds.  Note: the watchdog

;       keeps running if you shell to DOS using Alt-F9, so keep shell

;       sessions short or the node will think you've forgotten to return

;       and will reboot!

;

WATCHDOG=120

;

;       Address (hex) of parallel port used to drive hardware watchdog.

;       If you set a valid address here, the least significant data bit

;       of the port will be toggled several times per second if the node

;       is working correctly.  You can use external circuitry to detect

;       if the output stops toggling for more than a few seconds, and use

;       it to reboot the computer.  Note that your circuit must look for

;       an AC signal, because the output could lock up high or low if the

;       the computer crashes.

;

; WATCHADDR=378

;

;       Colour Settings:

;

;       Note the defaults have been chosen for their relative luminances

;       and contrast, and you may find certain combinations may not be

;       visible.

;

;       Permissible TEXT Colours are:

;

;       BLACK, NAVY, GREEN, CYAN,     RED, MAGENTA, BROWN, SILVER,

;       GREY,  BLUE, LIME, TURQUOISE, PINK, CERISE, YELLOW, WHITE

;

;       Permissible BACKGROUND colours are:

;

;       BLACK, NAVY, GREEN, CYAN, RED, MAGENTA, BROWN, SILVER

;

;

;       Top status bar background colour

;

TopWinBgColor=CYAN

;

;       Top status bar text colour

;

TopWinTxtColor=BLACK

;

;       Main window background colour

;

MidWinBgColor=BLACK

;

;       Main window text colour

;

MidWinTxtColor=WHITE

;

;       Command line background colour

;

CmdWinBgColor=NAVY

;

;       Command line text colour

;

CmdWinTxtColor=YELLOW

;

;       Bottom menu bar background colour

;

BotWinBgColor=CYAN

;

;       Bottom menu bar text colour

;

BotWinTxtColor=BLACK

;

;       Colour for displaying outgoing (transmitted) data

;

TxColor=RED

;

;       Colour for displaying incoming (received) data

;

RxColor=GREEN

;

;       Colour used for echoing Sysop's commands to main window.

;

EchoColor=YELLOW

;

;       Screen saver interval in seconds. (0 = disable screen saver)

;

SAVER=300

;

;       In the following section there is no limit on the number of

;       lines of text you can specify, but no line must exceed 255

;       characters.  The end of text is marked by *** on a line by itself.

;

;       This text is sent to an incoming caller.  CTFLAGS controls which

;       callers receive the text.

;

CTEXT

Kidderminster spur AX25/IP Router.

Type ? for list of commands.

***

;

;       This text is the response to the plain 'I' command.

;       Note that you may create info "topics" which the user can access

;       using the "I <topic>" command.  Each topic should occupy a

;       seperate, appropriately named file in a directory named "INFO",

;       and the filenames must have the extension ".INF"

;

INFOTEXT

You are connected to part of the Kidderminster Network Node system,

run on behalf of Fourpak by Paula G8PZT.

To access the local Bulletin Board, use the command "BBS".

For information on Fourpak, contact G4FPV @ GB7GLO, or see the web site

at www.g8pzt.pwp.blueyonder.co.uk/

***

;

;       This ID message is sent every IDINTERVAL.

;       Only the first line is sent.

;       If your APRS-format static position code is included, starting

;       within the first 40 characters, you will be visible on APRS maps

;       and the MHeard function will record distances to heard stations.

;       The format is "!ddmm.mmN/dddmm.mmE#" where dd represents degrees

;       of latitude/longitude and mm.mm represents minutes to two decimal

;       places. "N" and "E" may be replaced by "S" and "W" as appropriate.

;

IDTEXT

!5224.00N/00215.00W# Kidderminster Router (KDRMIN) 44.131.91.4, Chat=KDRCHT:G8PZT-8

***

;

;       CTFLAGS controls which connects receive CTEXT.

;       Add together the following numbers:

;

;               1       Send ctext if connect is to Node/port alias

;               2       Send ctext if call is to Node/port call

;               4       Send ctext on L4 connects.

;               8       Send ctext to TCP (TELNET) callers.

;

;       Default is 9 (Alias and TCP only).

;

CTFLAGS=1

;

;       You may have up to 5 "virtual" consoles, upon which the sysop

;       may conduct independent sessions.  Consoles are selected by

;       using Alt-1 through alt-5, or the left and right arrow keys.

;

NUMCONSOLES=3           ; No. of virtual consoles (max=5)

;

;       Usage Log:

;       Setting LOG=1 will log all connects, disconnects, user-entered

;       commands and chat server activity. LOG=0 disables this function.

;       Make sure you have enough disk space.  Not recommended for long

;       term use on floppy-based machines.  Can be overridden by LOG

;       command at the command line.

;

LOG=0

;

;       Overall limit on no. of concurrent sessions per user, across

;       all ports.  You might like to restrict "troublesome" users

;       this way! Max setting = default = 255

;

SESSLIMIT=255

;

;       L4 PARAMETERS

;       =============

;

;       (Don't adjust these unles you *really* understand all the

;       implications.)

;

;       No. of seconds between L4 retries and L4 connect/disconnect

;       attempts.

;

L4TIMEOUT=90

;

;       No. of seconds to delay a L4 ack in case further frames arrive.

;       10 secs is probably OK on normal AX25 links, but is excessive

;       on wire links.  However, the system will attempt to adjust this

;       this value to cope with prevailing conditions.

;

L4DELAY=10

;

;       No. of unacked L4 frames allowed before we stop to await an

;       ack.

;

L4WINDOW=4

;

;       No. of L4 connect/disconnect or retransmission attempts before

;       link is abanodoned.

;

L4RETRIES=3

;

;       L3 PARAMETERS

;       =============

;

;       Obsolescence counter initial value

;

OBSINIT=5

;

;       Obsolescence counter minimum to broadcast

;

OBSMIN=3

;

NODESINTERVAL=60        ; Mins between nodes b/casts. (0 = disable)

L3TTL=25                ; Max L3 hops

;

;       If this is set to 1, nodes whose alias begins with "#" will

;       not be displayed.

;

HIDENODES=1

;

;       Minimum quality to add to node table.  This is the global value

;       which will apply to all ports unless overridden by a port minqual.

;       If not specified, the default is 10.

;

MINQUAL=10

;

;

;       Ax25 Level 2 Global Parameters

;

T3=180                  ; Link check interval (180 secs).

IDLETIME=900            ; Idle link shutdown timer (900 secs)

IDINTERVAL=15           ; Minutes between ID broadcasts (0=disable)

;

; =======================================================================

; Interface definitions - These MUST come before any port definitions

; =======================================================================

;

;       Unlike BPQ, you first define the interfaces with the outside world,

;       then you define the ports that use those interfaces.  This is

;       because some interfaces (e.g. ASYNC) can support several channels

;       by use of the appropriate protocol (e.g. KISS).  Whereas BPQ

;       requires you to specify IOADDR, INTNUM and SPEED in each port

;       definition (which is tedious and error-prone when several ports

;       share a single com port), in PZTNODE you only have to specify them

;       once, in the interface definition.  Each interface has a number

;       by which the ports identify it.  With this method you may easily

;       switch a whole group of ports onto a different com port if you get

;       a hardware breakdown for example.

;

;       TYPE:           ASYNC           ; Serial port

;                       LOOPBACK        ; For test purposes

;                       EXTERNAL        ; User supplied driver (as BPQ)

;                       BAYCOM          ; Baycom USCC card.

;                       RLC100          ; Thor RLC100 4 port SCC card

;

;       COM:            For ASYNC interfaces, this is the Com number (1 - 16)

;                       For SCC cards, it is the channel (1 - 4)

;

;       (For ASYNC interfaces, ioaddr and intnum are optional for com 1 - 4)

;       IOADDR          IO device address (hex)

;       INTNUM          Interrupt number (decimal)

;

;       SPEED           The serial port baud rate for ASYNC interfaces, or

;                       the radio baud rate for HDLC cards.

;                       50-115200 bauds (Decimal). Don't include a comma.

;                       If set to zero (HDLC cards only), the modem must

;                       supply a x1 clock (TXC on RTXC pin, RXC on TRXC pin).

;

;

;       PROTOCOL:       Protocol to use on the interface:

;                       NONE    - Use this with type=loopback.

;                       ASCII   - Remote consoles (TTY) via ASYNC ports

;                       SLIP    - For TCP/IP over wire

;                       KISS    - For driving KISS TNCs or wired links.

;                       NETROM  - Not yet implemented

;                       HDLC    - For use with SCC cards and some EXTERNAL

;                                 drivers.

;

;       KISSOPTIONS:    Options for KISS interfaces only...

;                       POLLED  - For TNCs which send only when polled by node.

;                       CHECKSUM - Packets protected by checksum.  You can

;                                  only use this option if your TNC supports it.

;                       SLAVE   - The node will act like a polled KISS TNC,

;                                 sending only when commanded to do so.

;                                 (Polled and slave are mutually exclusive)

;                       (BPQKISS eproms require POLLED and CHECKSUM.)

;

;       FLOW:           Flow control options (ASYNC interfaces only):

;                       0 = No flow control

;                       1 = Hardware (RTS/CTS) flow control

;                       2 = Software (XON/XOFF) flow control (TTY link only)

;                       3 = Hardware AND software flow control

;                       If not specified, flow control defaults to NONE.

;                       Don't use Xon/Xoff with KISS.

;

;       MTU:            Specifies largest IP packet.  Setting this over 256

;                       will crash most BPQ nodes, or at the very least cause

;                       the packet to be lost.

;                       The router can handle single frames up to 1500 bytes,

;                       but BPQKISS TNC's are limited by 340 byte buffers.

;                       Received IP datagrams larger than MTU are

;                       fragmented to suit the outgoing link MTU.

;

;       Each interface definition starts with INTERFACE= and ends with

;       ENDINTERFACE.  The number following INTERFACE= is merely a label

;       allowing ports to refer to the interface.  It must be specified,

;       but the numbers themselves, and the order in which they're defined

;       are not important.

;

INTERFACE=1

TYPE=ASYNC

COM=1

;       IOADDR=3F8

;       INTNUM=4

SPEED=9600

PROTOCOL=KISS

KISSOPTIONS=CHECKSUM

MTU=256

ENDINTERFACE

;

;

INTERFACE=2

TYPE=ASYNC

COM=2

SPEED=4800

PROTOCOL=KISS

KISSOPTIONS=CHECKSUM

MTU=256

ENDINTERFACE

;

;

;       Example "loopback" interface, allowing self-connects without going

;       via an external system

;

INTERFACE=3

TYPE=LOOPBACK

PROTOCOL=KISS

MTU=576

ENDINTERFACE

;

;       Example interface to be used with BPQ "external" driver such as

;       baycom or ethernet driver.

;

;INTERFACE=4

;       TYPE=EXTERNAL           ; BPQ-compatible "External" driver

;       PROTOCOL=HDLC

;       INTNUM=96

;ENDINTERFACE

;

;       Example of an interface for TTY (remote console).  You would

;       connect the com port via a null modem to a dumb terminal or

;       computer running a terminal emulator program, such as TELIX.

;

;INTERFACE=5

;       TYPE=ASYNC

;       COM=1

;       SPEED=19200

;       MTU=256

;       PROTOCOL=ASCII

;       Flow control is optional. Be VERY careful if you are monitoring

;       over a flow controlled link, because if you pause the display for

;       a long time, the data will back-up in the router until it becomes

;       strangled.

;       FLOW=2          ; Xon/xoff flow control

;ENDINTERFACE

;

;       Example of an interface for BAYCOM USCC card.  You require one

;       interface per channel on the card.  Use the same base address

;       for each channel.

;

;INTERFACE=6

;       TYPE=BAYCOM

;       COM=2           ; Channel B

;       IOADDR=300      ; Card base address=300H

;       INTNUM=5        ; Card IRQ=5

;       SPEED=0         ; DF9IC 1200/9600 NRZ modem

;       PROTOCOL=HDLC   ; Only HDLC supported at present

;       MTU=256

;ENDINTERFACE

;

;

; =====================================================================

; Port definitions. Each one begins with PORT=n and ends with ENDPORT

; =====================================================================

;

;       The number following PORT= is the port number as displayed by

;       the P[orts] command. They do not need to be sequential, you

;       may use any number, but you must define them in the order in

;       which they are to appear in the list.

;

PORT=1

;

;       Text string to identify port on "PORTS" display

;

ID=Link to KIDDER

;

;       Interfacenum specifies which interface this port should use.

;

INTERFACENUM=1

;

;       The rest of these are optional, since there are defaults

;       built into the program.

;

;       PORTCALL=G8PZT-1        ; Additional L2 callsign for port

;       PORTALIAS=PZT1          ; Additional L2 alias for this port

;       PORTALIAS2=RELAY        ; Yet another alias, for APRS

CHANNEL=A               ; Channel to use on interface (A - P)

;       PACLEN=160              ; Overrides global paclen for this port

MAXFRAME=2              ; Only 1-7 at present.

;       TXDELAY=300             ; Tx keyup delay (millisecs)

;       TXTAIL=100              ; TX keydown delay (millisecs)

;                               ; Don't go less than 100 for SCC cards!

;       SLOTTIME=100            ; (millisecs)

;

;       I strongly recommend frack=7000 for 1200 bauds. (see manual)

;

FRACK=7000              ; L2 T1 time (millisecs)

;

;       Resptime should be *at least* ((paclen * 10000) / RFbauds) millisecs,

;       where "paclen" is the other end's paclen, otherwise you will send

;       unnecessary poll frames. 1500 is OK for 1200 bauds with paclen=120

;

RESPTIME=1500           ; L2 delayed ack T2 (millisecs)

;

;       Persist should be set to (255 / (no. of users on frequency)).

;       e.g. for a frequency with an average of 10 users on at any one

;       time you'd set it to 25. Default is 64.

;

PERSIST=64              ; Probability to transmit (0-255)

;

;       I can't see any point in setting retries more than 10, other than

;       for test purposes.  If you need so many retries it's a useless

;       link and you're just wasting everyone elses airtime.  The higher

;       you set this value, the longer users will have to wait to get

;       a "failure with" for a non-contactable destination.

;

RETRIES=10

;

;       If you set fulldup=1, the router will transmit whenever it needs

;       to, without waiting for the other end to stop.  Used only for

;       hardware which is capable of simultaneous transmission and

;       reception, such as full duplex radio or wire links.

;

FULLDUP=0               ; 1 = Full duplex, 0 = simplex/half duplex

;

;       Softdcd is used only by SCC cards.  If set to non-zero, the real

;       dcd will be ignored, and the driver will use the presence of

;       hdlc data as a DCD indication.

;

;       SOFTDCD=0

;

;       Rfbauds defaults to 1200 if not specified.  It is intended for use

;       with "real" tnc's attached via RS232, because the RF baud rate is

;       usually different to the serial baud rate.  It simply helps the

;       node make timing decisions.

;

;       RFBAUDS=2400

;

;       L3ONLY=0                ; 1 = Ban L2 downlinks on this port.

;

;       Enable/disable the MHEARD function on this port. The number

;       specifies how many callsigns to maintain in the list. Set to 0

;       to disable MHEARD.

;

MHEARD=10               ; Mheard enabled, 10 calls max

;       MHFLAGS controls which callsigns are recorded in the MH list,

;       and defaults to 255 (show everything).

;       The number is formed by adding the following values:

;

;               1       Show directly heard stations

;               2       Show directly heard digipeaters

;               4       Show digipeated stations

;

;

;       Digipeat flag for this port. 0 = no digipeating, 1 = digipeat UI

;       frames only, 255 = digipeat everything.

;

DIGIFLAG=255            ; Digipeat everything.

;

;       DIGIPORT=0              ; Port to relay digipeated frames on

;                               ; (0=this port)

;

;       List of destinations for "broadcasting".

;       Received non-digipeater UI frames, addressed to one of these

;       destinations, will be re-broadcasted on all ports which have a

;       matching address in their BCAST list.  This would for example be

;       used to broadcast mail beacons from a BBS onto several frequencies.

;

;       BCAST=MAIL,ALL

;

;       List of approved broadcasters.

;       If you wish to restrict the broadcast facility to certain senders

;       only, list the callsigns here.  If no calls are specified, the

;       facility is unrestricted.  Separate the calls by commas, and don't

;       include any spaces in the list.

;

;       BCFROM=GB7PZT,GB7MAX

;

;       Default quality for nodes whose broadcasts are received on this

;       port. Set to 0 to disable all L3/4 activity on this port.

;

QUALITY=10

;

;       Minimum quality to add to node table for nodes received via this

;       port.  If specified, this overrides the global minqual, and can

;       be used to exclude unreachable and marginal nodes.

;

;       MINQUAL=10

;

;       Port to transmit on. 0=this port.  You would typically use this

;       where a single transmitter is used in conjunction with several

;       receivers

;

;       TXPORT=0

;

;       Interlock is only used by SCC cards - KISS TNC's make their own

;       decisions when to transmit, and the node has no control over

;       that process.  If a non-zero value is specified, no two ports

;       with the same value will transmit at the same time.

;

;       INTERLOCK=4

;

;       Maximum no. of simultaneous users on this port. Default is 255

;       which means "no limit".

;

USERS=255

;

;       Sesslimit specifies the maximum simultaneous connects each user

;       is allowed.  You may wish to set a low value if you are a

;       control freak.

;

;       SESSLIMIT=5

;

;       The following two commands are mutually exclusive... Use one or

;       the other, but not both!

;       Callsigns should be seperated by commas or spaces, and there is no

;       limit to the number of calls. You can have multiple validcalls= or

;       exclude= lines, but you should be aware that, depending on your

;       hardware, there may be a performance penalty if you use long lists.

;

;       Validcalls allows L2 frames only from the specified users, and is

;       typically used to keep users from connecting to link-only ports.

;

;       VALIDCALLS=G6YAK,G6AMU  ; Accept only these users

;

;       Exclude allows L2 frames from everyone EXCEPT specified users,

;       and would typically be used on a user-access port to prevent

;       connections from trouble-makers and pirates.  Just because I

;       provide this command it doesn't mean I condone its use.

;

;       EXCLUDE=NOCALL,P1RAT     ; Ignore these users

;

CWID=G8PZT              ; Used only by SCC cards.

; Callsign is sent every 30 mins.

;

;       PIPE allows frames received (and optionally sent) on this port to

;       be copied to another port.  A typical use would be to allow a PMS

;       connected to one port to see the traffic on another port.  A

;       pipe is one way - in order to have two way traffic you must set up

;       a reverse pipe on the other port.  You may pipe several ports to

;       a single destination port, but you can only have one *outgoing* pipe

;       from any port.  This feature is capable of generating an immense

;       amount of traffic, so use it with care - your target port MUST be

;       capable of dealing with the traffic load.

;       To disable piping, set PIPE=0 or just omit the command.

;

;       PIPE=2                  ; Pipe frames from this port to port 2

;

;       PIPEFLAG is only used when piping is active, and controls which

;       frames are piped.  The default if not specified is 3.  The value

;       is made up by adding together the following numbers:

;

;               1       - UI frames *not* addressed to nodecall/alias.

;               2       - Non-UI frames *not* addressed to nodecall/alias.

;               4       - UI frames addressed to nodecall/alias.

;               8       - Non-UI frames addressed to nodecall/alias.

;               16      - UI frames transmitted by node.

;               32      - Non-UI frames transmitted by node.

;               64      - Allow budlisted users to be piped.

;               128     - Netrom frames

;               256     - IP / ARP frames

;

;       PIPEFLAG=5              ; Pipe all rcvd UI frames only

;

;       UNPROTO=CQ,G6YAK

;

;       Optional alternative IP address for use on this port.  If this

;       is specified, it will be used instead of the global IP address

;       for this port only.

;

;       IPADDR=44.131.91.5

;

;       If you set SYSOP=1, all users who connect on this port will get

;       full sysop status without answering a password challenge.  This

;       is intended ONLY FOR USE ON SECURE LINKS, such as RS232 or

;       Ethernet.  Be aware that, if the remote system is capable of

;       gatewaying or digipeating, users could downlink via the remote

;       system back into this port, thus gaining sysop status.  The default

;       for this parameter is of course zero!!

;

;       SYSOP=0

;

;       The APPLMASK parameter is used only if you are running applications

;       via the host interface.  It specifies which applications will be

;       directly connectible on this port.  Default is 255, which allows

;       all applications.  The value is made up by adding together the

;       following numbers:

;

;               1       - Enable Application 1

;               2       - Enable Application 2

;               4       - Enable Application 3

;               8       - Enable Application 4

;               16      - Enable Application 5

;               32      - Enable Application 6

;               64      - Enable Application 7

;               128     - Enable Application 8

;

;       If you want an application to be directly connectible on a port,

;       it must have a callsign, an alias or both, and the corresponding

;       bit in that port's applmask must be set.

;

;       APPLMASK=3

;

ENDPORT

;

PORT=2

ID=Link to BRUM

INTERFACENUM=2

CHANNEL=M

FRACK=7000

RESPTIME=1500

MHEARD=10

QUALITY=100

TXDELAY=500     ; Synthesiser is slow to lock!

ENDPORT

;

PORT=3

ID=Internal Loopback

INTERFACENUM=3

ENDPORT

;

;PORT=4

;       ID=External loopback

;       INTERFACENUM=4

;ENDPORT

;

; ====================================================================

;       Applications. Each must begin with APPL= and end with ENDAPPL

;       Application number must be between 1 and 8

;       Each field is optional - you may have an applname without a call

APPL=1

APPLNAME=PMS

APPLCALL=G8PZT-2

APPLALIAS=PZTPMS

APPLQUAL=50             ; Netrom quality to broadcast

ENDAPPL

;

;       In this example, the application has no callsigns or quality, so

;       it can only be reached by issuing the command "HOST" during a

;       command session.

APPL=2

APPLNAME=HOST

ENDAPPL

;

; ===================================================================

ROUTES

;

;       Routes to lock in - Syntax is as follows:

;

;       <callsign> <port> <quality> [! [maxframe [frack [paclen]]]]

;

;       You must specify at least Callsign, port and quality.  If you

;       include the lock flag ( ! ) the route will be locked in, and

;       will only be changed by a replacement entry with the lock flag

;       set. If you don't include the lock flag, the route will

;       eventually expire if not confirmed by the reception of nodes

;       broadcasts.  In either case, if the file PZTNODES is present,

;       its contents will override the entries here, subject to the

;       locking rules above, and the sysop may also edit routes while

;       the node is running.

;       Maxframe, frack and paclen are optional.  If specified they

;       override port defaults for that route.

;       Note FRACK is expressed in millisecs, e.g. 7000 = 7 secs.

;

;       End the section with *** as usual.

;

g6yak 1 100 !

***

;

;       Sysop-defined commands:

;

;       Up to 8 commands can be defined, allowing the sysop to set up

;       single-word aliases for frequently used command strings.  For

;       example you might wish to set up BBS, CONV and DXCLUSTER to

;       point to local systems.

;

;       Each command is defined using a seperate "COMMAND=" string,

;       and the arguments are <alias> <real_cmd>.

;       e.g. "COMMAND=BBS C 7 GB7PZT" means "create a new command

;       called BBS, which translates to the sequence C 7 GB7PZT"

;

COMMAND=BBS C 1 GB7PZT

COMMAND=CONV TELNET 44.131.90.1 3600

COMMAND=DXCLUSTER C GB7DXC

;

PZTNODES

~~~~~~~~

This file is identical in format and function to the BPQNODES file

used by BPQ for node & route table saving and recovery, except that it

doesn't allow digipeaters in ROUTE entries.

The nodes and routes tables are saved to this file every

NODESINTERVAL, at closedown, and whenever the SAVENODES command is

used (unless a different filename is specified).

The ROUTE entries are first, and have the following format:

ROUTE ADD <call> <port> <quality> [!] [<maxframe> <frack> <paclen>]

Where <call> is the callsign of the neighbour node, <port> is the port

upon which it is reached, and <quality> is a figure between 0 and 255

indicating how "good" the route is. The remaining parameters are

optional.  "!" indicates a locked route (i.e. one which will not

expire), and the rest are "non-standard" values which can override the

port defaults for that route.

For example:

ROUTE ADD G8UZV 1 240 ! 5 7000 120 shows that

neighbour node G8UZV is locked in on port 1 with a quality of 240, and

will use maxframe=3, frack=7000, paclen=120.

Following the ROUTE entries, the remainder of the file consists of

NODE entries, one per line. The format is as follows:

NODE ADD <alias:call> <route1> <port1> <qual1> [!] [<route2> ... ]

Where <alias:call> is the alias and callsign of the distant node,

<route1> is the callsign of the primary route to that node, for which

there must be a route entry, <port1> is the port used to reach that

neighbour and <qual1> is the quality of that route. "!" indicates a

locked route.  There can be up to 3 different routes listed for each

node.

For example:

NODE ADD #TLFRD:GB7IPT-7 G8PZT 1 142 ! G8UZV 1 139

NODE ADD BRUM:GB7BM G8PZT 1 94 G8UZV 1 92

NODE ADD BUXTON:GB7DAD-8 G8PZT 1 22 G8UZV 1 21

SECTION 3 - COMMAND REFERENCE

=============================

COMMANDS AVAILABLE TO EVERYONE

==============================

The following user commands are available on G8PZT routers:

@        ?         Arp       Bye       CHat      Connect   Finger

Help     Info      IProutes  Links     MHeard    Nodes     Ports

PIng     Routes    Stats     Telnet    TTylink   Users     Version

Commands are not case-sensitive, and only the portion shown in UPPER-

CASE is required.

@        Request sysop status

Syntax:    @ [string]

In order to gain access to sensitive commands, remote sysops

must first complete a password challenge.  Firstly, the

remote sysop enters "@" alone, and the system replies with a

matrix consisting of 5 lines of 5 numbers thus:

4 1 6 3 7

3 5 2 6 3

7 1 9 2 4

2 7 1 4 6

3 5 2 6 1

The remote sysop must then choose ONE of the lines, and send

the "@" command again, followed by the 5 characters from the

password string which correspond to the 5 numbers on the

chosen line.  There must be a space after the "@" but no

spaces between the characters.

If the challenge is correctly answered, the system replies

with "Ok", and the user has remote sysop status.

If the user does not have a password registered in

PASSWORD.SYS, the response to this command will be "Bad

command".

Note:  Console sysops already have full access and don't need

to use this command.

?        Obtain command list or syntax help

Syntax:    ? [cmd]

If used without arguments, this command sends a list of the

basic commands available to the user.

If the argument is an asterisk (e.g. "? *") it lists all the

available commands.

Using the form "? cmd" shows the syntax for "cmd".

ARP      Display Address Resolution Protocol table

Syntax:    ARP [ LIST ]

The ARP command is used to display the Address Resolution

Table, which is responsible for mapping IP addresses to

callsigns.

The LIST subcommand is available to everyone, and has the

same effect as using ARP by itself.  Other subcommands are

available to sysops only (see sysop command section).  The

response may look like this:

G8PZT:KIDDER}

Host             type        Hw Address

44.131.74.131    AX25        GB7WV-12

44.131.90.6      AX25        GB7IPT-9

44.131.95.240    AX25        GB7GH

44.131.93.240    AX25        G4FPV

44.131.91.245    AX25        G8PZT

44.131.91.4      AX25        G8UZV

44.131.91.2      AX25        GB7PZT-5

44.131.91.66     AX25        G8BKL-5

44.131.136.10    AX25        G1LOA-10

44.131.138.10    AX25        GB7MIP-8

(End of list)

<host> is the neighbour's IP address, <type> is either "ax25"

or "ether" (Ethernet) and <hw_address> is the hardware

address or callsign of the neighbour.

In order for this command to have any meaning, the router

must have an IP address and be connected to an IP-capable

network.

BYE      Disconnect from the router.

This command forces a disconnection, and is useful when the

user does not have easy access to a TNC's "command mode", or

when the uplink is from another node with the "stay" option

enabled.

Disconnection of an uplink terminates all dependent sessions.

CHAT     Connect to the chat server.

Syntax: CH[AT]

The CHAT command connects the user to the integral chat

server, which has its own set of commands detailed in a

separate section.

The chat server may also be reached directly by telnetting to

the router's TCP port 3600.

CONNECT  -- Make an outgoing AX25 connection

Syntax: C[ONNECT] [port] <call> [V[ia] digi[,digi...]] [S]

The CONNECT command, which may be abbreviated to "C",

instructs the router to make an outgoing (downlink) AX25

level 2 or 4 connection with another system.

If the target is a known node (i.e. one which is in the nodes

table) a port number is not required, and will be ignored if

supplied.  The router will attempt to make a level 4

connection with the target, using information from the

routing tables.  However, a level 2 connection with an

immediately adjacent node may be forced by using the target's

alias with an arbitrary SSID, specifying a port number if

required (e.g. C 4 MLVN-1)

If the target is not a known node, the router will attempt to

make a level 2 connection.  On multi-port systems, a port

number must be specified.

The "V" (via) parameter allows up to 7 digipeaters to be

specified, e.g.:  "C 3 G6YAK V G8NTU G8EPR"

The "S" (stay) parameter, e.g. "C <nodecall> S" causes the

uplink session to stay connected when the downlink session to

the target node is terminated.

See also:  TELNET (nake TCP connections)

FINGER -- Display information about users.

Syntax:    FINGER <user | user@host | @host>

If the command is of the form "FINGER <user>", the router

searches the FINGER sub-directory for a text file which

matches "user", and sends the contents of that file if it

exists.  The file may contain anything you like, and "user"

may be a callsign, nickname or other form of hostname

consisting of up to 8 legal DOS characters

If the form "user@host" or "@host" is used, the router will

attempt to resolve "host" into an IP address and establish a

TCP/IP contact with the finger server on that host.

Examples:  FINGER g8pzt        Info on local user g8pzt

FINGER g8jvm@iptlfd

Limitations:  This feature is very rudimentary at present,

requiring the user account files to be created

by the sysop.

HELP      Displays help for commands and other topics

Syntax:    H[ELP] [cmd | topic]

If no arguments are given, a short text gives directions on

how to access help.

If the argument is a topic or a command name, the contents of

the appropriate file in the HELP sub-directory are displayed.

The argument "*" will list all the help topics available.

Example:   H PING    will give help for the PING command.

Limitations:  The command or topic name must at present be

given in full, thus "H NODES" is acceptable,

but "H N" is not (unless you duplicate the

NODES.HLP file to N.HLP).  Future versions will

search for the closest match.

Note: The help available using the HELP command is intended

to be of use to both sysops and users alike, and is

kept brief in order to avoid wasting airtime.  The MAN

system gives more detailed information for sysops.

The sysop may customise the help files, or add his/her

own help topics to the HELP directory as required.  The

HELP directory is located beneath the router's working

directory, and the files are simply plain text files

with the .HLP extension.  Files without the .HLP

extension will not be listed or accessible.

See also:  ?, MAN, INFO

INFO     Displays information about the node and other topics.

Syntax:    INFO [topic]

If no arguments are given, the INFOMSG text is sent to the

user.

If the sysop has created other INFO topics, the user will

then be prompted to enter "INFO *" to display the list of

topics.

If [topic] is specified, the contents of the appropriate .INF

file, if it exists, are sent to the user.

Example:   I FOURPAK      Displays fourpak.inf file.

Note: The sysop may create INFO topics as required, and

there is no need to restart in order to activate them.

Each topic should be created as a plain text file with

the .INF extension and should be placed in the INFO

sub-directory located immediately under the router's

working directory.

Files without the .INF extension will not be listed or

accessible.

Limitations:  Topic names must consist of a maximum of 8

legal DOS characters.

See also:  ?,  HELP,  MAN

IPROUTE -- Display IP routing table.

Syntax: IPR[OUTE]

The IPROUTE command, which may be abbreviated to IPR,

displays the contents of the table responsible for routing of

IP datagrams.

This table is initialised from file IPROUTE.SYS when the

router is started, and may contain other entries "learned" by

the system, or entered by the sysop.  It is not required in

any way for normal AX25 and NETROM activities.

For each route it displays the IP address, the subnet mask,

the gateway address, the port and the mode (Datagram, VC or

Netrom).

The command "IP ROUTES" produces an identical result.

LINKS -- Displays the currently active level 2 sessions.

This lists level 2 user up/downlinks and inter-node links.

It is mainly of interest to sysops, and shows the callsigns

being used at both ends of the link plus some other data, for

example:

G8PZT:KIDDER} Links:

Remote Local  Prt Sta Ver Try  T3   Pac  Max  T/o  Txq Rxq Asm

G4FPV  G8PZT   5   5   2   0   177  160   1   904    0   0   0

GB7PZT G8PZT   7   5   2   0   181  240   7   908    0   0   0

GB7GH  G8PZT   9   5   2   0   176  120   1   903    3   1   0

(End of list)

Remote and Local are the L2 callsigns in use at either end of

the link.

Prt   - Port number

Sta   - Link state (2 = connecting, 4 = disconnecting,

5 = connected)

Ver   - AX25 version number

Try   - Retry count

T3    - Current state of the LAPB T3 countdown

Pac   - Paclen

Max   - Maxframe

T/o   - Timeout

Txq   - No. of L2 frames queued for transmission

Rxq   - No. of L2 frames on receive queue

Asm   - No. of frame fragments awaiting reassembly

Note: The exact format of this response may differ in future

versions.

MHEARD -- List recently heard stations.

Syntax:    MH[EARD] <portnum>

If the facility is enabled on the specified port, the MHEARD

command lists the most recently heard stations on that port,

along with the date / time of reception, and the number of

frames heard.

This is useful for users to discover who else the router can

hear, to aid the search for suitable digipeaters, and to

diagnose problems.  Even on linking-only ports, where there

is only usually one partner, it provides a useful indication

when the frequency is being encroached, either by deliberate

squatting, unauthorised attempts to link, or lift conditions.

If you have included an APRS-style position report in your ID

beacon, the router will know its own position and will

display position, distance and bearing for any stations

which broadcast APRS positions.

The command may be abbreviated to "MH".

Example: MH 3 gives a heard list for port 3....

G8PZT:KIDDER} Heard list for port 3:

Callsign Last-Date/Time  Frames

G1LOA-10     09/06 13:42   309        NODE

G3TQG-2    * 09/06 13:19   599

GB7PZT-15  * 09/06 13:18   708

DY25         09/06 10:19   4

An asterisk (*) indicates a digipeated signal.  The far right

column indicates the reception of Netrom, IP and ARP frames.

Note: For each port, the MH facility can be enabled /

disabled and the maximum length of the list specified

by appropriate entries in the .CFG file.

NODES -- Display contents of the Nodes table.

Syntax:  N[odes] [call | * | T]

When used without arguments, this command lists all the

NetRom nodes (but not KA nodes) known to the router, except

those "hidden" nodes whose alias begins with the hash (#)

character.

If the argument is an asterisk (*), all nodes, including

"hidden" nodes will be displayed.

If the argument is a known node call or alias, the preferred

route to the specified node, and up to two alternative routes

will be displayed.  Example: N MLVN  The response looks like

this:

G8PZT:KIDDER} Routes to: MLVN:G4FPV  RTT=28  FR=3538  Q=0

> 150  5  5 G4FPV

110  5  9 GB7GH

0  4  2 G1DKI-7

RTT stands for Round Trip Time and is a running average of

the time taken to get a response from that node.

FR indicates the number of level 3 frames sent to that node.

Q is the number of Level 3 frames currently queued for that

destination.

A chevron ">" in the left-most column indicates the currently

active route.

The first number shows the overall path quality to the target

node, the second is the "obsolescence count", and the third

number is the port designator.  The obsolescence count is a

measure of how recently the route was heard about or used.

It is usually reset to 5 upon hearing a nodes broadcast from

the neighbour node, and decrements by one every time this

node makes a node broadcast (typically once per hour). If it

drops below 4 the route is considered to be obsolete.

If the requested nodecall or alias is not in the table, an

error message results.

Entering  N T  will list the round trip times and frame

counts for all nodes which have non-zero figures, producing a

display like this:

G8PZT:KIDDER} Nodes:

WV12:GB7WV-12     RTT=20  FR=255

WV13:GB7WV-13     RTT=14  FR=26  Q=2

WV22:GB7WV-2      RTT=14  FR=31

Examples:

N           - List nodes except those beginning with #

N *         - List nodes including those beginning with #

N MLVN      - Display routes to MLVN node

N T         - List all nodes with non-zero RTT and FR counts

PING -- Send ICMP echo request(s).

Syntax:  PING <hostname | ipaddr> [length [interval]]

Sends ICMP echo request(s) to the specified IP address or

hostname for the purposes of route testing.

An optional data portion of "length" bytes may be specified,

and the echo request may optionally be repeated every

"interval" seconds.

If there is a reply it will be displayed.  For repeating

pings the system displays the number sent/rcvd, the average

round trip time in milliseconds, and the success rate.  The

"wait for reply" process may be cancelled at any time by

entering <CR> by itself.

If you specify a hostname (e.g. gb7pzt.ampr.org) instead of a

numeric IP address the request may take longer to action if

the hostname isn't found in DOMAIN.SYS, because the name will

have to be resolved by sending a DNS request.

Examples:

PING 44.131.91.2           Single ping of minimum size

PING 44.131.91.2 50        Single ping with 50 bytes data

PING gb7pzt                Uses DNS to resolve host.

PING 44.131.91.2 512 10    Ping 512 bytes every 10 secs

The response for a single ping looks like this:

G8PZT:KIDDER} PING: Pinging 44.131.91.2: hit <RETURN> to

quit...

44.131.91.2: echo reply - rtt 495 msec

And for a repeating ping it looks like this:

G8PZT:KIDDER} PING:

Target             Interval   Sent   Rcvd   %  Ave Rtt

44.131.91.2            9955      1      1  100  880

44.131.91.2            9955      2      2  100  880

44.131.91.2            9955      3      3  100  880

Limitations: The ROUTER must have an IP address and have IP

routing defined for this command to work.

Unrealistic ping rates are prevented.

Availability: The PING command is currently available to all

users, but due to the potential for abuse the

repeating ping option may be made sysop-only in

future versions.

PORTS -- List the available ports.

Syntax:  P[ORTS]

Displays the router's port numbers along with their brief

descriptions as specified by the PORTID fields in the CFG

file.

The command may be abbreviated to "P".

ROUTES -- List neighbouring nodes which can be heard directly.

Syntax:  R[OUTES] [*]

The ROUTES command, which may be abbreviated to "R", lists

the immediately adjacent NetRom nodes, i.e. those who can be

heard directly, providing those nodes are making NetRom nodes

broadcasts.

For each neighbour node the display shows the port number,

the neighbour's callsign, the route quality, and the number

of nodes accessible through that neighbour.  A chevron (>) in

the left-most column indicates a route which is in use, and

an exclamation mark (!) in the right-most column indicates

that the data has been "locked in" by the sysop. See

example...

If any argument is supplied, it will give additional

information mainly of interest to sysops.  The additional

fields are the current MAXFRAME, FRACK and PACLEN settings,

the number of information frames sent, the number of

information frames re-sent, the retry rate, which is the

ratio of the two preceding figures (or * if both are zero),

and the time a nodes broadcast was last heard from the

neighbour.

Examples:

If no arguments are supplied, the output looks like this:

G8PZT:KIDDER} Routes:

Port Callsign  Qty Nod

>  5 G4FPV     150  70!

>  7 GB7PZT    250   1!

>  8 GB7WV-12  100  32!

>  9 GB7GH     150 104!

10 GB7CL     150   1!

> 11 GB7IPT-7  150   3!

12 G1LOA-10  150   2!

The R R form produces an output similar to this:

G8PZT:KIDDER} Routes:

Port Callsign Qty Nod Max Frack Pac  Sent Resent Rty%  Last Heard

>  5 G4FPV    150  70! 5   7000 160 15550   1507   9% 09/06 13:46

>  7 GB7PZT   250   1! 7   5000 240 25387      4   0% 09/06 13:47

>  8 GB7WV-12 100  32! 1   4000 120 12170   3426  28% 09/06 13:35

>  9 GB7GH    150 104! 4   7000 120  7442   1335  17% 09/06 13:47

10 GB7CL    150   1! 3   7000 240   330      1   0% 09/06 13:23

> 11 GB7IPT-7 150   3! 2   7000 120 19401   2928  15% 09/06 13:47

12 G1LOA-10 150   2! 4   7000 120 10745   3379  31% 09/06 13:39

STATS -- Display router statistics.

Syntax:    S[TATS] [*]

Displays information about the performance of the router,

such as the uptime, the no. of packets routed, error rates

etc.  See elsewhere in this manual for a detailed explanation

of the output.

Entering S by itself will give a single page synopsis,

whereas "S *" will produce the full stats.

Note: On multi-port systems the display may wrap, and on busy

systems some of the fields may run together.  The exact

format of the display is not yet finalised and the

problems will be addressed in future versions.

TELNET -- Establish a TCP "connection".

Syntax:    TEL[NET] <hostname | ipaddress> [port]

The TELNET command allows users to "connect" to other TCP/IP

systems, using a "shell" account, i.e. the user does not need

to be running TCP/IP as the router does all the TCP/IP <>

AX25 translation.

The optional <port> parameter specifies the desired service

on the target host.  If not supplied, the default is 23, i.e.

the Telnet (remote login) port.  Common port numbers are 21

(FTP), 23 (Rlogin), 25 (SMTP), and 87 (TTYLINK).

Examples:  TEL 44.131.90.6  21  - Connect to gb7ipt FTP.

TEL gb7lgs.ampr.org  - Remote login to gb7lgs

TEL gb7pzt 25        - Connect to gb7pzt SMTP

Limitations: This command will only work if the router has an

IP address and IP routing has been defined.  It

should be obvious that the router also needs to

be connected to an IP-capable network!

Notes: Specifying target hosts by their IP addresses will

often result in faster connection if the hostname is

not in domain.sys, as it will not have to wait for DNS

resolution.

See also: PING

TTYLINK -- Chat directly to another TCP/IP station

Syntax:  TT[YLINK] <hostname | ipaddress> [port]

The TTYLINK command allows users of any type (i.e. AX25,

console, wire link, TCP/IP) to chat directly to TCP/IP users.

This is similar to TELNET, except that the default port

number is 87, which is reserved for keyboard to keyboard

chat.  Other port numbers may be specified.

Examples:  TTY 44.131.91.4

TTY g8pzt

Limitations: In order for this command to work, the router

must have an IP address and have IP routing

defined.

Note: If the target hostname is not known to the router, it

will be more efficient to specify the target as an IP

address, since this will not require DNS resolution.

See also:  TELNET

USERS Shows who is using the router.

Syntax: U[SERS]

The USERS command displays the circuits which originate or

terminate at the router.  "Through" connections are not

shown.

No arguments are required, and a typical response is as

follows:

G8PZT:KIDDER} Users:

Session Start time  S Uplink type/user

807  09/06 13:25  1 L4  (G8PZT@GB7PZT) <--> (G6YAK-1) L2

UPLINK is an incoming connection, and DOWNLINK is an outgoing

connection.  The type may be L2 (ax25 level 2), L4 (ax25

level 4), TCP (TCP/IP), CON (console i.e. sysop), or TTY

(Teletype, i.e. an ASCII wire link).

Established circuits are shown by <-->  and circuits being

set up are shown thus:  <~~>.

SESSION is a unique session number for the circuit.

For TCP/IP connections the IP address and TCP port number are

displayed in place of callsign - this may be subject to

future review if I decide to make TCP users supply a callsign

at login.

Unlike the equivalent BPQ command, this does not show the

version number - use the VERSION command for that, or buffer

count (this system does not use fixed buffers).

VERSION  Display software version.

Syntax: V[ERSION]

The VERSION command displays the router version, author and

compilation date.  You should quote the version number when

reporting any bugs.

CHAT SERVER COMMANDS

====================

The following commands are available within the chat server only.

/?       /BYE      /CHANNEL  /EXIT   /HELP     /JOIN     /LEAVE

/LINKS   /MSG      /NAME     /PERSONAL /QUIT   /TOPIC    /USER

/WHO     /VERSION

CHAT SERVER COMMANDS IN DETAIL

==============================

Note that the server is still experimental, and the syntax of these

commands may be changed in future versions.  Additional commands will

be added as required.

/?       Display commands / syntax help.

Syntax: /? [cmd]

When used without arguments, the /? command lists the

available commands.  The syntax for any of the commands thus

listed may be shown by specifying the command as an argument

to the /? command.

Examples:  /?          List available commands.

/? /who     Display syntax for the /WHO command.

/BYE    Exit the chat server.

Syntax: /B[ye]

The /BYE command, which may be shortened to /B, disconnects

the user from the chat server, and informs everyone that he's

left.  There is no need for the user to /leave any logged

channels before issuing this command.

If the user accessed the server via the router's CHAT

command, he will be returned to the router's main command

prompt, otherwise he will be completely disconnected.

The /EXIT and /QUIT commands also perform this function.

/CHANNEL  Display / Change logged channel(s).

Syntax: /C[hannel] [number]

The /CHANNEL command displays / changes the channel(s) the

user is logged to.  When no argument is supplied, the logged

channel(s) is / are displayed.  If a valid numeric argument

is supplied, the user is logged to the specified channel.

Examples: /C        Displays current logged channel(s)

/C 22     Change to channel 22

When a new channel is selected, the user remains logged to

any previous channels, (so he can "monitor" several channels

at once) but any subsequent text he sends will go to the new

channel (unless targeted otherwise).

Note: Channels 0 to 255 are "local" to each chat server, and

channels 256-32767 are "global", i.e. they are linked with

all other servers.

The /JOIN command has a similar function, and /LEAVE is used

to de-select unwanted channels.

/EXIT    Exit the chat server.

Syntax: /E[xit]

The /EXIT command, which may be shortened to /E, disconnects

the user from the chat server, and informs everyone that he's

left.  There is no need for the user to /leave any logged

channels before issuing this command.

If the user accessed the server via the router's CHAT

command, he will be returned to the router's main command

prompt, otherwise he will be completely disconnected.

The /BYE and /QUIT commands also perform this function.

/HELP    Obtain help.

Syntax: /HELP [topic]

When used without arguments, the /HELP command gives brief

instruction on how to access various levels of help.

If a topic is specified, detailed help for that topic (if

available) is displayed.  The topic may be a command name, or

any other chat server related topic.  A list of the available

help topics can be obtained by specifying "*" as a topic.

Examples:  /H          Display general instructions.

/H *        List available help topics.

/H /who     Display help for /WHO command.

Note:  When using /H to display help for a command, the

leading slash for that command may be omitted.  Thus

"/H /who" and "/H who" are equally permissible.

/JOIN    Join (log onto) a channel.

Syntax: /J[oin] <channel>

The /JOIN command logs the user to a channel, and performs a

similar function to the /CHANNEL command.

When a new channel is selected, the user remains logged to

any previous channels, (so he can "monitor" several channels

at once) but any subsequent text he sends will go to the new

channel (unless targeted otherwise).  (Unwanted channels may

be de-selected using the complementary /LEAVE command.)

Example:  /J 22   Join channel 22

Note: Channels 0 to 255 are "local" to each chat server, and

channels 256-32767 are "global", i.e. they are linked with

all other servers.

/LEAVE   Leave (log off) a channel.

Syntax: /L[eave] <channel>

The /LEAVE command logs the user off the specified channel.

When a user joins a channel, he remains logged to any

previous channels, so this command allows him to de-select

unwanted channels.

Example:  /L 22   - Leave channel 22

/MSG     Send a short message to a channel or a single user.

Syntax: /M[sg] <channel | callsign> <text>

The /Msg command is used to send a short message (70 chars

max.) to any specified channel or single user.  You may for

example use this command to direct a message to a channel you

are monitoring, but not actually logged to.

If you direct a message to a specific user, he may be on this

or any other chat server.  The private nature of the message

will be indicated to the recipient by asterisks around the

sender's call, e.g. <*g8pzt@kdchat*> (Paula): Meet me on

channel 69.

Examples: /M 32 Hello People

/M g6yak Meet me on channel 69

The first form sends "Hello People" to all the users of

channel 32, and the second form sends a private msg to g6yak

only. Providing G6YAK is logged on to any chat server, the

message will find him.

Note: As with all things Packet, the term "private" is

relative, as nothing is truly private when it is broadcast!

/NAME    Set name.

Syntax:  /N[ame] <your name> [channel]

The /NAME command sets the user's name, which will be

displayed on the user list and prefixed to everything he

sends to others.  Users are not allowed to join any channels

until they have supplied a name (12 chars max), so it acts as

a "log on" command.  The name need be supplied only at the

initial logon, and may be changed as the user wishes.

On the first use of this command, the user may optionally

specify a channel to join instead of the default (channel 0).

TCP/IP users must first use the /USER command (see below) to

enter their callsign.

Examples: /N Paula       Set name to "Paula"

/N Paula 23    Set name and join channel 23

/PERSONAL  Display / change personal description.

Syntax: /P[ersonal] [text | @]

The /PERSONAL command is used to display or change the user's

personal description.  This is a short text of up to 32

characters, which is displayed on the user list.  It may

typically contain the user's home town and "brag"

information.  If the user logs onto any "public" channels

(i.e. those above channel 255), this information will appear

on the user lists of all other chat servers.

If used without arguments, the /PERSONAL command displays the

user's current text.

If the argument is "@", the existing text is removed.

Examples: /P                        - Displays current text

/P Kidderminster, sysop   - Set new text.

/P @                      - Clear previous text.

/QUIT    Exit the chat server.

Syntax: /Q[uit]

The /QUIT command, which may be shortened to /Q, disconnects

the user from the chat server, and informs everyone that he's

left.  There is no need for the user to /leave any logged

channels before issuing this command.

If the user accessed the server via the router's CHAT

command, he will be returned to the router's main command

prompt, otherwise he will be completely disconnected.

The /BYE and /EXIT commands also perform this function.

/TOPIC   Display / Change channel topic.

Syntax: /T[opic] [channel] [text | @]

Every channel has an optional topic, and the /TOPIC command

can be used to display the existing topic or change it.  The

topic can be up to 12 characters, and is displayed on the

/Who list.

Examples: /T                        - Show current ch. topic

/T 32                     - Show channel 32 topic

/T 32 TCP/IP discussion   - Set topic for ch. 32

/T @                      - Clear topic.

/USER    TCP/IP logon.

Syntax:  /U[ser] <callsign> [name]

The /USER command is available only to TCP/IP users.  It sets

the user's callsign (and optionally his name), which will be

displayed on the user list and prefixed to everything he

sends to others.  The user will not be able to join the

conference without supplying both callsign (9 chars max) and

name (12 chars max), but if the name is omitted from this

command he may enter it in the normal way with the /Name

command.

Examples: /U g8pzt          - Set callsign to "g8pzt".

/U g8pzt Paula    - Set callsign and name.

/VERSION  Display chat server version.

Syntax: /V[ersion]

The /VERSION command displays the chat server version, author

and compilation date.  Please quote it if reporting bugs.

/WHO     List channels and users.

Syntax: /W[ho] [*]

The /WHO command lists who is logged onto the chat server,

and what channels they are on.

If no arguments are supplied, the active channels are listed,

along with the callsigns of their users.

If an asterisk is supplied as the argument, each user is

displayed in more detail.  The display would typically show

the user's callsign, name, personal text and logon date/time.

Examples:  /W     Lists channels & users in brief format

/W *   Lists users in detail

SYSOP-ONLY COMMANDS

===================

The following commands are available only to sysops.  Some of them,

e.g. ARP are repeated from above, but with extra facilities.

ARP -- Display / Edit the ARP table.

Syntax: ARP [ADD <host> <hwtype> <addr>]

ARP [DROP <host> <hwtype>]

ARP [LIST]

The ARP command is used to display and edit the Address

Resolution Table, responsible for mapping IP datagrams to

callsigns.

If no arguments are supplied, the table is listed.

The form ARP ADD is used to add an entry to the table, ARP

DROP is used to delete an entry and ARP LIST just lists the

table.

<host> is an IP address in dotted quad form.

<hwtype> is the hardware type, i.e. "ax25" or "ether".

<addr> is the hardware address, i.e. callsign or ethernet.

Examples:

ARP ADD 44.131.91.2 ax25 gb7pzt-5    Add ax25 entry

ARP DROP 44.131.91.7 ax25            Delete ax25 entry

ARP LIST                             List the table

ARP                                  List the table

Availability:

The LIST subcommand is available to everyone, but ADD and

DROP are sysop-only.

Note:

In order for this command to have any meaning, the router

must have an IP address and be connected to an IP-capable

network.

AXROUTES -- Display AX25 level 2 circuits

Syntax: AX[ROUTES]

The AXroutes command displays information about currently and

recently active ax25 level 2 connections.

It allows the sysop to monitor the performance of the links

between the router and its users.

Entries are purged 24 hours after they were last active.

Note:  This facility was intended for the author's benefit

only, and is liable to change or disappear altogether in

future versions.  If you find it useful, let me know.

CAPTURE -- Enable / disable tracing to disk file.

Syntax:  CAP[TURE] [on | off]

The CAPTURE command is used to enable or disable the tracing

to disk of incoming and outgoing traffic.

When capture is enabled, anything which is displayed in the

central 22 line window is written also to disk.  This

includes session activity and TRACE'd data.  Whereas the

screen display is filtered to prevent it being garbled by

binary data, the capture is pure binary, so it is useful for

diagnostics.

The optional argument may be "ON" or "OFF".  If no argument

is supplied, the current setting is reported.

This command overrides the <F5> (toggle capture) key.

The console may override a remote sysop's settings.

Example:  CAP ON  -  Enables capture.

Note: You are advised not to enable long-term capture on a

floppy based system otherwise performance will be

compromised.

See also:  MONITOR

DATE -- Enquire / set system date

Syntax:  DATE [dd/mm/yy[yy]]

If no argument is supplied, the existing system date is

shown.

If the user is a sysop, and the argument is a valid date of

form dd/mm/yy or dd/mm/yyyy the system date is set to the new

value.

Example:  DATE 24/2/99

See also: TIME

DIGIFLAG -- Display / Set digipeat options.

Syntax: DIGIFLAG <port> [0-255]

The DIGIFLAG command is used to display and/or set the

digipeat options for a specified port.

New settings override those read from the CFG file, and

remain in force until changed, or the system is restarted.

Options are:     0    No digipeating on this port

1    Digipeat UI frames only

255  Digipeat all types of frame

The minimum abbreviation of this command is DIGIF.

Examples: DIGIF 3        - Enquire setting for port 3

DIGIF 3 255    - Set port 3 digiflag to 255

See also: DIGIPORT - Set port to digipeat on

DIGIPORT -- Display / Set port to digipeat on.

Syntax:  DIGIPORT <port> [destport]

Displays or specifies the port upon which digipeated frames

should be transmitted.

If "destport" is not specified, the current setting is

displayed.

If "destport" is zero, frames are transmitted on the port

upon which they are received, otherwise they are transmitted

on the port specified.

The minimum abbreviation of this command is DIGIP.

Example: DIGIP 3 5   - Digipeat from port 3 to port 5

Limitations:  Frames may currently be digipeated only to one

port, therefore it is not possible for a BBS on

one port to broadcast unproto headers to

several ports at once.

See also:  DIGIFLAG - Control type of frames digipeated

DOS -- Enter PZTDOS command mode.

Syntax: DOS

The DOS command switches the command interpreter into "DOS

mode", allowing local and remote sysops to use familiar DOS

commands to maintain the system without taking it off line.

In this mode, the normal command prompt is replaced by a DOS-

style $P$G prompt showing the current drive and directory,

and only the PZTDOS commands are understood.

The operator must use the EXIT command to return to normal

command mode.

Sessions automatically return to command mode upon

disconnection.

PZTDOS accepts both the MSDOS backslash (\) and the UNIX

forward slash (/) pathname separator characters, and they may

be freely mixed.  For the sake of clarity however, the prompt

always displays the path using the MSDOS convention, as this

is familiar to the most users.

As in MSDOS and UNIX, when specifying pathnames, a single dot

(.) represents the current directory, and a double dot

represents the parent directory.  Thus a command such as "CD

../FRED" would change to directory FRED, which is located off

the current directory's parent.

Unlike MSDOS, PZTDOS does not maintain a separate "current

directory" on each drive, and you will be logged to the root

when changing drives.  The normal A: B: etc. commands can be

used to change drive, and the CD (change directory) command

can also accept a drive letter.

Notes:

PZTDOS is fully multitasking, i.e. normal router operation

continues while PZTDOS mode is operating.

Console sysops may also invoke a conventional dos "shell"

using the <F9> key, for running commands and programs not

available within PZTDOS.  This mode is NOT multitasking and

normal router operation is suspended for the duration of the

shell.  The operator must limit the duration of the shell

session otherwise the software watchdog (if enabled) will

reboot the system.  TSR's and programs which interfere with

the timer or serial ports should not be invoked from a shell

as they may lock up the router or affect its performance.

Shell sessions are ended with the EXIT command.

FRACK -- Display / Set FRACK for a port.

Syntax:  FRACK <port> [msec]

Displays the current FRACK (AX25 level 2 T1 timer) setting

for the specified port, or allows it to be changed.

If the second argument is omitted, the current value is

displayed, otherwise the value in milliseconds is used.

The new setting overrides the value specified in the CFG

file, and remains in force until changed or the next restart.

Examples:  FRACK 2          - Display port 2 frack setting

FRACK 2 6000     - Set port 2 frack to 6 seconds

Note: A typical FRACK setting for 1200 baud is 7000 (7 secs),

and specifies how long AX25 will wait for a level 2 ack

before requesting confirmation.  Setting this figure

too low is antisocial, achieves nothing, and could

result in the link retrying out.

FULLDUP -- Display / Set full duplex mode.

Syntax:  FULLDUP <port> [0 | 1]

Displays the current duplex setting for a port, and allows it

to be changed without restarting the system.  If port is

operating in full duplex mode, DCD is ignored and the port

will transmit whenever it has anything to send.

If two numeric arguments are supplied, the duplex flag is set

to the second argument, otherwise the current setting for the

specified port is displayed.  Values are: 0 - simplex, 1 -

full duplex.

Modified settings remain in force until changed or system is

restarted.

Example:  FULLDUP 3    - Display current setting for port 3

FULLDUP 3 1  - Set port 3 into full duplex mode

Note: On KISS ports, allow up to 5 minutes for modified

settings to reach the TNC.

IP -- Display / Change contents of IP routing table.

Syntax: IP ROUTES

IP ROUTE ADD <host>[/len] <gateway> <port>

IP ROUTE DROP <host> <len>

IP ROUTE DEFAULT <port> [gateway [d|v|n]]

The IP commands are used to display and alter the contents of

the table responsible for routing of IP datagrams.

This table contains a series of entries which are checked

whenever a datagram is routed.  The search stops at the first

entry which matches the address and mask.

The command arguments are as follows:

<host>      Target host IP address in dotted quad form.

<len>       No. of bits to be matched (from left) 0-32

<gateway>   Destination gateway IP address in dotted quad.

<port>      Port number on which to route the datagram.

<d|v|n>     Mode: (d)atagram, (v)irtual circuit or (n)etrom.

The DEFAULT subcommand sets up a default route which is used

when no other route is found.  If no gateway is specified,

the target will be assumed to be a direct neighbour.  If not

specified, the mode defaults to datagram.

The ADD subcommand adds an entry to the routing table.  The

first argument is the target host IP address, with optional

mask. e.g. 44.131.90.1/32 means "match all 32 bits", whereas

44.131.90.0/24 means "match the most significant 24 bits",

and would route all 256 addresses from 44.131.90.0 to

44.131.90.255.  The second argument is the "gateway" address,

i.e. the address of the system which can handle the datagram.

The third argument is the port to route the datagram on, and

the last argument is the mode.  Datagram mode should be used

on links with low loss rates.  Better performance can be

obtained on lossy links by using Virtual Circuit mode.

Netrom mode is inefficient, but can "tunnel" datagrams across

non-ip parts of the network.

The DROP subcommand removes an entry from the table.  Both

the target host and the mask must match.

Examples:  IP ROUTE DEFAULT 3 44.131.90.6 v

IP ROUTE ADD 44.131.95.0/24 44.131.95.240 9 d

IP ROUTE DROP 44.131.97.1 32

Availability: The ADD DROP and DEFAULT subcommands are not

available to non-sysops.

Note: The IP routing table is necessary only for IP, and does

not take any part in normal ax25 and Netrom activities.

See elsewhere for details on how to set up the IP

system.

See also: IPROUTE, ARP.

KILL -- Terminate any session

Syntax:  KILL <session>

Immediately terminates the specified session, disconnecting

any links.  Useful for disconnecting troublemakers or failed

sessions.

Session numbers are shown by the U(sers) command.

Example:  K 301  - Terminate session numbered 301

Limitations:  You cannot kill your own session.

LOADNODES -- Load the nodes and routes tables from a disk file.

Syntax:  LO[ADNODES] [filename]

The LOADNODES command loads the nodes and routes tables from

a "recovery" file.  If a filename isn't specified, PZTNODES

is the default.  It can be used to reconstruct the tables

without having to wait for nodes broadcasts to be received.

This occurs automatically when the router is started, but if

the PZTNODES file was empty, you may wish to quick start from

a daily or weekly backup file which you previously created

with SAVENODES.

LOG -- Enable / disable activity logging.

Syntax:  LOG [on | off]

The LOG command is used to enable or disable the logging of

router activity, such as connections, disconnections, errors,

user commands etc.  It overrides the default setting

specified in PZTNODE.CFG.

The optional argument may be "ON" or "OFF".  The former

enables logging, and the latter disables it.  If no argument

is supplied, the current setting is reported.

The console may override a remote sysop's settings.

Examples:  LOG ON  -  Enables logging.

LOG OFF -  Disables logging.

Note: Do not enable long-term logging if you are running a

floppy based system, as you will seriously compromise system

performance.

MAN -- Display Online Sysop's Manual pages.

Syntax:  MAN [command | topic]

The MAN command is used to access the online sysop's manual,

which is a collection of pages, each covering a different

command or topic.

The MAN pages are designed for sysop use, and usually give

more detail than the corresponding HELP pages.

Entering MAN alone displays a list of available pages.

To read a specific page enter MAN followed by the command or

topic for which you require help.

The manual is divided into several sections.  Section 1 is

the command reference section.

Example:  MAN ARP      Display manual page for ARP command.

The following conventions are used in MAN pages:

< >   Command argument - These braces and the text enclosed

within them represent any piece of text typed after the

command.  The <> do not form part of the argument.

[ ]   Optional argument - These braces enclose anything

which is not always present. If used within a command

name (e.g. N[ODES]), it indicates that the command may

be abbreviated to a minimum form while remaining

unique.

Availability: Although this command is currently available to

all users, it is not shown in the list of

normal commands because it generates a lot of

output, some of which is not relevant to non-

sysops.  Its use by non-sysops may be withdrawn

at a later date if it proves to be a problem.

Limitations: Although most commands may be abbreviated, it is

not currently possible for the MAN command to

correctly identify the required page from an

abbreviated argument.  Thus "MAN NODES" is

acceptable, whereas "MAN NO" is not.  This

problem will be addressed in a later version,

but a temporary work-around might be to make

several copies of each MAN file for all

permutations of the command.  Beware though,

this may make the list very confusing!

Note: Due to its size it is difficult to ensure that everyone

has a completely up to date sysop document, but the MAN

pages will always contain the latest information.  They

will be kept updated by distributing new pages as

required.

See also:  H -- General help system.

MAXFRAME -- Display / Set port MAXFRAME value.

Syntax: MAX[FRAME] <port> [n]

The MAXFRAME command is used to display and set the AX25 L2

MAXFRAME (window) value for the specified port.  This is the

maximum number of unacknowledged frames allowed before the

link must stop and wait for an ack.  The normal range is

between 1 and 7, although maxframes of up to 127 are allowed

in modulo-128 links.

If the second argument is omitted, the current value is

displayed.

If the setting is changed, the new setting remains in force

until it is changed again, or until the router is restarted,

in which case it returns to the default value specified in

the port entry in the CFG file.

Examples: MAX 2     - Display current setting for port 2

MAX 2 6   - Set maxframe for port 2 to 6.

MMASK -- Select which protocol(s) to monitor (trace)

Syntax:  MM[ASK] [0 - FFFFh]

The MMASK command selects which protocol layers will be

monitored when traffic tracing is enabled.

The optional argument is a HEX number between 0000h and

FFFFh, which is calculated by adding together the desired

values from this table:

0001 - Incoming frames          0100 - ICMP

0002 - Outgoing frames          0200 - TCP

0004 - AX25 layer 2             0400 - KISS

0008 - AX25 info frames         0800 - SLIP

0010 - AX25 layer 3             1000 - PASSALL

0020 - AX25 layer 4             2000 - Hex Dump

0040 - IP frames

0080 - ARP frames

If no argument is supplied, the current setting is reported.

The default setting for the first console is 03FF, which

displays all incoming and outgoing traffic from AX25 layer 2

upwards.

This command duplicates the function of the <F4> key.

The console can override a remote sysop's settings.

The PASSALL option enables frames which fail the validity

checks at one layer (e.g. KISS checksum) to be passed to the

next layer above for tracing.

Examples:  MMASK 0201  -  display incoming TCP frames only

MMASK 0242  -  display outgoing TCP and IP

Note: Don't try to trace too much detail via a remote link -

see warning in MONITOR below.

See also:  MONITOR  -  Enable / disable monitoring.

MPORT    -  Select port(s) to monitor.

MONITOR -- Enable / disable traffic monitoring (tracing)

Syntax: MON[ITOR] [on | off]

The MONITOR command is used to enable or disable the display

of incoming and outgoing traffic.

The optional argument may be "ON" or "OFF".  The former

enables the display, and the latter disables it.  If no

argument is supplied, the current setting is reported.

If used at the console, this command enables and disables

tracing to the console window, and works in conjunction with

the <F2> (monitor toggle) key.

If used by a remote sysop, monitored frames are sent to him

instead of the console window.  In order to avoid overloading

the link, the MPORT setting is cleared every time monitoring

is enabled, so the sysop must then issue the MPORT command to

select which port(s) to monitor.

The console may override a remote sysop's settings.

Examples:  MON ON  -  Enables tracing.

MON OFF -  Disables tracing.

Only one sysop may receive the trace display at any one time.

Note:  Remote sysops must ensure that their link with the

router is capable of carrying the large volume of traffic

resulting from tracing.  Attempting to trace too many ports /

too much detail on a slow link may result in poor

performance.  You have been warned!

See Also:  MPORT  -  Select port(s) to monitor.

MMASK  -  Select type of activity to monitor.

MPORT -- Select which port(s) to monitor (trace)

Syntax:  MP[ORT] [0 - FFFFh]

The MPORT command selects which ports will be monitored when

traffic tracing is enabled.

The optional argument is a HEX number between 0000h and

FFFFh, which is calculated by adding together the desired

values from this table:

Port  HEX    Port  HEX    Port  HEX     Port  HEX

1   0001      5  0010      9  0100      13   1000

2   0002      6  0020     10  0200      14   2000

3   0004      7  0040     11  0400      15   4000

4   0008      8  0080     12  0800      16   8000

If no argument is supplied, the current setting is reported.

This command duplicates the function of the <F3> key.

If used by a remote sysop, monitored frames are sent to him

instead of the console window.  In order to avoid overloading

the link, the MPORT setting is cleared every time monitoring

is enabled, so the sysop must then issue the MPORT command to

select which port(s) to monitor.

The console can override a remote sysop's settings.

Examples:  MPORT 800  -  Trace port 12.

MPORT 1803 -  Trace ports 13, 12, 2 and 1.

Note:  Remote sysops cannot trace activity on the port on

which they are uplinked

See also:  MONITOR  -  Enable / disable monitoring.

MMASK    -  Select type of activity to monitor.

PACLEN -- Display / Set global or port paclen values.

Syntax: PACLEN <port> [value]

The PACLEN command allows the global and port-specific PACLEN

settings to be displayed and changed.  Paclen is the maximum

data field length within a packet.

The global PACLEN is specified in the L2 global parameters

section of the config file, and is the default value, used

where not overridden by a port-specific paclen.

All frames originating at the router use the global or port

paclens specified, but Netrom frames originating at other

systems can not be fragmented, so we have no control over

them, and they may be larger than our paclen.

If a single numeric argument is supplied, and its value is

greater than 0, the current paclen for that port number is

displayed.  If the argument is 0 the global paclen is

displayed instead.

If two numeric arguments are supplied, the first specifies a

port number and the second specifies a new paclen value for

that port.  Port 0 sets the global paclen.

Examples:  PACLEN 0          - Display global default paclen

PACLEN 0 120      - Set default paclen to 120

PACLEN 10        - Display paclen for port 10

PACLEN 10 160    - Set port 10 paclen to 160

PERSIST -- Display / Set the persist value for a port.

Syntax:  PER[SIST] <port> [0-255]

The PERSIST command is used to display and set the PERSIST

value for a port.  This is the "probability to transmit",

used as part of the CSMA channel access procedure.  A low

value is used when there are several stations sharing the

channel, giving each a fair chance to transmit.  A high value

is used when the channel isn't shared.  The optimum setting

is 256/n where n is the number of station sharing the

channel.

If the command is used with a single numeric argument, the

current setting for that port number is displayed.

If two arguments are supplied, the PERSIST value for the port

specified by the first is set to the value of the second.

The new setting remains in force until changed again or until

the router is restarted, in which case the value specified in

the CFG file will apply.

Examples:  PERSIST 5        - Display value for port 5

PERSIST 5 64     - Set port 5 PERSIST to 64

Note: On KISS ports, you should allow up to 5 minutes for a

new setting to become active.

See also:  SLOTTIME -- CSMA interval timer

QUALITY -- Display / Set default quality value for a port.

Syntax: Q[UALITY] <port> [0-255]

Displays or sets the default quality for nodes heard on the

specified port.  It should be set to zero to suppress all

level 3/4 activity on a port, e.g. on user access

frequencies.

If a single numeric argument is supplied, the current quality

for that port number is displayed.

If two numeric arguments are supplied, the quality for the

port specified by the first is set to the value of the

second.  The new setting remains in force until changed, or

until the router is restarted, in which case the setting

reverts to that specified in the config file.

Example:  QUALITY 3      - Display quality for port 3

QUALITY 3 40   - Set port 3 default quality to 40

REBOOT -- Reboot machine.

Syntax: RE[BOOT]

Reboots the machine immediately and un-gracefully.

It is typically used by remote sysops to restart the machine

after editing autoexec.bat/config.sys, installing a new

version of the software, or to recover from potentially

dangerous situations such as a corrupt hard drive.

Console sysops can use the "Vulcan Nerve Pinch" (Ctrl-alt-

del) or interrupt the power instead.

Notes:  Remote sysops should only use this command if the

router program has been started from AUTOEXEC.BAT,

otherwise it won't restart.

It is recommended that you include an automatic disk

check and repair utility within AUTOEXEC.BAT to

remove any problems which resulted in the REBOOT

command being used.

RESPTIME -- Display / Set L2 delayed ack timer for port.

Syntax: RESPTIME <port> [millisecs]

The RESPTIME command allows the value of the AX25 T2 (delayed

ack) timer for a port to be displayed or altered.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter.  The new setting remains in force until

changed, or until the router is restarted, in which case the

value specified in the CFG file is re-applied.

Example:  RESPTIME 3       - Display resptime for port 3

RESPTIME 3 150   - Set port 3 resptime to 1500 ms

Notes:

The RESPTIME parameter specifies how long the router will

wait, after receiving a frame, before sending an ack for that

frame.  It helps to improve the efficiency by cutting out

unnecessary acks.  It allows a single ack to be sent when a

transmission contains several frames, instead of acking each

frame in turn.

The value must therefore be at least the length of time it

takes to transmit a single packet.  At 1200 bauds (120

bytes/sec) a 120 byte packet lasts 1 second, a 180 byte

packet lasts 1500 millisecs and a 256 byte packet lasts just

over 2 secs.  Therefore resptime should reflect the paclen

used by the sender.  1500 millisecs is a good compromise, but

if the other end regularly uses high paclens, 2000 or 2500 ms

would be more appropriate.  At 9600 baud, 200 millisecs is

probably adequate.

Too high a value will cause the link to be too "relaxed",

whereas too low a value will cause too many acks.  Both

extremes reduce the link efficiency.

See also:  FRACK, SLOTTIME

RETRIES -- Display / Set the retries parameter for a port.

Syntax: RETRIES <port> [value]

The RETRIES command allows the value of the level 2 maximum

retry value for a port to be displayed or altered, i.e. the

maximum no. of attempts at sending a frame without a reply

being received.  The usual setting is 10.  A setting of 0

means "try forever", and should be avoided except for

testing.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter.  The new setting remains in force until

changed, or until the router is restarted, in which case the

value specified in the CFG file is re-applied.

Example:  RETRIES 3       - Display setting for port 3

RETRIES 3 15    - Set port 3 max retries to 15

SAVENODES -- Save the nodes and routes tables to disk.

Syntax:  SAVENODES [filename]

The SAVENODES command dumps the nodes and routes tables to

a file.  If a filename isn't specified, PZTNODES is the

default.  This file is read by the router at start up, and

used to reconstruct the tables without having to wait for

nodes broadcasts to be received.

Table dumping occurs automatically every hour, and when the

router is closed down using Alt-x, but this command allows

you to dump and examine the tables at any time, without

taking the router off line.

Note: The PZTNODES file is identical to the BPQNODES file

used with BPQ node, and may be used to "quick start"

when changing between the two systems.

SHELL -- Run a command or program in a DOS shell.

Syntax:  SHELL <cmd> [args...]

The SHELL command is used for running DOS commands and simple

programs in a temporary DOS shell which terminates upon

completion of the program or command.

Example:  SHELL DIR /W

Limitations:  Trying to run interactive commands or programs

via this means may fail, and may crash the

router.

At least 60k of free memory is required for the

DOS shell itself in addition to any program

loaded above it.

Notes:  This command is not yet functional.

Console users may use the F9 key to invoke a DOS

shell which persists until closed.

SLOTTIME -- Display / Set the slottime parameter for a port.

Syntax: SLOTTIME <port> [millisecs]

The SLOTTIME command allows the value of the CSMA interval

timer for a port to be displayed or altered.  The usual

setting is 100 milliseconds.

If a single numeric argument is supplied, the slottime value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter.  The new setting remains in force until

changed, or until the router is restarted, in which case the

value specified in the CFG file is re-applied.

Example:  SLOTTIME 3       - Display slottime for port 3

SLOTTIME 3 150   - Set port 3 slottime to 150

See also:  PERSIST

TIME -- Display / set system time.

Syntax:  TIME [hh:mm]

If no argument is supplied, this command displays the current

setting of the DOS clock.

If an argument of the form HH:MM is used, it sets the DOS

clock to the specified time. (24 hour format)

Example:  TIME        - Display current time.

TIME 23:06  - Sets DOS time to 23:06

See also:  DATE -- set system date

TXDELAY -- Display / Set the transmit keyup delay for a port.

Syntax:  TXD[ELAY] <port> [millisecs]

The TXDELAY command allows the value of the transmit keyup

delay for a port to be displayed or altered.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter in milliseconds.  The new setting remains in

force until changed, or until the router is restarted, in

which case the value specified in the CFG file is re-applied.

Example: TXDELAY 3       - Display txdelay for port 3

TXDELAY 3 300   - Set port 3 txdelay to 300 ms

Notes:

TXDELAY is the interval between the transmitter being keyed

up and the start of a transmitted packet.  It allows time for

the RF to reach its full value, and for the TX audio circuits

to stabilise.  Some synthesised rigs require a large txdelay

(500 or more) to allow the synthesiser to swing between RX

and TX frequencies, and many rigs have audio stages which

take 100 ms to stabilise while oversized electrolytics charge

up.

One factor which is often overlooked is the other end's

receiver.  It takes a finite time for the squelch to open,

and if the rig has just been transmitting it may take a while

to stabilise back to receiving, especially with synthesised

rigs.  If your txd is too short, you will be sending replies

before the other end is ready to hear you, and unnecessary

retries will result.

On KISS TNC's, you should allow up to 5 minutes for any new

setting to take effect.

See also:  TXTAIL

TXOK -- Display / Set "ok to tx" for a port.

Syntax:  TXO[k] <port> [0 | 1]

The TXOK command displays the current txok flag setting for a

port, and allows it to be changed.  This flag controls

whether or not the transmitter is allowed to key up.  If set

to zero, nothing will be sent on the port (see notes).  By

default, all ports have this flag set to a non-zero value.

If two numeric arguments are supplied, the txok flag is set

to the second argument, otherwise the current setting for the

specified port is displayed.  Values are: 0=don't transmit,

anything else=ok to transmit.

Modified settings remain in force until changed or system is

restarted.

Examples: TXOK 3        - Display current setting for port 3

TXOK 3 0      - Stop port 3 transmissions.

Notes:

Setting TXOK=0 does not actually disable the

transmitter itself, it merely prevents any frames

being sent to the TNC, so if the latter has a built-

in CWID, it will still key up the TX so send the

CWID.

It should be obvious, but if you set TXOK=0 for the

port you are using to send commands, you will lose

control of the node!

TXTAIL -- Display / Set the transmit keydown delay.

Syntax:  TXD[TAIL] <port> [millisecs]

The TXDELAY command allows the value of the transmit keydown

delay for a port to be displayed or altered.  This is a

parameter used only by KISS TNC's, and you should refer to

the TNC documentation for an explanation.

If a single numeric argument is supplied, the current value

for that port number is displayed.

If two numeric arguments are supplied, the first specifies

the port number, and the second specifies the new value for

the parameter in milliseconds.  The new setting remains in

force until changed, or until the router is restarted, in

which case the value specified in the CFG file is re-applied.

Example:  TXTAIL 3       - Display txtail for port 3

TXDELAY 3 100  - Set port 3 txtail to 100 ms

Note: After modifying this value, you may have to wait up to

5 minutes for it to take effect.

See also:  TXDELAY

PZTDOS COMMANDS

===============

The following commands are available only within PZTDOS mode, i.e.

after issuing the "DOS" command.

CD    -- Change Working Directory

COPY  -- Copy file(s).

DEL   -- Delete one or more files

DIR   -- Obtain a directory listing.

EDIT  -- Edit a text file.

EXIT  -- Leave PZTDOS mode

MD    -- Make Directory.

MOVE  -- Move file(s).

REN   -- Rename file(s).

RMDIR -- Remove Directory.

TYPE  -- Display a text file.

Each command is discussed in more detail in the following section:

PZTDOS COMMANDS IN DETAIL

~~~~~~~~~~~~~~~~~~~~~~~~~

CD -- Change Working Directory.

Syntax: CD <[drive:]<dir>

The CD command changes the "working directory" to the one

specified by <dir>.  It is available only within PZTDOS.

If <dir> is an absolute pathname, beginning with \ or /, it

is used as specified, otherwise it is interpreted relative to

the current working directory.  The command recognises "."

and "..", and both forward and reverse slashes may be freely

mixed.

Unlike the MSDOS command of the same name, if <dir> is

preceded by a drive spec the PZTDOS version will also change

the current drive.

Examples: CD ../FRED    CD C:\WINDOWS\SYSTEM

COPY -- Copy file(s).

Syntax: COPY <source> [dest]

The COPY command copies the file(s) specified by the pathname

<source> into the file(s) or directory specified by [dest].

<source> may be a directory, a single file, or multiple files

specified using wildcards, and may contain a drive and path

spec.  If <source> is a directory, all the files in that

directory will be copied.  If <source> contains wildcards,

all matching files will be copied.

[dest] may be a directory, a single file, or blank.  If

[dest] is a directory, the source file(s) are copied into it

as separate files.  If [dest] is a single file, the source

file(s) are concatenated into it.  If [dest] is not

specified, the file is copied into the working directory and

retains the original filename.

Examples:  COPY C:\MYPROG.EXE  A:TESTPROG.EXE

COPY A:FWD.SYS

COPY WHAT.SYS  A:

COPY C:\TMP1 D:\TMP2

COPY *.HLP HELP.DOC

Portability:  Normal dos syntax is followed, except that

concatenation using the "+" operator is not

supported.

Note:  Overwrites destination files without warning.

DEL -- Delete one or more files.

Syntax:  DEL <filespec>

If <filespec> describes one or more files, they are deleted.

The number of files deleted is reported upon completion.

Examples:  DEL FRED.TXT

DEL \TMP\LOG??98.*

DF -- Display free space on drive.

Syntax:  DF [drive]

The DF command displays the free and total space in bytes on

the specified drive, plus sector and cluster size

information.

If no drive is specified, the current drive is assumed.

The drive letter need not include the colon.

Examples:  DF

DF E:

DIR -- Obtain a directory listing.

Syntax:  DIR [drive:][path\][mask]

The DIR command lists the contents of the specified directory

using the optional specified mask.

If a drive is not specified, the "current drive" is assumed.

If a path is not specified, the "current working directory"

is assumed.

If mask is not specified, "*.*" is assumed.  If only the

filename part is specified, the extension ".*" is assumed.

Examples:  DIR MSGHDR??.*

DIR \LOGS\

DIR E:\PUB\DOCS\*.TXT

Limitations:  Does not at present accept pipes, redirection

or switches.

EDIT -- Edit a Text File

Syntax: E[DIT] [drive:][path\]<filename>

Invokes the text editor, allowing text files to be viewed or

edited.

See separate section detailing Line Editor commands.

EXIT -- Leave PZTDOS mode

Syntax:  EXIT

Closes a PZTDOS session and returns to normal command mode.

The complementary command is "DOS", which causes entry to

PZTDOS mode.

MD -- Make Directory.

Syntax: MD [drive:][path\]<dirname>

MD creates a directory with the specified pathname.  It is

identical to the MSDOS version.

If drive or path are not specified, the current working drive

and/or path are assumed.

The specified path may be absolute, or relative to the

current directory.

Examples:  MD fred

MD d:\usr\pub\packet

See also:  RMDIR -- Remove Directory.

MOVE -- Move file(s).

Syntax: MOVE [drive:][path\]<filename1>

[drive:][path\][filename2]

Moves the file(s) specified by the first argument (source) to

the new location and/or name(s) specified by the second

argument (target).  Both filenames may contain wildcards.

"source" must not be a directory.  If it doesn't contain a

path, the current working directory is assumed.

"target" may be a directory.  If a filename is not specified,

the original name(s) are used.  If target is a single file,

the source must also be a single file.

If source and target filenames are different the file(s) will

be renamed at the same time.

Examples:  MOVE fred.bat test.bat            - Rename only.

MOVE test.bat c:\                 - Move only.

MOVE c:\test.bat d:\tmp\fred.bat  - Move & rename.

Portability: This command follows normal MSDOS v 7 syntax,

apart from the ability to rename a directory,

which is not implemented.

See also: COPY, REN

REN -- Rename file(s).

Syntax: REN [drive:][path\]<oldname> [drive:][path\]<newname>

Renames one or more file(s) from <oldname> to <newname>.

If the source path is not specified, the current working

directory is assumed.

If the destination path is not specified, the renamed file(s)

remain(s) in the source directory.

If you specify two different paths, this command will move

the file to the new path, providing it is on the same drive.

The filenames may contain wildcards.

Examples:  REN FWD.SYS  FWD.OLD

REN  C:\AUTOEXEC.BAT  AUTOEXEC.BAK

REN  C:\TMP\FRED.DOC  \JUNK\JILL.DOC

REN  *.SAV *.19

Limitations: Source and destination pathnames must be on the

same drive.  (Use the MOVE command for renaming

to a different drive)

Portability: This command follows normal dos syntax.

See also: MOVE -- Move a file with optional rename.

RMDIR -- Remove Directory.

Syntax:  RMDIR [drive:][path\]<dirname>

Deletes a directory with the specified pathname, if it

exists.

If drive or path are not specified, the current working drive

and/or path are assumed.

The specified path may be absolute, or relative to the

current directory.

Examples:  RMDIR fred             Delete sub-directory FRED

RMDIR c:\windows       Improve your system!!

See also:  MD -- Make Directory.

TYPE -- displays a text file.

Syntax: TYPE [path/]<filename>

The TYPE command displays the specified text file without

pagination.

Example: TYPE ../DOCS/FRED.DOC

Limitations:  For use with text files only.

There are no limitations on the size of file

which can be displayed.  The command does not

(yet) allow concatenation of files, and the

output may not (yet) be redirected to a file.

Wildcards are not accepted.

LINE EDITOR COMMANDS

====================

In this section, n1, n2 and n3 refer to line numbers, which must be

greater than zero, and within the range of line numbers used in the

file.  The space between the command and the first number may be

omitted, and the numbers can be separated by spaces, commas, colons,

tabs etc.   Parameters in square brackets [] are optional.  When

copying, moving or inserting lines, the "source" text is inserted at

the "destination" line, i.e. "in front of" the original text.

?              Displays a brief list of commands.

A <text>       Append line <text> to the file.  The editor ignores

exactly one space between the A and the text to be

inserted, so if you want to enter a line with leading

spaces, add one extra.

C n1 n2 [n3]   Copy line(s).  If n3 is omitted, line n1 is copied into

line n2, otherwise the block n1 to n2 inclusive will be

copied into the position beginning at n3.

Note: n3 must not be within the block being copied.

D n1 [n2]      Delete line(s).  If both n1 and n2 are specified, the

block of lines between and including n1 and n2 will be

deleted.  If n2 is omitted, or is the same as n1, only

n1 will be deleted.

I n1 <text>    Insert <text> into line n1.

L n1 [n2]      List (display) line(s).  If n2 is specified, lines n1

to n2 inclusive will be displayed, otherwise it will

display 10 lines beginning at n1.  If n1 is omitted,

the first 10 lines of the text will be displayed.

M n1 n2 [n3]   Move line(s).  If n3 is specified, the block of lines

n1 to n2 inclusive will be moved to the position

beginning at n3, otherwise the single line n1 will be

moved to the n2 position.

Q              Quit.  The text is abandoned and the original file is

left intact, providing you hadn't used the W command.

S              Save the text.  Syntax and function identical to W.

W [<file>]     Write (save) file being edited to disk.  If <file> is

specified, the work is written there instead of the

original name, but will not overwrite an existing file.

W! <file>      Write the text being edited to <file>, overwriting any

existing file of that name.

WQ             Write the modified text back to the original file and

quit editor. (Same as ZZ)

ZZ             Same as WQ (see above)

FTP SERVER COMMANDS

====================

PZTNODE's FTP server accepts the following commands:

ABOR    CDUP    CWD     DELE    HELP    LIST    MKD     MODE

NLST    NOOP    PASS    PORT    PWD     QUIT    RETR    RMD

RNFR    RNTO    STOR    STRU    SYST    TYPE    USER

All commands and arguments are case-insensitive, thus "cwd", "CWD" and

"cWd" are equally valid.

Both UNIX style (e.g. /pub/fred) and DOS style (e.g. \pub\fred)

pathname conventions are accepted.

Unlike DOS, the server does not maintain separate "working

directories" for each drive, so pathnames which include a drive letter

must start at the root of that drive.  e.g. "c:mydir\fred.txt" is not

valid, whereas "c:\mydir\fred.txt" is.

The FTP commands are described in detail below:

ABOR -- Abort data connection.

Syntax: ABOR

The ABOR command tells the server to abort any data transfer

currently in progress and close the data connection.  The

control connection is not closed.

If the data connection is not open, this command has no

effect.

CDUP -- Change Directory Up By One Level

The CDUP command changes the current working directory up one

level to the parent directory, i.e. it performs the function

of "CWD .."

This command has no arguments, and if already at the root it

has no effect.

See also: CWD -- Change Working Directory

CWD -- Change Working Directory

Syntax:  CWD [drive:\]<path>

The CWD command changes the current working directory (and

drive if necessary) for the FTP session.

Examples:

CWD FRED        Change to subdirectory FRED.

CWD ..          Change up one level to parent directory

CWD /           Change to root directory

CWD /FRED/JIM   Change to JIM subdirectory of FRED directory.

See also:  CDUP -- Change up one directory;

DELE -- Delete file(s).

Syntax:  DELE [drive:\][dir\]<mask>

Examples:

DELE JIM.TXT         Delete JIM.TXT from current directory.

DELE /FRED/DOG.EXE   Delete DOG.EXE from directory /FRED

DELE *.BAT           Delete all files with .BAT extension.

Notes:  Wildcards '*' and '?' are accepted.

HELP -- Display help for FTP server commands.

Syntax:  HELP [command]

Examples:  HELP        Displays basic info and list of cmds.

HELP CWD    Gives help for the CWD command.

Notes:

Some FTP clients may intercept the HELP command to give help

on client commands.  In this case, the REMOTEHELP command, if

it is implemented should translate to a server HELP command.

If not, the client may have a command which passes commands

"RAW" to the server.  If all else fails, TELNET to port 21

and the HELP command will work.

LIST -- lists FTP server directory contents.

Syntax:  LIST [drive:\][dir\][mask]

The LIST command causes a directory listing to be sent from

the FTP server to the client over the data connection.  If

the data connection cannot be established, the command will

fail.

The optional argument consists of a directory path and

filename mask.  If no path is specified, the current

directory is assumed.  If no mask is specified, "*" (all

files) is assumed.  Wildcards '*' and '?' are accepted.

Examples:

LIST              Displays all files in current directory

LIST C:\PUB       Lists all files in PUB subdir of drive C:

LIST /USR/*.EXE   Lists all .exe files in /USR directory

See also:  NLST -- List names only

MKD -- Make new directory.

Syntax:  MKD [drive:\]<pathname>

The MKD command creates a new directory of the specified

name.  If pathname is not fully qualified, the new directory

is created within the current working directory.

Examples:  MKD FRED

MKD C:/JIM/BILL

See also:  RMD -- Remove directory

MODE -- Specifies the data transfer mode.

Syntax:  MODE <mode_code>

MODE specifies how the data is to be formatted and

transferred via the data connection.  Mode codes are as

follows:

B - Block         Data is sent in blocks

C - Compressed    Data is compressed

S - Stream        Data sent as stream of characters

The default transfer mode, which is the only one currently

implemented, is Stream.  The command is included to prevent

unnecessary error replies.

Examples:  MODE S   Sets (S)tream transfer mode.

NLST -- (Name List) lists directory contents in short form.

Syntax:  NLST [<filespec>]

The NLST command causes a directory listing to be sent from

the FTP server to the client over the data connection.  If

the data connection cannot be established, the command will

fail.

The optional argument consists of a directory path and

filename mask.  If no path is specified, the current

directory is assumed.  If no mask is specified, "*" (all

files) is assumed.  Wildcards '*' and '?' are accepted.

The listing consists of filenames only, without size, date

and other supplementary information.

Examples:  NLST

NLST K:\PUB

NLST /USR/*.EXE

See also:  LIST -- List directory contents

NOOP -- (NO OPeration) does nothing.

Syntax:  NOOP

The NOOP command does not affect anything, and its only

action is to cause the server to send an "OK" reply.  It is

perhaps useful for testing that the control connection is

still functioning.

PASS -- Specifies user password at login.

Syntax:  PASS <password>

The argument to the PASS command is a string of up to 5

characters in response to the secure password challenge.  The

string may not contain spaces.

The command must be immediately preceeded by the USER

command, which causes the system to replies with a matrix

consisting of 5 lines of 5 numbers thus:

4 1 6 3 7

3 5 2 6 3

7 1 9 2 4

2 7 1 4 6

3 5 2 6 1

The remote sysop must then choose ONE of the lines, and send

the PASS command followed by the 5 characters from the

password string which correspond to the 5 numbers on the

chosen line.  There must be a space after PASS but no

spaces between the characters.

Example:  PASS WATER

Notes:

If the sysop has connected on a port which has SYSOP=1 in the

config file (e.g. a secure wire link), the response to this

command is ignored, and the sysop is granted full access.

See also:  USER -- Specify your username.

PORT -- Specifies the IP address and port for the data connection.

Syntax:  PORT h1,h2,h3,h4,p1,p2

The PORT command specifies the IP address and TCP port number

to be used by the data connection.  The argument is the

concatenation of a 32 bit IP address and a 16 bit TCP port

number, broken into 8 bit fields, each field being

transmitted as a decimal number.  The fields are seperated by

commas, and the high order fields are transmitted first.

Example:  PORT 44,131,91,2,4,1

Specifies IP address 44.131.91.2 and TCP port number 0401

(1001 decimal)

Notes:

Under normal circumstances this command is not needed.  The

data connection defaults to TCP port 20 at the client's IP

address of the control connection.  However, it allows data

to be sent to a completely diffent host if required.

PWD -- Print Working Directory.

Syntax:  PWD

The PWD command causes the full path of the user's current

working directory to be displayed via the control connection.

See also: CWD -- Change Working Directory

QUIT -- Terminates an FTP session.

Syntax:  QUIT

If a file transfer is not in progress, the QUIT command

terminates the FTP session and closes the control connection.

If file transfer is in progress, the control connection will

remain open until transfer is complete, allowing the result

code to be transmitted.  The control and data connections

will then close.

An unexpected close on the control connection will abort any

data transfer currently in progress.

See also:  ABOR -- Abort current command.

RETR -- Retrieve (download) file.

Syntax:  RETR [drive:\][dir\]<filename>

The RETR command causes the named file to be sent from the

FTP server to the client over the data connection.  If the

file can't be found, or access is denied, or the data

connection can't be opened, an appropriate error message is

returned.

Examples:  RETR JIM.TXT

RETR ../FRED/DOG.EXE

RETR C:\CONFIG.SYS

Notes:  Single files only, wildcards not accepted.

See also:  PORT -- Specify host/port for data connection.

STOR -- Send a file to the server.

STOR -- Store (upload) file.

Syntax:  STOR [drive:\][dir\]<filename>

The STOR command requests the FTP server to accept data via

the data connection and store it as a file with the specified

name.

If the file specified in the pathname already exists at the

server, it is overwritten by the new data.  The overwrite

doesn't take place until the file has been correctly

received, which prevents a critical file being lost if the

data connection breaks.

Examples:  STOR JIM.TXT

STOR /FRED/DOG.EXE

See also:  RETR -- Retrieve a file from the server.

RMD -- Remove Directory.

Syntax:  RMD [drive:\]<pathname>

The RMD command deletes the directory specified by <pathname>

from the FTP server, providing you are the owner of that

directory and have write access.

Examples: RMD FRED

RMD K:/JIM/BILL

See also:  MKD -- Make directory

RNFR -- Rename From

Syntax:  RNFR [drive:\][dir\]<filename>

The RNFR command specifies the old pathname of a file which

is to be renamed, and must be immediately followed by an RNTO

command.  The two commands together cause a file to be

renamed.

If the file isn't found the request will be refused.

Examples: RNFR ../JIM.TXT

RNFR K:/FRED/DOG.EXE

See also:  RNTO -- Rename To

RNTO -- Rename To

Syntax:  RNTO [drive:\][dir\]<filename>

The RNTO command specifies the new pathname of a file which

is being renamed, and must immediately follow an RNFR

command.  The two commands together cause a file to be

renamed.

If the new pathname isn't valid, the request will be refused.

Examples:  RNTO DOG.TXT

RNTO K:/FRED/CAT.EXE

See also:  RNFR -- Rename From.

STRU -- Specify File Structure.

Syntax:  STRU <structure_code>

STRU specifies the internal structure of the files being

transferred, i.e.  how the data is organised within them.

Structure codes are as follows:

F - File       No record structure (contiguous bytes)

R - Record     File is collection of sequential records

P - Page       File is composed of independant pages

The default structure, which is the only one currently

implemented, is (F)ile.  The command is included to prevent

unnecessary error replies.

Example:  STRU F   Sets (F)ile structure.

SYST -- Operating system enquiry.

Syntax:  SYST

The SYST command is used to find out what type of operating

system is being used on the server.

The first word of the reply is one of the agreed system

names, in this case MSDOS.  The rest of the reply gives the

server version number and byte size.

TYPE -- Specifies the data representation type.

Syntax:  TYPE <type_code>

The argument to the TYPE command specifies how the data is to

be translated between storage and transfer.  Type codes are

as follows:

A - ASCII      Text.  End of line indicated by <CR><LF>

I - Image      No translation.  Bytes stored as received.

L <bytesize>   Size of local storage bytes

Examples:  TYPE A    Specifies Ascii type.

TYPE L 8  Specifies local byte size of 8 bits

USER -- Specify user name for login.

Syntax:  USER <username>

The argument to the USER command is a string of up to 8

characters specifying the user's login name (usually

callsign).  The string must not contain spaces.

Users are not allowed access to the system without logging

in.

If the username (or callsign) exists in the PASSWORD.SYS file

the server sends a grid of 5x5 numbers (see PASS command)

generated from the user's stored password.  If the username

is not found, an error message is sent instead, and access

will not be permitted.

The USER command must be immediately followed by the PASS

command.

Example:  USER SYSOP   Enters your login name as "SYSOP"

USER G8PZT   Use callsign

Note: Because the FTP server allows you to use any string of

characters as a name, you may set up different passwords for

different login names, and these may be in addition to your

callsign, which is the one used for ax25 sysop access.

See also:  PASS -- Specify password.

(end of document)