Using PZTNODE with Ethernet

===========================

Using Ethernet, PZT routers can be linked with each other and with other

ethernet-capable systems such as Linux, xNOS, BPQ (and Windows?).

There are two options, both of which use the TYPE=EXTERNAL interface,

but using different PROTOCOL= settings.  The first option is somewhat

limited in capability.

Option 1 - Use BPQ's ODIDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

PZTNODE will interface directly to BPQ's ODIDRV program, allowing

AX25-only links between any number of PZT routers, and between PZT

and BPQ systems.

For this option, the interface should be specified as TYPE=EXTERNAL,

PROTOCOL=HDLC, INTNUM=125 (adjust this to suit your needs). The MTU

should be set at 256 since ODIDRV can only handle small frames. You

can attach one PORT to this interface.

You will need to obtain BPQ's ODIDRV.EXE and the appropriate bits of

Novell Netware, and have a reasonable working knowledge of how to

set it all up!

Follow the instructions in BPQ's DRIVERS.DOC for setting up the ODI

system.  PZTnode currently uses a fixed PID of 0x8FF, which is the

default value for BPQ, and frames are addressed to the multicast

address FF:FF:FF:FF:FF:FF, so the only change you need to make to

BPQ's example is to change ETH_ADDR to that value.

The load order is as follows:

LSL.COM		(Part of Novell Netware)

NE1000		(Ditto - adjust to suit your Ethernet card)

ODIDRV 125	(BPQ program.  Number must match INTNUM)

PZTNODE

If you wish to communicate with BPQ nodes via Ethernet, the PID and

multicast addresses must match.

Because ODIDRV only supports HDLC protocol, you won't be able to use

TCP/IP on the link unless it is first encapsulated in AX25 frames.

This should happen automatically if you try to route IP via the ethernet

port, but obviously this encapsulated IP cannot talk directly with

real TCP/IP systems which don't understand AX25.

Option 2 - Use PZT's ETHDRV.EXE driver

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This option allows simultaneous connection between any number of

PZT routers, between PZT and BPQ, and between PZT and other systems

capable of TCP/IP over Ethernet.  It doesn't require any Netware

components, and is relatively easy to set up.

The software components you require are:  ETHDRV.EXE (supplied with

PZTnode), an Ethernet card driver to match your card e.g. NE1000.COM

(You can get these free from Simtelnet), and PZTNODE itself.

The interface should be set up as TYPE=EXTERNAL, PROTOCOL=ETHER, and

you may set MTU as high as 1500 if you wish. Set INTNUM to a spare

software interrupt in the range 60 - 127 (125 is used as an example).

You can attach one PORT to this interface.

If you are not familiar with Ethernet card drivers (aka "packet" or

"Crynwyr" drivers), 3 arguments are usually specified when starting,

e.g...

NE2000 0x60 5 0x300

Where:	NE2000	is the driver matching your card.

0x60	is the software interrupt used to talk to the card,

specified in hex (0x60 is decimal 96, 0x61 is 97 etc.)

5	is the IRQ used by the card.

0x300	is the I/O address of the card.

The I/O address and IRQ are often settable by jumpers on the card,

especially on older cards, but some cards have fixed settings.

0x60 is the default software interrupt used by ETHDRV.EXE to communicate

with the card driver.  If you specify an alternative number (e.g. if you

have several cards), you must specify that number when starting ETHDRV.

ETHDRV.EXE normally requires a single argument, namely the INTNUM which

should match the one specified in the INTERFACE block.  It optionally

takes a second argument, if you wish to use an interrupt other than

0x60 to communicate with the card driver.

e.g.	ETHDRV 125		(intnum=125, software int=0x60)

ETHDRV 125 0x63		(Using a different card interrupt)

ETHDRV	125 -U		(To unload the driver)

My startup sequence is as follows:

NE2000 0x60 5 0x300

ETHDRV 125		(Interface intnum=125)

PZTNODE

----------------------------------------

If you have any questions or feedback PLEASE let me know, don't leave it

to others because they probably won't bother!  I've probably omitted some

vital information in the rush to get this written, so don't hesitate to

chastise me ;-)

73, Paula  g8pzt@gb7pzt.#24.gbr.eu or g8pzt@cableinet.co.uk

27th February 2001