CAT980 Ver 2.0                                        Appendix A  Page 1

APPENDIX A

WRITING THE CAT980.CFG CONFIG FILE

==================================

What IS the Cat980.cfg File?

----------------------------

This file, written in plain text (ASCII), allows you to

customize your operation when you first call up CAT980.  The program

will initially look for this file and execute the valid statements you

have included.  If the file is absent CAT980 will attempt to continue

using the defaults.

You have control over what this file can do for you.  It can

initialize colors, set up time displays, pick a com port, define band

limits, activate the AGC scan function and load other operating files

such as menu (.mnu), memory (.mem) and log (.log) files.  You can

rewrite and add to this file as you learn more about what it can do and

what you want it to do.

HOW DOES IT WORK?

-----------------

When you initially call up the program you will first see the

header information and an invitation to 'hit any key'.  When you do,

CAT980 will look for the .cfg file and a box will appear on your screen

which might look like this:

------------------------------------------------------

|Reading Configuration File - ........................|

|....................................                 |

|                                                     |

|                                                     |

|Configuration complete                               |

|60 Line read in from cat980.cfg file...              |

| 0 errors detected  <Press any key to continue>      |

-------------------------------------------------------

The string of dots inside the box on the first two lines represent

the number of lines which have been 'read' by CAT980 and will appear on

the screen in real time as they are encountered in your .cfg file. Blank

lines are not counted but comment lines are. If you should have a bad

statement in your file that is detected the screen could look like this:

------------------------------------------------------

|Reading Configuration File - ........................|

|........................e                            |

|                                                     |

|                                                     |

|Config file error in line  49                        |

|TimeFreq=3H00000                                     |

| <Press any key to continue>                         |

-------------------------------------------------------

A-1

CAT980 Ver 2.0                                        Appendix A  Page 2

"TimeFreq" is the statement that allows you to set the frequency

of another time standard with <ShftF10> - the default is WWV on 5 MHz.

In this example the .cfg writer may have been trying to install that

frequency but typed an 'H' instead of a '5' - and the error was

detected. Make a note that the error occurred on line 49, you can

actually count down that number to find the bad statement and correct it

later.  After hitting another key the screen will look like this:

------------------------------------------------------

|Reading Configuration File - ........................|

|........................e...........                 |

|                                                     |

|                                                     |

|Configuration complete                               |

|60 Line read in from cat980.cfg file...              |

| 1 errors detected  <Press any key to continue>      |

-------------------------------------------------------

- provided there are no more errors.  If there are they will be dealt

with as explained above.

Be sure and check out the file cat980.cfg included with this

package. You can do it with a simple ASCII editor.  It has a lot of tips

in the comment section and is purposely written as a tutorial.  It is

intended to complement this appendix so print it out.

Where To Start

--------------

First off, the one thing you do NOT want to do in this file is

include a COMM= statement until you are absolutely sure which port you

are going to use.  Leaving this statement out allows CAT980 to default

to COM1 and will let you use a <^p> to change the port from the keyboard

prior to capturing the radio when you first enter the program.

When you have decided on a port and you know your interface and

everything is working OK, then by all means include the statement in

this file so CAT980 will always know automatically which port to use

when you call it up - you won't have to use <^p> again.  ONE of the

following is allowed:

COMM=1

COMM=2

COMM=3

COMM=4

Lower case is OK but include no spaces and keep this statement

on a line by itself. Your supplied cat980.cfg file contains a commented

out '* COMM=2' statement which, because of the asterisk, does not take

effect and allows a default com port 1 and the use of <^p> for set up

purposes.

A-2

CAT980 Ver 2.0                                        Appendix A  Page 3

*YOU MUST HAVE THE HARDWARE INSTALLED IN YOUR COMPUTER*

*IN ORDER TO USE ANY GIVEN PORT!*

A word about COM ports; they are tricky.  You probably already know

this if you've been using internal modems and TNC's and whatever. There

are times it seems you just have to reboot the machine to get your

program to work.  In general the following statement is true:

1. You can not use COM1 and COM3 simultaneously.

2. You can not use COM2 and COM4 simultaneously.

If you have a TNC on COM1 then put CAT980 on COM2 or COM4. If you

have a TNC on COM2 then put CAT980 on COM1 or COM3.  Get the idea?

Versions 1.8 and 1.9 used direct hardware addressing and a polling

system.  This was (is) OK on the older slower systems, but with the

advent of Windows and super fast machines, CAT980 now uses a complete

interrupt system.  It should adapt to more environments and be more

crash-proof.

THE SCREEN CLOCK

================

CAT980 derives its time and date from your system clock so it is

assumed you will have that set correctly on boot up or you have a

permanent system hardware clock installed.

Depending upon where in the world you are, you can include a

TIMEZONE= statement up to 5 printable ASCII characters.  Some of the

most common are:

timezone=CST

timezone=CSDT

timezone=EST

timezone=X9IQK

No, that last one is not a misprint even though it is

meaningless.  The point is whatever 5 characters you choose, that is

what will be shown in upper case in your clock box on the screen.

*IF YOU OMIT THE TIMEZONE= STATEMENT THE TIME BOX*

*WILL DISPLAY THE WORD 'LOCAL'*

Because hams often live on UTC the clock box displays both your

local time and UTC time - and day/dates too.  Since CAT980 doesn't know

where you are operating from, you can use the ZULUSHIFT= statement to

set the UTC time.  Some examples follow.

A-3

CAT980 Ver 2.0                                        Appendix A  Page 4

STATEMENT        QTH

-------------   ------

zulushift=-2    Cairo

ZuluShift=-9    Tokyo

zulushift=8     Seattle

zulushift=6     Dallas

zulushift=5     Dallas on daylight savings time

zulushift=10    Hawaii

*CAT980 IS FRIENDLY TO 9V1'S IN SINGAPORE!*

If you live in a part of the world that is on the half hour in

respect to UTC the time box will still work for you.  In Singapore this

statement will work:

zulushift=-7.5

Any other half hour time zone can be set in the same manner.

* DEFAULT FOR CAT980 IS C.S.T. *

*IF YOUR QTH IS IN ZERO TIME ZONE (GMT) THAN ONLY ONE TIME*

*WILL BE DISPLAYED IN THE TIME BOX*

The CAT980 screen clock shows Zulu day and date as well as local

day and date.

CHECK FOR TRANSMIT

==================

One allowable statement the author isn't particularly fond of is

the:

CHECKTX= statement.  It's only allowable form is:

checktx=on

This tells CAT980 to continually interrogate the FT-980 from the

command prompt to see if it has changed from receive to transmit and the

other way around.  Somehow the steady stream of data flowing between the

radio and the computer lacks a certain eloquence.  The advantage is that

in operation like VOX, the screen would show a transmit condition when

the operator speaks and keys the radio.  The disadvantage is it can slow

down the keyboard commands.  If a key is held down by the operator, say

a manual sweep stepping 1 Khz up the band <Home>, then at some point the

commands outnumber the ability to respond and the interface could be

lost.  It depends on your habits (do you hold down a key for long

periods?) and the speed of your computer.

A-4

CAT980 Ver 2.0                                        Appendix A  Page 5

However no difference in response should be noticed with

singular commands during normal CAT980 operation. If you want to use the

CheckTx= statement go ahead and experiment. It has been omitted in the

supplied configuration file.

*IF YOU DON'T INCLUDE THE CHECKTX= STATEMENT*

*YOU CAN STILL USE THE FEATURE FROM THE KEYBOARD WITH A ^O*

By the way, if you are keying the radio through the RTS or DTR

lines than you will get a screen indication of a transmit condition the

moment the line changes state provided the FT-980 goes into transmit

condition.  The author finds this more satisfactory at this QTH and

simply leaves the CHECKTX= statement out of the config file. (Yeah, I

hate it.)

* NOTE THAT WHEN AN ASTERISK APPEARS IN YOUR FILE IT MEANS A

* COMMENT LINE AND WILL PROCESS YOUR FILE FASTER! *

LOADING MENUS, MEMORIES AND LOG FILES FROM THE CONFIG FILE

===========================================================

LOAD=   statement

-----------------

In the main documentation you learned how to load a .mem file to

fill the slots and/or the FT980 16 memories. But you can also load a

menu when you bring CAT980 up.  In the supplied config file note these

two command lines:

load=examp20.mem

load=demo20.mnu

*DO NOT USE THE STATEMENT TO LOAD A LOG FILE!*

The first statement above loads the memory file examp20.mem -

that's what caused all the initial action on the screen when you first

tried this program. (If you're 'tired' of it loading each time simply

delete the line OR put an asterisk in front of it.)

*ALL YOUR MEMORIES SET PREVIOUSLY BY THE FRONT PANEL WILL*

*BE AUTOMATICALLY RESTORED AFTER A SESSION*

The second statement loads the menu file, demo20.mnu, into

readiness.  After capture of the radio you can simply tap the space bar

and the menu you have loaded will appear.  The file  must be in the root

directory - as all files supporting CAT980 should be.  The extension

should be .mnu for trouble-free operation though it can be something

else (you will continually be prompted if you stray from .mnu!)

A-5

CAT980 Ver 2.0                                        Appendix A  Page 6

* KEEP 'LOAD=' STATEMENTS ISOLATED ON LINES BY THEMSELVES. *

You can NOT install a .log file with the 'load=' statement, however the

statement LogFile= does the job.

LogFile= statement

------------------

This is a special load statement so that you may install log files

from the config file. For example:

logfile=k5yef.log

BFO= statement

-------------

This statement should be set to match the switch position on the

back of the FT-980 which sets the CW tone. (Bet you have it on 700!)

Allowable statements (and switch settings) are:

BFO=500       BFO=600        BFO=700

The default, no statement necessary, is indeed 700.  Your choice

will be used in a virtual BFO indicator on the CW filter displays <F8>

and play an important part in the various filter configuration options.

AFSKmark= statement

-------------------

This can be set to any value from 100 to 4000.  It defines the mark

tone of an external TNC or RTTY modulator hooked to the audio input of

the FT-980.  The default is based on the PK-232 mark tone:

AFSKmark=2100

This value will play a role in the filter configuration shifts

offered by CAT980.

AFSK= statement

----------------

If AFSK=YES then this statement shifts the relationship of the CW,

FSK, USB and LSB filters in a proportionate way related to the AFSK Mark

tone (see above).  The default is AFSK=NO which is the standard Yaesu

filter configuration.

A-6

CAT980 Ver 2.0                                        Appendix A  Page 7

TIMEFREQ= statement

-------------------

The key <F10> brings up WWV on 10 MHz and by default <ShftF10> will

deploy the radio to WWV on 5 MHz.  But you can select a time standard

station of your choice for the <ShftF10> key other than 5 MHz.  If, for

instance, your .cfg file contains the statement:

TIMEFREQ=1500000

then the <ShftF10> key will call up WWV on 15 MHz. (Note we are still

observing the listing of any CAT980 frequency in 7 digits!)  Some unique

time broadcasts presented in CAT980 7 digit format are:

Australia:  0542500  0751500  1200500

Canada:     0330000  0733500  1467000

You might have a local news/time/weather AM broadcast station you

would like <ShftF10> to call up.  Just list the frequency in your .cfg

file.  The only limitation is the mode; <ShftF10> will always be AM

whatever frequency you list.  When you hit another key your radio will

go back to whatever frequency AND mode that was present prior to the

'time' request.

AUTOVFO=YES statement

---------------------

If you include this statement then every time you choose memory,

that is a channel in the FT-980 memory picked by <F7>, then it will

automatically be translated into the VFO - without prompting you for

approval.

With AUTOVFO=YES and the active channel is 16 for instance, vfo,

mode and frequency, will be switched from memory into the vfo mode , yet

will still be retained in the FT-980 as channel 16..

AUTOMEM=YES statement

---------------------

This is just the opposite of AUTOVFO - which means you can not use

the AUTOVFO= statement if you use AUTOMEM=. When a channel is selected

the system will remain in memory control. That's the way Version 1.8

works all the time so if you include this statement then you will cause

the version 2.0 series to behave like Ver 1.8 with respect to memory/vfo

control.

(NONE OF THE ABOVE)

-------------------

If you leave out both AutoVFO or AutoMEM statements then version

1.9 will prompt you on the command line as to wether you want to stay in

memory or switch it into VFO.  Note: a <^e> will toggle through these

same options from the keyboard.

A-7

CAT980 Ver 2.0                                        Appendix A  Page 8

MONO=YES statement

------------------

This command may come in handy for use with certain lap top

computers.  It will force the program into a black & white mode which

the color commands or <AltC> cannot change.  If, on any mono system, you

have any trouble seeing any text, etc., give this command a try too.

The default is color with the MONO= statement entirely missing.

DIRECTSCREEN=YES

----------------

With today's modern PC's writing directly to the screen seems to be

the way to go. It's certainly quick and is the default for the version 2

series.  However, if you're experiencing screen presentation problems

then try letting CAT980 write thru the machine BIOS. You can do this

with the statement:  DIRECTSCREEN=NO

COLOR COMMANDS

--------------

See Appendix E for a list of color statements that can be used in

the .cfg file.

FILE UTILITY STATEMENTS            WHAT IT DOES

-----------------------            ------------

LOGFILE= (valid DOS file name)     Defines file called by <^p>

TIMESTAMP=YES                      Supplies time/date for log files.

TIMESTAMP=NO                       Removes time/date for log files.

ZSTAMP=YES                         Supplies time in UTC (Zulu)

ZSTAMP=NO                          Supplies local time.

All of these statements can be part of your .cfg file - and as you

will learn in Appendix B and Appendix C, you can also use many of them

in .mnu and .mem files. Appendix F is a helpful aid for this activity.

SET TABS STATEMENT

------------------

SETTAB(band)= (upTab loTab)

Example for 80 meter default SetTab80=3999999 3500000

TABSRESET=YES  This will reset all tab frequencies to United States ham

band edges - the default. But Appendix D will show you how to redefine

these bands for your country's regulations.

AUTOTABS=YES   This will actually engage the tabs from band to band,

etc.

A-8

CAT980 Ver 2.0                                        Appendix A  Page 9

These three powerful commands require more space than are detailed

here.  Please read Appendix D for the full story.

MISCELLANEOUS STATEMENTS

=======================

KEYNUM=YES

----------

When this statement is in your .cfg file the key pad on the

keyboard outputs numbers instead of up/down, right/left arrows. You may

prefer this type of operation. If you do, give it a try.  KEYNUM=NO also

works (default) and can be used in .mnu or .mem files so you can engage

this feature for special operating modes when you so choose.

BELL=NO

--------

BELL=YES is the default.  If you don't like the 'beep beep' when

you make a mistake or CAT980 makes a mistake(!), then this will turn

most of the annoying beeps off. BELL=YES can be put in other files to

turn it back on for other activities. Again, you can toggle the feature

for changing operating modes and moods (and keep sleeping harmonics

sleeping).

It is recommended that the bell be on for calibrating the AGC

trigger level point.  The game-sound produced for this adjustment allows

you to keep your eyes off the screen while setting up.

CLEARSLOTS=YES

--------------

This statement compliments the high speed slot and slot load

commands for the 300 memory slots. When encountered it clears 'em all

out.  Just like a <^d> from the keyboard.

BIGFREQNUM=YES

--------------

This brings up a large frequency display (3 cols by 3 rows each

digit) and rearranges the screen to the version 2.x display format.  It

is the default.

BIGFREQNUM=NO will cause the frequency display to revert to the version

1.8/1.9 display format.

A-9

CAT980 Ver 2.0                                       Appendix A  Page 10

AGC=YES

-------

This causes CAT980 to be cognizant of the state of the CTS line.

With an external circuit you can set up a trigger point which

corresponds to an 'S' meter level. In a scanning mode the scan can be

made to halt when this level is detected. See a suggested circuit in

Appendix G.  The program defaults to this feature being off which can be

accomplished with the statement:

AGC=NO

SHOWFILES=YES

-------------

This statement will produce a box showing the active files being

used, or not used, by CAT980. The .mnu, .mem and .log files are all

shown.  The default for this feature is on but you can turn it off with

the statement:  SHOWFILES=NO

WATCHDOG=21

------------

CAT980 has a tune function and the author, being notorious for

creating unwanted clouds of smoke, installed this statement to keep from

burning up the radio.  When you do a TUNE the radio goes into FSK and

the watchdog counter heads toward zero and turns off the radio via the

RTS line.  You might even consider lowering the default from 21 seconds

to a safer level.

BLINK=YES

----------

There are times in the program when certain messages blink.  When

running in Windows 3.1 there are some situations where these messages

don't blink. It is recommended that if you find a blinking message

offensive or you are running in a multi application environment, that

you set this parameter to:

BLINK=NO

SPECIAL PARAMETER - COULD BE TROUBLE

====================================

PORTTIME=6

----------

This is the default for a number that is related to a port 'time

out' sequence. It may be, may be, necessary to increase this number one

integral at a time if you have a very fast machine.  It is recommended

that you make absolutely sure that you H/W interface is OK and your port

H/W is OK before you use this value. If your FT-980 is being captured

(the EXT led coming on and the loss of front panel controls) but then

looses control - this parameter would be worth experimenting with.

Otherwise - leave it be.

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=== END APPENDIX A ===

A-10

CAT980 Ver 2.0                                       Appendix B  Page 1

APPENDIX B

Writing CAT980 Memory Files

===========================

You may write an 'unlimited' number of memory files to

compliment your operating preferences. These files must be plain text

files that you have created in an ASCII editor such as DOS 5.x EDIT. The

formatting ground rules for these files is outlined below.

There are two main functions that are controlled by this

file, (1. the loading of any one up to all of the FT-980 internal 16

memories, 2( the loading of any one or all of CAT980 300 memory storage

slots.

Loading The FT-980 Internal Memories

------------------------------------

Each or any of the radio memories can be loaded by this file

using the following format:

Channel=xx Freq=2810000  Mode=USB vfo=HAM

|      |             |       |

|      |             |       |

|      |             |       |- Can be HAM GEN

|      |             |

|      |         -Can be LSB USB CWW CWN AMW AMN FSK FM

|      |

|      | -Must be 7 digits.

|

|

|- One of the FT-980 memory channels. Must be 1 thru 16.

The FT-980 memory channel operated upon is in the statement

'Channel=xx'.  Any parameters following such a statement such as

frequency, mode and vfo will pertain to that channel number.  When the

next channel number statement is encountered than all the info following

will pertain to that channel, etc.  You can load them in any order you

desire starting each entry on a separate line.

Allowable Range of Parameters:

Channel=xx Freq=2810000  Mode=USB vfo=HAM

|      |             |       |

|      |             |       |_HAM GEN

|      |             |

|      |             |_USB LSB CWN CWW FSK AMW AMN FM

|      |

|      |_0015000 to 2999990 (150 Khz to 29.999.990 Mhz)

|       Freq MUST always be entered 7 digits long!!

|

|_1 to 16

B-1

CAT980 Ver 2.0                                       Appendix B  Page 2

EXAMPLE:

Channel=15 Freq=2960000  Mode=fm vfo=ham

FORMATTING .mem RULES

---------------------

1. The 'Channel=xx' statement comes before any of the parameters

that will apply to it.  If no parameters are listed then the

present configuration of the radio will be stored in that

channel. Consider that last revelation; there may be some

unique way to use this situation [??] - One idea is listed

later in 'HINTS'.

2. The freq= statement must always contain 7 digits or the

channel may be loaded with an unpredictable result - or not

at all.

3. Statements may be on consecutive lines or the same line if it

pertains to the same channel number.

LOADING .mem FILES

------------------

A memory file can be loaded in one of 4 ways.  1) using the

<ShftF1> key and supplying the name of a .mem file or, 2) type 'load

fname.mem' <CR> from the command line, 3) the .cfg file can specify an

automatic load of a particular file. (see Appendix A) or, 4) a Menu

(.mnu) file selection can load a given .mem file. (see Appendix C).

HINTS ON WRITING .mem FILES

---------------------------

One 'trick' to use is to load a channel to capture the

configuration of the radio as it was when it was captured.  This is done

by NOT supplying parameters after a channel statement.  For instance if

the first statement in the .mem file is:

Channel=16

then channel 16 will be loaded with the default frequency, mode and vfo

that was in effect when the .mem file was called.

If CAT980 was in prior control and then exited with a <z>

leaving the FT-980 locked to the last configuration, a .mem file will

NOT be automatically read upon reentry to CAT980. You can use the load

<ShftF1> function if the file is required - or if the .cfg file calls

for a load you will be prompted to see if you want to reload it.

When CAT980 is exited by <AltX> then the FT-980 memories will

revert to the original parameters previously stored by the operator from

the front panel...PROVIDED YOU HAVE A BATTERY INSTALLED IN THE FT-980!

B-2

CAT980 Ver 2.0                                       Appendix B  Page 3

* A LOW BATTERY IN YOUR FT-980 CAN MAKE CAT980 ACT SCREWY! *

Here are two forms of valid load statements:

Channel=1               *load channel 1 (this is a comment)

Freq=1400100  Mode=cwn vfo=ham

Channel=2 Freq=2810000  Mode=USB vfo=HAM

LOADING THE CAT980 300 MEMORY SLOTS

===================================

Another unique function of your memory file is that it can load

special memory slots in the program for recall or scanning purposes. To

load an individual slot use the following statement format:

SLOT=0350000

This is the only allowable slot statement, i.e. a 7 digit

frequency. Note that NO slot number is provided or even allowed.  The

number of the slot which is loaded, 1 to 300, will depend on the last

slot loaded either from a .mem file or the keyboard.  Thus if the first

five slots were loaded from the keyboard with a <^s> then the statement

above will load slot 6 with 3.5 Mhz. The following would then load slots

7 thru 10:

SLOT=0360000

SLOT=0370000

SLOT=0380000

SLOT=2850000

* IMPORTANT: ALL MEMORY FILES MUST HAVE THE EXTENSION .MEM ! *

'HIGH SPEED' SLOT LOADING FROM THE .mem FILE

--------------------------------------------

To type up 300 such statements, say to do a spectrum scan, would

try the patience of the most experienced typist.  So CAT980 allows a

general symmetrical load of any string of statements up to 300 loads

with a special SLOTXX= statement formatted as:

B-3

CAT980 Ver 2.0                                       Appendix B  Page 4

SLOTXX=0100000/01000/202

|       |      |

|       |       - The number of slots to load. This must

The XX in      |       |         be 3 digits. So 8 slots would

SLOTXX=is NOT  |       |         be 008.

a number, it's |       |

exactly as     |       |

shown.         |        -The frequency DIFFERENCE between each slot.

|   This must be 5 digits. Range can be

|         from 10 hz (00001)

|         to 999,990 hz (99999).

|

-The actual frequency (here, 1 Mhz) the 1st available

slot will occupy. This must be 7 digits.

Again, this statement will begin the load in the first available

unloaded slot. In the previous example 10 slots had already been loaded.

Therefor this SLOTXX statement will begin its load in slot 11 and will

continue to high speed load the next 202 slots, each slot being 10 Khz

higher than the last.  If this statement were to load slots beyond the

FT-980 capability (29.999.990), or an attempt to load more than 300

slots - an appropriate error message will result.

The slash '/' characters in the 'high speed' statement are

necessary.

* A .MEM FILE CAN CONTAIN EITHER CHANNEL LOADS, SLOT LOADS OR BOTH *

* PRINT OUT THE .mem FILES IN THIS PACKAGE - THEY ARE TUTORIAL *

FILTER CHARACTERISTICS IN .mem FILES

------------------------------------

You can also command the FT-980 filter shift and width controls

from within a .mem file using the form:

ifwidth=xxx

|

|

|

|-100 to +100 (+/- 100%)

ifshift=xxxx

|

|

|

|-1500 to +1500 (+/- 100 hz increments)

B-4

CAT980 Ver 2.0                                       Appendix B  Page 5

Statement       Effect

------------    --------------------------------------------

IFWidth=000     centers the filter width

IFShift=0000    centers the filter shift

IFShift=-300    shifts the filter up band by 300 hz

IFShift=-350    INVALID! Will yield error msg -

Must be multiples of 100.  0000,+0100,-0100 etc.

IFWidth=-30     narrows skirt toward higher freq by about 30%

IFWidth=+40     narrows skirt toward lower freq by about  40%

* PLUS SIGN (+) IN FILTER STATEMENTS ARE NOT MANDATORY *

*IF YOU INCLUDE FILTER STATEMENTS MAKE THEM*

*THE LAST STATEMENTS IN THE .MEM FILE*

Examine the ASCII file examp20.mem supplied on your disk.  This

file is automatically loaded by the supplied .cfg each time the radio is

captured by CAT980 - unless reentering after a <z> exit. In that case

all 300 memory slots are cleared but the FT-980 memories will still

retain the initial load. You can reload the 300 slots from the keyboard

<ShftF1> by resupplying the examp20.mem file name.

Of course you can rewrite the config file to NOT load this file

or to load a .mem file of your creation.  See Appendix A.

* IMPORTANT: AFTER YOU LOAD A .MEM FILE YOU CAN RETURN TO THE *

* PREVIOUS SET UP BY TYPING 'RESTORE' OR <ShftF3> *

Study the included .mem files.

*ALWAYS PUT AN ASTERISK IN FRONT OF A COMMENT!*

-------------------------file examp20.mem start--------------------

* examp20.mem   5/2/92

*This is a major load - clear all slots first

ClearSlots=Yes

*Unlike .mnu files, keep load info on a single line for each channel

Channel=1  Freq=1425000  Mode=usb vfo=ham

Channel=8  Freq=1410300  Mode=fsk vfo=ham  *{packet madhouse freq}

Channel=12  Freq=0108000  Mode=amn vfo=gen  *{am broadcast band}

Channel=15 Freq=2960000  Mode=fm vfo=ham *This is a simplex FM freq

Channel=16 Freq=2966000  Mode=fm vfo=ham *This is a repeater output freq

B-5

CAT980 Ver 2.0                                       Appendix B  Page 6

---------------------file examp20.mem continued-----------------

Slot=0350000   *this is how you load slots individually

Slot=0360000

Slot=0370000

Slot=0380000

Slot=0390000

Slot=0400000  *this slot will work in GEN but not HAM: 3.999.990 is tops

*The following is a high speed multi slot load command. It will load

*the 1st available slot (#7 - since 6 slots were loaded above) and each

*slot will be 100 Khz higher than the previous one. The command for 229

*slots:

SlotXX=0710000/10000/229

*If we try one more slot we'd pass 2999999 and get

*an error msg.  Try it(change 229 to 230)!

*Just like these comments, you can put them in a .mem file too.

*   K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

---------------------file examp20.mem  end----------------------

Let's examine one other .mem file that is a little different. It shapes

filters, sets up a mode and frequency and loads the slots with a pre

scan set of frequencies.

---------------------file bcon20.mem start---------------------

* This file loads 10 Meter beacon frequencies for use with the scanner

* function - this command will load a slot every 300 hz from the entry

* frequency. We don't have to but let's save the present set up in the

* FT-980 memory channel number one.

channel=1

*That did it - ok, now on with the memory load.

* ALWAYS DO THE MEMORY LOAD STATEMENTS BEFORE ANY FT-980 COMMANDS

* The next statement clears out the buffer and then loads the slots.

ClearSlots=yes

SlotXX=2810000/00030/300

*This loaded 300 slots every 3000 Hz

B-6

CAT980 Ver 2.0                                       Appendix B  Page 7

---------------------file bcon20.mem continued-----------------

* NOW THAT THE LOAD HAS BEEN 'FED' TO THE PROGRAM, IT'S OK TO COMMAND

* THE RADIO INTO A DESIRED CONFIGURATION.

* So now let's build our own 'cw' filter from the FT980 LSB filter

ifshift=-900

ifwidth=-57

mode=lsb

* Then let's set up on the 1st beacon frequency ready for a scan!

vfo=ham

freq=2810000

* To start the scan hit <F9> and pick the marker scan from the menu.

* AFTER THE LOAD YOU CAN RETURN TO THE ORIGINAL SET UP BY ENTERING

* <ShftF3> OR TYPING 'RESTORE' <CR> - AND THIS FILE ALSO SAVED THE

* ORIGINAL SET UP IN THE FT-980 MEMORY CHANNEL NUMBER ONE.

---------------------file bcon20.mem end------------------------

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=== END APPENDIX B ===

B-7

CAT980 Ver 2.0                                       Appendix C  Page 1

APPENDIX C

WRITING CAT980 MENU FILES

=========================

Writing menus is probably one of the most satisfying and fun

things to do to make CAT980 come alive as a day to day operating aid.

Everyone's interests are different and every op has his own special

operating preferences.  You have complete control of your menus and each

menu selection virtually has complete control of the radio at the stroke

of a key.  You can define modes, filter characteristics, FSK shift,

frequency and even call up split operation for each and every entry on

the menu can be written to your specifications.

You can define band limits and even define entire short wave

bands.  You can nest the menus, that is have one menu selection call up

a brand new menu. Or crate a master menu where all the selections call

other menus.  And add a menu selection to "back you out" to the last

menu - etc. etc.

You can develop special interest menus.  Digital operation,

sideband, am, cw, fm (including 10m repeaters!) and unique shortwave

broadcast station listings.

You can color code the result of a given menu selection. Screen

colors can be commanded along with deployment of the FT-980.

*SHARE YOUR UNIQUE KNOWLEDGE OF THE ELECTROMAGNETIC SPECTRUM*

*BY SHARING YOUR MENUS WITH OTHER CAT980 USERS!*

Lets take a look at general menu construction and, like all the

CAT980 files you write, be sure and create it as an ASCII file.

THE MENU TITLE

--------------

You can supply an ASCII string up to 60 characters long which

will display at the top when you call in the menu from the program.  Any

printable characters are allowed.  Example:

TITLE= T H I S   W I L L  P R I N T  A S  A  T I T L E

title=So Will This!

*IF YOU OMIT THE TITLE= STATEMENT THEN THE DEFAULT IS 'Menu'*

C-1

CAT980 Ver 2.0                                       Appendix C  Page 2

MENU ENTRIES

============

'Two Line' Construction

-----------------------

Each entry consists of 2 or more lines, the first line is an

ASCII text line that starts with two closed brackets enclosing a number.

A menu with 3 selections would have the following form:

[1] Write your 1st menu description here. Entire line will display.

(this is the 2nd line - we'll talk about it in a moment)

[2] Write your 2nd menu description here. This line will display.

(the 2nd line contains command statements for this selection)

[3] Write your 3rd menu description here. This line will display.

(the 1st line displays on your screen, these 2nd lines do NOT)

The First Line

--------------

The consecutive [1], [2] and [3] is very important as this is

how CAT980 understands your selection sequence. You can have 75

characters in the 1st line including the [xx] portion.  You may have

from [1] to [21] entries for any one menu file. Use any upper/lower case

characters but do NOT use brackets, [], except at the beginning of the

line for the selection number.  Keep your selections numbered and in

order.

All recognized statements will be honored on every line until a

new menu selection [xx] is detected.

* ONLY USE BRACKETS [] AT THE START OF AN ENTRY *

A valid 1st line might look like this:

[1]  3.850111 Wheelin' & Dealin' Southwest Swap Net

The Second Line

---------------

Actually the 'second' line can be any number of lines after the

bracketed [] first line.

CAT980 will execute all valid commands that appear on these

lines, the '2nd line', immediately after the menu entry line, the 1st

line.  An example for a valid 2nd line might look like:

MODE=LSB VFO=HAM FREQ=0385010

C-2

CAT980 Ver 2.0                                       Appendix C  Page 3

Now let's combine the 1st line we created above along with our

second line. Now we have:

[1]  3.850111 Wheelin' & Dealin' Southwest Swap Net

MODE=LSB VFO=HAM FREQ=0385010

If this menu were to be called up we would see the 1st line on

the screen - but not the 2nd. And if we choose selection [1] the radio

will 'instantly' deploy to 75 meters, lower sideband, HAM vfo and a

frequency of 38501000 Hz - not as line 1 shows 38501110, Hz because that

is not a command, just plain text and purposely shown as a 'typo' for

our example.

After you select [1] your plain text 1st line will now appear at

the top of the screen to remind you of what you have chosen. The rest of

the screen will reflect the changes which took place as a result of the

2nd line.

OK, if we added some more selections to the menu it might begin

to shape up like this:

[1]   3.850111 Wheelin' & Dealin' Southwest Swap Net

MODE=LSB VFO=HAM FREQ=0385000

[2]  20 Meter Packet Mail Box (14.107)

FREQ=1410745  VFO=HAM CONTROL=VFO MODE=FSK FSK=170 IFSHIFT=-400

IFWIDTH=76

[3]  SPLIT 10 M FM Operation (W2IBJ Virgin Islands - and others!)

MODE=FM VFO=HAM FREQ=29560000 CHANNEL=16 CONTROL=RXM

Looking again at selection 1, [1], note that on the 2nd line

only those commands actually needed are included.  Invariably this will

be MODE, VFO and FREQ for all menu entries that are intended to deploy

the radio.  But more may be required for special listings.

For example take a close look at selection 2, [2], which is for

a packet set up. What's needed? An additional command to insure an FSK

shift of 170 hz is included. (Why? Well maybe you had been up on 10

meter packet and were running wide shift, 850 Hz.) And here's a case

where the screen shows the frequency as 14.107 but to achieve packet

reception with the FT-980 the radio is tuned higher. (See 'Hints and

Kinks', QST, June 1992)

Now let's get tricky; study selection 3, [3].  Looking at the

2nd line, the radio is commanded into an operating FM mode on a HAM vfo

frequency of 29.56 Mhz. The next command points to channel 16 in the

FT-980 which was previously loaded with the 10 m. FM frequency of 29.66

Mhz (as it was by the included file examp20.mem). Now comes a RXM

control command meaning that the radio will receive on the selected

memory channel (16), 29.66 MHz, and transmit on the ham vfo frequency of

29.56 MHz. It could have been the other way around if the statement had

C-3

CAT980 Ver 2.0                                       Appendix C  Page 4

been CONTROL=RXV - but that would have put you in a reverse mode known

as QRM City.  In either case an FT-980 memory channel has to contain at

least one of the frequency set ups for split operation.  This can be

done from the keyboard prior to calling up the menu - or better yet,

automatically with a .mem file as above.

If you have brought the program up in its initial supplied form,

then the file demo20.mnu has already been loaded. (If not you can load

it with a <ShftF1>.)  Hit the <SpaceBar> and choose selection [3] either

by entering <3><CR> or use the arrows to get on [3] and do a <CR>. Now

you can see the action of the above discussion.  Notice the meaningful

analog screen presentation with Tx frequency identified and the

displayed info showing the receiver tuned to an FT-980 memory channel

(RxM).

* FOR SPLIT OPERATION AN FT-980 MEMORY CHANNEL MUST CONTAIN *

* ONE OF THE FREQUENCY PAIR! *

The most obvious use of such a menu selection as [3] are the 10

meter FM repeaters around the world.  Any split operation such as DX'ing

on 40 meter SSB; listening to 7.07 Mhz and transmitting on 7.2 Mhz.

Don't forget the <RightShift> frequency spotting key available to help

keep the peace!

THE PARAMETERS AVAILABLE FOR A MENU SELECTION

=============================================

STATEMENT       RANGE OF ALLOWABLE PARAMETERS

---------       -----------------------------

VFO=            HAM GEN

MODE=           LSB USB CWW CWN AMN AMW FSK FM

FREQ=           0000150 to 2999990 must be 7 digits

FSK=            170 425 850 (FSK hz shift)

CONTROL=        VFO MEM RXM RXV

CHANNEL=        1 thru 16 (FT-980 memories)

IFWIDTH=        -100 to +100 (+/- 100%)

IFSHIFT=        -1500 to +1500 (+/- 100 hz increments)

Those last two? - YES, you can control the filter

characteristics with each menu selection!  Here's how:

Statement       Effect

------------    --------------------------------------------

IFWidth=000     centers the filter width

IFShift=0000    centers the filter shift

IFShift=-300    shifts the filter up band by 300 hz

IFShift=-350    INVALID! Will yield error msg -

Must be multiples of 100.  0000,+0100,-0100 etc.

IFWidth=-30     narrows skirt toward higher freq by about 30%

IFWidth=+40     narrows skirt toward lower freq by about  40%

C-4

CAT980 Ver 2.0                                       Appendix C  Page 5

* PLUS SIGN (+) NOT MANDATORY IN FILTER STATEMENTS *

You will find statusing of the FT-980 from CAT980 to show a

slight discrepancy of the IF width from the entered menu values.  The

Heisenberg uncertainty principal could be cited but you would be correct

in not believing that.  It is due to the FT-980 hardware response to the

command. It is so close that no human ear can detect it and it was not

worth the code to make it exact.  But enough difference for the human

eye to detect it so this writer thought it prudent to bring it up to

ward off any incoming fire.

Bottom Line: You can believe your entries will yield the desired

results.

* EACH MENU SELECTION AUTOMATICALLY CENTERS THE FILTERS BEFORE IT *

* LOOKS FOR ANY FILTER SHAPING COMMANDS! *

The values for these two filter parameters are probably best

picked during actual operation.  Let's say you are in a 40 m. cw QSO

with W1XXX and using the filter display feature <F8> you find the best

copy is with an IF width of 38.58% and a positive shift of 200 hz.  You

and W1XXX decide to set up a schedule so you want to add him/her to the

menu.  Your entry might look like:

[4] W1XXX 40 M. Sked. Tuesday nites - 2000 local

freq=0703185 mode=cww vfo=ham

ifshift=+200 ifwidth=+39

* THE 'SECOND LINE' CAN ACTUALLY BE 2 OR MORE LINES *

* UNTIL CAT980 DETECTS THE NEXT MENU SELECTION BRACKETS [] *

* REMEMBER - YOU CAN ALWAYS CENTER ALL FILTERS FROM THE KEYBOARD *

* WITH <ShiftF8> *

CAT980 is structured to perform your statements in the correct

order demanded by the radio - even though you may put the commands out

of sequence (I think). However, if it seems a particular entry isn't

behaving go back and sequence your command statements in the same order

you would manually have performed them from the front panel to get the

desired result. (Do you detect a touch of uncertainty in Dr.

Frankenstein about his monster's behavior? You're right! Hi hi)

* WORRIED ABOUT THE ORDER OF THE STATEMENTS?  USE COMMON SENSE. *

* WRITE THE MENU SEQUENCE AS YOU WOULD MANUALLY *

* PROGRAM FROM THE FRONT PANEL *

C-5

CAT980 Ver 2.0                                       Appendix C  Page 6

By the way, there is often confusion and an actual

difference from where a receiver is tuned compared to a published

frequency - like RTTY center and/or mark frequencies.  You can have your

1st line displaying the accepted published frequency - but your 2nd line

can put the rig right were it belongs!

* M.A.R.S. OPS CAN CREATE MENUS FOR QUICK CHANGE OPERATION *

* ESPECIALLY IF THE FT-980 HAS THE GEN TX MODIFICATION! *

DEMO MENU

=========

To get familiar with menu operation it is strongly suggested that

you print out the included ASCII demo files (print *.mnu).  Then try the

various selections as you examine the statements that produced them.

* THE SUPPLIED .MNU FILES ARE TUTORIAL IN NATURE *

* PRINT 'EM & READ 'EM - NOTICE THE LIBERAL USE OF COMMENT LINES *

COMMENT LINES: For faster file processing put an asterisk (*) at

the beginning of each comment line.

*COOK UP SOME MENUS AND SHARE THEM WITH US!*

ADVANCED MENU WRITING

=====================

Besides just setting up frequency/mode/filter combinations menu

selections, menus can also call other menus, set up frequency spectrums,

change colors, turn the beeper on and off, load other files including

memory (.mem) files  There's more. Let's look at the features one at a

time.

* REMEMBER THERE IS ALSO AN AUTOMATIC MENU BUILDING FEATURE *

* SIMPLY ENTER 'MENU' <CR> OR <AltP> *

MENU SELECTION 'ZERO' - THE ENTRY MENU HEADER COMMANDS

------------------------------------------------------

A number of commands may be included prior to the first

selection, [1],  and will be executed when the menu is first called to

the screen via <SpaceBar>. We've already examined the statement TITLE=.

Presented here is part of the header from SWL20.MNU but with line

numbers in parenthesis added for this discussion.

C-6

CAT980 Ver 2.0                                       Appendix C  Page 7

---------------------------------------------------

| (1) title=Shortwave Broadcast Bands

|

| (2) *Let's make this menu a different color

| (3) MenuColor=k,y          *a reverse video effect

|

| (4) *Then let's change the color of the bottom status boxes (except

|     *clock)

| (5) box2=k,g

|

| (6) * And Let's have the utility files deal with Zulu time and we'll

|     * create or append a log file just for this activity.

|

| (7) Zstamp=yes LogFile=SWL.LOG

|

| (8) [1] 25 Meter Band (11.65 - 12.05 Mhz)

| (9)  SetTabGen=1205000 1165000 AutoTabs=yes vfo=gen mode=amw

|  *Note that these events occurred simply by loading this menu -before

|  *any, if any, selection will be made...

|  *==================================================================

- - - - - - - - - - - - - - - - - - - - - - - - -

Zstamp=yes LogFile=SWL.log

So what's happening here?  First keep in mind that when an

asterisk * is encountered everything after it for the rest of the line

will be ignored by CAT980 as a comment.

So line (1) will set up the title for the menu and will appear

at the top when you call the menu.

The next line (2) is a comment but line (3) contains a color

change command statement for the menu - followed by a comment. (WHAT?!

Read another appendix?!  Yes, the color statements are covered in

Appendix E.)

Line (4) is a comment, line (5) changes the color of the bottom

status boxes - again, see Appendix E.

Line (6) is a comment and then comes two important command

statements on line (7): the first turns on the UTC (zulu) date and time

stamp for a log file named SWL.LOG, defined by the second command

statement in this line - followed by a comment.  You can create as many

different log files as you choose. You could have an SWL log, WAS log,

W1AW log, BC log.  You can create these menu on the fly.  After the log

file exists you can call it with a menu file or bring it in with

<ShftF1> or the 'load fname.mnu' command. Note that the SWL.LOG now

shows in the active file display box.  To access the log file simply

type 'LOG' <CR>.

C-7

CAT980 Ver 2.0                                       Appendix C  Page 8

OK, at this point the header commands are over and the familiar

Remember, all the commands prior to line (7) happened before

any, if any, menu selection is made by the operator.  When the menu

appears these actions have already taken effect. See Appendix F for a

listing of the command statements which can be used in this part of the

menu.

* HOW CAN YOU KEEP TRACK OF ALL THE ALLOWABLE STATEMENTS/COMMANDS? *

* APPENDIX F IS A TEMPLATE AID IN HELPING YOU WITH THIS CHORE *

MULTI COMMAND LINES FOR EACH MENU SELECTION

-------------------------------------------

REMEMBER, YOU CAN INCLUDE MORE THAN ONE COMMAND LINE FOLLOWING THE

DISPLAY LINE IN A MENU.  The 'second line' can actually be the second,

third, etc.; all recognized statements will be honored on every line

until a new menu selection [xx] is detected.

Look at selection [2] in the menu file qrp20.mnu:

[2] CW - 80 Meters - 3.560 MHz Shift to CAT980 Filter Configuration

FREQ=0356000  VFO=HAM CONTROL=VFO MODE=CWW

cat980=yes

The second line puts the radio on the low end of 80 meters in CW

mode.  But notice the next line: CAT980=YES.  This changes the filter

relationships to each other such that a CW signal producing a tone

matching the back panel setting, say 700 Hz, will also produce a 700 Hz

tone in USB, LSB and FSK!

Whenever a selection is made all filters are first centered and the

filter configuration is put back into YAESU's original mode.  You can

see this action by observing the screen display while first selecting

[2] and then selecting [1].

LOAD A MENU ON CALL-UP OR FROM OTHER MENUS

------------------------------------------

YOU CAN LOAD A MENU FILE ON PROGRAM CALL-UP WITH CAT980.CFG

FILE. This means you can load an initial menu from the .cfg file which

will be ready for use (See Appendix A). Instead of going through the

<ShftF1> file load process, you simply hit the space bar and the menu

will appear.  Of course you can still use the <ShftF1> feature when you

want to. To load a menu just include the following statement in your

cat980.cfg:

load=demo20.mnu

C-8

CAT980 Ver 2.0                                       Appendix C  Page 9

This same command may be used in a menu selection itself -

indeed, the only command of a given menu selection may be the load=

statement. That boils down to the fact you can create menus that call

other menus! Take time to look at the file demo20.mnu to see how this is

done. Selection 14 from that menu is:

[14]Shortwave Broadcast Menu

Load=swl20.mnu

And check the action when you choose selection 14. The new menu

pops up on the screen ready to use - and notice that the screen colors

have changed!  Color code your operation (red for contests?).

THE POWERFUL SHOTGUN 'DOUBLE BARREL LOAD'

-----------------------------------------

Here is menu selection number 6 from the menu file DEMO20.MNU:

[6] 10 Meter FM Repeater Menu

load=tenrpt20.mem

load=fm_rpt20.mnu

It does what you might expect:  1) load a memory file, 2) load a

menu file.  And the result is complementary.  In this case the memory

file loads the FT-980 memories with 10 meter repeater output frequencies

(and a bit more, actually) and then the menu pops up ready for all the

standard 10 meter repeater splits. If you'll give this tool some thought

it is probable that you can come up with many uses for this type of

combination.  Take time to check out this action and read the comments

in all three files involved.

SETTING AND USING THE FT-980 BAND TABS

======================================

SET UP FREQUENCY SPECTRUM BLOCKS WITH A MENU SELECTION

------------------------------------------------------

Check out selection 19 in demo20.mnu:

[19] Novice Bands - Set Band limits for 80, 40, 15 and 10 meters

VFO=HAM

SetTab80=0375000 0370000 SetTab40=0715000 0710000

SetTab15=2120000 2110000

SetTab10A=2850000 2830000 mode=cww freq=0371000 AutoTabs=yes

This one selection will set the FT-980 TABS to ALL HF U.S.

Novice band edges and then configure the radio for some CW 80 meter

action. It could have been written to settle down in the 10 meter Novice

phone band on USB just as easily.

But remember when you construct a menu you should use the same

sequential logic you would use if you were manually commanding the

C-9

CAT980 Ver 2.0                                      Appendix C  Page 10

FT-980 from the front panel. Most important you would put the VFO in HAM

in this case.  Your menu command should do the same.

Note also the use of the command:

AutoTabs=yes

The TAB functions and how they work are covered in Appendix D.

This statement insures that the radio will activate the tabs

automatically. Tabs in the band of operation, in this case 80 meters

dictated by the 'freq=' command, will be activated.  Now do a band-up,

<^PgUp>, and notice that the tabs in the 40, 15 and 10 meter bands are

also set to the new values and are activated automatically. Using the

<HOME> <END> keys will take you from edge to edge and <^j> will put you

in the middle.

Unlike a menu selection calling another complete menu, this

selection retains the menu from which it was called.  That is if you

were to hit <SpaceBar> the same menu comes up.  But you could have

created a HAM BANDS menu with selections for NOVICE, ADVANCED, EXTRA,

CW, PHONE - on and on and on.

A selection like this also would ignore the tab statements during a

menu scan. As this selection comes by it will be treated as:

vfo=ham mode=cww freq=0371000

* TO RESET ALL FT-980 TABS FROM THE KEYBOARD ENTER <^w> *

* OR TYPE 'CLEAR TABS' <CR> *

READ ONLY IF YOU ARE UPGRADING FROM VERSION 1.8

===============================================

Your Version 1.8 menus are compatible with Ver 1.9 operation

except that:

1. PLEASE REVERSE THE SIGN OF ANY 'WIDTH=' STATEMENTS IN YOUR VERSION

1.8 MENUS! (i.e., if you have a 'width=50' then change it to 'width=-50'

and if you have 'width=-10' make it 'width=10' or 'width=+10'!)  DO NOT

CHANGE ANY SIGN ASSOCIATED WITH THE 'SHIFT=' STATEMENTS!

2. And - when you get time please put an asterisk * at the start of any

comment or comment line.  Your present 1.8 menus should still work OK

without this addition, but they will be processed much quicker when you

add the *.

Do not, of course, put an asterisk * before a desired, valid command

statement otherwise it will be ignored.

3. Your Ver 1.9 files should require no changes.

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=== END APPENDIX C ===

C-10

CAT980 Ver 2.0                                      Appendix C  Page 11

C-11

CAT980 Ver 2.0                                       Appendix D  Page 1

APPENDIX D

WORKING WITH FT-980 TAB STATEMENTS

----------------------------------

The 'SetTab=' command statement makes very good use of the

FT-980 tab features which allows you to configure your radio into

specific band segments. For instance, Advanced and Extra segment

privileges can easily be defined and installed through menu selections.

And the SWL enthusiast should be delighted with this tool which can

define shortwave broadcast spectrums to be scanned and explored.

Chances are you never really used the TAB features very much in

manual operation.  (I didn't; they seemed a bit awkward to me.)  Yet

when coupled with the power of our computers they can offer some real

solid help for everyday operating.

OK, the format for setting tabs takes the following form:

SetTabBandNo=(7 digit Hi Tab Frequency),(7 digit Low Tab Frequency)

On 80/75 meters this would be:

SetTab80=0399999 0350000

Why wasn't the high tab set to 0400000, the high end of the

75/80 meter band? Keep in mind how the radio itself behaves. In the ham

vfo you can't achieve 4.0 MHz from the radio. (Who says?! No way; try it

from the front panel.)

The syntax of the SetTAb statements is:

SetTabxxx Hi freq|Lo Freq

--- -------|-------

|   |      ||

|   |      ||-must be seven digits

|   |      |-(you can use a comma or a space)

|   |

|   |_must be seven digits

|

|METER BAND

|_(160,80,40,30,20,17,15,12,10A,10B,10C,10D,GEN)

D-1

CAT980 Ver 2.0                                       Appendix D  Page 2

CAT980 Tab Defaults

-------------------

CAT980 already knows the US band limits.  Here are the tab

defaults of CAT980. If you were to include the following statements

nothing would change:

SetTab160=0199999 0180000

SetTab80=0399999 0350000

SetTab40=0730000 0700000

SetTab30=1015000 1010000

SetTab20=1435000 1400000

SetTab17=1816800 1806800

SetTab15=2145000 2100000

SetTab12=2499000 2489000

SetTab10A=2849999 2800000

SetTab10B=2899999 2850000

SetTab10C=2949999 2900000

SetTab10D=2999999 2950000

SetTabGen=2999999 0015000

Defaults, yes, but you have control over these and you can

change any or all to redefine the band edges. Hams in countries other

than the U.S. can customize these limits to fit their regulations.  In

fact you are encouraged to do so. You can do this with a set of

statements in the config (CAT980.CFG) file - or better yet have your

.cfg file load a special .mem file for your band edges.

Note that the GENeral vfo tabs are set for the entire general

spectrum of coverage. If you wanted to have the config file or a menu

selection set up the 19 meter shortwave broadcast band, say 15.1 Mhz

thru 15.6 Mhz, the statement would read:

SetTabGen=1560000,1510000

If you wanted to set up the 10 meter novice phone band the

statement is:

SetTab10A=2849999 2830000

* WANT TO HAVE A MENU SELECTION RESET ALL TABS? *

TabsReset=YES

TabsReset=YES is a related statement which will reset all tabs

to the original program state as defined in the table above. Suppose you

had different defaults set up by the config file? You should create a

special .mem file to reload band edges that are different from the

D-2

CAT980 Ver 2.0                                       Appendix D  Page 3

defaults; look at CWtabs.MEM. (You could have done it with a special

single menu selection too!)

* NOT SURE WHERE THE TABS ARE SET? *

* SIMPLY TYPE 'SHOW TABS' <CR>! *

These statements can be used within menus as well as the .cfg

file and multi statements per line are allowed. In one menu selection

you can redefine a number of bands.  Here from DEMO20.MNU is an example

of this capability:

[19] Novice Bands - Set Band limits for 80, 40, 15 and 10 meters

VFO=HAM

SetTab80=0375000 0370000 SetTab40=0715000 0710000

SetTab15=2120000 2110000

SetTab10A=2850000 2830000 mode=cww freq=0371000 AutoTabs=yes

Take the time to examine the use of the SetTab family of

statements in the supplied menu (.mnu) files.  When used creatively it

could add a whole new dimension to your operating tool box.

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=======END OF APPENDIX D=======

D-3

CAT980 Ver 2.0                                       Appendix E  Page 1

APPENDIX E

WORKING WITH CAT980 COLOR STATEMENTS

====================================

* IF YOU DON'T HAVE A COLOR MONITOR PUT THE FOLLOWING STATEMENT *

* IN THE CAT980.CFG FILE *

MONO=YES

USING COLOR WITH CAT980

-----------------------

Color can be an important part of CAT980. Nevertheless if you're

a Hercules user like the author was during the early development of this

program, you will still have the benefits of mono graphics. (Cat980,

Version 1.8 was developed initially on a Hercules monitor.)  If you

don't have a color monitor or are using a B&W lap top, then use the

statement, 'mono=yes', in your cat980.cfg file.

But control of the color by those who wish it can be had by

including certain statements where appropriate in the .cfg, .mem and

.mnu files.  Basically there are two sets of colors that are part of the

program which we will examine next.

COLOR SETS

==========

Color Set No. 1 below (k,b,g...) can be used for both text and

background colors.  Color Set No. 2 (K,B,G...) are for text colors only.

Depending upon your monitor you may spend sleepless nights wondering who

named some of these colors!  These names are coupled with software

triggers that are an industry standard. So if you don't like 'brown',

well.... that's what the big boys call it so for consistency we will

observe convention.

Here's how it works in writing your .cfg, .mnu or .mem files.

When specifying colors in CAT980 files use the following letters to

designate the color of choice:

____________        _________________

| COLOR SET  |      |  COLOR SET      |       COLOR STATEMENTS ARE

|   NO. 1    |      |    NO. 2        |       CASE CONSCIOUS.  USE

|____________|      |_________________|       LOWER CASE WHEN RE-

| k  black   |      |  K grey         |       FERRING TO COLOR SET

| b  blue    |      |  B light blue   |       NO. 1, AND UPPER CASE

| g  green   |      |  G light green  |       WHEN REFERRING TO

| c  cyan    |      |  C light cyan   |       COLOR SET NO. 2.

| r  red     |      |  R light red    |

| m  magenta |      |  M light magenta|

| y  brown   |      |  Y yellow       |

| w  white   |      |  W bright white |

|____________|      |_________________|

USE THIS SET         USE THIS SET FOR

FOR TEXT &           TEXT COLORS ONLY

BACKGROUND

E-1

CAT980 Ver 2.0                                       Appendix E  Page 2

ANALOG DIAL COLORS

==================

Let's start with a simple command that can be included in any

CAT980 user files:

HamDial=C

When this statement appears your analog ham dial will be colored

'light cyan' according to the tables above. This happens to be the

default so you need not include it to realize it. If you want 'red' then

make the statement:

HamDial=r

Since the analog dial is made up of text type characters you can

use a color from either color set 1 or 2.  A similar statement will

color the General VFO dial:

GenDial=y

This is also the default for the general dial. You can change

dial colors with each selection of a menu file if you like.

WARNING: You can pick not only an ugly color, you can pick an invisible

color!  The statement:

HamDIal=k

will display black characters - on a black background! Try to read that

without x-ray vision!

COLOR COMBOS - THE BOXES OF CAT980

==================================

PAIRING BOX COLORS

------------------

A number of box color combinations are used which you can also

change from your files. The format is:

Box0=W y

| | |

| | -- Background color from set 1

| |

|  ---Text color from set 1 or set 2

|

---Box identifier, see table below

E-2

CAT980 Ver 2.0                                       Appendix E  Page 3

Where in this case Box0 is white characters on a brown (looks

red to me!) background. Box0 is the Help File activated by pressing <F1>

and the reverse, brown on white, will be the default of any menu that

doesn't specify a different color combination.  Use the following to

experiment (or don't!) to bring out the artist in you. Remember, you can

choose not only the 'ugly' but the invisible.

Box0=k r

- - -

| | |_ BACKGROUND - Any letter from Color Set 1

| | (you can use a comma instead of a space: Box0=k,r)

| |

| |_ FOREGROUND - Any letter from Color Set 1 or 2

|

---|

Box0   Help File (<F1>)

User Menus (displays reverse of)

Box1   Exit Options

Box2   Bottom row Status Boxes(OP FREQ, etc)

Box3   Pop Up Controls (<F3-F5-F6>)

Box4   F9 Pop Up (SCAN)

Box5   Screen Clock

Box6   Active Filter DIsplay (F8)

Box7   Active Memory DIsplay (F7)

Box8   Pop Up Radio Status (<^q>)

Log Utility (<^p>)

Box9   Pop Us System Status (<^q>)

Construct Menu Utility (<AltP>)

These are the defaults of the color boxes. You can put multiple

Box0=W y    Box1=w k    Box2=k y    Box3=W c    Box4=Y b

Box5=k c    Box6=k g    Box7=B y    Box8=Y k    Box9=G k

HOW TO PICK AND RESET COLORS <AltC>

-----------------------------------

You can get into 'color trouble' after loading a few menus and

memory files - at least the potential for a wild color mix is there.

Simply enter <AltC> or type SHOW COLORS <CR> and a color chart showing

the latest will appear.  You also will be prompted as to whether you

want to reset all colors to the defaults shown above. If you answer 'Y'

then the chart will change to those colors.

This chart can be of great value to you to choose the colors you

may wish to use in your files.  You can actually see what the colors

look like on your monitor - and maybe even agree with me that brown

looks like it's red. hi hi

E-3

CAT980 Ver 2.0                                       Appendix E  Page 4

WHICH FILES ACCEPT COLOR STATEMENTS?

====================================

ALL your CAT980 readable files will read and use the color

statements. Use them in your cat980.cfg, .mnu (menu) and .mem (memory)

files.

In your menu files the color statements can be used in two ways.

If they appear at the top of the menu prior to the selections then they

will take effect when the menu is called. Statements may also be used

with individual menu selections and will take effect only when that item

is selected.

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=========END OF APPENDIX E========

E-4

CAT980 Ver 2.0                                        Appendix F Page 1

APPENDIX F

CAT980 VERSION 2.0 QUICK REFERENCE USER FILE COMMAND STATEMENTS

������������������ͻ

Use to write .CFG, .MEM & .MNU files...     �    OK TO USE IN  �

�������������������������������������������������������������������͹

�STATEMENT    �      ALLOWABLE PARAMETERS        �.MNU �.MEM � .CFG �

�������������������������������������������������������������������͹

�AUTOTABS=    � YES                              �  *  �  *  �  *   �

�BFO=         � 500  600  700                    �     �     �  *   �

�BELL=        � YES NO                           �  *  �  *  �  *   �

�KEYNUM=      � YES NO                           �  *  �  *  �  *   �

�������������������������������������������������������������������Ķ

�HAMDIAL=     � Color Statements Are Of The Form �  *  �  *  �  *   �

�GENDIAL=     �     STATEMENT=Fcolor,Bcolor      �  *  �  *  �  *   �

�BOX0=        �  Where Fcolor=kbgcrmywKBGCRMYW   �  *  �  *  �  *   �

�thru Box9=   �       And Bcolor=kbgcrmyw        �     �     �      �

�             �      Example Box7=B,y            �     �     �      �

�RESETCOLORS= � YES                              �  *  �  *  �  *   �

�������������������������������������������������������������������Ķ

�TIMEZONE=    � xxxxx (Any 5 or less characters) �     �     �  *   �

�ZULUSHIFT=   � +/- 1 to 12                      �     �     �  *   �

�������������������������������������������������������������������Ķ

�LOAD=        � .mem .mnu file names             �     �     �  *   �

�������������������������������������������������������������������Ķ

�LOAD=        � .mem .mnu file names in a selection  * �     �      �

�������������������������������������������������������������������Ķ

�LOGFILE=     � .log file names                  �   * �  *  �  *   �

�TIMESTAMP=   � YES NO                           �   * �  *  �  *   �

�ZSTAMP=      � YES NO                           �   * �  *  �  *   �

�������������������������������������������������������������������Ķ

�CLEARSLOTS=  � YES                              �   * �  *  �  *   �

�SLOT=        � any 7 dig freq 0015000 to 2999999�   * �  *  �  *   �

�SLOTXX=      � 7 dig freq/5 dig freq diff/3 dig �   * �  *  �  *   �

�             �Example 0100000/01000/030 will begin loading slots   �

�             �       at 1 MHz, every 10 KHz for 30 slots    �      �

�������������������������������������������������������������������ͼ

(continued next page)

F-1

CAT980 Ver 2.0                                        Appendix F Page 2

CAT980 VERSION 2.0 QUICK REFERENCE USER FILE COMMAND STATEMENTS

(continued from previous page)

������������������ͻ

Use to write .CFG, .MEM & .MNU files...     �    OK TO USE IN  �

�������������������������������������������������������������������͹

�STATEMENT    �      ALLOWABLE PARAMETERS        �.MNU �.MEM � .CFG �

�������������������������������������������������������������������͹

�DIRECTSCREEN=� YES                              �     �     �  *   �

�COMM=        � 1 thru 4 (port to use)           �     �     �  *   �

�CHECKTX=     � ON                               �     �     �  *   �

�TIMEFREQ=    � any 7 dig freq 0015000 to 2999999�     �     �  *   �

�MONO=        � YES                              �     �     �  *   �

�KEYNUM=      � YES NO                           �   * �  *  �  *   �

�BIGFREQNUM=  � YES NO                           �   * �  *  �  *   �

�SHOWFILES=   � YES NO                           �   * �  *  �  *   �

�BLINK=       � YES NO                           �   * �  *  �  *   �

�WATCHDOG=    �                                  �     �     �      �

�������������������������������������������������������������������Ķ

�TABSRESET=   � YES                              �   * �  *  �      �

�SETTAB160=   � UpperTab (7 dig),LowerTab (7 dig)�   * �  *  �  *   �

�thru         � Example SetTab20=1417500,1415000 �     �     �      �

�SETTAB10D=   � for 20 meter Extra phone privileges    �     �      �

�������������������������������������������������������������������Ķ

�VFO=         � HAM GEN                          �   * �  *  �      �

�MODE=        � LSB USB CWW CWN AMN AMW FSK FM   �   * �  *  �      �

�FSK=         � 170 425 850                      �   * �  *  �      �

�CHANNEL=     � 1 thru 16                        �   * �  *  �      �

�FREQ=        � any 7 dig freq 0015000 to 2999999�   * �  *  �      �

�CONTROL=     � VFO MEM RXM RXV                  �   * �  *  �      �

�IFWIDTH=     � +100 to -100 (for +/- 100 %)     �   * �  *  �      �

�IFSHIFT=     � +1500 to - 15000 ( for +/- Hz)   �   * �  *  �      �

�������������������������������������������������������������������Ķ

�FT980=       � YES  {YAESU's filter config}     �   * �  *  �      �

�CAT980=      � YES  {BFO filter config}         �   * �  *  �  *   �

�AFSK=        � YES  {AFSK mark filter config}   �   * �  *  �  *   �

�AFSKmark=    � 100 to 4000 (freq of mark tone)  �   * �  *  �  *   �

�������������������������������������������������������������������ͼ

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=======END OF APPENDIX F=======

F-2

CAT980 Ver 2.0                                                   Appendix G 1

APPENDIX G

YAESU(tm) FT-980 TO IBM COMPUTER INTERFACE CIRCUIT

The circuit on page 2 of this appendix is all that is necessary

to get your FT-980 and IBM compatible computer 'talking' to each other.

It is what the creator of CAT980 is using.  The circuit is universal in

as much as the prime requirement of RS-232-to-TTL and TTL-to-RS-232

levels are accomplished. You can adapt this circuit to virtually any CAT

transceiver on todays market provided you substitute the correct radio

connector and pin outs (see your manual). You can eliminate the 2N2222

transistor and it's components if you are not going to key the PTT line

of the radio with your computer.

On page 3 there are additional components added to the interface

circuit to enable AGC level detection for scanning purposes. This is

more radio-specific than page one so it is presented here primarily for

FT-980 and CAT980 users.  For a 'DELUXE' circuit, print out the included

PCX file (you can do it from paint programs and of course Windows)

ULTRA_IO.PCX. It's the same circuit but easier to read than the ASCII

characters used in the following pages.  Plus it includes a 555 IC

circuit to eliminate the need for two batteries.

If you're using commercial or other interface hardware you will

probably be dismayed to see that the AGC feature is not available.

Simply build the portion of the circuit on page 3 to implement the AGC

function and continue to use what you already have on line to control

the radio.

G-1

CAT980 Ver 2.0                                                   Appendix G 2

YAESU(tm) FT-980 TO IBM COMPUTER INTERFACE CIRCUIT by K5YEF

+9v to +12v (9v battery OK)

+| | -  Add DPST switch at X's

TO IBM/COMPATIBLE  ----------------O -X-||||--|

!                  |               |\   | |  _|_         TO FT-980

RS 232             |               < \       ///  6 PIN DIN CONNECTOR

25 PIN CONNECTOR   |          10 k <  \                      !

!                  |               <   \---------------|     !

!                  |               |                   |     !

7<-------------------------------------------------o---------->1 GND

!                  |               |              |    |     !

!                  |/|2    100 k   |              |    |     !

!                 4/+|-----/\/\----|------------------------->2 TTL

!                 /  |             |              |    |     !  SIGS

!      3.3 k   1 /   |             |              |    <     !  FROM

3<------/\/\-----/ U1A|             |              |    <1.5K !  RADIO

!               \    |             |              |    <     !

!                \   |             |      |\ |    |    |     !

!                 \  |3            |      | \|    |  ____    !

!                  \-|-----/\/\----o------| /|----o  \  / 1N914 OR LED

!                   \|     100 k          |/ |    | __\/__   !

!             1N914                       1N914   |    |     !  KEY

!              |\ |                               |    o----->4 LINE

!     100 k    | \|                               |  |/      !  TO

4<-----/\/\-----| /|----------------------------------|\e     !  RADIO

(RTS)           |/ |                               |  | |2N2222-(Radio

!                                                 |    |     ! Shack

!            -----\/\/-------      (Radio Shack   |   _|_    276-2009)

!            |     100 k    |       276-1122)     |  ////    !

!            |  |\          |                     |          !

2<------/\/\--o--|-\         | (1N914 Radio Shack  |          !

!       18 k   6|  \        |  276-1620)   | /|   |          !

!               |   \ 7     |              |/ |   |          !

!               |U1B \------o----/\/\------|\ |----------o--->3 TTL

!               |    /            47       | \|   |      |   !  SIGS

25 PIN TO 9 PIN  |   /11                   1N914   |      |   !  TO

TRANSLATION      |  /|                     OR LED  |      |   !  RADIO

TABLE       -----|+/ |   U1  LM324 Quad Op Amp     |      |   !

25  9      |   5|/  |   (Radio Shack 276-1711)    |    ------

# | #      |        |                             |      /\1N914

=====      |        |  Build circuit on bread-    |     /  \  !

7 | 5      |        |  board(Radio Shk 276-175)   |    ------ !

3 | 2      |        |                             |      |    !

4 | 7      |        |            - | | +          |      |    !

2 | 3      |        O --X---------||||---|        |      |    !

_|_         -9v to -12v  | |  _|_      _|_    _|_   !

///         (9v battery OK)   ///      ///    ///

For Use With CAT980 Or Any Interface Activity.

Schematic in PUBLIC DOMAIN.

(K5YEF / Art Harding / PO Box 861719 / Plano, Tx. 75074 / USA)

For full details on this and additional CAT interface circuits send $3.00 for

a reprint of the article 'UNIVERSAL CAT INTERFACE' by K5YEF, 73 Mag, Aug '91

to:  73 Magazine /  WGE Center /  Forest Road /  Hancock, NH 03449

G-2

CAT980 Ver 2.0                                                   Appendix G 3

AGC INTERFACE CONTROL CIRCUIT FOR SCANNING WITH THE FT-980

Add the following components to the circuit on page 2 to expand the interface

to provide the AGC function. Note that no additional IC's are required. If

you are building this AGC portion of the circuit only, be sure to apply +9 to

+12 volts to pin 4(+Vcc) and -5 to -12 volts to pin 11(-Vcc) to power the

chip.

+9v to +12v (9v battery OK)

TO IBM/COMPATIBLE  ----------------O

!                                  |                     TO FT-980

RS 232                             <              6 PIN DIN CONNECTOR

25 PIN CONNECTOR             100 k <                         !

!                                  <                         !

!                                  |                         !

7<------------------------------------------------------------>1 GND

(GND)              +Vcc              |                         !

!               4o  /|             |  100k POT               !

!                | /+|-------------o--/\/\/\                 !

!                |/  |12               /-\ SET FT-980        !  SIGS

!      3.3 k  14 /   |                  |  AGC TO DESIRED    !  FROM

5<------/\/\--o--/ U1D|                  |  'S' METER LEVEL   !  RADIO

(CTS)         |  \    |                 _|_  TO RAISE PIN 5   !

!            |  |\   |                ////  ON THE RS-232.   !

!            |  o \  |13                    (see below)      !

!            | 11  \-|-------------o                         !

!            | -Vcc \|             |                         !

!            |                     |                         !

!            |----------/\/\/\-----o                         !

!                       100 k      <                         !

!                                  < 4.7k                    !

!                                  <                         !

!                                  |                         !

!            ---------------o------|                         !

!            |              |                                !

!            |  |\          |                                !

!            |--|-\         | (1N914 Radio Shack             !

!              9|  \        |  276-1620)                     !

!               |   \ 8     |                                !

!               |U1C \------o                                !

!               |    /                                       !

25 PIN TO 9 PIN  |   /   Sections A&B of U1 are used in       !

TRANSLATION      |  /    the circuit on page 2.               !

TABLE       -----|+/     U1  LM324 Quad Op Amp                !

25  9      |  10|/      (Radio Shack 276-1711)               !

# | #      |                                                 !

=====      |           Build circuit on bread-               !

7 | 5      |           board(Radio Shk 276-174 instead of 276-175)

5 | 8      |                                                 !

|                  10 MEG                         !

------------------/\/\/\--------------------------->  5 AGC

(modified 9/2/92)                                                !

G-3

CAT980 Ver 2.0                                                   Appendix G 4

HOW TO ADJUST THE AGC POT USING CAT980 AND THE FT-980

1. Tune to a quiet frequency and crank in 30 db attenuation and/or

disconnect the antenna (we're trying to get a no-signal situation.)

2. With CAT980 up and running do a <AltF7> or type 'CAL' <CR>. This

brings up the calibration screen.

3. With a no signal condition adjust the AGC pot on the interface

unit to produce the following low CTS message:

������������������������������Ŀ

�CTS PIN IS LOW (-)            �

��������������������������������

4. Turn the RF GAIN control on the FT-980 counter clockwise until the

FT-980 'S' meter is at the setting where you want to stop a scan. You can set

this from about an S2 to +20 db over S9.  Ignore any screen changes and

sounds from CAT980 as you set up this AGC point.

4. Now very slowly turn the interface AGC pot until the following

high CTS messages appears (complete with sound):

����������������������������������������Ŀ

�"SIGNAL DETECTED" - CTS PIN IS HIGH (+) �

������������������������������������������

5. Consider calibrating your pot so that in the future you can

quickly set these scan level points up. You do this by performing these steps

for each 'S' unit and 5 db points above S9.

Note that this feature is made available to everyone in the spirit of

amateur experimentation.  Registered users can use the calibrated circuit to

cause their scan to be AGC sensitive.

(K5YEF / Art Harding / PO Box 861719 / Plano, Tx. 75074 / USA)

=======END OF APPENDIX G=======

G-4

CAT980 Ver 2.0                                              Appendix H Page 1

APPENDIX H

PLAIN TEXT COMMANDS

-------------------

NOTE: THIS APPENDIX ONLY APPLIES TO REGISTERED USERS...

The following are plain text commands you may enter from the

keyboard from the command line. Many of these commands are brand new to

the Ver 2.0 series and have no function key or key combination

equivalents.  But many do and the table below lists such keys when

appropriate. The effect of each command is listed.

When presenting a plain text command in this appendix note that

both upper and lower case with certain underlined characters are used.

The underlined upper case characters are the minimum key strokes

necessary for the command to take effect.  For example: SHOw FILter

--- ----

means that entering SHO FIL is the minimum required to <show filter>.

Also note that the space in the string SHOW FIL is a must-type

character.

{DISPLAY Commands====================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

DISplay FILter            none          This brings up the filter

---    ----                             display plot. It is not active,

or                                  that is it is not the same as

SHOw FILter                             <F8>, but the SHOw FILter

--- ----                                display will remain current as

you shape the filter or change

modes. <ESC> removes display.

DISplay MEMory            none          This brings up the FT-980

---    ----                             display of internal memories. It

or                                  is not active, that is it is not

SHOw MEMory                             the same as <F7>, but the

--- ----                                display will remain current as

you modify any memory channels.

<ESC> removes display.

DISplay SLOts             <^f>          This brings up the CAT980

---    ----                             display of the 300 memory slots.

or                                  <ESC> removes display.

SHOw SLOts

--- ----

H-1

CAT980 Ver 2.0                                              Appendix H Page 2

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

DISplay STAtus            <^q>          This brings up the CAT980

---    ----                             & FT-980 Status Display. <ESC>

or                                  removes the display.

SHOw STAtus

--- ----

DISplay TABs              none          This shows the upper/lower tab

or                                  limits and their present state:

SHOw TABs                               on, off and/or auto tab.

--- ----

DISplay COLors            <AltC>        This brings up the present

---    ----                             colors being used by CAT980

or                                  displays. It also offers the

SHOw COLors                             chance to reset to the original

--- ----                                colors in case certain menus,

etc. have produced an undes-

irable color combination.

{File Utilities======================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

FILEs                     <ShftF1>      This brings up the display

----                                    prompt to 'Load Menu/Log/Memory'

files. <ESC> drops the display.

LOAD fName.ext            none          This loads any legitimate .mnu,

--------------                          .mem or .log file.  For instance

fName.ext can be the supplied

file DEMO20.MNU. If you supply a

file CAT980 can not find it will

give you the opportunity to

create it provided it is a .log

or .mnu file. (.mem files must

created in an editor only.

<ESC> aborts the display.

H-2

CAT980 Ver 2.0                                              Appendix H Page 3

{Scanning operations=================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

SCan                      <F9>          This brings up the Scan Menu

--                                      Display to choose a scanning

operation. <ESC> drops the

display.

SCan BAnd                 <^b>          This starts the scan band

--  ---                                 sequence. <ESC> aborts the

sequence.

SCan MEMory               <^n>          This starts the scan sequence

--  ----                                of the FT-980 internal memories.

<ESC> aborts the sequence.

SCan SLOts                <^v>          This starts the scan sequence of

--  ----                                the CAT980 300 memories. <ESC>

aborts the sequence.

SCan MENu                 <^m>          This starts the scan sequence of

--  ----                  <RETURN>      the active menu. <ESC> aborts

the sequence.

{MAIN VFO COMMANDS===================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

CONFIGuration             <F5>          This brings up the configuration

------                                  menu to choose MEM/VFO/RXV/RXM.

<ESC> aborts the operation.

MEMory configuration      none          Unlike the <\> toggle this

---                                     commands the mode directly into

the MEM configuration.

VFO configuration         none          Unlike the <\> toggle this

---                                     commands the mode directly into

the VFO configuration.

H-3

CAT980 Ver 2.0                                              Appendix H Page 4

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

SPLit                     none          This configures the FT-980 into

---                                     SPLIT operation with the

or                                 transmit frequency being the VFO

RXM configuration                       and the receive frequency being

---                                     the active FT-980 memory

channel.

RXV configuration         none          This configures the FT-980 into

---                                     SPLIT operation with the receive

frequency being the VFO and the

transmit frequency being the

active FT-980 memory channel.

{TAB Commands========================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

TABs                      <^t>          This toggles the FT-980 TAB ---

function on and off.

TABs ON                   none          Unlike the toggle <^t> this

--- ---                                 commands the TAB function to be

ON immediately.

TABs OFF                  none          Unlike the toggle <^t> this

--- ---                                 commands the TAB function to be

OFF immediately.

SET  Upper tab            <^u>          Whatever frequency is active

--- --                                  will become the upper tab limit.

SET  Lower tab            <^l>          Whatever frequency is active

--- --                                  will become the lower tab limit.

RESET Tabs                <^w>          CAT980 maintains a table of

-------                                 default TABS which correspond to

the U.S. band edges. This resets

the tabs to those defaults.

H-4

CAT980 Ver 2.0                                              Appendix H Page 5

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

AUTO tabs                 <^k>          This toggles the ON/OFF auto

----                                    tab function.  When ON the tab

limits will automatically adjust

to band changes, keeping the

tabs ON.

AUTO tabs ON              none          Unlike the toggle <^k> this

----     ---                            commands the AUTO TAB function

to be ON immediately.

AUTO tabs OFF             none          Unlike the toggle <^k> this

----     ----                           commands the AUTO TAB function

to be OFF immediately.

{FREQ commands=======================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

FREQ                      <F2>          This brings up the frequency

----                                    digit display so you can enter

the desired frequency. <ESC>

aborts the sequence.

xxxxxx to xxxxxxxx      none            You can enter the frequency

------    --------                      direct from 6 to 8 digits. The

bottom of the GEN band, that is

the lowest frequency of the

FT-980 is 150,000 Hz. So you may

enter 150000 (6 digits) and if

you are in GEN the radio will

deploy to that frequency.

The highest frequency of the radio is limited to

29,999,990 Hz.  You may enter that frequency or

any frequency in between.  Remember the radio

limitations: if you are in HAM the radio will

not go to 12 MHz, etc.

H-5

CAT980 Ver 2.0                                              Appendix H Page 6

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

xxxK  to xxxxxxK        none            You can use the multiplier 'K',

----     -------                        which will be interpreted as

and decimal OK!                         'times 1000'. So entering 150k

puts the radio on 150 KHz.

Entering 28000k will deploy the

radio to 28,000,000 Hz and

entering 2800.1k sends the radio

to 28,000,100 Hz.

* YOU CAN EXPLORE THE FT-980 BELOW 150 Khz *

* BY STEPPING BELOW IT WITH <PageDn> *

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

xM    to xxM            none            You can use the multiplier 'M',

--       ---                            which will be interpreted as

and decimal OK!                         'times 1000000. So entering 10m

puts the radio on 10 MHz.

Entering 28m will deploy the

radio to 28,000,000 Hz and

entering 28.1m sends the radio

to 28,100,000 Hz.

UP xx to UP xxxxxxxx    none            This allows you to move up a

---      ---                            desired amount specified by x.

It can be from 10 (10 Hz) to, if

the radio is not ordered out of

range, 29000000. (Similar to

<+>).

DN xx to DN xxxxxxxx    none            This allows you to down  a

---      ---                            desired amount specified by x.

or                              It can be from 10 (10 Hz) to, if

DOWN xx to DOWN xxxxxxxx                the radio is not ordered out of

-----      -----                        range, 29000000. (Similar to

<->).

EXAMPLES-FROM 10MHz     RESULT

====================    ========

UP 10101010             20101010

UP 10101k               20101000

UP 10M                  20000000

H-6

CAT980 Ver 2.0                                              Appendix H Page 7

{Special FREQ Calls==================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

WWV                     <F10>           This brings up the 10 MHz time

----                                    standard frequency.  <ESC> will

or                                      return to initial settings.

TIME

----

ALTernate time          <ShftF10>       This brings up the alternate

---                                     time standard frequency set

by the config file. The default

is  5 MHz. <ESC> will return to

initial settings.

{FILTER Commands=====================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

FILter                  <F8>            This brings up the active filter

---                                     menu display.  <ESC> will make

the display inactive, another

<ESC> will drop the display.

CENter filter           <ShftF8>        This centers both filter width

---                                     and shift.

CENter WIDth            none            This centers the filter width

---   ----                              without affecting filter shift.

CENter SHIft            none            This centers the filter shift

without affecting filter width.

WIDth xxx %             none            This narrows the band pass

---  ----                               filter from the 'left' skirt

toward a lower frequency, or

down band, by the entered xxx

amount. Values up to 100 % are

accepted.

WIDth -xxx %            none            If a negative value is entered,

---  -----                              the filter band pass narrows

from the 'right' skirt toward a

higher frequency, or up band, by

the -xxx amount entered. Values

up to -100 % are accepted.

H-7

CAT980 Ver 2.0                                              Appendix H Page 8

Unlike <ShiftF6> and <ShiftF7> which changes the

filter width a small amount with each entry, the

WIDth command goes directly to the entered

amount.  Entering the per cent sign is optional.

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

SHIft xxxx              none            This shifts the band pass

---  -----                              filter up band by the entered

xxxx amount. Values up to 1000

are accepted in multiples of

100.

SHIft -xxxx             none            This shifts the band pass filter

---  ------                             down band, by the -xxxx amount

entered. Values up to -1000 are

accepted in multiples of 100.

Unlike <ShiftF4> and <Shift57> which changes the

filter shift by 100 Hz with each keystroke, the

SHIth command shifts the filter directly to the

entered amount.

CAT980                  none            This repositions the USB, LSB,

---                                     FSK and the CW filters in their

relation to each other such that

a signal in any of these filters

whose position from the carrier

matches the BFO - that signal

will appear in the same place in

all of the above filters.  (The

BFO shift is set by a switch on

the rear of the FT-980 and set

by the config file.) Unlike the

<AltF3> toggle, this command is

immediate.

AFSK                    none            This repositions the USB, LSB,

----                                    FSK and the CW filters in their

relation to each other such that

a signal in any of these filters

whose position from the carrier

matches a given mark frequency -

will maintain a compatible

position in all of the above

filters.  (The AFSK shift is set

by the config file.) Unlike the

<AltF4> toggle, this command is

immediate.

H-8

CAT980 Ver 2.0                                              Appendix H Page 9

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

FT-980                                  This resets the USB, LSB, FSK

--                                      and the CW filters in their

relation to each other to the

YAESU configuration. Unlike the

<AltF3> toggle, this command is

immediate.

{Other===============================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

STATUS                   <^q>           This brings up the CAT980

------                                  & FT-980 Status Display. <ESC>

removes the display.

HELP                    <F1>            This brings up four pages of

----                                    help information.  <PageUp> and

<PageDn> walks thru the pages.

<ESC> removes the display.

TUNE                    <`>             First this stores the present

----                    <~>             FT-980 set up and then puts the

radio into FSK mode, raises the

RTS line for transmit and begins

a count down (21 sec default)

toward 0 and then drops the RTS

line and returns the radio to

its previous mode.  <ESC> aborts

the tune function.

OFFSet display          <^End>          This turns on the FT-980 offset

----                                    display on the radio itself.

This function is only evident by

looking at the FT-980 digital

display.

RTS

---                     <TAB>           This raises the RTS line, pin  4

on a 25 pin RS-232 connector

(pin 7 on a 9 pin) which you are

encouraged to use to key the

FT-980 (see Appendix G).

H-9

CAT980 Ver 2.0                                             Appendix H Page 10

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

DTR                     <ShftTAB>       This raises the DTR line, pin 20

---                                     on a 25 pin RS-232 connector

(pin 4 on a 9 pin) available for

any external control ideas.

TX                      none            This raises the RTS line, pin  4

--                                      on a 25 pin RS-232 connector

(pin 7 on a 9 pin) to transmit.

Unlike the toggle <TAB> this

command is immediate.

RX                      none            This lowers the RTS line, pin  4

--                                      on a 25 pin RS-232 connector

(pin 7 on a 9 pin) to receive -

the default. Unlike the toggle

<TAB> this command is immediate.

QUIT                    <AltX>          This brings up the exit-option

----                                    menu.

EXIT                    none            This closes the program CAT980

----                                    but leaves the FT-980 in a

captured state (you can't

control it from the front

panel!).

BYE                     none            This command releases CAT

----                                    control of the FT-980 completely

and closes the program.

{FT-980 MEMORY commands==============================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

WRITE                   <ShftF9>        This places (writes) the current

-----                                   vfo, mode and frequency into the

current selected FT-980 memory

channel.

PUT                     none            This places the current selected

---                                     FT-980 memory channel vfo, mode

and frequency into the active

vfo, mode and frequency.

H-10

CAT980 Ver 2.0                                             Appendix H Page 11

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

CHAN xx                 none            This goes directly to the

-------                                 FT-980 memory channel selected

or                                    by the value xx. Valid xx

GO xx                                   entries may be from 1 thru 16.

-----

LOCk memory             none            This locks the FT-980 current

---                                     memory channel such that the

frequency can not be changed.

Unlike the toggle <DEL> this

command is immediate.

UNLock memory           none            This unlocks the FT-980 current

---                                     memory channel such that the

frequency can be changed. Unlike

the toggle <DEL> this command is

immediate.

MEM980                  <F7>            This brings up the active

------                                  FT-980 Memory Display Menu.

{Cat980 Memory Commands==============================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

PICk                    <^r>            This brings up the prompt to

---                                     select any memory slot from 1 to

max (300 possible).

MArk                    <^s>            This stores the current

--                                      frequency into the next

available memory slot.

CLEAR Slots             <^d>            This brings up the prompt to

-------                                 empty the 300 CAT980 memory

slots.

SPEEd                   <^a>            This brings up the multi load

----                                    'speed load' prompt sequence.

H-11

CAT980 Ver 2.0                                             Appendix H Page 12

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

SAVE set up             <ShftF2>        This stores the current mode,

----                                    frequency, vfo, and filter

parameters into a special CAT980

memory - not part of the 300

memory slots.

RESTore set up toggle   <ShftF3>        This toggles between the set up

----                                    stored by 'SAVE' and whatever

the present set up is.

{RIT commands=========================================================}

RIT                     <F6>            This brings up the clarifier

---                                     (RIT) menu. Arrow keys move

about the RX TX RX/TX options.

This is a toggle command.

RIT OFF                 none            This turns the RIT off. This

-------                                 command is immediate.

RIT ON                  none            This puts the RIT into the RX

------                                  mode, that is any frequency

or                                    change will be experienced

RIT RX                                  during receive only.

------

RIT TX                  none            This puts the RIT into the

------                                  TX=RX mode. This command is

immediate.

{DOS operations=======================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

DIR                     <^x>            This brings up the DOS directory

---                                     of all files in the directory

from which CAT980 is running. If

the DOS file MORE.COM is in that

directory then scrolling is

enabled.

H-12

CAT980 Ver 2.0                                             Appendix H Page 13

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

DIR *.MNU               none            This brings up the DOS directory

---------                               of all menus, .mnu, files in the

directory from which CAT980 is

running.

DIR *.MEM               none            This brings up the DOS directory

---------                               of all memory load, .mem, files

in the directory from which

CAT980 is running.

DIR *.LOG               none            This brings up the DOS directory

---------                               of all log, .log, files in the

directory from which CAT980 is

running.

DOS                     <^z>            This allows you to enter the DOS

---                                     shell - that mystery zone where

you may or may not be able to

perform miracles.  To return to

CAT980 type EXIT.

{User Utilities======================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

MENU                    <AltP>          This brings up the active menu

----                                    with the next available

selection ready to store. If the

active menu is full (21

selections) then the default

CAT980.MNU will appear. <ESC>

aborts the sequence.

SWAP MENU               <AltY>          This allows you to toggle

---------                               between the active menu and the

default CAT980.MNU.

LOG                     <^p>            This brings up the active log

---                                     file ready for your entry. <ESC>

aborts the sequence.

H-13

CAT980 Ver 2.0                                             Appendix H Page 14

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

SWAP LOG                <^y>            This allows you to toggle

--------                                between the active log and the

default CAT980.LOG.

AGC                     <AltF5>         This toggles the AGC sensing

---                                     function of CAT980 use to halt

scans. The hardware in Appendix

G must be implemented.

AGC ON                  none            This enables CAT980 to sense a

------                                  preset AGC or 'S' meter level to

halt a scan.  This command is

immediate.

AGC OFF                 none            This disables the ability of

-------                                 CAT980 to sense the AGC or 'S'

meter level.  This command is

immediate.

CALibrate agc           <AltF7>         This produces a special screen

---                                     to set up a desired threshold

AGC level using an external pot

(see Appendix G).

{MODE commands=======================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

MODE                    <F3>            This toggle command brings up

----                                    the mode menu.  Arrow keys move

thru the mode options.

LSB                     none            This command engages the LSB

---                                     FT-980 filter immediately.

USB                     none            This command engages the USB

---                                     FT-980 filter immediately.

CWw                     none            This command engages the CWW

--                                      FT-980 filter immediately.

CWN                     none            This command engages the CWN

---                                     FT-980 filter immediately.

H-14

CAT980 Ver 2.0                                             Appendix H Page 15

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

FSK                     none            This command engages the FSK

---                                     FT-980 filter immediately.

FSK xxx                 none            This command engages the FSK

-------                                 FT-980 filter and also sets the

FSK shift by the amount xxx.

Allowable parameters are 170,

450 or 850.

AMw                     none            This command engages the AMW

filter immediately.

AMN                     none            This command engages the AMN

---                                     FT-980 filter immediately.

FM                      none            This command engages the FM

--                                      FT-980 filter immediately.

{Vfo Configuration===================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

GEN                     none            This engages the FT-980 GEN

---                                     VFO. This command is immediate.

HAM                     none            This engages the FT-980 HAM

---                                     VFO. This command is immediate.

EQUAL ham/gen vfo       <CtrlHome>      This command works in the

-----                                   following way:

or                                      1. If in the HAM vfo when

HAM=gen frequency                         executed then the GEN vfo

----                                      will move to the same

or                                       frequency as the HAM vfo.

GEN=ham frequency                         2. If in the GEN vfo AND

----                                      within a HAM band when

executed then the HAM vfo

will move to the same

frequency as the GEN vfo.

3. If in the GEN vfo AND the

frequency is NOT within a HAM

band when executed, then the

HAM vfo will remain where it

was and an error message will

appear.

H-15

CAT980 Ver 2.0                                             Appendix H Page 16

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

UPDate cat980           none            This sends a command to the

---                                     FT-980 to return the complete

status stream, 148 bytes, to the

computer.  This is included as a

'bug' chaser. If in doubt about

any displayed info or plots this

command will forcibly update the

system.

{BAND commands========================================================}

COMPARABLE

TYPED COMMAND           FUNCTION KEY    EFFECT OF COMMAND

======================  ============    ===============================

BANd UP                 <CtrlPgUp>      This command shifts the band

--- ---                                 up one band as defined by the

FT-980 segments.

BANd DN                 <CtrlPgDn>      This command shifts the band

--- ---                                 down one band as defined by the

or                                    FT-980 segments.

BANd DOwn

--- ---

TOP of band

---                     <Home>          This command moves the frequency

to the top of any given band as

defined by either the FT-980 or

CAT980. The US band limits are

the defaults used. If TABS are

engaged then other limits

defined by you can be used.

BOT of band             <End>           This command moves the frequency

---                                     to the bottom of any given band

as defined by either the FT-980

or CAT980. The US band limits

are the defaults used. If TABS

are engaged then other limits

defined by you can be used.

MID band                none            This command moves the frequency

---                                     to the center of any given band

as defined by CAT980.

H-16

CAT980 Ver 2.0                                             Appendix H Page 17

The following commands may be used to go directly to the mid point of

any given band. If in HAM vfo the band will change with the command. If

in GEN vfo the band remains in GEN but the midpoint of the ham band will

still be reached. Note there are two ways of reaching each band using

plain text commands.

COMMAND                 OR              COMMAND                FREQUENCY

========================================================================

160 MEters                              ONE sixty              1.900.000

-----                                   ----

80 MEters                               EIGhty meters          3.750.000

-----                                   ---

40 MEters                               FORty meters           7.150.000

-----                                   ---

30 MEters                               THIrty meters         10.125.000

-----                                   ---

20 MEters                               TWENty meters         14.175.000

-----                                   ----

17 MEters                               SEVenteen meters      18.118.000

-----                                   ---

15 MEters                               FIFteen meters        21.250.000

-----                                   ---

12 MEters                               TWELve meters         24.940.000

-----                                   ----

10A Meters                              TEN A  meters         28.250.000

-----                                   -----

10B Meters (or 10 Meters or TEN Meter)  TEN B  meters         28.750.000

-----          ----         -----       -----

10C Meters                              TEN C  meters         29.250.000

-----                                   -----

10D Meters                              TEN D  meters         29.600.000

-----                                   -----

K5YEF/Art Harding/PO Box 1719/Plano, Tx. 75074/USA

=======END OF APPENDIX H========

H-17