GSH-PC                         - 1 -                          SHW-1.00

----------------------------------------------------------------------

GSH-PC SSTV - CONVETER

======================

Copyright (C) 1995 G.Szabados-Hann

All Rights reserved.

About this program

==================

GSH-PC is a program for sending and receiving SSTV (Slow-Scan

Television) pictures. This program is not a multi-mode decoder for

FAX/RTTY/PACTOR/etc., it was designed specially for SSTV.

Howerer the program needs a very simple modem circuit, it offers

a very good picture quality.

Features:

- Supports all common used speed (ROBOT, SCOTTIE, SC-2and MARTIN),

- Load / save BMP und TIF image formats.

- Needs a very simple hardware, only two OPs.

- Up to 16 Mio. colors by VESA-norm compability.

- Supports the realtime frame-grabber (VD-720).

- Split window for RX/TX.

Hardware requirements

=====================

1) PC and grafik adapter

The programm is running on  MS-DOS PC with at least 640 KB mermory.

It needs an 1MB VGA grafik adapter supporting the 640x480 VESA-mode,

15, 16 or 24 bits each pixel (VESA mode 272, 273 or 274).

It is suggested to use at least 386DX porcessor and VLB-VGA board.

The color resolution is, depending on the VESA-mode, 32.000, 64.000

or 16 mio. colors per pixel. The best available resolution will be

selected automatically.

2) Modem Interface

==================

To convert the audio signal between receiver and PC a very simple

modem circuit is used. See for the proposed circuits in appendix-1

at the end of this manual. This kind of interface circuit is used

by several other RTTY and SSTV programs.

GSH-PC                         - 2 -                          SHW-1.00

----------------------------------------------------------------------

3) Video Digitizer

==================

Using the digitizer board  VD-720 you are able to freeze an image

from any video source (FBAS input). The image is loaded directly

into the TX-Window and can be transmitted immidietly.

Ordering information:

VD-720C: (Typ-FG01)

Gatz und Hartmann Fehrbelliner Str.32

13585 Berlin. Tel.: 030-375 55 68

This is an optional facility, you can operate GSHPC without the

digitizer board too.

Installing of GSPHC

===================

Copy the compressed file "GSHPCZIP.EXE" into the directory you want

start the program, for example: c:\sstv\ghspc. This file is an self-

extracting programm, start simply the executable file. After

"unzipping", following files are available:

GSHPC.EXE       SSTV Converter-Program.

UNIVESA.EXE     VESA-Emulationsprogram (TSR-Prg.)

VESAMOD.EXE     Lists the supported VESA-mode of the VGA adapter.

VGATEST.EXE     Testprogram for VGA-adapter boards.

SSTV.BAT        Batchfile to start GSHPC, incl. callsign and name.

GSHPC_D.DOC     Operator Manual for GSHPC (german).

GSHPC_E.DOC     Operator Manual for GSHPC (english).

NORMS_D.DOC     Description of the SSTV-Modes (german).

NORMS_E.DOC     Description of the SSTV-Modes (english).

UNIVESA.DOC     Manual for the VESA-Emulator (english).

Demo-Images

===========

BIGTXT.TXT      Example (ASCII) for generating of textfiles.

NORTXT.TXT      Example (ASCII) for generating of textfiles.

AUTHOR.BMP      24-Bit BMP-format.

FRAU.TIF        24-bit TIFF-format.

SHUTTLE.BMP     8-Bit Color-Map BMP-file.

PAPAGEI.TIF     8-Bit Color-Map TIFF-file.

SSTV.BMP        Main-Maske in 24-Bit BMP-format.

GSH-PC                         - 3 -                          SHW-1.00

----------------------------------------------------------------------

Starting GSHPC

==============

Before first starting GSHPC on your PC you should delete the

configuration file MODE_DEF.CFG. It is maybe an old version or a

version from your ham-friend and can contain directrories not

existing in your PC.

Start the program by typing:

-  GSHPC DL1XXX  <retrun>             ; if only callsign is used

-  GSHPC "DL1xxx  op: Name" <return>  ; if space in the text

note: do not use space in your callsign !

The callsign and name are optional, they will be placed at the top

of the transmitted image.

If youe have trouble:

---------------------

- Check the VGA-adapter, use the VESAMOD.EXE and VGATEST.EXE programs.

- Install UNIVESA.EXE if the VGA-adapter does not supports VESA.

- Remove/disable not used driver e.g. EMM-386, CD-ROM, Soundblaster.

Operation:

=========

The program is controlled by function-keys and pull-down selection

lists. The main functions can be selected by pushing the first

(underlined) character of the function-name. If opened, the neigh-

borous functions can be reached by means of the TAB or Curs <-/->

keys. After selecting the desired item (Pg-up/Pg-down), the selected

function is executed by pushing the RETURN key. Use the ESC key

to exit the function without change.

+--------------------------------------------------------------------+

|  Mode  Martin M1      | DL4SAW SSTV Converter | Version: SHW 1.00  |

+-------+-------+-------+-------+-------+-------+------+------+------+

|  Mode | Load  | Save  | Fill  | Paint | Word  | Tx ##| Hold | Rx ##|

+-------+-------+-------+-------+-------+-------+------+------+------+

|                                                                    |

|                                                                    |

+----------------------------------+---------------------------------+

|     TX window  PICTURE.TIF       |     RX window  PIC_0001.BMP     |

+----------------------------------+---------------------------------+

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

+------+------+------+------+------+------+------+------+------+-----+

| /osc/| conf.| F-run| VIS  | Fsave| Tx<Rx| Tx>Rx| Zoom+| Scope|Exit |

| F1 ##| F2 ##| F3 ##| F4 ##| F5 ##| F6   | F7   | F8   | F9 ##| F10 |

+----------------------------------+---------------------------------+

GSH-PC                         - 4 -                          SHW-1.00

----------------------------------------------------------------------

MAIN FUNCTIONS

--------------

Mode:   Selection of SSTV-mode (default = Martin-1).

=====   The file "norms.doc" shows an overview of the used SSTV norms.

The so-called "quasi" mode are not implemented.

Load:   Load images in BMP, TIFF or TXT-format to the TX-window.

=====

Bitmap-format images must have the filename-extension

"xx.BMP", TIFF-Format images "xx.TIF".

In case of BMP-format images, only the 8-bit Color-Map and

24-bit RGB formats (without compression) are supported.

The image size is limited to 1000 x 800 pixels.

TIFF has a lot of versions, on that reason I had to limit

the impementation supporting only the mainly used formats.

GSHPC supports only the "INTEL"-format with single planar-

configuration and without LZW-compression. By loading the

images they are fitted into the TX-window, the x/y ratio

is not changed. Similar to BMP-format only the 8-bit Color-

Map and 24-Bit RGB-formats are supported.

Textfiles in special GSHPC-format (.txt) can be loaded too.

The both example files shows how to compose this files.

You can use any text editor to create and change them.

Save:   Store images from RX-window:

=====

1)  Open the Save-Window and select a directory.

2)  Select TIF oder BMP format. If you want to store the

image using the current autosave-number (PIC_xxxx.)

enter RETURN. Otherwiese enter the desired filename.

Use ESC to exit this function without change.

The autosave-number is incremented (if used) automatially

and stored together with the savepath in the configuration

file.

GSH-PC                         - 5 -                          SHW-1.00

----------------------------------------------------------------------

Fill:   Testcards:  Following built-in testicards are available

=====

* Graystep.

* Color-bars.

* testcard for color-resolution.

* testcard with graystep and color-bars.

* testcard for horizontal resolution

* Background, the R,G,B components can be adusted

independently.

Paint:  Opening this window, you can select a color and draw in the

======  TX-window using the mouse. A standard mouse-driver should be

installed before on the second COM-interface.

Word:   Enter text into the TX-window (16 x 8 characters).

=====

1) Available funktions in Edit-Mode:

* Cursor, Page-up, Page-down, Home, <-,->  Pos-1, Ende

* TAB    Change to Color-Adjustment Mode.

* Alt-C  Delete text, the background is transparent.

* Alt-I  Append callsign. If entered at invoking the program,

the text (up to the first space) will be copied.

* Ctrl-Return  Copy the text into the TX-window.

2) Funktions im Color-Adjustment Mode;

* Cursror-Keys:  Set background and foreground colors compo-

nents.

* Page-up/Page-down: Toggle between background and foreground

color adjustment.

* The characters can be displayed "transparent" wihtout

background color. First, one of the background color compo-

nents shoud be set to zero. After pushing the cursor-down

key all component values are set to "_", signaling that the

transparent background have been activated.

* TAB    Change to Edit-Mode .

* CTRL-Return:  Enter the text into the TX-window.

Operating-Modes

================

Tx:   Transmitting of images in the preselected SSTV-mode

===

* Press Tx-key once; image will be sent only once, TX-LED is yellow.

* Press Tx-key again: Endless transmitting, TX-LED is red.

GSH-PC                         - 6 -                          SHW-1.00

----------------------------------------------------------------------

* Break the transmitting;  ESCAPE-taste.

* Pressing all other keys are entering the "mono" mode. The TX-LED

transits to yellow. Transmitting will be stopped after sending

the current image.

Hld:  Standby (default state),

====  All funktions are available.

Rx:   Receiving images in the preselected SSTV-mode.

===

* Press Rx-key once; endless receiving, RX-LED is green.

* Press Rx-key again; Break receiving by reaching the bottom of

the image, the RX-LED is yellow.

Pressing all other keys are set GSHPC back in the stanby-mode.

Exeptions: the following function keys are available during

receiving.

Horizontal and vertical synchroninsation

========================================

-  F1 H-Res: Horizontal-Reset / Search - Mode.

---------------------------------------------------

After starting the receive function, the program is looking

for synchron impulses according to the preselected speed.

In case of noisy signal or receiving in a not correct speed-

mode, the F1-LED is yellow, signalling that receiving cannot

be started. If you are in the correct mode, it starts to

write the image automatically after detecting the synchron

impulses. In other case, you can force to search the corret

speed, by means of pressing the F1-key. During the search

procedure the F1-led is set to red. After a certain time,

depending on the siganl quality, it switches to the correct

speed and starts image reception. The F1-LED is set than to

green. Each time, the image have been synchronised uncorrect-

ly, you can re-synchronize the image by pressing the F1-key.

During resinchronization the F1-LED is set to yellow.

There are some limitations, because some SSTV-mode are using

the same horizontalfrequenz. It cannot be distiguished

between:

* Martin-1  und Martin-3    (selects to M-1)

* Scottie-1 und Scottie-3   (selects S-1)

* 32s S/W   und 16s B/W     (selects 32s)

* 30s SC-2  und 60s SC-2    (selects SC-2 60s)

* 12s S/W   und 24s B/W     (selects 24s B/W Robot)

The so-called "quasi-mode" (256-line mode with a low horizon-

tal resolution) are leading to a selection of the corresponding

128-line mode (e.g. Martin-2 -> Martin-4).

GSH-PC                         - 7 -                          SHW-1.00

----------------------------------------------------------------------

-  F2 V-Res: Vertikal-Restart

--------------------------

Pressing the F2-key, the receiver cursor (small green arrow)

jumps to the first line and a horizontal resynchronization

is forced.

Die F2-LED shows the reason of the last vertical reset;

* LED-off;  Idle-state / manual restart by pressing "F2".

* LED-green;

- Erronous VIS-Signal was received (parity error)

- Speed-mode ist not supported.

- No VIS-code in the vertical synchron signal.

* LED-red;

New speed-mode have been entered caused by the VIS signal.

* LED-yellow:

Valid VIS-signal was received, speed mode is not changed.

The VIS mode-selection was barred (F4) or GSHPC was

already in the correct speed mode.

The F2-LED is set off after the 10th line.

-  F3 F-Run: Free-Run  (See F-key description)

-  F4 VIS:   Change of speed mode on VIS signal enabled/disabled.

-  F9 Osci:  Display-mode of the Scope function:

- Analogue            F9-LED: yellow

- spectrum analyser   F9-LED: green  (default)

- off                 F9-LED: off

FUNKTION - KEYS

---------------

F1: PC-crystal calibration / available only in HOLD-State !

======================

The absolute precision of the PC-quarz does not allow to

receive pictures without parallel shift. On that reason you

have to calibrate the timing in the following way;

1) Receive an image containing vertical lines in Martin-M1 mode.

2) Press F1, to start the calibration.

3) Set the red line parallel to a vertical line of the image.

4) Ctrl-Retrurn: store the corrected value.

Use Ctrl-D to set the default values. Pressing ESC you can exit

the calibration without storing the new value.

If calibrated while in Martin-1, GSHPC computes the calibrating

factors for all speed-modes. It means, that all other mode can be

received than without parallel drift. In case, if a light indivi-

dual correction of a single mode is neccessary, it can be done

starting the correction in this mode. This correction is stored,

but has no effect on the adjustment of other mode.

GSH-PC                         - 8 -                          SHW-1.00

----------------------------------------------------------------------

Note, that there is special possibility to correct "old" mode

(e.g. B/W mode) in Free-Run. Using the Curs-up, down <-, -> keys

you are allowed to adjust the frequenz online, while receiving.

This adjustment will be not stored ant it is valid up to the next

change of the speed mode. Also pressing the end/pos-2 key restores

the original values (see also F3).

F2: Configuration

=============

The required parameter field can be choosed by Curs-Up/Down.

Use "Curs <-/->" to select the correct parameters.

Following paramters can be adjusted.

1) Serial-Interface.

Valid values: COM-1 to COM-8.

2) SSTV output signal via:

- Off

- PC-speaker

- TxD-pin of the adjusted COM-interface.

- PC-speaker and TxD-pin of the selected COM-interface.

3) PTT-Control:

- Off.

- RTS-Pin of the selected COM-interface.

4) Gray-step header on the top of the SSTV-image;

- Off:  Header is not overlayed.

- On :  Header is overlayed (not in ROBOT-mode).

5) Show directory for the Load and Save Functions.

- Off:  Do not show load/save directory.

- On:   show load/save directroy.

6) Fast-Save on/off.

- Off:  Fast-Save (F5) is disabled.

- On:   Fast-Save is enabled.

The fast save function can only be used after selecting

a save directory and image-file type (TIF/BMP).

The F5-LED shows the overall readiness of the Fsave

function.

By pressing Ctrl-D you can restore the default parameter values.

The autosave-number is resetted to PIC_0001. Use ESC to exit the

menue without storing the selected parameters.

F3: F-Run  Free-Run Mode on/off

===========================

Die "new" SSTV-Mode like Martin-Mode can only be received in

Free-Run mode. In this mode it is not possible to switch off

the F-Run function.

GSH-PC                         - 9 -                          SHW-1.00

----------------------------------------------------------------------

To enjoy the advantages of receiving images without synchron

disturbance, new SSTV mode were developed. These new mode are

joining the advanteges of SSTV and FAX. The horizontal synchron-

impuls allows the receiver to resynchronise the image during the

image transfer. Normally, the picture is sinchronized only at the

beginning of the transer. The receiver is running than decoupled

from the synchron impulses. It is absolute necessary for the

receiver and transmitter to have a precise clock signal.

Also the "old" sstv-mode (B/W and ROBOT) can be received Free-

Run, supposed both equipments are running precise with the same

frequenz. Small differences can be corrected "online" using the

following keys:

-  coarse to left:  "UP"

-  coarse to right: "DOWN"

-  fine to left:    "<-"

-  fine to right:   "->"

-  Set default values: Pos-1/home or Pos-2/end

These adjustment will not stored and they are valid up to the

next change of sstv mode.

F4: Automatic change of speed mode on VIS-Signal enable/disable

===========================================================

A lot of SSTV transmitter use the VIS-signal (Vertical Inverval

Signal) to force the receiver into the correct speed mode.

The VIS signal is 300 ms long and modulated by a 8-bit code,

additionally a start- and a stopbit. See further information in

"NORMS.DOC".

F5: FSave: Fast-Save Funkton.

=========================

The FSave function allows storing the received images without

opening the save menue. The FSave function must be previously

enbled. See also F2-key description.

GSH-PC                        - 10 -                          SHW-1.00

----------------------------------------------------------------------

F6: TX<<RX  Copies Rx-window to Tx-Window

=====================================

F7: TX>>RX  Copies Tx-window to Rx-Window

=====================================

F8: Zoom  Rx-Window

===============

This function allows to magnify the image in the RX-window.

Pressing any key restores the main window. The image in the

Tx-window will be deleted.

Shift-F8  Zoom Tx-Window

==============

Similar to F8, magnifies the image in the Tx-window.

Pressing any key restores the main window. The image in the

Rx-window will be deleted.

Ctrl-F8: Show fast video input / freeze

==============================

This function is available only when the digitizer board VD-720

have been installed correctly.

* First pressing of Ctrl-F8: Shows the fast video signal bl/wh.

It is not a real-time facility, the scan rate is depending on

the speed of your PC (ca. 3 pic/s. using 486DX-40).

* Second pressing of Ctrl-F8: The last video sequence will be

stored in true color mode. The max. color resolution is

16 Mio.-colors.

F9: Scope:  Shows the SSTV video signal in two different mode

=========================================================

Pressing the F9 selects the display mode;

* LED is off; Function is not activated.

* LED is green; AF-spectrum.

* LED ist yellow; oscilloscope.

Using the oscilloscope function, the receiver must be adjusted

so, that the video signal remains between 1500 and 2300 Hz.

Approximatly +/- 100 Hz deviation is tolerated.

Note that displaying the video signal is an additional load

for the PC. If using slow PC and/or receiveing 128 line mode

which is displayed full size, the PC can reach its speed limit.

In this case, the received image is disturbed.

F10: Exit;  Exit the program, back to DOS.

=====================================

All windows must be closed before pressing the F10 key.

The currently use configuration, e.g. speed mode, load- and

savedirectory, scope mode are stored in the configuration file.

GSH-PC                         - 11 -                         SHW-1.00

----------------------------------------------------------------------

Limitation of this version:

==================================

- I/O address of digitizer board (300) is not changable.

Known errors:

- Pressing Ctrl-C and Ctrl-Break cause some disturbance of the main

window.

- Problems when loading or saving images from/to non ready drives.

Repeated pressing of keys, after receiving the error message, can

lead to program breakdown. Close the drive and than and press any

key. After a certain time it will work, have patience!

Do not try to store images on your CD-drive, it will not work!

It is not recommended to use drive A: and B: for storing images.

- Problems when using 32-bit (PCI) VGA-adapters. These boards are

using a (for GSHPC) unusual video-RAM organisation (4-byte for one

pixel).

Copyright and registering notes

===============================

(c) Copyright 1995 G.Szabados-Hann DL4SAW. All rights reserved.

Am Zndhtle 7a   76228 Karlsruhe / Germany

Tel./Fax.: 0721 / 47-53-19

For non commercial purposes (like amateur radio) you have the

right to use this software without further permission. You can

copy and share it, but you are not allowed to charge more money

for it as your cost for disk and mail (ca. $4).

Any commercial use of this software needs the written permission

of the author. Also the distribution by shareware dealers and

SSTV-modem manufacturers is prohibited.

This software, although thoroughly tested, can contain some bugs.

The sowftware is provided "as is", with no warranty expressed or

implied. You use this program at your own risk, the author is no

responsible for any damages resulting by the use of this software.

If you like this software, you can resister it simply sending

DM 50.- or equivalent to the above address.  Yor benefit:

- you obtain the newest version,

- yor are informed about news and updates,

- you support the further development of the software.

All brand and product names appeared in this manual may be trade-

marks or registered trademarks of their rspective companies.

GSH-PC                 Appendix - 1/1                         SHW-1.00

----------------------------------------------------------------------

Improved quality comparator interface

=====================================

Jumper   PC

Tx-Level         10K     10K                   o----> Speaker

+---------+--####-+--####------------------o

0.1uF  |         |       |                        o-+

<----||--->U 10K    ===     ===                         |

Tx-Mic.    |         |4n7    |4n7                       |

-+-       -+-     -+-     47uf      1N4148    |     RS-232

|---||-----+---|<|---+ |     9.pol.

+    |         | |     SUB-D

100k               +-------+         | |

+--####-------+       |       | MC-     | |

|             |   47k U       | 1458/2  | +--o (3) TxD

Rx  1uF   4k7  |  +----\     |       |       |         |

>---||---####--+--|-     \   |  1K   |     +-+--\      +----o (4) DTR

Audio-            | OP-1  >--+--####-------|-     \

Eingang       +---|+     /           |     | OP-2  >--------o (6) DSR

|   +----/         50K U<----|+     /

4k7 U    MC-1458/1  Offset |     +-+--/        +--o (7) RTS

|                      U 47k   |           |

-+-                     |       |           |  o (5) GND

+    |       |  1N4148   |  |

|---||--+-------+----|>|----+  |

Tx-PTT                          47uF                     |  |

<---------------\|     2k2                               |  |

npn |-----####--------|<|-------------------+  |

/|                1N4148                    |

-+-                                    |------+

Fig-1:  Circuit diagram of the suggested modem-interface

OP-1 acts as amplifier, OP-2 als Limiter. The supply voltage is

delivered by the modem lines DTR (+) and RTS (-). The program

sets this output pins in compliance with it.

The RTS pin can be used, if programmed so, for PTT-controlling.

You have two possibility to transmit SSTV images using this modem.

- send via PC speaker,

- send via Tx-d pin of the serial interface.

in both cases the jumper and the program parameter "Transmitting"

must be adjusted correctly (see this manual, F-2 key).

You can also use another simple interface. It is only important

to keep the square-wawe signal as symmetrical as possible.

Unfortunetly, the most OPs, like 741 have a big offset error. On that

reason an additional offset adjust in necessary. Minimizet the offset

error when receiving a strong single carrier singal (e.g. calibrator

signal of your receiver). Enter the scope function (F9) snd monitor

the video signal. Adjust the 50K offset trimmer to the minimum noise

ripple.

GSH-PC                 Appendix - 1/2                         SHW-1.00

----------------------------------------------------------------------

Another possibility is shown by fig-2. Using the CMOS-OP CA-3240 no

offset adjustment is necessary. Two diodes were added to protect the

input circuit against high input voltage.

Jumper   PC

Tx-Level         10K     10K                   o----> Speaker

+---------+--####-+--####------------------o

0.1uF  |         |       |                        o-+

<----||--->U 10K    ===     ===                         |

Tx-Mic.    |         |4n7    |4n7                       |

-+-       -+-     -+-       47uf    1N4148    |     RS-232

2x1N4148                         |---||---+---|<|---+ |     9.pol.

+-|<|-|    22K      1M                 +  |         | |     SUB-D

|        |-####-+--####-------+           |         | |

+-|>|-|         |             |           | CA-     | |

|       22K     |  CA-3240/1  |           | 3240/2  | +--o (3) TxD

|               |  +----\     |           |         |

|               +--|-     \   |   1K    +-+--\      +----o (4) DTR

|  1uF (bipolar)   | OP-1  >--+--####---|-     \

+------||-------+--|+     /             | OP-2  >--------o (6) DSR

|               |  +----/     |--####---|+     /

U 1K            U 22K             1K    +-+--/        +--o (7) RTS

Rx |               |                         |           |

>--+              -+-                        |           |  o (5) GND

+     |  1N4148   |  |

|---||---+----|>|----+  |

Tx-PTT                                 47uF              |  |

<---------------\|     2k2                               |  |

npn |-----####--------|<|-------------------+  |

/|                1N4148                    |

-+-                                    |------+

Fif-2:  Circuit diagram of the modified interface