GSH-PC                         - 1 -                       Norms_e.doc

----------------------------------------------------------------------

Description of the SSTV speed mode supporded by GSHPC

=====================================================

The SSTV video signal shows a color-bar image with following

order of colors (from left to right);

white / yellow / cyan / green / magenta / red / blue / black

1./ Martin Mode

===========

The Martin-Mode was developed by Martin H Emmerson. G3OQD.

The Martin-Mode are all free-run also called "New Mode".

speed      | Color  |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+------------

Martin-M1: | G-B-R  |   320 x 256    |   320 x 256    |    4:3

Martin-M2:*| G-B-R  |   160 x 256    |   160 x 256    |    4:3

Martin-M3: | G-B-R  |   320 x 128    |   320 x 256    |    4:3

Martin-M4: | G-B-R  |   160 x 128    |   160 x 128    |    4:3

*  M2 can be received setting M4.

#1 Resolution of image transfer

#2 Displayed resolution

Green           Blue             Red

|<------------->|<------------->|<----------->|

+-------+       +-+ +-+ +-+ +-+ +---+   +---+

|       |       | | | | | | | | |   |   |   |

|       |       | | | | | | | | |   |   |   |

|       |       | | | | | | | | |   |   |   |      |

--+ ++       +-------+ +-+ +-+ +-+ +-+   +---+   +---+ ++

| |                                                | |

+-+                                                +-+

5ms|(synch)      M1/M3 3x147 ms                     |

5ms|(synch)      M2/M4 3x 73 ms                     |

|<---------------------------------------------->|

2./ Scottie Mode

============

The Scottie Mode was developed by E.T.J Murpy GM3SBC.

The (modified) Scottie Mode are Free-Run mode.

speed      | Color  |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+-------------

Scottie-S1 | G-B-R  |   320 x 256    |   320 x 256    |   4:3

Scottie-S2*| G-B-R  |   320 x 128    |   160 x 250    |   4:3

Scottie-S3 | G-B-R  |   320 x 128    |   320 x 128    |   4:3

Scottie-S4 | G-B-R  |   160 x 128    |   160 x 128    |   4:3

Scottie-DX | G-B-R  |   320 x 256    |   320 x 256    |   4:3

*  S2 can be received setting S4.

#1 Resolution of image transfer

#2 Displayed resolution

GSH-PC                         - 2 -                       Norms_e.doc

----------------------------------------------------------------------

Green             Blue             Red

|<------------->|<------------->| |<------------->|

+-------+       +-+ +-+ +-+ +-+    +---+   +---+

|       |       | | | | | | | |    |   |   |   |

|       |       | | | | | | | |    |   |   |   |

|       |       | | | | | | | |    |   |   |   |

--+       +-------+ +-+ +-+ +-+ +-+ ++   +---+   +--+-

|                               | |               |

|                               +-+               |

|       S1/S3    3 x 140 ms  +  9ms (synch)       |

|       S4       3 x  90 ms  +  9ms (synch)       |

|       SDX      3 x 347 ms  +  9ms (synch)       |

|<----------------------------------------------->|

3./ Wraase SC-2 Color Mode

======================

SC-1 and SC-2 mode was developed by Volker Wraase in Kiel.

The old SC-1 mode are not supporded by GSHPC.

The improved SC-2 mode are Free-Run mode .

speed      | Color  |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+------------

SC-2  30s  | R-G-B  |   256 x 128    |   320 x 128    |   4:3

SC-2  60s  | R-G-B  |   256 x 256    |   320 x 256    |   4:3

SC-2 120s  | R-G-B  |   512 x 256    |   320 x 256    |   4:3

SC-2 180s  | R-G-B  |   512 x 256    |   320 x 256    |   4:3

#1 Resolution of image transfer

#2 Displayed resolution

Red                 Green                Blue

|<-------------->|------------------------->|<------------->|

+---+   +---+   +------------+             +-+ +-+ +-+ +-+

|   |   |   |   |            |             | | | | | | | |

|   |   |   |   |            |             | | | | | | | |

|   |   |   |   |            |             | | | | | | | |

-+ ++   +---+   +---+            +-------------+ +-+ +-+ +-+ ++ +

| |                |                          |              | |

+-+                |                          |              +-+

5ms|(synch)         |                          |              |

|      58 ms     |  SC_2 30s  117 ms        |     58 ms    |

|      58 ms     |  SC_2 60s  117 ms        |     58 ms    |

|     117 ms     |  SC_2 120s 235 ms        |    117 ms    |

|     235 ms     |  SC_2 180s 235 ms        |    235 ms    |

|<-------------->|<------------------------>|<------------>|

GSH-PC                         - 3 -                       Norms_e.doc

----------------------------------------------------------------------

3. ROBOT Color Mode

================

The Robot mode was developed by ROBOT RESEARCH, INC. San Diego CA9211

(e.g. Robot-1200c). Originally the Robot-Mode were not free-run mode,

modified Robot-1200c and compatible converter using Martin-Eprom and

high-performance oscillator can receive and transmit these mode using

the free-run facility. GSH-PC can receive all Robot mode normal or

free-run.

speed      | Color  |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+------------

ROBOT 12s  | Y-U-V  |   160 x 120 *A |   160 x 120    |   4:3

ROBOT 24s  | Y-U-V  |   320 x 120 *B |   320 x 120    |   4:3

ROBOT 36s  | Y-U-V  |   320 x 240 *A |   320 x 240    |   4:3

ROBOT 72s  | Y-U-V  |   320 x 240 *B |   320 x 240    |   4:3

#1 Resolution of image transfer

#2 Displayed resolution

*A Typ-A: Color information for each 2nd. line.

*B Typ-B: Color information for each line.

Typ-A: Color information (R-Y) in each odd line.

____

|    ----____                       ################

|            ----____              +################+

|                    ----____      |################|

--+ ++                            ------+                + +--

| |                                                    | |

+-+                                 | |                +-+

10,5ms| |   12s-Mode    90ms        4,5ms | |     45ms       |

7ms | |   36s-Mode    60ms          3ms | |     30ms       |

-/-/<------------------------------->/-/<-------------->|

B/W image  (Y)     col-sync.   R-Y

Typ-A: Color informatio (B-Y) in each even line.

____                             _

|    ----____                    | |################

|            ----____            | +################+

|                    ----____    |  ################|

--+ ++                            ----+                  + +--

| |                                                    | |

synch.+-+                                 | |                +-+

10,5ms| |   12s-Mode    90ms        4,5ms | |     45ms       |

7ms | |   36s-Mode    60ms          3ms | |     30ms       |

-/-/<------------------------------->/-/<-------------->|

B/W image  (Y)     col-sync.   B-Y

Typ-B. Color information (R-Y) und (B-Y) in each line.

___                                    _

|   ---___                  ###########| |############

|         ---___           +###########+ +############+

|               ---___     |###########   ############|

--+ ++                     -----+                          + +--

| |                                                      | |

synch.+-+                         | |           | |            +-+

12 ms | |   24s-Mode    88 ms     | |   44 ms   | |    44ms    |

12 ms | |   72s-Mode   138 ms     | |   69 ms   | |    69ms    |

/-/<----------------------->/-/<--------->/-/<---------->|

6 ms   R-Y    6 ms   B-Y

B/W image  (Y)    1.col-sync.   2.col-sync.

GSH-PC                         - 4 -                       Norms_e.doc

----------------------------------------------------------------------

Black/white Mode

=================

The Robot mode was developed by ROBOT RESEARCH, INC. San Diego CA9211

(e.g. Robot-1200c). Originally the Robot-Mode were not free-run mode.

Modified Robot-1200c and compatible converter using Martin-EPROM and

high-performance oscillator can receive and transmit these mode using

the free-run facility. GSH-PC can receive all Robot mode normal or

free-run.

speed      | Color  |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+--------

SC-1   8s  | Bright.|   128 x 128    |   160 x 128    |  1:1

SC-1  16s  | Bright.|   256 x 128    |   320 x 128    |  1:1

SC-1  16sq | Bright.|   128 x 256    |   160 x 256 *  |  1:1

SC-1  32s  | Bright.|   256 x 256    |   320 x 256    |  1:1

Robot  8s  | Bright.|   160 x 120    |   160 x 120    |  4:3

Robot 12s  | Bright.|   320 x 120    |   320 x 240    |  4:3

Robot 24s  | Bright.|   320 x 240    |   320 x 240    |  4:3

Robot 36s  | Bright.|   320 x 240    |   320 x 240    |  4:3

#1 Resolution of image transfer

#2 Displayed resolution

*  SC-1 16s "quasi" can be received in SC-1 8s mode.

white

____----+

____----        |

black    ____----                |

--+ +----                        ++ +--

| |                             | |

+-+                             +-+

|                               |

SC-1   8s    |5ms           60 ms            |

SC-1  16s    |5ms          120 ms            |

SC-1  16sq   |5ms           60 ms            |

SC-1  32s    |5ms          120 ms            |

ROBOT  8s:   |10ms          66 ms            |

ROBOT 12s:   |7ms          100 ms            |

ROBOT 24s    |12s          100 ms            |

ROBOT 36s:   |12s          150 ms            |

|<----------------------------->|

GSH-PC                         - 5 -                       Norms_e.doc

----------------------------------------------------------------------

The Vertical Interval Signal (VIS)

==================================

The VIS-code, introduced by ROBOT, is used by lot of SSTV-stations to

indicate the used speed-mode at the beginning of the transmission.

The VIS-code, decoded by the receiving stations allows the automatic

selection of the correct speed mode. The VIS-code is parth of the

vertical sync pulse, and consist of 10 bits each 30 ms long. The first

and last bit (start and stop bits) are represented by 1200 Hz tone.

The VIS-code is coded in the remaining 8 bits, including one bit

(even) parity. There are 128 possible combinations.

1900 Hz         1900 Hz

------------+ +------------+ +-start-bit                     +-

gray     | |    gray    | |   example-code = 43 dez.      |

| |            | | b1 b2 b3 b4 b5 b6 b7 parity   |

| |            | V                               |

| | "0"=1300 Hz|  +--+  +--+  +--+  +--+  stop   |

+-+     1200 Hz+--+  |  |  |  |  |  |  |  +--+---+

10ms "1"=1100 Hz      +--+  +--+  +--+  +--+

|           | |            |                             |   |

| ca.300 ms | | ca.300 ms  |      10x30ms = 300ms        | t*|

|<----------/-/<---------->|<--------------------------->|<->|

*t is the horizontal sync pulse of the first line, the length is

depending on the current sstv-speed. This length is very impor-

tant for the correct horizontal image position.

The meaning of the bits (with some exceptions);

+---+---+---+---+---+---+---+---+

| P | b7| b6| b5| b4| b3| b2| b1| <--LSB  least significant bit

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   | +---+---+

|   |   |   |   |   | | 0 | 0 | -> Color composite video

|   |   |   |   |   | | 0 | 1 | -> b/w   Red image

|   |   |   |   |   | | 1 | 0 | -> b/w   Green image

|   |   |   |   |   | | 1 | 1 | -> b/w   Blue image

|   |   |   |   |   | +---+---+

|   |   |   |   |   |     Horizontal resolution

|   |   |   |   | +---+   =====================

|   |   |   |   | | 0 | -> 128/160  Pixel

|   |   |   |   | | 1 | -> 256/320  Pixel

|   |   |   |   | +---+

|   |   |   |   |         Vertikalal resolution

|   |   |   | +---+       =====================

|   |   |   | | 0 | -----> 128/120  lines

|   |   |   | | 1 | -----> 256/240  lines

|   |   |   | +---+

|   |   |   |         Manufacturer / speed -typ

| +---+---+---+       =========================

| | 0 | 0 | 0 | --->  ROBOT

| | 0 | 0 | 1 | --->  Wraase SC-1

| | 0 | 1 | 0 | --->  Martin  (G3OQD)

| | 0 | 1 | 1 | --->  Scottie (GM3SBC) & Wraase SC-2

| | 1 | 0 | 0 | --->  AVT-Mode & Scottie-DX

| | 1 | 0 | 1 | --->  r.f.u. (reserved for futur use)

| | 1 | 1 | 0 | --->  r.f.u.

| | 1 | 1 | 1 | --->  r.f.u.

| +---+---+---+

|

+------------------> Parity-Bit (Even)

GSH-PC                         - 6 -                       Norms_e.doc

----------------------------------------------------------------------

The comlete list of the available VIS-codes;

The LSB of the VIS-code (bit-1) is sent as first, as usual by serial

data communication. Logical "1" is represented by 1100 Hz and

log "0" by 1300 Hz. The start and stopbits are modulated with 1200 Hz.

+---------------+---------------+----------+

| Norm-Nickname |msb  bits   lsb| decimal  |

|               | 7 6 5 4 3 2 1 |  value   |

+---------------+---------------+----------+

| ROB 12s col.  | 0 0 0 0 0 0 0 |     0    |      1: red image

| ROB  8s b/w   | 0 0 0 0 0 x x |   1,2,3  | ---> 2: green image

| ROB 24s col.  | 0 0 0 0 1 0 0 |     4    |      3: blue image

| ROB 12s b/w   | 0 0 0 0 1 x x |   5,6,7  |

| ROB 36s col.  | 0 0 0 1 0 0 0 |     8    |

| ROB 24s b/w   | 0 0 0 1 0 x x |  9,10,11 |

| ROB 72s col.  | 0 0 0 1 1 0 0 |    12    |

| ROB 36s b/w   | 0 0 0 1 1 x x | 13,14,15 |

+---------------+---------------+----------+

| 24s col SC-1* | 0 0 1 0 0 0 0 |    16    |

|  8s b/w SC-1  | 0 0 1 0 0 x x | 17,18,19 |

| 48s col SC-1* | 0 0 1 0 1 0 0 |    20    |

| 16s b/w SC-1  | 0 0 1 0 1 x x | 21,22,23 |

| 48q col SC-1* | 0 0 1 1 0 0 0 |    24    |

| 16q b/w SC-1* | 0 0 1 1 0 x x | 25,26,27 |

| 96s col SC-1* | 0 0 1 1 1 0 0 |    28    |

| 32s b/w SC-1  | 0 0 1 1 1 x x | 29,30,31 |

+---------------+---------------+----------+

| Martin-M4     | 0 1 0 0 0 0 0 |    32    |

| Martin-M3     | 0 1 0 0 1 0 0 |    36    |

| Martin-M2 *   | 0 1 0 1 0 0 0 |    40    |

| Martin-M1     | 0 1 0 1 1 0 0 |    44    |

+---------------+---------------+----------+

| Scottie-4     | 0 1 1 0 0 0 0 |    48    |

| Scottie-3     | 0 1 1 0 1 0 0 |    52    |

| Scottie-2 *   | 0 1 1 1 0 0 0 |    56    |

| Scottie-1     | 0 1 1 1 1 0 0 |    60    |

| Scottie-DX    | 1 0 0 1 1 0 0 |    76 #  |

+---------------+---------------+----------+

| SC2 col 30s   | 0 1 1 0 0 1 1 |    51    |

| SC2 col 180s  | 0 1 1 0 1 1 1 |    55    |

| SC2 col 60s   | 0 1 1 1 0 1 1 |    59    |

| SC2 col 120s  | 0 1 1 1 1 1 1 |    63    |

+---------------+---------------+----------+

| AVT col 24s * | 1 0 0 0 0 0 0 |    64    |

| AVT col 90s * | 1 0 0 0 1 0 0 |    68    |

| AVT col 98s * | 1 0 0 1 0 0 0 |    72    |

| AVT col 188s* | 1 0 0 1 1 0 0 |    76 #  |

| AVT b/w 125s* | 1 0 1 0 0 0 0 |    80    |

+---------------+---------------+----------+

# Scottie-Dx und AVT-188s share the same VIS-code.

* Not supported from GSH-PC.

- When in B/W mode GSH-PC sends always "green" code.

Martin mode was developed by Martin H. Emmerson G3OQD /England.

SC-1 und SC-2 converter were developed by Volker Wraase /Germany.

The Scottie mode was developed by E.T.J. Murphy GM3SBC /Scotland.

ROBOT is a trademark of ROBOT Research Inc. /USA.