BLASTER COLOR SLOW SCAN

using the

SOUND BLASTER

COLOR SLOW SCAN for the Sound Blaster is now available! This

shareware program will send and receive ROBOT 36 COLOR,

Scotty 1, and now in version 1.5 Martin 1. This program has

evolved from a 8 second black & white version that I released

in December of 1992. Many changes have taken place, making

the program better than ever. The response has been good and

lots of suggestion and ideas have been incorporated.

Thanks to all!

Required equipment:

IBM (or compatible) computer (286 minimum), the faster, the better!

Math co-processor not necessary, but also is nice, as the pictures

display faster.

VGA display, capable of 640 x 480 - 256 colors. This requires a VGA

card with at least 512K memory.

Sound Blaster (or compatible) sound card. The program has been

tested with the Sound Blaster V1.5, V2.0, Pro,and SB16, as well as Media

Visions Pro Audio Spectrum 16 and Fusion 16 (let me know of others

as you try them!).

This program is designed to display COLOR slow scan

pictures that you can copy off the ham radio bands. The

frequencies that you will normally find slow scan are:

3.845  MHZ

7.171  MHZ

14.230 MHZ (and 14.233 MHZ)

21.340 MHZ

28.680 MHZ

The most popular frequency is on 20 meters at 14.230 MHZ, with

14.233 MHZ being used on weekends when there are too many on

14.230.

The following diagrams might help with the electrical hookup. For

receive, the audio from your receiver needs to be connected to the

microphone input on the Sound Blaster. The audio from your receiver

may come from the speaker output. In my case, I used an audio

output from the back of the radio (Icom 730) that is to be used for

RTTY, Slow Scan, packet, etc. The audio out of the back does not

vary with the volume control, so I can keep the audio at a

reasonable level and still copy pictures.

For transmit, take the audio out (one channel, left or right) to

the microphone on your transmitter. Again, I went into the back of

my radio. With the IC730, you have to use an amplified microphone.

Mine just happens to have a level control under the base so the

level into the microphone can be adjusted down to keep the room

noise from interfering with the picture. Use your microphone button

to key up the transmitter.

Sound Blaster Board V1.5 and V2.0

________________

|

|<---- Sound in from receiver

|

Volume -->|

Control   |

|----> Sound out to your microphone input

|

|

|<---> Midi input/output (not used for this

|      project)

________________|

Sound Blaster Pro

________________

|

|<---- Line in (not used for this project)

|<---- Microphone in - Sound in from receiver

Volume -->|

Control   |

|----> Sound out to your microphone input

|      for transmitting Slow Scan

|

|<---> Midi input/output (not used for this

|      project)

________________|

I did the development on the Sound Blaster Pro, but I kept the

sample rate at 12 KHZ (the top speed for the Sound Blaster V1.5 and

V2.0) so all Sound Blasters could be used.

Now that the program has been out for a short while, I've received

feedback from others who inform me that it also works on the PAS16

(Pro Audio Vision from Media Vision) and the Fusion 16 (also from

Media Vision). In this version I am packing the drivers, as some of

the cards, including the V1.5 and V2.0 from Creative Labs, do not

come with the ones needed. Make sure to read about the drivers as

they have to be set up just right for the program to work properly.

Copy the program to your hard disk (preferably make a separate

subdirectory called SS). Your computer should have several meg of

free space as the .VOC files will take up about 100k for every

8 seconds of recording. Also you will want to save some of your favorite

pictures in .PCX files so you can recall them later. The reason I

chose the .PCX format is that it generally a lot less space than

.VOC files. The display on your computer needs to be a VGA display

that will work in 640 x 480 - 256 color mode. Some VGA cards do not

have enough memory to do this. If it doesn't work on your system,

I need to know so I can make the necessary changes. When reporting

problems, please make sure to include as much information about

your computer system as possible (autoexec.bat, config.sys, path,

etc), so I can be of help.

The first time that you run the program type "ss". You will be asked

some questions the first time that you run the program. Your response

is saved in a file called "SS.CFG". The next time that you

run the program, you will not be given the choices as it knows from the

file "SS.CFG" what you selected the first time. If you need to make

changes, you can:

1. Erase the "SS.CFG" file from your hard drive, or

2. Type "ss v" instead of "ss" to start the program. This will

automatically erase "SS.CFG" so you can make your

selections again.

To start the program after it is loaded type "SS" ("SS V" to reselect

the configuation setup.

Once you have the audio connected, and a station ready to transmit,

pressing F2 will start the receiving process. The sound is stored

in a disk file named "TEMP.VOC". This file is overwritten every

time you press F2. You will not see anything happen until you press

ESC which should be pressed as soon as the audio for the picture

stops. The "TEMP.VOC" file will be a minimum of 1.5 MEG in size for

one SCOTTIE 1 picture, so you will need to make sure that you have

several MEG free on your hard disk where ss.exe is loaded (several

megabytes would be even better). You will learn after using the

program that it takes a little bit before it starts recording, so

you need to anticipate the start of the picture just a little.

If you anticipate too much, you can do the following:

Press ESC will stop the recording & start to display whatever

there was to that point

- Pressing ESC again will stop the display

- Pressing F2 will restart the recording.

Once you have a picture on the screen, if you Press F2 again you

will loose the one you have. Menu item F7 allows you to save the

picture as a .PCX file, or you can rename the "TEMP.VOC" file to

another name so when you press F2 again, you do not loose the

picture. You can not save in the shareware version.

Transmit has been added with two options.

1. Pressing F4 will transmit "SEND.VOC", the last picture

received.

2. Pressing F5 will give you a menu of ".VOC" files that

you have saved. By saving the "TEMP.VOC" file to other

names, you can later re-transmit the pictures you have

received. It's just like a tape recorder!

Help is built into the program and accessed by pressing F1. This

will give you a second menu of help items. Just press the F key

that will give you the information that you want.

I hope this program will give you some enjoyment. If it does, I

would also hope that you will register the program for $30.00. I

have spent over 4 years writing this (not every day mind you) and

it was an exercise for me in learning to program in "C". Well, I

guess something must have worked! Anyway, if I get enough response,

I will get to work and enhance the program with the other modes.

Also, I am interested in any ideas you may have to improve the

program or ideas for other programs for the Sound Blaster. I bought

their development kit, and might as well put it to work. So please

send your $30.00 and ideas for enhancement and other programs you

would like to see (how about copying RTTY or PACKET RADIO with the

Sound Blaster ???). Also, anyone sending in the registration with

$30.00 will receive credit towards any major upgrade to the next

higher level.

Speaking of UPGRADES!!!

BLASTER COLOR SLOW SCAN is available! It is the same as this program

with the other modes enabled and the SAVE features activated.

Audio Analyzer - Another NEW PROGRAM!

Now you can view the audio signals that you hear on the air! Audio

Analyzer will display the change in frequency in relation to time

so you can "SEE" what Slow Scan, RTTY, Packet Radio, Weather Fax,

and other audio signals 'look like'. The waveforms can be saved to

voice files or viewed REAL TIME! Waveforms saved to .VOC files

can be viewed over and over. If you want finer detail, you can EXPAND

the image or you can SHRINK the image to view more at one time.

The vertical frequency range can be adjusted up or down as needed.

This utility was useful in developing the SLOW SCAN programs and

understanding what is going on. A TIME MEASUREMENT feature allows you

to measure sync pulses, ect., in milliseconds.

Comments or questions can be left for me at:

atvq@aol.com

Questions and orders can be handled by calling (815) 398-2683. I'm

usually home by 6:00 pm (CST) unless I have meetings, etc. Our

fax number is (815) 398-2688.

Also check our web page for the latest on upgrades:

http://www.cris.com/~Gharlan

I have enjoyed helping those that have called to date. All the

comments and compliments were appreciated!!!

*********************************************************************

NEW - We are the NEW publishers of Amateur Television Quarterly !!!

*********************************************************************

Amateur Television Quarterly

published by Harlan Technologies

5931 Alma Dr., Rockford, IL 61108

tel (815) 398-2683  fax (815) 398-2688  orders (800) 557-9469

Internet:  http://www.cris.com/~Gharlan     email: atvq@aol.com

As of October 17, 1997, ATVQ has a new publisher, Harlan Technologies! We

have a little catch up to do, and the next issue (Summer 97) will be out

about November 1. This will be followed by the fall issue around

December 1. After that we expect to be back on schedule and it will

be our goal to stay on schedule!

We are always looking for new authors. If interested in writing about

the following topics, please send email to let us know.

Amateur Television Quarterly, the publication for the video aspects of

ham radio, brings you the latest in: ATV ACTIVITY ON EARTH AND SPACE,

PUBLIC SERVICE ATV ACTIVITY, SLOW SCAN TELEVISION, BUILD-IT PROJECTS,

FIRST HAND ACCOUNTS OF ATV ACTIVITIES, NEWS, ARTICLES AND FEATURES,

INSIGHTS FROM THE EXPERTS, NEW PRODUCT REVIEWS, TECHNICAL INFO,

AND LOTS MORE ! Dont miss the action, SUBSCRIBE NOW !

RATES          (Effective 10-17-1997 - Subject to change without notice)

USA             $18.00 / Year

CANADA          $20.00 / Year

ELSEWHERE       $26.00 / Year    ( All in US $ )

NAME ________________________________________  HAM CALL  _______________

ADDRESS ________________________________________________

CITY  ________________________ STATE  _______  POSTAL CODE  ____________

COUNTRY  ______________________

PHONE __( ____ ) ___________________  EMAIL  ___________________________

_____  VISA  _____  M/C  _____ AMEX  _____ CHECK

CARD NUMBER __________________________________ EXPIRES  ________________

SIGNATURE  ________________________________________________

ARE YOU CURRENTLY ON ATV ? ____  YES  ____  NO

ARE YOU CURRENTLY ON SSTV  ____  YES  ____  NO

DO YOU USE AN ATV REPEATER  ____  YES  ____  NO

IF SO, CALL OF THE REPEATER  _____________  LOCATION  __________________

FREQUENCIES: INPUT _________ OUPUT _________ 2 METER TALK BACK ______

ATV NET NIGHT AND TIME ____________________________________________

DOES THE REPEATER HAVE A WEB PAGE?

IF SO ADDRESS http://___________________________________________________

WOULD YOU BE INTERESTED IN WRITING FOR ATVQ?  ____  YES  ____  NO

ANY COMMENTS? __________________________________________________________

________________________________________________________________________

______________________________________________________________________________________________________________________________________

______________________________________________________________________________

****************************************************************

ORDER FORM FOR SSTV

****************************************************************

Please send the following registration form to:

Harlan Technologies

5931 Alma Dr.

Rockford, Illinois 61108

Registration Information

_________________________________________________________________

______ I have enclosed $30.00 and would like to register the Slow

Scan program.

_______NEW!!! Audio Analyzer - View the waveforms that you hear for

Slow Scan, RTTY, Amtor, Packet, ect. See the sync pulses and

be able to measure the time in milliseconds. This program

is only $39.95 plus $5.00 shipping (Illinois residents include

$2.50 tax) Real time samples can be taken or record and playback

from .VOC file.

PRICE REDUCTION !!! WAS $99.95 NOW $49.95 !

_______BLASTER COLOR SLOW SCAN - NOW AVAILABLE! only $49.95 plus $5.00

shipping (OVERSEAS SHIPPING $10). (Illinois residents

include $3.12 tax) Get in on all the fun TODAY!

VISA / MASTERCARD / AMERICAN EXPRESS ACCEPTED! Call Today!

(815) 398-2683 (VOICE) or (815) 398-2688 (FAX)

ORDER INFO

____VISA ____ MASTERCARD ____ AMERICAN EXPRESS ____ CHECK ____ COD ($5.00)

CARD NUMBER ________________________ EXP DATE ______________

SIGNATURE _____________________________

PLEASE PRINT THE FOLLOWING

Name___________________________________ Call Sign_______

Street_________________________________

City___________________________________ State __________ Zip__________

Phone _________________________________ Email ________________________

Type/speed of computer __________________________________

Size of hard drive ______________________________________

Type of display card ____________________________________

Ram - how much do you have ______________________________

Sound Blaster _____ Sound Blaster Pro _____ SB20 _____SB16______

AWE32 ______ AWE64 ______

PRO AUDIO SPECTRUM 16_____ FUSION 16 _____ OTHER __________________

3 1/2 disk _____  5 1/4 disk _____ (Disk size preferred)

Other information - ideas - suggestions

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________