BPQ-DED    (C)1995 Brian E. Cauchi 9H1JS    31-July-1995

Provides interface between G8BPQ host mode & WA8DED host mode.

I wrote this for use with TSTHOST, 8 channels & unproto/monitor.

It provides a TFPCX like interface.

Default interrupt numbers are 127 for BPQ, and 255 for the TF interface. They

may be changed by specifying them as decimal numbers:

BPQ-DED /B126 /T253

The driver may be unloaded by using /U

So, a typical batch file would look something like this:

BPQCFG

BPQAX25 ... if you use a baycom type modem

BPQCODE

BPQ-DED

TSTHOST /T /I255

BPQ-DED /U

BPQNODES

What needs to be configured

---------------------------

TSTHOST.CFG parameters for automatic connection

HCHANNEL        1

HNODE           SWITCH

NPROMPT         }

TSTHOST.PSW

If you use a password to connect your master bbs, add a replica of the

line in the TSTHOST.PSW file, with SWITCH in place of the BBS call.

BPQCFG.TXT

Add UNPROTO=<bbscall> in the respective 'Port' section

Known bugs & shortcomings

-------------------------

1. The USERS command of the node causes a 'Division by zero' error.

For now, don't use it! Use the LINKS commmand instead.

2. Node broadcasts and Net/Rom information is not fully decoded.

Any info appreciated.

Finally, send bug reports & comments to:

9H1JS @ 9H1AX

or, by post to:

Brian E. Cauchi,

Gawhra, Domenico Cachia Street,

Birkirkara BKR06, MALTA.