BPQLog

Martin Schick, KA4IWG

1996  Version 1.0

Revisions:

1.1 removes the immediate unload hotkey since it causes crashes and changes the toggle key combination to <ctrl><L> to avoid PacketCluster functions.

Program:

BPQLog is a TSR that is installed between BPQ code and an application program, such as PacketCluster.  The purpose of BPQLog is to monitor the various radio ports of  a BPQ node and print the monitored packets on the computer system’s line printer, LPT1.

BPQLog was developed as a troubleshooting tool for sysop’s of PacketCluster nodes.  It is not anticipated that the program would be used on a continual basis, but only in helping to determine causes of communication problems between the node and users or other nodes.

BPQLog was written in Turbo Pascal and utilized TPU’s supplied from TurboPower Software for the development of the TSR and ISR segments of the program.  Most of the procedural names of these routines are self explanatory.

The monitoring can cover all the packets heard including supervisory frames, limit monitoring to  those packets associated with a given callsign, limit the monitoring only to information frames, limit the monitoring to only one radio port of the node, or combinations of these.

At present, in the isolated callsign arrangement, BPQLog will only allow for the monitoring of a single callsign.  Likewise, in the isolated radio port arrangement, it will only allow the monitoring of a single radio port.

It is important to mention that BPQLog does not do the actual monitoring.  It simply diverts the monitoring performed by the application program to the line printer.  Thus, for example, in the case of the PacketCluster program, PacketCluster must be in the monitor mode in order for BPQLog to be operational.  When PacketCluster is returned to normal command mode, BPQLog will be inactive.

Usage:

BPQLog must be loaded into memory after the BPQ code has been installed in the computer memory.  BPQLog must be located in the same directory as the BPQCFG.BIN file in order to load properly.  This is due to the fact that BPQLog examines BPQCFG.BIN to determine what interrupt BPQ code is using as an interface.  BPQLog will only load under BPQ version 4.07B or higher.  This is due to the method used to determine the presence of BPQCODE which was introduced in version 4.07B.

BPQLog has been tested on a 80386 computer with a DOS based platform.  This system utilized QEMM as a memory manager.  It has not been tested under Windows or Desqview.  A simple dot matrix printer was used as a test printer.  PacketCluster was used as the test application.

It is best to load BPQLog from a cold start of the system rather than shelling to DOS from the application or simply shutting down a running application and installing.  This is due to the fact that BPQ may remain active for some time after an application is shut down and the loading of BPQLog while BPQ is active can result in interrupt conflicts.  However, from a cold start up, BPQLog can be installed from within a batch file or from the DOS prompt.

Presently, BPQLog makes use of BIOS routines to interface only with LPT1.  Consequently, it is important that a suitable printer be connected to this port and be available for printing.

Options:

BPQLog is loaded by entering the command line “BPQLOG [Options} <enter>“, where the options consist of the port selection, callsign, help screen, packet types or unload command.  The options can be prefaced with a “/” or a “-” character.  If a “/” is used, no space is needed following the program name.  If the “-” is used, a space must be left between the program name and the options.  If additional options are entered after the first, a space must separate the options.

If no options are given on the command line, BPQLog will monitor all packets heard.  If this options is selected, be sure to have plenty of paper in the printer.  Its somewhat amazing to see how fast the printout stack grows.

The “/C=aanaaa” or “-C=aanaaa” option will install BPQLog in such a manner so that it will only monitor packets addressed to or sent from the station with a callsign of “aanaaa”.  An example would be “/C=KA4IWG”, in which case only packets with this callsign in them would be printed.  Notice that if an alias is used for this call, they will not be monitored.  Also, if a station is using a SSID, it must be supplied as part of the option since BPQLog will distinguish between stations with various SSID’s.

The “/P=n” or “-P=n” option will install BPQLog in such a manner that it will monitor only the port designated by the number “n”.  Obviously, this must be an existing radio port.  This port is the actual port number that the radio is assigned, not a user stream or channel number.

The “/I” or “-I” option will install BPQLog in such a manner that only the information frames of the designated system will be monitored.

The “/U” or “-U” option will remove BPQLog from memory.

The “/?” or “-?” option will display a short help screen explaining these options and halt the routine.  If it is used as an option, the help screen will appear, but the routine will not be loaded.

Various combinations of these options can be used on a command line to narrow down the scope of the monitoring.  However, each option type can only be used once on the command line.  Thus it is possible to issue “/C=KA4IWG /P=2 /I” to monitor only information packets sent from or addressed to KA4IWG on port 2.  But issuing an option list such as of   “/C=KA4IWG /C=N9UQI /C=W9OFA /P=2 /P=3” will not result in the intended objective.

Hotkeys:

There is one hotkey combination available in BPQLog,  “<ctrl><L>“.  This hotkey can be utilized from within the application, such as PacketCluster, by shelling to DOS and then issuing the key combination.

The “<ctrl><L>“ combination will toggle the monitor logging off and on.  When the program is loaded into memory, it is loaded with the monitoring off.  Thus, the user must shell to DOS and issue the “<ctrl><L>“ combination in order to activate BPQLog monitoring.  The user may toggle the monitoring between off and on at any future time by repeating this sequence.

I hope this routine will be helpful to folks and would welcome comments on its usage.

Martin Schick, KA4IWG

BPQ, BPQCODE and BPQCFG are software routines written by John Wiseman, G8BPQ.  They are not public domain and may be used only by licensed radio amateurs.

Turbo Pascal is a trademark of Borland International

PacketCluster is a trademark of Pavillion Software

Desqview is a trademark of Quarterdeck Office Systems

Windows is a trademark of Microsoft Corporation

The author of this software does not assume any liability for the use of the BPQLog routine and in no event will the author be liable for damages arising out of or inability to use this program.