BPQBoot

Version 1.10

by Martin Schick KA4IWG

1996

Revisions:

1.1 removes the immediate unload hotkeys since these caused crashes.  Also, the toggle key was changed to <ctrl><B> to avoid PacketCluster functions.

Program:

BPQBoot was written in Turbo Pascal and the source code is included.  It utilizes TPU’s from TurboPower Software which allow for the development of TSR’s and ISR’s.  Most of the procedural names for these units are self evident in their purpose.

BPQBoot sits between the BPQCODE and PacketCluster routines and scans incoming packets to the cluster.  If a packet is received with a specific text string,  the system computer is reset with a warm boot.  This routine can be used as a method of rebooting the system remotely.

Needless to say, the system must be running in order for BPQBoot to function.  It will not work if the system has hung and is not processing the incoming packets.  Thus it is not foolproof.  However, it does add a means of resetting the computer for such things as phantom nodes, during changes in parameters, loops and other traditional problems with PacketCluster operations.

Usage:

In order to properly install BPQBoot, it must be loaded from the same directory in which the BPQCFG.BIN  file, the configuration file for the BPQ code, resides.  This is due to the fact that BPQBoot must examine this file to determine which interrupt BPQCODE is using at the present time.

BPQCODE must be installed prior to installing BPQBoot.  Since BPQBoot redirects the interrupt vector for the given interrupt of BPQCODE to BPQBoot, the vector must have already been installed in order to redirect it.  Only the version 4.07B and above series of BPQ are acceptable to BPQBoot since it utilizes the method of determining if BPQ is present introduced in the version 4.07B.

BPQBoot should not be installed from within the PacketCluster program by use of the DOS shelling routine.  Since the PacketCluster program usually enters the DOS shell by swapping memory and may still issue interrupts to BPQ while shelled, installing BPQBoot at that stage will give unpredictable results.  However, the hotkeys, mentioned later, can be used from within a DOS shell.

From my experience with usage on a PacketCluster node, it has been best to completely reboot the computer before loading BPQBoot.  When I have tried to shutdown PacketCluster and either load or unload the routine, the computer has crashed.  I suspect this is due to trying to swap interrupt vectors while they are still in use by BPQ.  I have never had a problem when installing the routine from a cold start with the routine’s command line in a batch startup file.

When BPQBoot is initially installed, the scanning process is active.  The scanning can be deactivated by use of the “<ctrl><B>“ key combination.  In a PacketCluster system, this can be done by simply shelling to DOS from a running PacketCluster routine and then issuing the “<ctrl><B>“ combination.

The initial version of BPQBoot has been tested only under a DOS based 80386 system with the QEMM memory manager.  There has been no evaluation of the routine under Desqview or Windows, so no information can be given to its reliability under these platforms.

Options:

BPQBoot is installed by entering “BPQBOOT [OPTIONS] <enter>“.  The options may be “/?” or “-?’, “?U” or “-U”, or the desired reset character string.  Only one option is allowed on the command line, since only one is necessary.  Also, like DOS commands, there must be a space between the program name and the options when using the “-” key, but the space is not necessary if the “/” key is used.  The “/?” or “-?” option will bring up a short description of the various options available.  The “/U” or “-U” option can be used to remove a copy of BPQBoot from RAM memory.

To enter the desired reset character string, the “-$” or “/$” option is used and the “$” is followed by a short, unique string of characters.  For example, “BPQBOOT /$FOOBAR” would install the routine so that any incoming packet containing the text “FOOBAR” would reset the computer.  This string could be send using a number of techniques, such as “SH/FOOBAR” or “T FOOBAR”.  Since the string is detected before PacketCluster reads it, the context is not critical.  It is important, however, to maintain the case of the string.  Thus, upper and lower case characters in the string are unique.  There is one hotkey combination which can be used in BPQBoot.  This is the “<ctrl><B>“ combination.  DOS must be in control of the system when these key combinations are used.  Therefore, by shelling out to DOS using the “DOS” command, these hotkeys can be utilized.

The “<ctrl><B>“ combination will toggle the scanning process between active and inactive.  WHEN BPQBOOT IS INSTALLED, THE SCANNING IS ACTIVE!  It can be deactivated by use of the “<ctrl><B>“ combination at a later time.  The scanning process can be reactivated by repeating the “<ctrl><B>“ combination.

Testing:

I have tested this routine on my PacketCluster node, an 80386 CPU, and have found it to work well.  I have given the reset string to several of the local operators with permission to reset the node if the system is misbehaving.  I also have a watchdog board in the system for catching instances when the computer completely hangs up.  Between these two systems, it should be possible to cover most upsets in the system.

I hope this routine will be helpful and would appreciate any suggestions or feedback on its usage.

Martin Schick  KA4IWG

BPQ, BPQCODE and BPQCFG are software routines written by John Wiseman, G8BPQ.  They are not public domain and may be used only by licensed radio amateurs.

PacketCluster is a trademark on Pavillion Software.

Desqview is a trademark of Quarterdeck Office Systems.

Windows is a trademark of Microsoft Corporation.

Turbo Pascal is a trademark of Borland International

The author of this software does not assume any liability for the use of the BPQBoot routine and in no event will the author be liable for damages arising out of the use or inability to use this program.