PC/FlexNet 3.3e

USER  MANUAL

(English version written by Eric BERTREM, F5PJE @  HB9IAP © 1995 - 1996)

[Final editing and presentation by Ian Wade, G3NRW — ian@dowrmain.demon.co.uk]

PC/FlexNet is © Gunter Jost, DK7WJ @ DB0ZDF

PC/FlexNet WWW : http://www.imaginet.fr/~ferracci/flexnet

Update : 1st May 1996

IMPORTANT: PC/FlexNet is still under development. All notes and comments included in this document, as well as driver specifications, may change without notice. This manual was not produced by the FlexNet team. PC/FlexNet must not be distributed via Internet, packet radio, Fidonet or similar networks with open accounts. Also, it must not be sold or used for professional purposes. If you appreciate PC/FlexNet, please don’t forget to encourage the developers by sending them a contribution which will help further developments.

1.  INTRODUCTION

First, always remember that PC/FlexNet can be used with or without a digipeater function. If you need a comprehensive FlexNet digipeater, run FLEXNET.EXE and FLEXDIGI.EXE. If you only need a TFPCX-like driver, only use FLEXNET.EXE and the associated Level 1 drivers.

Now, let’s pay more attention to PC/FlexNet technical specifications.

First distributed in October 1994, PC/FlexNet 3.3 is fully compatible with RMNC/FlexNet (RMNC = hardware, FlexNet = software). You simply need to use a PC-compatible computer, instead of 6809-based CPU cards. Here is the minimum configuration you need to get your PC/FlexNet on the air:

l Hardware and Software:

PC/XT, or PC/AT if possible, with at least 512 KB RAM. PC/FlexNet needs about 200 KB RAM for its own use, excluding the memory needed by the Leve1 1 drivers and utilities. The computer speed will determine the maximum radio speed usable. Software takes less than 1 MB on the HD.

l DOS:

MS-DOS (3.1, or better 5.0, or even 6.2). With 6.2, packets will be loaded high in UMBs. 6.0 cause trouble, please don’t use it.

l Outputs:

According to your needs and the available drivers. The easiest interface to implement uses BayCom USCC cards, with 2, 4, or 8 radio channels (with various high-speed modems, from 300 baud AFSK up to 38400 FSK).

PC/FlexNet uses various Level 1 drivers. These drivers allow the use of any hardware with PC/FlexNet: BayCom serial modems, BayCom 9600 FSK LPT modems, USCC cards, Ethernet drivers, sound cards, etc. You simply have to program a new driver if your hardware can’t be used. A driver kit is available — ask for it if you’re a programmer.

For users who don’t need to run a real digipeater, a “limited” version has been created, without routing capabilities, and without a built-in digipeater. It is called FLEXNET.EXE and has to be run with another program, like FBB, AA4RE, TOP, SP, Graphic Packet, etc. Please check the DOC file to find out more about this driver.

For sysops, the use of FLEXNET.EXE and FLEXDIGI.EXE is mandatory, unless you don’t need the digipeater function.

2.  How to get PC/FlexNet ?

First, remember than PC/FlexNet can’t be distributed via open accounts on packet-radio, internet, fidonet, or similar systems.

If you need an updated version, first try to contact PC/FlexNet author via packet (DK7WJ@ DB0GV). If you get no answer, then try the following addresses :

l packet:		F5PJE @ HB9IAP

l Email:		ebertrem@curis.gatelink.fr.net

l Fido:		Eric BERTREM, 2:323/9.2

l Fax:		(+33) 7808-1806

l Snail mail:	Eric BERTREM
			207 Rue Des Combes
			69250 Curis Au Mont D’Or
			- FRANCE -

There are two ways to get updates from myself :

l Mail: I suggest you send 6 IRCs + two 3½" diskettes to cover two shipments.

l Modem: if you can use a high-speed modem (14400 bps minimum), we can easily 
	  exchange data. Please contact me via fax to decide a sked.

3.  PC/FlexNet drivers

As already mentioned above, PC/FlexNet uses different Level 1 drivers. Here is a short description of the use of these drivers (each of them has a DOC file, please read it first, thank you):

| Driver Name | FLEXDIGI.EXE |
| --- | --- |
| Command Syntax | FLEXDIGI |
| Functions | This driver adds the digipeater function to FLEXNET.EXE. If you don’t load it, you won’t be able to connect to PC/FlexNet, but will only be able to use it like a powerful TFPCX, using any Level 1 driver you want (USCC card, Baycom serial modem, etc.). |

| Driver Name | DUMMY.EXE |
| --- | --- |
| Command Syntax | DUMMY #_of_channels |
| Example | DUMMY 4 |
| Functions | DUMMY isn’t a driver but a “place holder”. Since ports are numbered according to the order in which drivers are started, it can sometimes be useful to deactivate a port without having to change the parameters for other ports. Consequently, use DUMMY to fill in one or more ports. Thus, for example, the command DUMMY 4 reserves 4 ports. Type DUMMY /? to get help. |

| Driver Name | KISS.EXE |
| --- | --- |
| Command Syntax | KISS com [/P=com_address /I=com_IRQ] |
| Examples | KISS 1 KISS 4 /P=0x2e8 /I=5 |
| Functions | KISS.EXE is used to run KISS mode on any COM port. If you intend to use KISS-CRC, add “c” in the MODE command (e.g. MODE 3 9600c). 16550 UARTs are detected but haven’t been tested yet. If you use a COM between 1 and 4, there’s no need to specify the address and IRQ, unless they are not standard. Type KISS /? to get help. |

| Driver Name | PIF.EXE |
| --- | --- |
| Command Syntax | (see PIF.DOC) |
| Example | (see PIF.DOC) |
| Functions | Link two computers via their LPT ports. |

| Driver Name | USCC.EXE |
| --- | --- |
| Command Syntax | USCC /P=add /I=IRQ /C=chans /B=buffer /T=tail /S |
| Example | USCC /P=0x300 /I=7 /C=8 /B=80 /T=1 /S |
| Functions | /P = card address /I = card IRQ /C = number of channels /B = frame buffer (s=channels * 10) /T = tail-time for full duplex (seconds) /S = use DOS clock tick instead of scc timer for Level 1 timing USCC.EXE can use the following BayCom USCC cards: BayCom USCC>4 (4 channels) USCC9k6 (2 channels) Old SCC>4 with 8 channels USCC>8 (8 channels) To use the open-squelch DCD on a channel, use parameter “c” in the MODE command; e.g. MODE 1 1200c. |

| Driver Name | DG1SCR.EXE |
| --- | --- |
| Command Syntax | (see DG1SCR.DOC — in English and German). |
| Example | (see DG1SCR.DOC — in English and German). |
| Functions | Allows the use of the DSP card developed by DG1SCR, with 1200 or 9600 baud FSK, plugged into a COM port. |

| Driver Name | DSKC26.EXE and DSKC50.EXE |
| --- | --- |
| Command Syntax | (see the DOC files — in English and German). |
| Example | (see the DOC files — in English and German). |
| Functions | Use the DSP Starter Kit from Texas Instrument, a DSP processor card which can run 1200 or 9600 FSK. |

| Driver Name | PSADRVR.EXE |
| --- | --- |
| Command Syntax | (see PSADRVR.DOC) |
| Example | (see PSADRVR.DOC) |
| Functions | Supports the use of PSA ICs from Analog Devices and run 1200 and 9600 baud. |

| Driver Name | SBLAST.EXE |
| --- | --- |
| Command Syntax | (see SBLAST.DOC) |
| Example | (see SBLAST.DOC) |
| Functions | 1200 baud AFSK with a Sound Blaster card. You need a fast computer (486 minimum for example) and will only be able to use half-duplex. |

| Driver Name | SNDDIAG.EXE |
| --- | --- |
| Command Syntax | (see SNDDIAG.DOC) |
| Example | (see SNDDIAG.DOC) |
| Functions | Make a diagnostic on your sound driver’s setup. |

| Application Name | WSS.EXE and WSS_9k6.EXE |
| --- | --- |
| Command Syntax | (see the DOC files) |
| Example |  |
| Functions | Driver for use with wiNdOoze SoundSystem compatible soundcards |

| Driver Name | IPPD.EXE |
| --- | --- |
| Command Syntax | (see IPPD.DOC) |
| Example | (see IPPD.DOC) |
| Functions | This driver encapsulates AX.25 frames within an IP header and sends them on the Ethernet network. It has been developed to connect a KA9Q-like program to PC/FlexNet through a LAN. |

| Driver Name | 6PACK.EXE |
| --- | --- |
| Command Syntax | (see 6PACK.DOC) |
| Example | (see 6PACK.DOC) |
| Functions | This driver allows the use of a maximum of 7 TNC-2 on each COM port. a BIN file is provided and has to be used in the TNC-2 27C256 Eprom. The use of a 16550 COM port is strongly recommended... |

| Driver Name | IPXN.EXE |
| --- | --- |
| Command Syntax | IPXN |
| Example | IPXN |
| Functions | Novell driver. (Needs a TSR IPX-Novell driver). Allows the interfacing of your digipeater to a Novell LAN. Only PCs using the same physical segment are reachable, with no possibility of using routers, gateways or multiport servers. IPX uses the IPX driver which has to be loaded beforehand. Traffic is sent via Ethernet broadcast, just like on a radio link where all stations can hear each other. Therefore, you don’t have to handle the IPX routing. In addition, you can monitor all channels of networked PC/FlexNet. IPXN is started without parameters. Warning: IPX Novell drivers aren’t freeware but commercial software. One more hint: PDIPX.COM (public domain from Intel) can’t be used since it can receive IPX broadcasts. If you find a solution, please let us know. Specifying a speed and mode is useless when you use IPXN.EXE. |

| Driver Name | IPXD.EXE |
| --- | --- |
| Command Syntax | IPXPD packet_interrupt_vector |
| Example | IPXPD 0x60 |
| Functions | Ethernet driver for IPX. Needs a TSR packet driver. It supports a connection to Ethernet using the IPX protocol. Only PCs using the same physical segment can be reached, without gateway, router or multiport servers. IPXPD uses the packet driver which has to be loaded before. Traffic is sent via Ethernet broadcast, just like on a radio channel where all stations can hear each other. IPXPD can only be used if no IPX Novell shell is in use on the computer. Warning: the Novell IPX driver isn’t shareware, but commercial software. IPXPD uses Ethernet II frames with frametype 8137, like on the new Novell 3.12. You may have trouble with older versions. If so, use “ECONFIG” to solve the troubles. IPXPD is started specifying the packet driver IRQ number, between 0x60 (96) and 0x7F (127). |

| Driver Name | FLEXPAS.LZH [Turbo Pascal Interface] |
| --- | --- |
| Command Syntax |  |
| Example |  |
| Functions | Written by Michael, DG9FDL, this package provides a PC/FlexNet programming interface, allowing Turbo Pascal 6.0 or Borland 7.0 programmers to easily interface with this kind of a digipeater. A terminal program is included as an example, with a detailed and commented listing. Therefore, you should be able to interface everything with PC/FlexNet: weather nodes, I/O cards, traffic monitoring, etc. |

| Driver Name | FLEXAPPL.LZH [Turbo C Interface] |
| --- | --- |
| Command Syntax |  |
| Example |  |
| Functions | This package provides a PC/FlexNet programming interface, allowing Turbo C programmers to easily interface with this kind of a digipeater. An example is provided, with a detailed and commented listing. Therefore, you should be able to interface everything with PC/FlexNet: weather nodes, I/O cards, traffic monitoring, etc. |

4.  APPLICATIONS

Once the digipeater is loaded, PC/FlexNet runs like a TSR and can be used by programs running on the same computer (FBB, TOP, SP, Graphic Packet, etc). Here are the available applications which should help interfacing:

| Application Name | TFEMU.EXE |
| --- | --- |
| Command Syntax | (see TFEMU.DOC) |
| Example | (see TFEMU.DOC) |
| Functions | TFEMU is a host-mode emulator (WA8DED / TF). It allows the use of host-mode compatible programs on a PC/FlexNet digipeater. For example, it is now possible to run a digipeater and FBB 5.15c on the same computer (don’t use a 286...hihi). Graphic Packet, TOP, SP, Die Box, AA4RE, TERM, etc can be used. In addition, TFEMU can simulate a DRSI or TFPCX driver. New host commands have been added in order to improve the compatibility with PC/FlexNet. |

| Application Name | ETHEREMU.EXE |
| --- | --- |
| Command Syntax | (see ETHERMU.DOC) |
| Example | (see ETHERMU.DOC) |
| Functions | ETHEREMU is a packet driver emulator for Ethernet on one side and PC/FlexNet on the other side. Standard TCP/IP systems can be used on the air, such as NCSA TELNET or KA9Q. |

| Application Name | ETHER.EXE |
| --- | --- |
| Command Syntax | (see ETHER.DOC) |
| Example | (see ETHER.DOC) |
| Functions | This driver allows the direct coupling of computers with an Ethernet interface using the G8BPQ protocol. Only PCs on the same physical segment are reachable. Traffic over routers, bridges or multiport routers is not possible. |

| Application Name | FSET.EXE |
| --- | --- |
| Command Syntax | (see later in this document for an example) |
| Example | (see later in this document for an example) |
| Functions | FSET.EXE is used to set up speed, mode or txdelay on a system running without the digipeater function. You’ll use it to interface PC/FlexNet with FBB, TOP, Graphic Packet, etc. An example is given later in this manual. |

| Application Name | CALIB.EXE |
| --- | --- |
| Command Syntax | CALIB channel minutes |
| Example | CALIB 1 5 |
| Functions | Calibrates a radio port. Must only be used when drivers are loaded (USCC.EXE for example). |

| Application Name | SYSNUM.EXE |
| --- | --- |
| Command Syntax | SYSNUM sysop_access_code DOS_digi_path |
| Example | SYSNUM 12345 c:\PCFLEX |
| Functions | Used to enter the sysop access code used to remote control the digipeater: SYSNUM sysop_code config_file_path (flexnet.fpr) |

| Application Name | KILLAPLL.EXE |
| --- | --- |
| Command Syntax | KILLAPLL |
| Example | KILLAPL |
| Functions | Kills any applications running, such as TFEMU. |

| Application Name | SHOW.EXE |
| --- | --- |
| Command Syntax | SHOW |
| Example | SHOW |
| Functions | Monitors all radio channels. |

| Application Name | STAT.EXE |
| --- | --- |
| Command Syntax | STAT |
| Example | STAT |
| Functions | Displays the digipeater’s statistics without having to make a local connect. |

| Application Name | BCT.EXE |
| --- | --- |
| Command Syntax | BCT [callsign] |
| Example | BCT F5PJE |
| Functions | BCT uses the same interface than BayCom 1.60, but has been adapted to be used with PC/FlexNet (from 3.3d only, not before). Read BCT.DOC to know more. |

| Application Name | CONVERS.EXE |
| --- | --- |
| Command Syntax | (see CONVERS.DOC) |
| Example |  |
| Functions | Conference bridge between PC/FlexNet or TheNetNode digipeaters. It provides a widespread network on which users can talk, connected to their local digipeater, without having to go any further. |

| Application Name | TNC.EXE |
| --- | --- |
| Command Syntax | (see TNC.DOC) |
| Example |  |
| Functions | TNC provides an easy to use Terminal, with very limited commands. |

5.  Starting PC/FlexNet (with digipeater function)

In this chapter we will explain how to start PC/FlexNet with its digipeater function (FLEXDIGI.EXE). For a comprehensive example, please read the LOADFLEX.BAT file located on the distribution disk.

First, load-high FLEXNET.EXE. Then load all Level 1 drivers, according to your hardware (BayCom USCC cards, Ethernet interface, etc), followed by FLEXDIGI.EXE to activate the digipeater function.

Most of the available drivers can use one or more ports (type “P” when connected to the digipeater to have a look at these ports, or “STAT” to discover what drivers are in use). These ports will be activated in the order called. Imagine you want to use two USCC>4 (4 channels on each card) and a KISS port. We’ll load USCC.EXE for the first card, then KISS.EXE, and finally USCC.EXE again for the second USCC card. Here are the ports you will then get:

|  | P0 | USCC1, 1st channel on card 1 |
| --- | --- | --- |
|  | P1 | USCC1, 2nd channel on card 1 |
|  | P2 | USCC1, 3rd channel on card 1 |
|  | P3 | USCC1, 4th channel on card 1 |
|  | P4 | KISS port |
|  | P5 | USCC2, 1st channel on card 2 |
|  | P6 | USCC2, 2nd channel on card 2 |
|  | P7 | USCC2, 3rd channel on card 2 |
|  | P8 | USCC2, 4th channel on card 2 |
|  | P15 | internal port (used by applications to chat with FlexNet) |

On the other hand, if you load both USCC drivers first, then the KISS one, you’ll get:

|  | P0 | USCC1, 1st channel on card 1 |
| --- | --- | --- |
|  | P1 | USCC1, 2nd channel on card 1 |
|  | P2 | USCC1, 3rd channel on card 1 |
|  | P3 | USCC1, 4th channel on card 1 |
|  | P4 | USCC2, 1st channel on card 2 |
|  | P5 | USCC2, 2nd channel on card 2 |
|  | P6 | USCC2, 3rd channel on card 2 |
|  | P7 | USCC2, 4th channel on card 2 |
|  | P8 | KISS port |
|  | P15 | internal port (used by applications to chat with FlexNet) |

Set up LOADFLEX.BAT according to the configuration you need. DUMMY.EXE can be loaded to replace a missing driver, and therefore fill in a hole, avoiding changing the whole setup. DUMMY does not use a radio link. The port will be considered as unused and won’t be shown in the parameter list (command “P”).

If possible, load all drivers high (LH KISS.EXE 2 for example) in order to free as much memory as possible for other applications.

Start LOADFLEX.BAT. The first time, you’ll be asked to create a configuration file called FLEXNET.FPR. Answer “Y”. Drivers will then load into memory. If no errors are displayed, PC/FlexNet is now resident and running in the background, as a TSR. Use FLEX /U to unload it.

Here is a typical LOADFLEX.BAT:

|  | rem rem Example of PC/FlexNet with digipeater rem rem Setup of a KISS port on COM2, an USCC>4., rem and start the digipeater function rem (flexdigi.exe) rem lh flexnet 80 if errorlevel 1 goto exit lh kiss 1 if errorlevel 1 goto exit lh uscc /p=0x300 /I=7 /C=4 if errorlevel 1 goto exit lh flexdigi if errorlevel 1 goto exit flex goto end :exit flex /u :end |
| --- | --- |

Let’s now make our first local connection using BCT.EXE, with the command: BCT F5PJE. Remember, BCT is a user-friendly terminal, working just like BayCom 1.60.

To connect the digipeater, type ESC C FLEXNET (FLXNET is the callsign used when the digi is started for the first time).You’ll then automatically gain sysop privileges and will have to enter all of the required setup parameters: txdelay, links, port SSIDs, etc. At present there is no way of preparing a configuration file before starting FlexNet, like it can be done with RMNC. All commands remain identical to the ones used with FlexNet 3.3e.

Now, let’s set all the parameters, and give PC/FlexNet a real callsign using the MY command. Eg : MY F6XXX 0 8  => you’ll be able to connect using SSIDs from 0 to 8, 8 being very often the one used by the BBS (linked on a wire kiss link).

Let’s now pay attention to the configuration of the user port and links. Imagine the following situation : you’re using a BayCom USCC>8 card wit only 3 modems : one 1200 Baud, two 9600 Bauds. The digipeater uses F6XXX 0 to 9, 8 being the FBB 5.15C’s SSID, and 9 the TCP/IP’s SSID, both application being connected to the digipeater through a nul-modem kiss link at 19k2 full-duplex.

Voici comment, une fois connecté au FLXNET du départ, configurer le digi et tout préparer pour accueillir les deux BBS :

MY F6XXX 0 9

P S 0 0		* SSID 0 port 0, user access 1200 Baud

MO 0 1200C	* 1200 Baud on port 0, with automatic DCD

P T 25 0	* Txdelay 25 on port 0

MO 1 9600trz	* 9600 Baud NRZ, external Tx & Rx clocks on port 1 (DF9IC modem)

P T 10 1	* Txdelay 10 on port 1
L 1 F8PX-2	* Link to FlexNet F8PX-2 on port 1

MO 2 9600trz	* see above, but on port 2
P T 10 2	* Txdelay 10 on port 2
L 2 F6BIG-2	* Link to FlexNet F6BIG-2 on port 2.

MO 3 19200d	* 19200 Baud full-duplex on port 3

P S 8 3		* SSID 8 on port 3 (FBB kiss link)

L 3 F6XXX-8 @	* mandatory link test

MO 4 19200d	* 19200 Baud full-duplex on port 4
P S 9 4		* SSID 8 on port 4 (TCP/IP BBS kiss link)

L 4 F6XXX-9 @	* mandatory link test

Then, conclude with « Q » to make a local-disconnect and connect back to F6XXX-0. Type P to check if all is ok. Now, you’ll have to set the BBS (see the FBB setup described in this document). Don’t forget to use your own callsign, not F6XXX :-))

To get remote control of the digipeater, you need to enter a sysop code. Simply use SYSNUM.EXE , providing the sysop code and configuration file path. For example:

SYSNUM 12345 c:\PCFLEX

6.  Using FLEXNET.EXE alone, without digipeater

FLEXNET.EXE is simply a kernel, for users and service computers, without a digipeater function (unless you load FLEXDIGI.EXE when starting the system). All channels can use different parameters and run with any Level 1 driver (USCC, SER12, IPXPD, etc). You have a maximum of 8 channels, which should be large enough for normal users.

Use LOADHIGH or LH to load all drivers in high-memory (MS-DOS 5.0 minimum). If you don’t have enough memory, simply load them in conventionnal memory. You can check the place used by each driver using MEM/D. Try to fill the holes.

PC/FlexNet setup, without digipeater, can be done using a BAT file. Here, each channel will be set up with FSET [parameters] because it isn’t possible to read FLEXNET.FPR (parameter file) if you don’t use the digipeater function. Example: FSET MODE 1 9600TRZ

Don’t forget to specify a txdelay and speed. If a problem occurs, COMMAND.COM will abort the BAT file.

FLEXNET.EXE must be started first, followed by all Level 1 drivers. You can include an optional parameter: the size of the work environment (in KB). 15 KB per channel is the very minimum. Most of the time, you’ll need 30 or 40 KB.

When all drivers are loaded, activate PC/FlexNet with the FLEX commmand. No more drivers will then be allowed. Use FSET to set up each channel, but don’t forget that the MODE command can be different, according to the driver you’re using. If you are in trouble, start the driver with the /? parameter (ask for help and syntax).

Now, we can start the applications (terminal software for example) using TFEMU, ETHEREMU, etc.

If you start with the command FLEX /U, all drivers and interfaces will be removed from the memory. KILLAPPL will only remove applications (TFEMU, ETHEREMU, etc). Please don’t start any other TSRs when PC/FlexNet is activated; this may cause trouble.

Here is an example of a BAT file (with comments alongside):

|  | LH FLEXNET 20 | Kernel with 20 KB of working memory |
| --- | --- | --- |
|  | if errorlevel 1 goto end | Option: abort if already installed or syntax error |
|  | LH SER12 2 | Channel 0: BayCom serial modem on COM2 |
|  | if errorlevel 1 goto error | Option: abort if already installed or syntax error |
|  | LH KISS 1 | Channel 1: KISS mode for a PC-to-PC link on COM1 |
|  | if errorlevel 1 goto error | Option: abort if already installed or syntax error |
|  | FLEX | Start PC/FlexNet and its drivers |
|  | FSET mode 0 1200c | Mode on channel 0: 1200 baud half-duplex, Soft-DCD |
|  | FSET txd 0 15 | TxDelay channel 0 150ms |
|  | FSET mode 1 38400cd | Mode on channel 1: 38400 bps full-duplex, mode CRC |
|  | LH TFEMU | Option: start the Hostmode emulator |
|  | goto end |  |
|  | :error |  |
|  | FLEX /u | unload if a mistake occurs |
|  | :end |  |

The above mentionned BAT file can be integrated in the APPEL.BAT used on an FBB BBS, in order to link your mail server to a PC/FlexNet digipeater (using KISS or Ethernet modes). In such a case FLEXNET.EXE will replace BPQ or TFPCX, with more capabilities. In PORT.SYS, specify interface 5 (TFPCX); TFEMU will do the rest of the job. See the chapter concerning FBB in this manual.

Most of the time, you will only use one single channel. If you intend to use two or more, remember to specify the output port after the callsign you want to connect. For example:

CONNECT F6AAA 1
or
	CONNECT F1XYZ via 1 F6NUL (via can be omitted).

For an incoming connection, the choice of a channel depends on the application used.

7.  Wire link between PC/FlexNet and a FBB bbs

The same way you would connect a BBS to a RMNC/FlexNet node, it is also possible to link a BBS to PC/FlexNet, using a COM port on each computer this time.On the BBS side, use PC/FlexNet without its digipeater function instead of TFPCX or BPQ Node : it’s more reliable, and has more capabilities.

Please note you can also run PC/FlexNet & FBB on the same computer. You’ll then need around 4 Mbyte Ram, and a 386DX minimum.

On PC/FlexNet, load the KISS driver (eg: KISS 2 if you intend to use COM2) in the startup BAT file, and make a local connection using BCT.EXE. Set the speed to 19200D (19200 Baud full-duplex), or higher if your COM ports can afford it (16550 Ics). TXDELAY must be set to 0.

Now, any user must be able to directly connect to the BBS, using the digipeater in a transparent manner. First, the digipeater and the BBS must use the same callsign. In addition, the SSID range on PC/FlexNet must include the SSID used for the BBS; e.g. F5PJE 0-9 on the digipeater, and F5PJE-8 as a BBS.

Following our example,  set SSID 8 on the port used by KISS.EXE; e.g. P S 8 1 if port 1 is KISS.EXE. Lastly, check that it is possible to from PC/FlexNet to the BBS; e.g. L 1 F5PJE-8 @

That’s all. Now, F5PJE-8 will be sent to the network, included in F5PJE 0-9. Any user connecting with  C F5PJE-8 will directly access the BBS. It doesn’t matter if the user is using one of your PC/FlexNet ports or arriving through the network.

Now, let’s take a look at FBB’s setup files. The following examples are used on F5KPO-8 BBS. Thanks Yvan, F1UNA @ F5KPO, for your precious help and the copy of the system files of your BBS.

AUTOEXEC.BAT

@ECHO OFF

SET PATH=C:\;C:\DOS;C:\FBB\BIN

SET FLEXNET=C:\PCF

LH KEYB FR,,C:\DOS\KEYBOARD.SYS

LH C:\DOS\MSCDEX.EXE /D:MSCD001 /M:10 /L:D

CD \PCF

LH FLEXNET 20

LH KISS 2

FLEX

FSET MODE 0 38400CD

LH TFEMU

CD \FBB

CALL APPEL.BAT

APPEL.BAT

In order to increase the size of the available memory during the maintenance, FlexNet drivers are unloaded. Therefore, at the end of the process, we reboot the computer (REBOOT.COM), instead of simply starting back SERV.EXE.

@echo off

cd \fbb

break off

serv %1

if errorlevel 2 goto direct

if errorlevel 1 goto suite

goto fin

:suite

cd \pcf

flex /U

cd \fbb\system

cls

echo Sorting HROUTE.SYS...

copy \fbb\system\hroute.sys temp.$$$

type temp.$$$ | sort | find /v "# " > \fbb\system\hroute.sys

del temp.$$$

echo

echo Backup of important files...

copy \fbb\system\dirmes.sys \packet\fbb\backup\dirmes.s1

copy \fbb\system\inf.sys \packet\fbb\backup\inf.s1

echo Cleaning up message-files...

del epurmess.res

rem   epurmess

cd \fbb\system

MAINTINF 3 /a

COPY C:\FBB\SYSTEM\INF.NEW C:\FBB\SYSTEM\INF.SYS

cd \fbb

del c:\fbb\system\dirmes.new

cd \fbb\system

clr_user

cd \fbb

:direct

cd \pcf

flex /u

cd \fbb\system

tlabel

cd \fbb\system\sat

if not exist amsat.txt goto boot

satupdat amsat.txt /n > satupdat.res

satupdat amsat.txt >> satupdat.res

del amsat.txt

goto boot

:boot

echo End of the maintenance...

echo The computer will reboot in 3 seconds...

sleep 3

reboot

:appel

cd \packet\fbb

echo Program is started, use ^C to interrupt within 3 seconds

sleep 3

serv -t

if errorlevel 2 goto direct

if errorlevel 1 goto suite

goto fin

:fin

cd \pcf

FLEX/U

echo Program ended.... Driver unloaded...

break on

Echo on

cd\

PORT.SYS

Attention  : one single port is usable !!

#

#########################

# PORT.SYS for FBB 5.15 #

#########################

# File for programming of channels and TNCs.

#

# Ports : How many ports (COM1, COM2, Etc...)

# TNCs : How many TNCs and modems in use. With multiplexer

# there can be up to 4 TNCs per port.

#

#Ports TNCs

1       1

#

# Com      : COM-number (1,2,...8)

# Interface: 1 = External ESS/COMBIOS driver

#            2 = BPQ-node V 4.05 or up.

#            3 = Telephone-modem with FBBIOS

#            4 = DRSI card with driver (WA8DED host-mode)

#            5 = TFPCR/TFPCX interface (WA8DED host-mode)

# Adress   : Adress of port in hexadesimal. Ignored by COMBIOS/BPQ/MODEM.

# Baud     : Ports baud rate. Ignored by BPQ.

#

# Use same number of lines as number of ports.

#

#Com Interface Adress (Hex) Baud

8      5        3E8          9600

#

# TNC     : Number on TNC in use.

# NbCh    : Number of channels I want to use in the TNC.

#           Maximum available channels depend on firmware.

# Com     : Number of the COM-port. Com1, Com2 etc.

# MultCh  : Number of channel if port-multiplexer is used, otherwise 1.

#           In DRSI use values from 0 to 7, by KAM use 1/VHF and 2/HF.

# Paclen  : PACLEN on this TNC.

# Maxframe: The maximum nb of frames the TNC will send at a time.

# NbFwd   : Number of channels for OUTGOING forward at same time.

# MxBloc  : Size of forward-block in kb.

#

# M/P-Fwd : Minute of the hour for start of forward, and period

#           (how many minutes between each forward-start).

# Mode    : One of these:

#           B : BBS-mode.

#           G : "Guest"-mode.

#           U : Normal-mode.

# Type host-mode, one of these:

#           D : WA8DED

#           K : KAM hostmode.

#           P : PK-232

#           Q : BPQ v 4.x

# Addition: One of these letters can be used too:

#           L : Send unproto beacon after each arriving mail.

#           M : Telephone-modem.

#           Y : Yapp allowed on this QRG.

#           W : Gateway allowed TO this QRG.

# Freq.   : Text to describe this port (max 9 characters, no space)

# Same number of lines as number of TNCs.

#

#TNC NbCh Com MultCh Pacln Maxfr NbFwd MxBloc M/P-Fwd Mode    Freq

1     10   8   1      250   7     2     10     00/60  UDLY    FLEXNET

# Special callsigns and modes for some channels.

# SSID d'acces special pour les bbs.

#TNC Nbs Callsign-SSID Mode

#1    1    F5KPO-9       B

#

# End of file.

#

BEACON1.SYS

Use the user port SSID in order to let the unproto frames going out (F5KPO-9 in the following example).

%15

! F5KPO-8 V F5KPO-9

Date : $d, Time : $T. This is F5KPO-8 located in $c

in $?. There are $N active messages. New messages to :

$Q

INITTNC1.SYS

MUISC

8. PC/FlexNet and TSTHOST

TSTHOST seems to be more popular than TPK. We thought it could be usefull to explain how to use it with PC/FlexNet. therefore, here are the basic configurations. Thank you to those who helped, providing me with their system files.

de F1TVQ @ F6KBF.FRPA.FRA.EU

@echo off

rem Startup of PC/FlexNet (with digi).

flex -u

set flexnet=c:\flex

lh flexnet

lh ser12 2

lh flexdigi

flex

lh tfemu

fset mode 0 1200c

fset txd 0 30

cd..

cd tsth141

lh tsthost /t /i253 /r100 /u500

cd..

cd flex

flex /u

cd..

Parameters  /r100 and /u500 allow to recover some memory, since my PC is a 386SX4Mb, with only 1 Mbyte available, thanks to MsDos 6.22.

The TSTH setup file remains the same for TFPCX or PCFLEX. Of course, parameters like PERSIST, SLOTTIME, TXD are useless with PC/FlexNet.

# TSTHOST CONFIGURATION FILE EXAMPLE

# BEFORE USE, SET UP YOUR PARAMETERS

#

# setup tnc interface

AX MY    MYCALL

AX MON   UISC

AX MAX   2

AX RET   15

AX FRACK 7

AX PE    64

AX SLO   20

AX RES   20

AX TXD   30

AX UN    BALISE f6kbf-12

AX US    4

AX BE    0 <beacon3.ini

AX CT    Connection made with F1TVQ.....

AX PO    1 MYCAL

AX PO    2 MYCALL

AX PO    3 MYCALL-1

AX PO    4 MYCALL-1

AX PO    5 MYCALL-1

AX PO    6 MYCALL-1

AX PO    7 MYCALL-1

AX PO    8 MYCALL-8

PAram 0 @PO 00--000-

Param 0 @C 25

#

# setup work directory

USERDIR C:\TSTH141\USERS\

WORKDIR C:\TSTH141\MSG\

#

# setup pms section

HOMEBBS     F6KBF-1

FTIME 0

FRE OFF

PYAPP ON

HI          .F6KBF.FRPA.FRA.EU

LBH ON

LPH ON

YTIME 1800

BMail off

#

# setup unproto section

UL Enable

UL Time 30

UL Rpath F6KBF-4

UL P OFF

UL A +F1TVQ +reseau +APRS

ul N +achat

UL I P

#

# other setup

AU ON

YAPP ON

7pc on

SCR 15

LO on

SERVER +7PNOU +MULTST

#

73's de F1TVQ @F6KBF.FRPA.FRA.EU using TSTHost 1.41

Loc JN19CA - Taverny - 95 - FPAC : 2080 178202 (F6KBF-13)

9. Conclusions

In CONFIG.TOP, use the following setup :

CFG:TNC1=TFPCX,10,70,0,0,2,2F8,5,19200

2F8,5  means COM2, IRQ5, 19200 being the speed.. These parametersare certainly not necessary, but this way you won’t forget your hardware setup :-))

The BAT file is :

CD \PCF33D

LH FLEXNET			* loading main kernel

LH SER12 2 /I=5		* BayCom 1k2 modem on COM2, IRQ5

LH FLEXDIGI			* digi function loaded

FLEX				* start the whole PC/FlexNet system

LH TFEMU /DR /B10000	* Loading TF emulator with 10 Kb buffers

CD \TOP150			* Start  TOP 1.50

TOP

You now have a beautiful system, being able to be used as a digipeater or as a mere packet-radio terminal. One last tip : when you want to make a radio-connection, you don’t have to go throught the digipeater first : simply ask the connection, directly, since it will be issued on a transparent manner.

10.  Experiences

PC/FlexNet and FBB 5.15c have been tested on the same computer (386DX25, 2 MB RAM), but without enough users to know if this solution is really feasible. There is a lot of experimentation left to be done with FBB.

11.  New commands

PC/FlexNet has got some new commands whichare also available on RMNC/FlexNet 3.3e (except STAT) :

|  | L ** | Display the links and system runtime and the 16 last values of the link timings. |
| --- | --- | --- |
|  | L * | Display the links and system runtime |
|  | M | As a user, you’ll be connected to the BBS set up by the sysop. As a sysop, type “M callsign” to validate a BBS (its callsign has to be known in the network’s destinations). For example: M HB9IAP-8. You’ll then access HB9IAP-8 simply by entering “M”. Type « M? » to know the BBS callsign. |
|  | STAT | STAT displays statistics. Therefore, you can discover how many frames have been sent/received, how many frame errors occured, the digipeater’s runtime, etc. |
|  | MODE | Syntax: MODE n speed [x] the X option avoids all transmission on the mentionned radio port.Eg : MODE 1 1200x |
|  | MHEARD | Syntax: MHEARD port# or MHEARD callsign Display a list of stations heard, and the port used. |

12.  Known bugs

There are many problems with FPAC or ROSE digipeaters which keep sending a UA-frame, before they really get connected. This harms link testing via FPAC or ROSE networks; values are never accurate .

Most of the PC/FlexNet 3.3d bugs have now been removed, thanks to the numerous reports sent to the FlexNet team. 3.3e is now fully operationnal.

12.  Cards and interfaces

BayCom USCC and RMNC cards are distributed in France by INFRACOM, with English or French manuals. The USCC ones are strongly recommended if you intend to run a PC/FlexNet digipeater using 4 or more ports (USCC have up to 8 modem ports).

In other countries, please contact your local BayCom dealer or check the WWW server at http://www.baycom.de

13. Conclusions

This documentation is supposed to be as accurate as possible. However, errors are still possible; we are only human after all, not machines.

Please always read carefully the ASCII DOC files provided with any new drivers (if needed). It’s the only way to find out what has changed. All modifications are integrated in this manual as quickly as possible, but it takes time, a lot of time (this manual exists in French and English now...).

In addition, if you use PC/FlexNet, please send the authors all bug reports, and make them as accurate as possible. It’s the only way to fix the remaining problems. Thanks a lot in advance for your participation in this adventure.

73 QRO, have fun with PC/FlexNet !!

Eric BERTREM / F5PJE

Email: 	ebertrem@curis.gatelink.fr.net
Fax: 	+(33) 7808-1806
Fido: 	2:323/9.2

[Final editing and presentation by Ian Wade, G3NRW — ian@dowrmain.demon.co.uk]

August 1995