DxNet

version 1.37

© 1994, 1995 Olivier Le Cam, F5MZN

F5MZN' DxNet'software

General points.

Upgrades

Questions and suggestions

Copyright

Acknowledgements

Installation

Equipment required

Configuration of the G8BPQ'SWITCH

Configuration of DxNet parameters

SYSTEM\DX.DO file

HLP file (HELP directory)

Script files (SCRIPT directory)

SYSTEM\DXTODO.DAT file (system directory of DxNet)

SYSTEM\DXSPOT.BIN file (system directory of DxNet)

LOG\DX.LOG file (LOG directory)

SYSTEM\CONNECTS.DO (system directory)

SYSTEM\WWVSPOT.BIN file (system directory of DxNet)

SCRIPT\CallSign.MER (script directory of DxNet)

DXSPOT.BIN  file ( SYSTEM directory)

DX.LOG  file (LOG directory)

SYSTEM\CONNECTS.DO  file (SYSTEM directory)

SYSTEM\WWVSPOT file (SYSTEM directory)

SCRIPT\CallSign.MER  files (SCRIPT directory)

Callsign.PC  files (SCRIPT directory)

Callsign.IN  files (SCRIPT directory)

Callsign.OUT  files (SCRIPT directory)

SYSTEM\CONFIG.BIN (system directory of DxNet)

BASE.CTY  file (DATABASE directory)

SYSTEM\REMOTE.CFG file

SYSTEM\LOCAL.CFG file

Starting up the node

TNC

Starting

DXNET.BAT file :

Exchange protocol between DxNets

Definition of protocol

Lifetime of message.

Setting lifetime protocol in step with node

Limited protocols (External net ).

Loops

Files editor

Mini file-editor

Mini shell DOS

Console

DataBase

DXCC database

Acces to REMOTE database

Acces to HAMCALL database of Buckmaster

USERS commands list

Announce Msg

Announce/Full Msg

Bye

Remote commands

CONFerence

DX Freq Dxcall Comments

Help

Help COMMAND

SET/Here

SET/NOHere

SHow/Cluster

SHow/CONFiguration

SHow/DX #

SHow/Prefix

SHow/Users

SHow/VERsion <CallSign>

Show/WWV #

Talk Call

Talk Call Msg

Quit

Command SYSOP list.

CONnection/Available

CONnection/NOTavailable

DISC CallSign

DISCALLUSERS

DISCOTHER

DOS

DXCall MyDxCall

DXDiddle Time

DXEnable Callsign1 Callsign2 Callsign3 ...

DXNconnect ScriptFileName

EDIT FileName.Ext

QUIT

REBOOTSYSTEM

SET/AUTOREBOOT on/off

SET/BELL ON/OFF

SET/BLACKlist Callsign1 Callsign2 Callsign3 ...

SET/CDROM A: or B: or C: or D: ...

SET/FWDPC26

SET/HOPS Pc Hops

SET/LOCALUSER_LOGIN on/off

SET/NETUSER_LOGIN on/off

SET/NODE_LOGIN on/off

SET/NODUPE

SET/SSID

SET/SYSop Callsign1 Callsign2 Callsign3 ...

SHow/MEMleft

SYS

Incoming connection to DxNet from user.

Another incoming connection to DxNet management.

Simultaneous Connections to several DxNets of seam net.

# F5MZN' DxNet'software

## General points.

A DX server (DxNet) is a service designed for Radio Hams who are very fond of DX (Long distance contacts). Its main function is to flow in a very short time some information about Dx traffic. A DxNet can work independently or be part of a network. Dx information (called SPOTS) flows from DxNets to DxNets, the users connected to them receive it as it goes along.

## Upgrades

To receive updates, simply send me a letter with a 1.44 formated disk enclosed. Please don't forget to mention your name, mail address and to your mail mailing adress, don'forget enclose a stamp and write to :

Olivier Le Cam, F5MZN

38, Rte de Melgven

29910 TREGUNC

France

Ax 25 : F5MZN @ F6KPF.FBRE.FRA.EU

Email : olecam @ imaginet.fr

DxNet is also available :

	On Intenet by anonymous FTP from  ham.ireste.fr (Many Thanks F1JLN).

	On  packet servers F6ABJ, F6KNL, F5ROC et F6FBB (contact the SysOp). Thanks jean-Louis F5HDX for your help to forward.

	Directly to Jean-Louis F5HDX,  who propose to  make an automatic distribution.

	On Ham BBS (7878-0302) if your computer use a modem down from 14400 bauds and ZMODEM protocol. Thanks Eric F5PJE.

## Questions and suggestions

Please feel free to contact me if you have any questions, suggestions or comments.etc...

## Copyright

This software is in the public domain and can be freely duplicated and distributed. Any commercial use of this software is strictly forbidden. The author does not guarantee it will work properly, and cannot be held responsible for any problems that may arise from its use.

Although this program is free, and can be used as a shareware, a contribution towards costs would be most welcomed (I suggest 100FF or 20 US dollars). It will make up for my initial expenses (documentation and C++ compilator...)

## Acknowledgements

Many Thanks to F5EXO, F1JLN, F5GHV, F5NAC, F5ITK, F5HDX, without whom the production of this software would not have been possible. Special thanks too to all OMs beta-testers for their reports and suggestions and also to ATEPRA (frensh association to help on packet radio developpements) for providing hardware and software assistance.

Congratulation also to F5EXO for translating the doc into english.

# Installation

## Equipment required

The minimum required for F5MZN'DxNet is a PC, XT computer, with 640Ko RAM memory, CGA sceen and 360 Ko floppy.

It uses the G8BPQ'PACKET SWITCH (BPQCODE.EXE), and offers 64 simultaneous channels

## Configuration of the G8BPQ'SWITCH

This DxNet has been tested with 4.05, 4.08 and 4.08a G8BPQ'SWITCH versions.

The BPQCFG.TXT file must include your own parameters, a text editor can be used for that. BPQCFG.TXT file delivered with G8BPQ is already in configuration to use the DxNet with a TNC 2 or clones. Nevertheless, it will then be necessary to define NET/ROM Node'Calls (NODECALL,NODEALIAS) as well as the  BSS (BBSCALL, BBSALIAS). The  DxNet'CALL will be this BBS. And do not forget to mention the address (IOADDR), the IRQ number (INTLEVEL) and serial port speed (SPEED), the TNC is connected with (heading PORT of BPQCFG.TXT file). Moreover, application name (BBS) may be replaced by DX.

Be careful. To validate BPQCFG.TXT modifications, do not forget to run the  BPQCFG.EX program.

Annotation  on  PACLEN parameter : Protocol exchange (PC10, PC11 ... see further ) will not work  properly if not transmitted in only one frame, you might even lose the connection. I advise you to use the largest PACLEN (255 for me) on transport links.

***   Since 1.18b and 1.30 new SYSTEM directory is created :

C:\----\DXNET---------+---\SYSTEM---+---DX.DO

|             +---DXTODO.DAT

|             +---DXSPOT.BIN

|             +---DXPASS.DAT

|             +---CONNECTS.DO

|             +---WWVSPOT.BIN

|             +---REMOTE.CFG

|             +---LOCAL.CFG

|

+---\LOG------+---DX.LOG

|

+---\HELP-----+---HELP.HLP

|             +---INFO.HLP

|             +---CTEXT.HLP

|             +---EDIT.HLP

|             +---BYE.HLP

|

+---\SCRIPT---+---CALLSIGN.FWD

|             +---CALLSIGN.MER

|             +---CALLSIGN.IN

|             +---CALLSIGN.OUT

|             +---CALLSIGN.PC

|

+---\DATABSE--+---BASE.CTY

|

|

+---DX.EXE

## Configuration of DxNet parameters

### SYSTEM\DX.DO file

For each running , the server executes all commands in DX.DO file. All server information must be in this file. See example below :

The server call is F6KBI-3, and  recognizes F5MZN, GJ6TMM-4 and F6KPF-3 as DxNets calls. DXDiddle is set to 600 seconds. Moreover, the time life counter (Hops) is limited to 3 for PC16 and PC19 protocols messages. The DX.DO file will then be :

DXCall F6KBI-3

DXEnable F5MZN GJ6TMM-4 F6KPF-3

DXDiddle 600

SET/HOPS 16 3

SET/HOPS 19 3

Please, note that all parameters must be initialised in this file. In fact, parameters modified on the keyboard will not be saved on the disk. And after every server shut down all modifications will be lost.

### HLP file (HELP directory)

###### HELP\HELP.HLP

Help message. This file is sent to the user after typing "Help" command.

###### HELP\INFO.HLP

Information message. This file is sent to the user after typing "Info" command.

###### HELP\CTEXT.HLP

Connect message text. This file is sent to the user as soon as the connection is established.

###### HELP\EDIT.HLP

Help message from small editor of files. This file is sent to the Sysop after typing HELP from sub-menu of small editor of files.

###### HELP\BYE.HLP

Disconnection message. This file is sent to the user after typing the BYE command to disconnect from the DxNet.

### Script files (SCRIPT directory)

A  script is a set of lines used to indicate that the  connection is in progress to a distant node. The files must have .FWD for extension (For example : F5MZN.FWD, GJ6TMM.FWD).

The first line of the sript file is the CALLSIGN (with  SSID) of the distant DxNet. THIS FIRST LINE IS VERY IMPORTANT, you might run a connection to a DxNet which is already connected !

The next lines are used to talk with the nodes (TheNet, ROSE, FPAC, Flexnet, ... if any) which are located between local DxNet and distant DxNet. For each node, three or four lines are necessary.

So for each given node, the features can be :

The first line begins with  "." followed by the command for the node.

The second beginning with "#" sets the number of seconds you have to wait before stopping using the node. THIS LINE IS NECESSARY.

The third begins with "+". The following characters are searched in the messages returned by the node to indicate the connection has been established. The conformability must be accurate (including CAPITAL/lower-case letters). A maximum of seven characters is available (the next ones are ignored). THIS LINE IS NECESSARY.

The fourth begins with "?". The following characters are searched in the messages returned by the node to indicate that the connection has  failed. THIS LINE IS NECESSARY.

The fifth also begins with "?" and works like the fourth one. This line is not necessary.

Important remark : To establish a connection, the server first connects itself to the G8BPQ node. The parameters of the first series of three or four lines must therefore be G8BPQ node commands.

In  the  following  examples , comments  after  "//"  are  only for your personal information. THEY MUST NOT BE ENTERED IN THE SCRIPT FILES.

Exemples

Example of script file  contents "SCRIPT\F5MZN.FWD" of F6KBI-3 to connect to F5MZN-3 DxNet, directly accessible on port 2 of G8BPQ node.

F5MZN-3

.C 2 F5MZN-3		//Connect request from BPQ node to F5MZN-3

#30			//Abort after 30 seconds

+to			//to is a part of msg Connected to F5MZN-3

?Failure			//If F5MZN-3 not responding

?Busy			//If no channels available on F5MZN-3

Example of script file contents "SCRIPT\GJ6TMM.FWD" of F6KBI-3 to connect GJ6TMM-4 DxNet, accessible on port 1 of G8BPQ via three TheNet nodes (F6KCF-2, F5JGS-2, F6KRM-2).

GJ6TMM-4

.C 1 F6KCF-2		//Connect request from BPQ node to F6KCF-2

#40			//Abort after 40 seconds

+to

?Failure			//No Busy ... but there could have been !

.C F5JGS-2		//Connect request to the second node

#60

+to

?Failure

.C F6KRM-2		//Then the 3rd

#60

+to

?Failure

?Busy			//Assuming that this node is often busy !

.C GJ6TMM

#120

+to

?lure			//instead of Failure...  it does work !

?Busy

Prior to defining the parameters of a script file, we must first define the best path and know the answer messages returned by each node to request connection, especially for the ROSE and FPAC systems (A "Connected  to"  message  doesn't  mean  a  real connection has been established).

Warning ! Don't insert a CR ( carriage return = Enter key) at the end of script file, you might then receive an error message when running it.

### SYSTEM\DXTODO.DAT file (system directory of DxNet)

This file can run some commands at certain times.

Starting hours must be formated as below :

HHMM Command

Thus if the SYSOP wants to run a connection request at 08:30 to GJ6TMM-4, next line in DXTODO.DAT will be :

0830 DXNconnect GJ6TMM

If this connection request must be run

	·Every hour, this line is :

??00 DXNconnect GJ6TMM

	·Every ten minutes at minutes 5 :

???5 DXNconnect GJ6TMM

	·Every minute (not recommended) :

???0 DXNconnect GJ6TMM

???1 DXNconnect GJ6TMM

???2 DXNconnect GJ6TMM

....

???9 DXNconnect GJ6TMM

Indeed, ???? is not authorized.

Commands numbers are unlimited. If DxNet is already connected the procedure will be aborted.

Example of DXTODO.DAT file of F6KBI-3

??05 DXN F5ITK      	//Every hour, at minutes 05

??35 DXN F5ITK      	//Every hour, at minutes 35

???9 DXN GJ6TMM     	//Every ten minutes, at minutes 9

Warning ! It's not possible to define more than one connection via the same node, the call sign used to connect request being always this DxNet (with same SSID). In this case, you can connect  via  this node.

### SYSTEM\DXSPOT.BIN file (system directory of DxNet)

This file is used to save and read DX information (spots). Sysop can't modify it.

### LOG\DX.LOG file (LOG directory)

The LOG\DX.LOG file contains the server LogBook, in text format.

### SYSTEM\CONNECTS.DO (system directory)

This file contains commands list which are run at the incoming connection. If you want to send users connected list after incoming message....

CONNECTS.DO file must contain next line :

SHow/CONFiguration.

### SYSTEM\WWVSPOT.BIN file (system directory of DxNet)

This file is used to save and read WWV information (spots). Sysop can't modify it.

### SCRIPT\CallSign.MER (script directory of DxNet)

This file contains two numbers wich are used by the DX/WWV merge request (PC25) during the initialisation sequence.

The two numbers are typed on a single line, and separated by a space. First number is the Dx-count, and the second is the WWV_count. "CallSign" is the callsign of the distant DxNet, without SSID.

The following example is a ".MER" file to request the 5 last Dx spots and the 3 last WWV spot logged on GJ6TMM-4 DxNet :

GJ6TMM.MER

5 3

If the ".MER" does not exist, DxNet dont request any DX/WWV merge.

### DXSPOT.BIN  file ( SYSTEM directory)

This file is used to save and read DX informations (spots). Sysop can't modify it

### DX.LOG  file (LOG directory)

LOG\DX.LOG file contains LogBook of the server, in text format.

### SYSTEM\CONNECTS.DO  file (SYSTEM directory)

This file contains  commands  list which  are run  at  the incoming connection. If you want to send users connected list after  incoming message ....

CONNECTS.DO file must contain next line :

SHow/CONFiguration

To send last three received spots CONNECTS.DO file must contain next line :

SHow/DX 3

### SYSTEM\WWVSPOT file (SYSTEM directory)

This file is use tu save and read WWV informations (spots). SysOp can't modify it.

### SCRIPT\CallSign.MER  files (SCRIPT directory)

This file contains two numbers wich are used by the DX/WWv merge request  (PC25) during the initialistion  sequence.

The 2 numbers are typed on single line,  and separated by a space . First number  is DX -count and the

second is WWV -count. "Callsing" is the callsign of distant DxNet without SSID.

The following example is a  ".MER" file to request  the  5 last DX spots and 3 the last WWV spots logged  on GJ6TMM-4 DxNet :

GJ6TMM.MER:

5 3

If  MER file do not exist, no request will sent from DxNet.

### Callsign.PC  files (SCRIPT directory)

Restrictive protocols for the node  Callsign . See Protocol section for more precisions .

### Callsign.IN  files (SCRIPT directory)

The text is sent to the users  as soon as the node Callsign  is just connected to the net . See SysOp command section (SET/NODE_LOGIN) for more precision.

### Callsign.OUT  files (SCRIPT directory)

The text  is sent to the users as soon as  the node Callsign is disconnecting from the net. See SysOp command  section  (SET/NODE_LOGIN)  for more precision.

### SYSTEM\CONFIG.BIN (system directory of DxNet)

This file is used to save and read the max number of users connected to the DxNet net.

### BASE.CTY  file (DATABASE directory)

DXCC prefix list in the  format  of  K1EA 'CT (CT is a  contest software from  Ken Wolf, K1EA). BASE.CTY is an ASCII file and it can be editing with DOS editor.

SHow/Prefix command  allow to the user to ask this database.

### SYSTEM\REMOTE.CFG file

Setting databases  REMOTE. A Remote database is a database  accessible  from an adjacent DxCluster . See database section for more precisions.

The format  of  ROMOTE.CFG  is :

Database1Name CallDxCluster1	(Database Base1Name available on  CallDxCluster1)

Database2Name CallDxCluster2	(Database Base2Name available on  CallDxCluster2)

Database3Name CallDxCluster3	(Database Base3Name available on  CallDxCluster3)

etc ...  up 20  différents databases

### SYSTEM\LOCAL.CFG file

Setting LOCALE database. See database section for more précisions. At this moment, only  BuckMaster  is available. In next version  SysOp will can create database entierlry  setting, and acces name will be defined  in this same file.

The format  of  LOCAL.CFG file is :

NomBuck1 BUCKMASTER

NomBuck2 BUCKMASTER

# Starting up the node

## TNC

If TNC-2 or clone is used, set the TNC in KISS mode manually with this command (If EPROM is not a special KISS):

Cmd:KISS ON

Cmd:RESTART

## Starting

Also load BPQCODE.EXE (G8BPQ switch), and then DX.EXE (the server).

As these two files may be in different directories ,it may be wise to create a batch file DXNET.BAT. It must  be placed either at the root of the hard disk or in another directory specified in the PATH of autoexec.bat file.

### DXNET.BAT file :

C:

CD C:\BPQ408

BPQCODE

CD C:\DXNET

DX

Even better, if the PC is 100% used in DX, the five previous lines will be in autoexec.bat and then will self run in case of power cut.

To get 43 or 50 screen lines on an EGA or VGA monitor add the parameter -s43 or -s50 to the running command (s in lower case).

Examples :

C:\DXNET>DX	<- normal, 25 lines

C:\DXNET>DX -s43	<- EGA and  VGA, 43 lines

C:\DXNET>DX -s50	<- VGA, 50 lines

It is possible that DxNet lose the directory path of his system files. It is the case when the server answord "Sorry, no file available" when you type a command. F5HDX has found a cure (thank you Jean Louis) :

Assume the following tree on your hard disk :

C:\----DOS

+----BPQ408

+----DXNET----+---LOG

+---SCRIPT

+---HELP

+---SYSTEM

+---DATABASE

Add the following line into your AUTOEXEC.BAT file :

APPEND=C:\DXNET;C:\DXNET\HELP;C:\DXNET\SCRIPT;C:\DXNET\LOG

APPEND=C:\DXNET\SYSTEM;C:\DXNET\DATABASE

# Exchange protocol between DxNets

## Definition of protocol

Exchange  protocol of F5MZN'DxNet is compatible  with this PacketCluster software (Trademark of Pavillon Software ,PO Box  803,  Hudson, MA 01749,  USA),  But is limited.

Exchanges between two DxNets are established with the following items:

	·PC10 : Forwarding of message to a user connected to an adjoining DxNet (after a Talk command).

	·PC11 (*) :  Forwarding of DX information to adjoining DxNets.

	·PC12 : Forwarding an announcement.

	·PC16 (*) : Indicates  a new incoming station to DxNet net.

	·PC17 (*) : Indicates a disconnecting from station to DxNet net.

	·PC18 : When connection has just been established, distant DxNet sends this initializing command to local DxNet.

	·PC19 (*) :  Indicates a DxNet is connected to the net (during initializing ), or has just been just connected (when initializing is completed).

	·PC20 : Local DxNet informs distant DxNet that configuration forwarding has been completed. Distant station will forward its configuration in turn.

	·PC21 (*) :  Indicates a DxNet station has just disconnected from net.

	·PC22 : Distant station has completed configuration forwarding. Initializing has been completed.

	·PC23 (*)  :  Used to forward WWV information (Information on solar activity).

	·PC25 : Used to request DX/WWV merge to the distant station.

	·PC26 : Distant station forward a DX merge info (after a DX/WWV merge request of the local station).

	·PC27 : Distant station forward WWV merge info (after a DX/WWV merge request of the local station).

	·PC38 : Gives connected DxNets list.

	·PC44 : Remote DataBase request.

	·PC45 : Remode DataBase response.

	·PC46 : Remote DataBase complete.

	·PC50 (*) : Local User Count.

(*) This Item has a limited and set lifetime .

Number version exchange

When connection has just been established,  distant clusters exchange these numbers version of software.

For  DxNet,  format of  version number is 4XXX  XXX are the three numbers of update version.

Example, DxNet v1.30e will send as number version 4130 when connection has been just established. SysOp can see number version of each DxCluster connected to the net in case of need SHow/VERsion command (see section SYSOP commands).

## Lifetime of message.

Some items have a limited time life. This allows, for example, to forward information on connections and disconnections to distant nodes only.

A timer (hops) is associated with these exchange protocols. Default setting is 99 : when an incoming user connects to F6KBI, he sends PC16 (hops=99) to F6KPF, the latter sends information to GJ6TMM and counts down the timer (hops=98) and sends information to F5KEQ which counts down timer too (hops=97) ... and so on until timer equals zero.

If dx incoming information  in local DxNet can interest most net users across country, it may not be the case for users connections which are limited to local DxNets (low hops).

SET/HOPS command allows to set time life for each exchange protocol message (see sysop commands list and DX.DO file).

Remark : PC16 and PC17 will have the same time life. PC19 and PC21 too. And PC16/PC17 must be lower or equal than PC19/PC21.

## Setting lifetime protocol in step with node

Since 1.34 version it's possible to set lifetime protocol  in step with adjacent node. If so, hopcount configuration for F5NOD node will be in SCRIPT\F5NOD.PC file.

If  one parameter is not defined in this file, default setting will be this defined by SET/HOPS command (or H99, if it's has not defined).

When no hopcount is assigned to the protocol (Example PC10), a nul value will define it as no forward. A no nul value  (Lower to 99) will define it as forward.

Remark to forward protocol of adjacent node : Next exemple ...

Hopcount of Dx messages on F5MZN-3 is setting to 50 to F6KNL. F5MZN-3 receive a DX info with a hops value of 99 from GJ6TMM-4 DxNet. This DX info will be forwarded to F6KNL but hopcount will be changed to 50. If so DxNet compare these two Hops values (This from adjacent and this defined in hopcount configuration)  and use the lower .

Example of content of  SCRIPT\F6KNL.PC file

10 1	*** 	PC10 are available

11 50	*** 	PC11 Hops setting to 50

12 99	*** 	PC12 Hops settin g to 99

19 1	***	PC19, PC21, PC16, PC17 Hops

21 1	*** 	setting to 1

16 1

17 1

44 1	***       	PC44, PC45, PC46 are denied

45 1

46 1

etc ...

## Limited protocols (External net ).

When the link between two clusters is fragile (e.g. HF link) il will be interessting to reduce protocols  exchange ( don't send DxCluster list of the net and also users). This type of interconnexion is called an external interconnexion.

If so only essential protocols are sent and all other are setting to 0.

However to establish the conection  some protocols are necessary : PC18, PC19, PC20, PC22...

Example of  SCRIPT\F5KEQ.PC  file to connect an external node :

10 0	*** 	Talks (PC10)  not available

11 99	***  	PC11 (DX)  Hops default setting to 99

12 99	*** 	idem PC12 (Annouces)

13 0	*** 	Not present in DxNet

14 0	*** 	Not present in DxNet

15 0 	*** 	Not present in DxNet

16 0	*** 	Do not  send users list.

17 0	*** 	Do not send  disconnections of users

18 0	*** 	Although 0 setting this protocol will be forwarded (necessary)

19 0	*** 	only ITSELF information logged on will be forwarded

0  99	***	HOPS of MY PC19 in limited protocole

20 0	*** 	Although  0 setting this protocol will be forwarded (because this is a necessary protocol)

21 0	*** 	Do not forward

22 0	*** 	Although 0 setting this protocol will be forwarded.

23 99	***  	PC23 (WWV) Hops is setting to 99

etc...

Remark: PC18, PC20, PC22, PC38, PC39 (no present at this moment - hi - ), can to not be in ".PC" file. Because all this protocol are necessary, DxNet will assume they will be actif. Setting or not setting this protocols are sent.

I hope that this explanations are sufficient... In case of need make you a diagram of you net

configuration to help yourself  to define protocols...

## Loops

During net establishing, one MUST avoid loops (the same DxNet connected to two other DxNets also being connected to each other). Otherwise dx information will be received and forwarded several times by the same DxNet.

# Files editor

## Mini file-editor

A mini file-editor is available.

Using format : EDIT Filename.Ext

Ex : EDIT HELP\INFO.HLP

Commands :

D              	Delete the current line

D #Line        	Delete the line #Line

D #Line #Num   	Delete #Num lines starting at the line #Line

E              	Save file and exit editor

H              	This message

I Str          	Insert the string Str before the current line

L #Line        	Set #Line as current line and list them

L  #Line  #Num	Set #Line as current line and list the  #Num live after

Q              	Quit editor without save

/ Str          	Replace the string of the current line with the string Str

* Str          	Insert the string Str at the end of file

## Mini shell DOS

SysOp can acces to mini dos shell  with  DOS command. Only some DOS commands are available.

CD (changedir)

To change directory.

CD SUBDIR to move into the directory  SUBDIR (CD SUBDIR1\SUBDIR2 is not authorized).

CD.. (or CD ..)  To go back to the previous directory -level.

COPY <Source> <Destination>.

Copy the file   <Source> to the file <Destination>. The path  can be defined for source and destination.

COPY C:\TEST C:\DXNET

COPY C:\BPQ\BPQCFG.TXT C:\

COPY C:\BPQ\BPQCFG.TXT (if so BPQCFG.TXT  will be copyed in the current directory ).

COPY BPQCFG.TXT C:\DXNET

It's not possible to copy a file on itself.

COPY cannot  copy more one file at once (COPY *.* is not available).

Copy of file can run  between two logical units but units mut be ready

Example COPY BPQCFG.TXT A:\ will crash system if  A:.is not ready ( no disk is present in unit a:).

Exit

To quit  DOS shell.

GETfile <Fichier>

To tranfer an ASCCI file from DxNet to SysOp . End of file is indicated by End Of File ------".

PUTfile <Fichier>

To transfer an ASCII file from SysOp to DxNet.

The file must be terminated by CTRL-Z (or/EX) to be validate. /ABORT to abort the transfer.

If the file already exist it will be replaced by the new file at the end of transfer.

During the  transfer the file is temporary recorded in the SYSTEM directory as name  PUTFILE.TMP, and then it will be copyed  as name defined by SysOp in current directory.

Quit

Like  EXIT.

# Console

All DxNet commands are available from console.

In next version, connection to G8BPQ node from console will be available (I have not found yet the technique which allows to use a different callsign of DxNet to trafic from console, for nstance F6KBI for F6KBI-3 DxNet) to use server like classic terminal (to connect BBS for instance).

# DataBase

## DXCC database

A DXCC database is accessible with command SHow/Prefix <CallSign>. For more information about this command see users commands section.

Prefix list is in the BASE.CTY file .This file is in CT format (Trademark of  Ken Wolf K1EA) and must be in  DATABASE directory.

## Acces to REMOTE database

With DxNet you can acces to database available on PacketCluster (Trademark of Pavillon Software). The configuration of REMOTE.CFG file in SYSTEM directory allow to set user commands to acces at its databases. Number of avalible databases is fixed up 20 .

In SYSTEM\REMOTE.CFG, file the name of databse is following by the call of cluster which is interrogate (with SSID if it has). Name and call must be on the same line and separated by space caracter.

Example  of SYSTEM\REMOTE.CFG file :

HAMCALL ON4DXB 		(HAMCALL database is available on ON4DXB)

IOTA ON4DXB 		(IOTA database is available on ON4DXB)

CONTEST F6EPY 		(CONTEST databse is available on F6EPY)

... etc ... (up  20 ).

For the users, available command are in this format  SHow/DATABASE NAME (Ex : SHow/HAMCALL F5MZN, SHow/IOTA EU-001, SHow/CONTEST CQWW etc...).

To inform users of the server that database is available SysOps will must include in HELP\HELP.HLP  file the list of help commands.

When the cluster having database is not connected DxNet answer  "*** Database is not available at the moment. Please try later...".

## Acces to HAMCALL database of Buckmaster

BuckMaster trade a CD-ROM including an internationnal callbook : over 1.250.000 callsigns are available (French call are not update : This damage is like ARRL CallBook ) . You can order CD-ROM  to BuckMaster, Route 4, Box 1630, Mineral, Virginia 23117, USA (tel : 540:894-5777, fax : 540:894-9141, Internet info@buck.com), for $50 + $8 cost carriage (1995 price).

HAMCALL database is accessible to DxNet users . It's neceessary to install a CD-ROM  drive

(double speed is sufficient) on the computer.

To define this CD-ROM access SYSTEM\LOCAL.CFG file must be created. The name of user command precede (before a space) the word BUCKMASTER. More than one name of command can be defined in it to access to the same database (now only BuckMaster is available).

Later (next release of DxNet), this same file will allowe to open other database wich the SysOp can modify.

###### Example of SYSTEM\LOCAL.CFG file

HAMCALL BUCKMASTER	(SHow/HAMCALL allowe to access at the database)

HAM BUCKMASTER		(SHow/HAM allowe also to access at  the database)

The unit of  CD-ROM must be defined in DxNet with command SET/CDROM (ex : SET/CDROM D:). No acces to BuckMaster will be available before setting this path. For this I suggest  you to include

this command in  SYSTEM\DX.DO  file.

And then the resident file BUCKTSR.EXE must be loading before starting DxNet. BUCKTSR.EXE is located in API directory of CD-ROM. Don't forget to run TSR before using BuckMaster  from DxNet instead of system will be scrach : DxNet don't check TSR presence...

###### To resume...

1.	Create and configure SYSTEM\LOCAL.CFG file

2.	Add the command SET/CDROM D: (or E: etc...) in SYSTEM\DX.DO

3.	Add the  line BUCKTSR in the  run Batch  of  DxNet (or in  lAUTOEXEC.BAT).

# USERS commands list

## Announce Msg

Send the message Msg to every user connected to the local DxNet.

Ex:  A Es on 2 meters !! Beaming to 9H1 now

## Announce/Full Msg

Sent  Msg message to every user connected to the net.

Ex:  A/F Es on 2 meters !! Beamming to 9H1 now

## Bye

Used to disconnect. HELP\BYE.FWD is sent to the users before complete disconnecting.

## Remote commands

Some  commands are defined in the  SYSTEM\REMOTE.CFG  file . Those  commands allowe to DxNet users  to access  on  the database PacketClusters  (Trade mark of Pavillon software).

## CONFerence

Entry in local conference mode.

## DX Freq Dxcall Comments

To send a spot. Frequency MUST BE in kilohertz. Commentaries are  optional.

Ex : 	DX 144330 LZ1VP 59++ in IN87

DX 14205.3 FM5CD

## Help

Sends the contents of HELP.HLP file.

## Help COMMAND

Sends the contents of HELP\COMMAND.HLP file to the user . Used to have a detailed help of particular command. SysOp must see to it that differents help files are update.

Ex:	To obtain the detail of DX command type H DX (ou HELP DX, or also? DX).

HELP\DX.HLP file will be sent.

Remark : MsDos don't allow fraction bar (/) in file name (conflicts are possible). Use caracter underbar (_) instead of.

Ex : 	To obtain the detail of  SH/CONF command type H SH/CONF (or HELP SH/CONF...)

HELP\SH_CONF.HLP file will be sent .

To obtain the detail of SHow/DX file type H SH/DX (...)

HELP\SH_DX.HLP file will be sents.

## SET/Here

This command inform clusters and other users that your are ready again.In the list of SHow/Users or SH/CONFiguration brackets around your call will be lose.

At your incoming connection to the DxNet default mode is HERE.

Associate command : SET/NOHere.

## SET/NOHere

This command inform clusters and other users that you are not ready (you leave your shack....) but

you are already connected to the DxNet. In the list of  SHow/Users and SHow/CONFiguration your call

will be in brackets.At your incoming connection to the DxNet default mode is HERE.

Associate command  : SET/Here.

## SHow/Cluster

Returns net cluster information.

## SHow/CONFiguration

Sends a list of net DxNets and users connected to.

## SHow/DX #

Sends a list of # last DX spots. Today only 50 last spots are free. # is optional.

Ex:	SH/DX		 <- 5 last spots list

SH/DX 20	<- 20 last spots list

## SHow/Prefix

Sends country and zone of callsign or prefix <CallSign> This command use DATABASE/BASE.CTY file  in CT format (Trademark of Ken Wolf, K1EA).

## SHow/Users

Sends callsigns connected to local DxNet (see also SHow/CON figuration).

## SHow/VERsion <CallSign>

Send the version number the the DxCluster <CallSign>.

Ex : SH/VER F6KPF-3

## Show/WWV #

Sends a list of last WWV spots or # last WWV spots. # is optional. Warning : today time only receive WWV spots is available.

## Talk Call

To send some messages to ToCall user. If Call is neither connected to local DxNet nor to distant DxNet, server answers by the error message : "Call is not logged in anywhere".

Ex:	T  G1ABC	<-  Ask to send next entries to G1ABC

Hello John !

I just QSO'd a EA3 on 144

bye CU soon

^Z		<- To stop entries and return to server prompt.

## Talk Call Msg

Send the message Msg to the user Call. If Call is neither connected to local DxNet nor distant DxNet, server replied with error message : "Call is not logged in anywhere".

Ex:	T G1ABC Hello John

TALK User > DxNet Msg allow to send the message to an user wich does not appair in this user (but you hi is connected to). This is the case when a limited protocol is in use.

## Quit

This command is like Bye.

# Command SYSOP list.

## CONnection/Available

Allow again incoming connection to the server

Associate command : CONnection/NOTavailable

## CONnection/NOTavailable

Disconnect all users and also all DxNets connected to the server except you and suspent any connection

untill you are disconnected or you send the CON/Available command.

Attempt to connection will have the message  BUSY.

If this command is sent from the console don't forget to allow again incoming connection by CONnection/Available command at the end of your maintenance.

Associate command : CONnection/Available

## DISC CallSign

Disconnect user CallSign from the server.

## DISCALLUSERS

Disconnect all users and also all DxNets connected to the server. In next version users will be advise by message that they being disconnected.

## DISCOTHER

Disconnect all users and also all DxNets connected to the server execpt you.

Users are not advised that they being disconnected and SysOp must advise them by local announce.

## DOS

To acces in DOS menu  (mini DOS shell).

## DXCall MyDxCall

MyDxCall is DxNet Callsign. This command must  be obligatory in DX.DO file, and Callsign must be the seame that BBS application of define BPQCFG.TXT file.

## DXDiddle Time

Time inactivity value expressed in seconds, before useless packet sending to connected stations to check link. This frame contains "CR" for DxNet and ">" for users.

## DXEnable Callsign1 Callsign2 Callsign3 ...

Set callsigns (with SSID) in DxNet liste as DxNet. The Call must be separated by space caracter. 80 caracters max are available.

Ex :	DXE F5MZN-3 F6BEE GJ6TMM-4

## DXNconnect ScriptFileName

ScriptFile Name is a name of scipt file being in server work directory. ".FWD" extension don't be present.

Ex :	DXN F5MZN	<- Run connection procedure writed in F5MZN.FWD file.

## EDIT FileName.Ext

Using to edit FileName.ext.

## QUIT

All users are disconnected and system return to DOS.

## REBOOTSYSTEM

To reboot system (reset). Use this command in final luck !

## SET/AUTOREBOOT on/off

Server available to reboot when acces file is bad.

Defautlt setting is OFF;

## SET/BELL ON/OFF

Bell filter only available if SET/BELL setting OFF. Only available from keyboard console.

## SET/BLACKlist Callsign1 Callsign2 Callsign3 ...

Set reject call (without ssid) in BLACKlist. Callsigns must be separated by spaces caracter and writed without ssid. Max 80 caracters are available.

Ex :	DXE TNC2 NOCALL F0XXX P1RAT

## SET/CDROM A: or B: or C: or D: ...

Set CD-ROM letter to access to the BuckMaster CD-ROM

## SET/FWDPC26

When SET/FWDPC26 setting ON DxMergeRequest (PC26) are forwarded to the adjacant DxNets . Default setting isOFF.

## SET/HOPS Pc Hops

Setting time live PC messages transmite between each DxNet. Default value is 99.

Ex : 	SET/HOPS 10 5		<- Set PC10 time live to 5

## SET/LOCALUSER_LOGIN on/off

When SET/LOCALUSER_LOGIN is ON, users are inquiryed of loging and logout of every local users . Default setting is ON.

## SET/NETUSER_LOGIN on/off

When SET/NETUSER_LOGIN is ON users are inquiryed of login and logout of every users of net cluster. Default setting  is ON.

## SET/NODE_LOGIN on/off

When SET/NODE_LOGIN is ON users are inquiryed of login and logout of the clusters wich constitute  the net. Default setting  is ON.

Also it's possible to send to the users a specific information about  connection of an accurate node.

The specific message must be in  file :

SCRIPT\INDICATIF_OF_NODE.IN  for connections

SCRIPT\INDICATIF_OF_NODE.OUT for disconnections

Ex : 	Content of  SCRIPT\F6EPY.IN  file on the  DxNet F6CTT-3

*** Etablissement de la liaison avec Paris. ***

Content of  SCRIPT\F6EPY.OUT  file on the  DxNet F6CTT-3

*** Liaison avec Paris rompue. ***

## SET/NODUPE

This command verify that DX spot which has just arrived is not already present. If the spot is already

existant and NODUP set ON then it will be not sent neither to the users  and adjacants DxNet.

This command avoids loops between clusters. Default  setting is ON.

## SET/SSID

Set SSID wich not clear when user incoming connection to DxNet. See below for more precisions.

Ex :      SET/SSID 5 6 7 8

## SET/SYSop Callsign1 Callsign2 Callsign3 ...

Set callsigns in distant SySop list (without ssid). Callsign must be separated by spaces caracteres. Max 80 caracters available

Ex :	SET/SYS F5MZN-5 F1NNI F5OAH-15

## SHow/MEMleft

Size of available memory left.

## SYS

A user sysop must use this command before may to the others SysOp commands. DXPASS.DAT file contains a caracters string (255 max) as acces password. After receive SYS command and if user callsign is considered as SysOp (with SET/SYSop command) server return a group of 5 numbers. Each number coresponding to each caracter position in password string.

Ex :	Let in DXPASS.DAT contains next line :

Voyezlebrickgeantquej'examinepresduwarf0123456789

V  Position  : 0

o  Position  : 1

y  Position  : 2

e  Position  : 3

z  Position  : 4

etc... etc...

SYS			<- User : Sysop acces required

F6KBI-3 > 10 36 2 9 32  	<- Server randow numbers

cayis			<- User : caracters sending

Ok			<- Server : code accepted

If caracters sending by user are wrong server replied "Unknow command, type H for Help". To try again user must disconnecting  and connecting again.

# Incoming connection to DxNet from user.

## Another incoming connection to DxNet management.

During initializing incoming connection, first test is DxNet recognizing (With ssid). If not, second test is remote SysOp.

After (when only one user) SSID is tested. Indeed all ssid not specified with SET/SSID are canceled from user callsign.

With this, it'n necessary to type "Talk F5GHV-15" every time we want sending a message to F5GHV (pathing trouhgt a TheNet node to connecte DxNet).

Its possible to set, with SET/SSID command, SSID list which are canceled (example SET/SSID 5 6 7 8).

## Simultaneous Connections to several DxNets of seam net.

It's not possible to a seam user to connecte more one DxNet at the same time .

Let in F5MZN is connected to F6KBI-3 DxNet. If F5MZN try to connecte GJ6TMM-4 (GJ6TMM-3 and F6KBI-3 being linked from the net) GJ6TMM-4 will send him the message ...

Only one DxNet allowed to connect at anytime.

You are being disconnected.

...and will be disconnected from GJ6TMM-4.