ASYSCC10

(c) 1995  IK5NAX  Lapo Pieri

Asyncronous communication program for SCC board

This program can be used with all the SCC board as the NSCC (designed by

me) and with the OptoPcScc by PA0HZP; I was not able to test the program on

other SCC board.

Running the program with no argument you will obtain all the information

necessary for its use:

===============================================================================

ASYNSCC  v1.1                                                  IK5NAX  1995

Use:  ASYNSCC <baseaddr> <irq> <port> <baud> <par> <bit> <stop>

<baseaddr> = base address of SCC board  (dip-switch) in HEX

<irq> = <5|6|7|9|10|11|12|15>

<port> = <0|1|2|3>

<baud> from 38 to 1228800 bit/s [auto rounding]

<par> = <n|e|o>

<bit> =<5|6|7|8>

<stop> = <1|2>

===============================================================================

As an example if you have the board mounted on hardware port 0x300 and IRQ 9, to

use port #0 at 1200 bit/s, no parity, 1 stop bit and 8 data bit, the command

line will be:

ASYNSCC 300 9 0 1200 n 8 1

SCC port number are (according to my design [NSCC]):

#0 - A

#1 - B

#2 - C

#3 - D

For every other info, dubt, etc., about the programm or about the NSCC (which

can be used with TSThost, NET/NOS, under Linux, and other), you can contact me

in one of the following mode:

- by mail:  Lapo Pieri  via A.dei Corbizi 9   50127  FLORENCE - ITALY

- by phone:  +39 55 410209

- via AX25:  ik5nax@iw5afk.itos.ita.eu

- via e-mail :  icesb@iroe.fi.cnr.it

73 de IK5NAX