This document describes the installation procedures for

7pserv V3.3 and how you use it. It is intended as for the Sysops

and not written for the general user. You as a Sysop is urged

to write down in your own language a user manual from the

contents of this manual.

With this program you can send and receive binaryfiles with

your BBS.

To see which version you have give the command 7pserv under

MS-DOS.

ACKNOWLEDGEMENT

This product uses the same algorithm as 7plus.exe a program

by Axel, DG1BBQ and it is built on his ideas and always made

compatible with the latest version of 7plus. The intention

is that users should use 7plus.exe to transmit and receive

the files. I wish to thank Alex for a great job without his

super ideas this program would propably be based on some

other more or less silly idea.

OBSERVE !!!!

THIS PRODUCT HAS NOTHING TO DO WITH IK1GJK:s SENDOUT OF

7PSERV. PSE DELETE ALL OLD VERSIONS OF 7PSERV AND USE

THIS NEW VERSION ONLY. IK1GJK HAS SENT OUT A PIRATE

VERSION OF 7PSERV AND IT IS STRONGLY RECOMMENDED THAT

YOU WILL NOT USE THAT ONE !!

Also please observe that this program is independet of

any FBB file and FILEAUT. This is said because of the vicious

rumors told by IK1GJK in his so called doc for "his 7pserv"

that 7pserv needs fileaut. You have propably understood that

IK1GJK is not one of my favorites !

GENERAL DISCLAMER

THIS PROGRAM 7PSERV.EXE IS HEARBY FREELY DONATED TO THE PUBLIC

DOMAIN, THERE IS NO FEE ATTACHED TO ITS USAGE THE AUTHORS

ONLY CONDITION IS THAT IT MAY BE DISTRIBUTED IN ITS ORIGINAL

UNMANIPULATED FORM ALONG WITH THIS DOCUMENTARY FILE. THE

AUTHOR ASSUMES NO RESPONSIBILITY WHAT SO EVER FOR THE EVENTUAL

DAMAGE THAT MAY BE CAUSED BY THE USAGE OF THIS PROGRAM.

The source code is also available but is not included in

the released files since the interest for these files are

not to high. If you are interested in getting the source

code you should send me a disk with 3 IRC:s and I will return

the disk containing the latest source files including project

files for Borland C++ 3.1.

Contributions is much aprechiated to:

SM0NCS, Tore Andersson

Bodastigen 1  6tr

151 64 Sdertlje

Sweden

Inside Sweden to Pg 650905-1170.

73 de Tore SM0NCS@SK0MK

Contents.

A			Whats new 3.3

1			Installation

1.1	    	7pserv.cfg

2 			How the server works

2.1             Example of CRON.SYS

2.2   		Special working fields

2.2.1   7PFrom

2.2.2   Destination

2.2.3   Pass

2.3             Password establishing

3			Correction file handling

4             		File access

5 	                Receiving files

6                       Automatic server to server

This program implements Axel superb algorithm of 7plus 2.02

and uses the fascilities I thought was useful such as splitting

everything in pieces since this version even COR files are

splitted. Axels source code is used as a basis for implementing

changes but it is not transported in its entirety instead

7pserv implements things by its own way but without Axel this

would not be possible !! This is said so that you should not

belive that you are running a 7plus.exe discaised as a server.

Also 7pserv does not leave any unescessary error codes to

the BBS since this is not a very smart thing to do. Instead it

deliveres error reports conserning the user to the user and

conserning failures to the Sysop. I have now tried to get down

the amount of error reports to user to a minimum.

7pserv minimizes the need for using the BBS to transfer files

and manipulating 7plus programs. This is done becaus the

BBS is one thing and the server another !!

Also this program does not write to the screen since this also

is a bad idea for servers since most BBS:es does not save the

screen when running the server program. Also what is the

use in writing to the screen when dealing with automatic

programs with no keyboard <-> screen user interactions ?

7pserv now also checks if the files are valid before

proceeding any further. This makes it impossible to store

invalid 7+ files and produce unwanted error reports.

The program can also receive the COR files before the

actuall 7+ files ! It looks to see if any Correction

files are available and uses them if necessary.	The correction

files are stored together with the uploaded files. There

are also a .SRV file stored if there have been produced

any error reports. The file is updated whenever an error

report is sent out.

And finally 7pserv minimizes the amount of data put in the

subject field of the letter since most BBS:es have limits

here. Instead it uses fields in the mail for information

exchange, it even eliminates the need for using return adress

by scanning the R:lines to look for a reply. The standard

@ reply in the subject field is although possible to use

if prefered.

A		Whats new 3.3

Version 3.3 presents you with a FBB compatible way of using

multiple virtual disks. It is used in the same manner as

FBB but 7pserv is still not dependent on INIT.SRV so another

line is added in the 7pserv.cfg file.

Version 3.3 have a couple of improvements in file saving

it looks at the file before saving it on the disk to see

if it really is a good 7plus file, it now also has the

ability to receive .inf files acording to 7plus standard

and you can receive these files at a directory of your

choice.

1		Installation

This server was originally written for F6FBB BBS and every

new version is tested on the latest FBB BBS product. I

have tried to make the program independent of the FBB

environment and I know that some are using it on AA4RE

for example. If you are not sure if your BBS software

can use this product I'm happy to help you with any

questions concerning 7pserv installation to your BBS

software. Before aproaching me with your question please

read through the documentation concerning servers,

external program execution, import and export in your

BBS documentation so that you can put a good description of

the capabilities of your BBS.

The program uses the 7plus algorithm to convert the files to

text format. It splits the files according to 7plus standard

with a size that you as a Sysop determines. The size of the

sent file also depends on how far the request has travelled.

The size of the file is substracted with a fixed amount for

every R-row in the received request. Therefore you can also set

the minimum size of files sent out from the BBS. All of these

configuration items is set in the 7pserv.cfg file. An example

of this file is shown below.

1.1		7pserv.cfg

# 7PSERV V3.3 Configuration file

#

# This is a comment.....

#

# First row is the number of mail per mail.in file

#

1

# Second row is maximum allowed size in bytes.

#

7152

#

# The third row denotes minimum size of mail sent

#

2123

#

# The fourth row denotes the name and path of mail.in file

#

c:\project\7pserv\mail.in

#

# Fifth row is what library the server should get files from

#

c:\fbb\users\

#

# Sixth row is the BBS call. This call is used as respons adress

#

sm0ncs

#

# The seventh row tells the server where the packed 7plus files

# should be stored

#

c:\fbb\upload\

#

# The eight row is new in 3.1. With this row you can make it

# possible so that the output queue will not fill up.

# This is done by limiting the number of mail.xxx files present

# at one time. This value is the maximum. If it exceeds information

# about it will be sent to the requesting user

# Set to 0 if you do not want to use this function

#

2

#

# The ninth row is the callsign of the sysop. The server uses

# this adress to send general failures.

#

sm0ncs

#

# The rest of the file is special permissions. There are three

# fields in connection to a permission.

# Field1 = Callsign, Field2=Password, Field3=permissions

# If you leave out field three that user is labeled as Sysop

# and has access to all partitions on your computer.

#

sm0ncs RRRRRR

sk0mk RRRRR

sm0ncs Tjosan d:\

sm0ncs AAAAAA c:\util\

sm0ncs VVVVVV c:\dos\

sm0orb CCCCCC c:\test

Comments in this file is entered in the same manner as in the

init.srv file and also in this file as in init.srv you cannot

change the order of the parameters.

2		How the server works

The server does NOT read the init.srv file (FBB), this is

done because it should be possible to use the server at

other BBS software aswell. It also allows you to portion

the mail.in files in mail.xxx files with a utility called

Mailren.com. This program is independet of 7PSERV.EXE so you

could use it for other purposess aswell. Mailren should be set

to run in the cron.sys file (FBB). Your choice in how often the

program is set to be run affects the load you cause to the

network. For example if you set the number of outgoing mails to

2 in 7pserv.cfg and the Mailren to exec every second hour then

two files will be going out every second hour. This operation

is merely a renaming of files which is made very quickly.

2.1		Example of CRON.SYS

IF D 0-6

IF H 0,2,4,6,8,10,12,14,16,18,20,22

X Mailren

ENDIF

ENDIF

2.2		Special fields

Due to the fact that the server has some automatic functions so

that two servers of the same type can exchange files without

any human involvement, some communication field must be used.

These are 7pfrom, destination, type and pass. They can also be

used by the users to manipulate the servers behavior. A field

consist of the keyword listed above, followed by a semicolon

and a space and the fieldinformation. The keyword is not case

sensitiv.

Keyword: Fieldinformation

2.2.1	7PFrom

This field specifies from which station the message is sent

from. This is because we want the servers to be able to adress

to themself without specifying an illegal from adress. An

example of such a field is 7PFrom: 7pserv@sm0ncs. Because of

this there could be many ways for the server to determine to

which BBS some reply should be sent. 7Pserv determines this in

the following order.

Priority .           Source.

1                 7PFrom field if it exists

2                 the route in the subject if exists

3                 Scanning of R:rows

2.2.2	Destination

This function is a copy function to get a file and put it in

another directory and or with another filename. The subject of

the message determines where the file will be put. The

destination  field is therefore switched with the subject in

the different serverlocations. An example of use is shown in

the chapter about fileaccess.

2.2.3	Pass

This is a field for the users to notify the user and to access

their special permissions given. If pass is not given or if it

is not correctly given the users rootdirectory is defaulted to

the user library stated in init.srv. The server also has a

function which allows two servers to copy messages from one

special permission directory at one server to another special

permission directory at the second server. This function is

however not much tested yet and I would very much apprichiate

any possible missfunctionreports. To use this function you

should specify a field lookin like.

Pass: """"" Newpass: """""

The newpass is then set to a Pass: field in the reply to allow

the file to be put in special permissions directory.

2.2.4	Type

The type field allows you to send the files as a bulletin. To

be allowed to do this you have to successefully entered a

password with the highest priviligies. A user that has the

highest priviliges is a user that is entered in 7pserv.cfg

list as a priviliged user with full access rights on your

hard disk.

2.3		Password establishing

As stated above if a user wants to access his special

permissions given by the Sysop at the BBS he should specify the

pass field in the message. The password is then checked with a

special algorithm. This algorithm is somewhat similar to the

way FBB uses its Sysop login. The differens is that it ofcourse

is not possible to randomise any numbers for the user to make

as lookup numbers in the passwordstring, because it is not an

interacting connection. Instead I choose to generate this five

numbers from the filename that the user wishes to get or upload

respectively.

The filename, not the path, are scanned through for letters in

the alphabet (a - z) not case sensitive. All other characters

such that underscores are ignored and the next characters are

taken instead. The first up to five, maybe fewer, is taken to

be based as the basis for the index to the password string. The

integer index is set to be the order it has in the intervall

(a - z), a = 1 b = 2 etc. If the case is that this number is

bigger then the length of the password the number is

substracted by passwordlength. E.g if the passwordlength i 10

and an index is found with the number 12 the new number is 2.

This is similar to begin counting from the begining of the

password again when reaching the end of it.

Example.

Imagine that sm0ncs wants to get a file from the BBS and that

we are using the 7pserv.pri file given as an example in this

documentation. That is the password we want to use is TestAr.

If the filename we want to fetch is autoexec.bat the password

that should be entered in pass field is:

Character         Order        Char in password

a               1               T

u               21              s

t               20              e

o               15              s

e               5               A

The get is made by:

sp 7pserv@bbscall

\autoexec.bat

Pass: TsesA

Because of the complexity of this password counting I have

given a utility program called 7ppass that does this counting

for you. You could give the users a copy of 7pserv.pri edited

so that only their permission is entered in it. And the 7ppass

program. Then they could just call 7ppass with the filename and

their callsign as an argument then the program will tell them

what string to enter in the message.

2.4	User priviliges levels

Because of the fascility of using passwords there is also a

way of knowing the users priviliges level that determines

what kind of manipulation the user is allowed to do. This

version of 7pserv suports three levels of priviliges, one

where the user is setup in the list of priviliged users as

a user with full permissions that is with no restrictions,

the second level is a user with in the same list but with

restrictions such as permissions on one directory level,

the third and last level is a user that is not mentioned in

the list as priviliged.

3	Correction file handling

Since 7pserv V3.1 the correction files are also splitted

if the size is to big. This also makes it possible for

7pserv to receive multiple correction file, but here is

a very important note ! Since there is no way (today) of

knowing how many .COR files there are going to be received

, you must let the user must let the server know. This is

done in the way that 7pserv NEVER sends an error report

if a COR file with extention .CXX is received, but it does

send additional error reports if there is a .COR file

received. The user should send the .COR file last of all

COR files so that the server will not send unnescessary

error reports. I expect this problem will be solved somehow

in the near future....

4		Fileaccess

The user access your files by sending a letter to the bbs

adressed like:

sp 7pserv@bbscall

\public\fv.com@bbs_route

As stated erlier in this text the server has different methods

for determining what route the reply should be sent to. One of

these methods are the @field in the subject noted above. If

none of the 7pfrom field an the @field above is found the

server scannes through the R:Lines in the received request to

find a reply BBS.

When a mail arraives to the server the path stated is checked

to see if it is a valid path. What root directory is used is

depending on if the user uses special permissions or not. If

the path does not exist a reply message stating it is sent to

the user.

The file FV.COM is splitted into pieces and sent to the user.

If the user when unpacking receives an error from his 7plus he

can request a correction file from the server by:

sp 7pserv@bbs_call

\public\fv.err

The correction file is then sent to him as a .cor file that

then could be used directly in his 7plus.exe program.

5		Receiving files

In addition you can also receive files automatically from users

that whishes to send you them. This is done by:

sp 7pserv@bbs_call

\upload\test.p01

The user should do this for every part he has splitted the file

into. Because of this the server must keep track when all files

has been received. If some file is missing nothing happens.

Whan all files has been received it tries to unpack it. If then

an error is detected it automatically sends a correction

request to the user in the form of a standard .err file that

could be entered in his 7plus.exe program. The user is then

supposed to send a correction file to the user in the following

format:

sp 7pserv@bbs_call

\upload\fv.cor@bbs_call

The message should contain the error file created by 7plus

program.

6		Automatic exchange

The server provide functions for sending files between two

servers of this type without human interaction.

Ex.

SM6XXX has installed this server and so has SK0XXX. SM6XXX

finds an interesting file in SK0XXX system and wants to get it

from him. He therefore enters the following.

sp 7pserv@sk0xxx

\BBSER\PRG\RLI.ARJ

Destination: \BBS\W0RLI.ARJ

7PFrom: 7pserv@sk0xxx

This is the only human intervention needed for transfering this

file to him. The servers is automatically sending .cor and .err

to each other switching names to inform the other which file is

intended.