1

TstHost version 1.36

--------------------

Features

--------

TSTHOST   is  a software  written for  TNC2 and  clones with   HOST

mode  EPROM type  TF8, TF23,  TF24 etc.  It is possible  to use the

software with other TNCs set in KISS mode.

A  TNC in  KISS mode  requires a  serial driver (like TFPCX/TFPCR),

but  don't forget  to set  the TNC  in  KISS  mode  BEFORE  running

TSTHost.

A  TNC in  WA8DED Host  mode  requires  a  serial    driver    like

COMBIOS,  MBBIOS etc.  The  GKJBIOS    driver    is    an    MBBIOS

compatible   driver  optimised for   TSTHost,  that   detects   the

special   16550A  UART.   For  the    driver    configuration,  run

GKBIOCFG.EXE and follow the instructions displayed on the screen.

As  far as  TSTHOST is  written for  8 channels,   you  cannot  use

the  software with  the original  WA8DED  or  TF4  EPROMs,  because

these  EPROMs have  only four  channels. It is possible the use the

software  with   EPROMs that  support more   of  8   channels,  but

only  8  are available.

TSTHOST  offers 8  independent  communication    channels,    where

every  channel has  a 300 lines receive buffer,  circular  keyboard

buffer  to recall  the  ten  previous  commands,  automatic    text

justification,  up   to    8  simultaneous  YAPP (with  resume  and

yappC  options) or  ASCII file  transfers,   up to   8 PMS channels

with  RLI-type forward  with  a master  BBS. All  received data can

be   saved   in   a   disk file. Every session may have a different

callsign.

There   is   a  separated window for monitoring, and it is possible

to capture in a file all the traffic displayed in that window.

The  text entered  from the  keyboard is  recognised as commands to

run  for every  COMMAND mode session, while in CONVERSATION mode is

sent  to   the   connected   station.   For this  reason it  is not

possible  to switch  in conversation  mode if  the session  is  not

connected.  The exception     is   the  Monitor  window,  where all

keyboard data  is transmitted as Unproto packets.

For   every  session   there is  a receive buffer, useful to review

all  the received  data. By  receiving data  from   a  channel  not

actually  selected, the  related callsign  in the  status line will

TSTHOST v1.36 (c) IK1GKJ                                     2

blink.   All  unconnected  channels  are  set  by  default  to  PMS

sessions.  So, when  an user  connects your  station, he  goes into

the   personal  mailbox system,  but if  YOU  connect  a  user  the

channel  is set   for  a normal  conversation mode.  Naturally, you

have  a specific command to switch  a PMS session into conversation

mode  and vice-versa.  During a  PMS session, the user can read and

write  messages, view  the users directory, up or download programs

with  the YAPP  protocol. The   PMS  session can  also run external

server   programs,   send   a   Break  to other PMS users, talk the

sysop.

TSTHost   automatically  recognises the  standard password  format,

and  the   extended MD2   algorithm.  The  standard password format

accepts  from   one to   ten  letters,    but  normally    all  the

programs require five letters for the password.

Other features are the Unproto list capability and a full forward

protocol  MBL-RLI or  FBB COMPRESSED  type, even  if  the  the  BBS

require a password for the access.

The  program is  written in   80286   code,  and does not work with

8088 microprocessors.

The  communication in   HOST-mode   between  the PC and the TNC has

a  3 seconds  timeout. After  this time,  if the  program does  not

receive  a reply  from the  TNC, attempts to synchronise the serial

port.

TSTHOST v1.36 (c) IK1GKJ                                     3

Tsthost with TNC in KISS or BAYCOM modem

----------------------------------------

You   need  the driver  TFPCR or  TFKISS for  KISS mode,  TFPCX for

modem  BAYCOM.   TFPCX version 2.10 supports both the KISS and  the

BAYCOM modes.

Using  a KISS-mode   TNC,  set the  TNC in  KISS mode  manually  or

with   the     appropriate  command     line    switch    of    the

TFPCR/TFPCX/TFKISS driver.

TFPCX  uses the  IRQ 253  for the  internal  communications,    the

other drivers uses the IRQ 254.

Load the driver

Load TSTHost with the command:  TSTHOST /T /I254 (or /I253)

/T  sets TSTHost   to   use   the  TFPCX/R  driver, and  /I254 sets

the IRQ for the driver communication.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the driver:  JHOST1, USERS 8, @U0

Example for TFPCR or TFKISS:

TFKISS command line parameters

TSTHOST /T /I254

Example for TFPCX:

TFPCX command line parameters

TSTHOST /T /I253

WARNING:   I  have noticed   that  TFPCR   version 3.30   does  not

work  properly with  high speed transfers over 400  cps (G3RUH 9600

baud).  I don't  know the  reason, but  version 2.10  of TFKISS and

TFPCX  does not have this bug.

TSTHOST v1.36 (c) IK1GKJ                                     4

Tsthost with the standard HOST mode

-----------------------------------

Configure  GKJBIOS or  other  COMBIOS  drivers  for  the    correct

serial port parameters. Load the driver in memory

Load TSTHost with the command TSTHOST /H /Cx /Bzzzz

where   /H  selects the  software to use the  standard host mode, x

is  the   serial port   where  you   have connected the TNC (1..4),

zzzz  is the  baud rate  of  the  port.  TSTHost  supports  a  data

transfer rate of 9600, 19200 and 38400 baud.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the TNC:  JHOST1, USERS 8, Z0, @U0

At  the end   of  the   program, you  must remove  from memory  the

serial driver with the command:  GKJBIOS U

For instance:

GKJBIOS

TSTHOST /H /C1 /B19200

GKJBIOS U

TSTHOST v1.36 (c) IK1GKJ                                     5

Tsthost with 2 or more tnc

--------------------------

You   may  connect   two or   more  interfaces to the program. This

can  be done with the TFPCX v2.10 driver. This  driver supports  up

to  8 interfaces   like  KISS,  BAYCOM,  MODEM  etc. This is only a

small  description,  because  the  complete  documentation  of  the

commands  is shipped with the driver files.

Since   the  driver   may support  up to  8 port,  the driver  have

a  command to  define which  channels assign  to  which  port.  The

extended  host command is @PO xxxxxxxx. Each 'x' is a value between

0  and the  number   of   installed   interfaces.   0 is  the first

interface,  1 the  second, etc. TSTHost uses 8 channels so you must

define 8 digits.

Of  course,   if you   use  the   driver for   one  interface only,

the  @PO  command  is  not  requested,  because  all  channels  are

assigned  to that interface.

Here are some examples:

1)   We  have two  TNCs,   connected to  COM1 and  COM2.  The  TNCs

are  already   set in  KISS mode,  the baud  rate is 19200 baud, we

want  to assign   channels  1234  to the TNC on COM1, channels 5678

to the TNC on COM2.

TFPCX -PKISS1 -PKISS2 -B19200:19200 -CH8

TSTHOST /T /I253

To assign the channels type the command:   PARAM 0 @PO 00001111

(This command can also be written in the tsthost.cfg file)

2)   We  have 2  TNC and  a  BAYCOM  modem.  The  BAYCOM  modem  is

connected  to COM1, 1200 baud; a TNC is on COM2 at 19200 bauds, the

second  TNC is   on  COM3,   IRQ10, 38400  baud. The  two TNCs  are

already  set in  KISS mode.  We to  want assign channels 123 to the

BAYCOM, 456 to the first TNC, and 78 to the second TNC.

TFPCX -PCOM1 -PKISS2 -PKISS3:03E8:10 -B1200:19200:9600 -CH8

TSTHOST /T /I253

To assign the channels, type the command PARAM 0 @PO 00011122

(This command can be executed from the tsthost.cfg file)

3)   A  TNC in  KISS mode,  already set  in   KISS mode,  on  COM1,

19200 baud.

TFPCX -PKISS1 -B19200 -CH8

TSTHOST /T /I253

In this case the command @PO is not requested.

TSTHOST v1.36 (c) IK1GKJ                                     6

Memory usage and EXPANDED memory

--------------------------------

TSTHost  requires at  least 450  KBytes of  RAM memory  to run, and

this  value can  drop to  210 KBytes  only, by  using the  expanded

memory.  This is  automatically detected  and used, if found in the

system.

The  memory  is  dynamically  allocated  and  released  when    not

anymore  needed,   but there   are some  function that require some

memory  to run.   For instance, the  Unproto  list  requires  about

24  KByte of  RAM memory  to store  the message  list.   Also   the

decoding  algorithm for   the   messages  in the  Unproto list  and

compressed  forward requires  19 KBytes,  that are released as soon

as the received message has been decoded.

The  RUN command  requires some  more memory to get the DOS prompt,

or  when any  user connected  to the  PMS esecutes a  PG or SERVER.

Usually  there   aren't   any   problem with a PC equipped with the

default  640 KBytes of RAM memory, even if the PC is not configured

to  use  the expanded memory.

If  you   have any  problem using  the  expanded  memory,  you  can

disable  the function  with the  command line  switch  /NOEMS.  For

example:

TSTHOST /H /B19200 /C1 /NOEMS

VGA mode

--------

Add  the parameter  /VGA to  the command  line, to  get  43  or  50

screen lines on an EGA or VGA monitor.

Example: TSTHOST /H /B19200 /C3 /VGA

TSTHOST v1.36 (c) IK1GKJ                                     7

Short cut keys

--------------

ALT F1             Automatic transmission of the password.

ALT F2             Displays the MHEARD list

(depending on the driver/TNC software)

ALT F3             Enables the monitor traffic to IUSC

ALT F4             Disables the monitor traffic

ALT F5             Set monitor traffic to U

ALT F6             Like the command ULIST VIEW

CONTROL ESC or F10 Switches command<>conversation mode.

PAGE UP            Activates review buffer, or if already active

displays the previous page.

PAGE DOWN          Displays next page in review buffer

CONTROL PAGE DOWN  Quit from review mode

CONTROL HOME       Top of review buffer

CONTROL END        Bottom of review buffer

ENTER              In command mode: executes the command.

In conversation mode: transmits the text.

In both mode, if the revision buffer is active,

it will be disabled.

CURSOR UP          Recalls the previous line from kbd buffer.

CURSOR DOWN        Recalls the next line from the kbd buffer.

F1...F8            Switches to session 1 to 8

F9                 Switches to the Monitor window

CTRL-F1..F10       User defined.

CTRL_BACKSPACE     Clears from cursor to the previous word.

CTRL_LEFT          Moves the cursor one word to the left

CTRL_RIGHT         Moves the cursor one word to the right

HOME               Moves the cursor at the beginning of the line

END                Moves the cursor at the end of the line

BACKSPACE          Deletes char to the left of the cursor

DELETE             Deletes char under the cursor

INSERT             Inserts mode toggle

ESCAPE             Clears the current line

CTRL-Y             Deletes the current line (internal editor).

TSTHOST v1.36 (c) IK1GKJ                                     8

Path of the files

-----------------

There  isn't a  particular directory  for the TstHost installation,

but  you MUST  create, within  that directory, three subdirectories

with the names: MAIL, PG, USERS.

This can be a standard installation:

\TSTHOST         <DIR>

|-PG           <DIR>

|-MAIL         <DIR>

|-USERS        <DIR>

|-TSTHOST.EXE

|-TSTHOST.DOC

|-TSTHOST.PSW  File for automatic password

|-TSTHOST.CFG  Configuration file for the program

|-TSTHOST.ENT  Input text when connect the pms

|-TSTHOST.HLP  Help file for pms

|-TSTHOST.SYS  Password file for pms DOS command.

|-TSTHOST.PGS  List/description of the available PG server

|-TSTHOST.ULS  Unproto list file, created/used by TSTHOST

|-TSTHOST.SHD  Tnc configuration file when shutdown program

|-TSTHOST.IN   Mail import file

|-TSTHOST.LOG  created from TSTHOST if LOG=ON

|-TSTHOST.BID  Bid file, managed by TSTHOST

|-TSTHOST.DMS  PMS messages database, created/used by TSTHOST

|-DBFMSG.OLD   Old messages database, created/used by TSTHOST

|-Files for source command (if you create source files)

Detailed description of files and directory:

PG - Server directory

You  must place the PMS server in this directory. If this directory

does not exist, the PMS PG command does not work.

The next section of the manual describes the PG Server.

MAIL - Messagges directory

The  bulletins/personal   messages of  the PMS  will be  stored  in

this  directory. If  this path  does not exist, the electronic mail

section of the PMS does not work properly.

TSTHOST v1.36 (c) IK1GKJ                                     9

USERS - Users directory

This  is the  directory   that   the  users  access with  the W, YU

YD  commands. This  is the   only   path   that  can point to other

parts  of your   hard  disk.   If not  otherwise specified, TSTHost

uses  the path  USERS from  the directory   where   tsthost.exe  is

loaded.   You   can  specify any  other  drive/directory  with  the

USERDIR command.

WORKDIR - Standard work directory

The  default value  is the  directory where  tsthost.exe is loaded,

but  you can  select any  other drive/directory  with  the  WORKDIR

command.  This path is used for the WRITEBUF, RECORD,  SEND,  YPUT,

YREC  and  MAILCOPY  commands.  With  these  commands,  unless  the

filename  does not contain a  specified  drive  and/or  path,   the

file   will   be   accessed in  the  WORKDIR  path.  For  instance,

by   executing   the  command WRITEBUF  TEXT, the  contents of  the

review  buffer will  be written  in   a file   called  TEXT  in the

directory  that  you have specified with  the WORKDIR  command, but

WRITEBUF  E:\ASC\TEXT will  write the buffer  into  a  file  called

TEXT   in  E:\ASC. The  purpose of  this command is to separate the

tsthost files  in  capture, YAPP receive and the other files.

TSTHOST.PSW - Automatic password file

This   file  contains the  password for  those systems that require

an access password, or for the SYS command for BBS and node sysop.

The  next manual  section describes  this aspect with more details.

If  you do  not need  any password,  or do  not need  an  automatic

password,  do not  create this  file. Anu  line starting  with  the

'#' symbol is a remark and is ignored.

TSTHOST.SYS - Password file for PMS DOS command.

This  file is  similar to  the tsthost.psw file. The user connected

to  the PMS interface can run  DOS  commands only if  his  callsign

is  registered   in this   file. The  format is  simple: every line

start  with   a callsign   without  SSID, a  space,  and  the  user

password   not  longer   than  255   alphanumeric  characters.  For

instance:

IK1GKJ aabbcc

IK1MSL 8873mnhbkjAST

TSTHOST v1.36 (c) IK1GKJ                                     10

TSTHOST.CFG - Automatic configuration file

This   is   an   ASCII    text  file,  that  contains  the  command

sequence  for   the   automatic  configuration.  This  file will be

searched  by TSTHost  when   started and,  if present,  the program

executes  all the  commands   in the  file. Every   line    is    a

command.   This   file  can contain  any TSTHost command.  Any line

starting  with the '#' symbol is  a remark  and  will  be  ignored.

The  line  length  MUST  not exceed 78 characters.

TSTHOST.SHD - Shutdown configuration file

Like   the  tsthost.cfg   file,  this  file  contains  the  command

sequence that is executed at the program shutdown.

TSTHOST.ULS - Unproto list file

This   file  is created  and  managed  by  the  program.  When  you

activate  the   unproto   list  the  program  reads  this  file, if

present,  so TSTHost   can  handle the  message list  from the last

number  received. When   you   close   the  list  or  shutdown  the

system,  the program  saves the  content of  the list  in this disk

file.  If   this   file  is not found or you delete it, the program

will  ask to  the HomeBBS  the Unproto message starting from number

one.

TSTHOST.ENT - PMS connection text

This  is an   ASCII   text   file.  When a remote user connects the

PMS  system,   TSTHost sends  the content  of this  text  file  (if

found) to the user before the PMS command prompt.

WARNING:  Do not use the square brackets '[' and ']' in this file,

because they are used for SID forward protocol.

TSTHOST.HLP - PMS help file

This  is an   ASCII   text  file. When the remote user connected to

the  PMS issues the  HELP  command,  TSTHost  sends  the content of

this  file to  the user,  otherwise sends  the message 'NO HELP' if

the file is not present on the system.

TSTHOST.PGS - PMS PG server list

Another   ASCII file.  It can be created by the sysop according  to

his  requests, because  it is  related to  the PG  section. When  a

user  connected   to   the   PMS   issues   the  command PG without

arguments,  TSTHost sends the contents of this file to the user, or

the  message NO PG AVAILABLE if the file does not exists. This file

can be used to list and describe the available PG server programs.

TSTHOST v1.36 (c) IK1GKJ                                     11

TSTHOST.BID - Bid file.

This  is a very important file, and must NOT be altered or deleted.

The  purpose of this file is to have a list of the received bid and

to  store the number of the last bid used by tsthost. But what is a

BID?  Simple, think  to a  bid like  a word  that unique identify a

message.  Every time a new message will be created, from tsthost or

other  bbs, a  unique bid  will be appended to the message, this to

avoid duplicate.

For  example, assume  that i forward the message XX to homebbs, the

message  have bid  YY. Next i use the command FA to force again the

forward  of this  message to  homebbs.  When  homebbs  receive  the

proposal,  it detect  that the  bid YY  was already  received,  and

refuse the message.

The  internal bid of tsthost are a sequential numbers and a string,

you callsign: 12345+IK1GKJ

(1->2->3->..65535->1->2 etc etc).

Now, if you delete or lose this file, you obtain 2 things:

1)  will be  lose the  list of  the received  bid, this is a little

problem,  at most,  if you  receive forward from more than one bbs,

is possible that you receive a duplicate message.

2)  tsthost must  restart the  bid sequence,  and this  is a  great

problem.  For example,  tsthost  have  created  and  forwarded  ten

message,  this messages  have bid  from 1 to 10. Now, if you delete

the  file, tsthost must restart the sequence from 1, but since your

homebbs  have already  received bid  1..10, it will refuse any your

message with bid number under 11.

To  avoid this  problem, you can use the command NBID, to assign an

high  value to  the bid  number.  This  is  a  emergency  solution,

because  assume that  you know what was the last bid number used by

tstshost.....  The best  thing is  to NOT  DELETE or  altered  this

file.  Pleas note  that the  NBID command  must be  used  ONLY  for

emergency situation, normally you MUST NOT usa this command.

When  tsthost start,  if the bid file do not exist, the program ask

for  its creation  (normally the  first time  you  start  tsthost).

Insert  Y to  create the  file, (and  if the  file was  lost do the

command NBID BID_NUMBER), or N to exist and provide a backup copy.

TSTHOST.BID  a routing file of 3001 record. Every record is long 13

byte,  and is  the bid  of the message (char[13]). The first record

is  an exception,  it  have  two  unsigned  integer  number  (ascii

format),  separated by  space; the  first is  the next internal bid

that  tsthost will  be use,  the  other  is  the  first  record  to

overwrite into the tsthost.bid file.

TSTHOST v1.36 (c) IK1GKJ                                     12

TSTHOST.LOG - Connections log

This  file will  be created  and   managed   by  TSTHost  when  the

LOG  command  is  ON.  It  contains  the  list  of the  connections

with callsign, date and time.

SOURCE files

These  files are  similar   to   the  tsthost.cfg  file,  but  will

be  executed  only  when  you  type  the  command.  This is  useful

to  activate specific  configurations on  demand. Any line starting

with the '#' symbol is considered a remark and is ignored.

TSTHOST.IN - Mail import file

This   is   an  ASCII text file. The purpose is to allow SERVER and

PG  programs   to   add   messages   to   the    PMS.     The  file

presence  is tested    every  minute  and  processed,  when  found.

TSTHost  searches in  this   file a  sequence of SP or SB commands.

Note  that you  MUST use  the   correct syntax.   When processed, a

line  starting with SP or SB is searched, discarding the others. If

such  line is  found, the  next one is considered the message title

and  the others  are the  message text.  A line  starting with  /EX

ends the message.

Example:

SP IK1AAA @ IK1MSL < IK1BBB

message title

message line 1

message line 2

....

/EX

SB ALL @ EU

message title

message line 1

message line 2

....

/EX

The   TO  field   MUST be  specified, the  ROUTE (@)  and FROM  (<)

fields

are  optional.   If the  route field is specified, the message will

be

forwarded   to  the   HomeBBS. If  the FROM field is not specified,

the

default value of the PMS callsign will be used.

NOTE: The line length MUST NOT exceed 80 characters.

TSTHOST v1.36 (c) IK1GKJ                                     13

TSTHOST.DMS - PMS messages database

This   is  a   binary file,   and is  used by  TSTHost to  maintain

the  messages   list in   the  MAIL directory. Do not alterate this

file  if you do not know what you are doing!. The easiest method to

destroy all messages is to delete this file, so be careful  ......

This   is    the    tsthost.dms    file    structure,    for  those

programmers interested in writing server programs:

struct dirmes {

char type;                  /* type of message B P */

char status;                /* status of message N F K Y */

long number;                /* number of message */

int  da_year;               /* creation date, year */

char da_day;                /* day */

char da_mon;                /* mounth */

unsigned char ti_min;       /* creation time, minutes */

unsigned char ti_hour;      /* hour */

unsigned char ti_hund;      /* hundredths of seconds */

unsigned char ti_sec;       /* seconds */

char to[7];                 /* destination, TO field */

char route[7];              /* destination BBS, @ field */

char from[7];               /* sender, FROM field */

char title[61];             /* message title */

char frombbs;               /* <> 0 if received in forward */

char startbbs[7];           /* BBS where the message started*/

char bid[13];               /* BID or MID of the message */

char rcvbbs[7];             /* BBS that have fwd the msg */

char free[4];               /* reserved */

};                 /* total 128 bytes */

All char fields end with a 0 byte, according to the C default.

The  message is  decoded with  the number  field. The message is in

the  MAIL   directory:   the   filename   correspond to  the NUMBER

field  and the  extension is  .MES. The  filename is always 8 chars

long,   right aligned by padding the remaining left positions  with

ASCII  '0's. For   instance,   if  NUMBER is  4,   the      message

filename   will  be 00000004.MES

The  message number  is consecutive   and,    when    the    server

appends  a new  message,   it   has   to  read the last record from

tsthost.dms  and increment  the number  field  to  obtain  the  new

message number.

WARNING:   all  fields in  the dirmes  structure must  be always be

set;  for instance,  if the  @ route is not requested, you MUST set

it, so *dirmes.route=0;

DO NOT ALTER THIS FILE WHILE TSTHOST IS RUNNING !

TSTHOST v1.36 (c) IK1GKJ                                     14

FORWARD,   UNPROTO   list,   CHANNEL   8   and (optional) automatic

password request.

--------------------------------------------------------------

TSTHost  connects the  HomeBBS  for  forward  or  unproto  list  on

channel  8 only.  If this  channel is  BUSY, the  program waits and

retries   to connect the station when this channel is free. For the

forward,  the program  always calls  the master  BBS to  allow  the

reverse  forwarding but  the unproto  list  is  requested  only  if

active and there are some selected messages.

NOTE:  in a  multiport system  (multiport TFPCX),  you MUST  ensure

that the channel 8 is associated to the same port of the HomeBBS.

Again,  since it  is possible  that the program  calls  the station

while you are already connected to the HomeBBS on another channel,

it is suggested to set for channel8 a  unique  SSID (AX25 PORTCALL

command).

It  is possible  to ask  for the  forward and  unproto list EVEN if

your  HomeBBS requires  a password  to access  the system.  To  get

this,   you  must   set an optional field  in the  HOMEBBS command.

This  optional field   must  be  set  ONLY if the HomeBBS  requires

an access password.

For  example, if my  HomeBBS IK1MSL-8 does not require a password,

the HOMEBBS command will be:

HOMEBBS IK1MSL-8

But, if my HomeBBS requires a password, and the prompt is:

PASSWORD? 11 22 33 44......

The command must be:

HOMEBBS IK1MSL-8 PASSWORD?

Of  course,   this prompt   and  the relative password must be also

set  in    the   TSTHOST.PSW   password   file.  Check the  section

about  the AUTOMATIC     PASSWORD   for more  informations on  this

matter.

TSTHOST v1.36 (c) IK1GKJ                                     15

FORWARD protocol

----------------

The   PMS  can forward  with a master BBS according to the standard

MBL/RLI  or FBB  COMPRESSED protocol.  Any BBS  may connect the PMS

and  forward data,  but the   PMS  will  send  the  message only to

the BBS defined with the HOMEBBS command.

The   PMS   will  send  to  the HomeBBS  all the messages that have

a  specified   route set  ( @Field not blank), and locally created.

If  a received   message   has  been forwarded  by another  BBS, it

will   be forwarded to the  connected  BBS  only  if  the  callsign

of   the  connected   BBS is not the same of the callsign stored in

the RCVBBS field.

For  personal  messages,  they  will  be  forwarded  only  if  the

addressee of the message has not read it.

In   the message list, the letter 'L'  near the type/status of  the

message  shows that   this  message was  written locally, otherwise

the message was received by the forwarding activity.

This is an example of a forward file for F6FBB software:

#

A IK1GKJ

#

P B

R

N

#

B IK1GKJ

F IK1GKJ

#

C C IK1GKJ

#

----------

#

If   you  also need   the   @AMSAT  messages, the  sysop  must  add

the G AMSAT statement after the F IK1GKJ line

TSTHOST v1.36 (c) IK1GKJ                                     16

TstHost  use to  call homebbs  for fwd  or unproto message a simple

protocol.  First, will  be connected  the bbs, using HNode or HDigi

if  set, or  directly. Next TstHost wait until receive the bbs SID,

the  string [FBB-5.15-ABFHM$]  or similar. The program MUST receive

this  string, because  from the sid the program can detect the type

of protocol to use.

After  received the  sid, if a password is defined, tsthost wait to

receive the password request.

And  now, tsthost  wait for  a bbs  prompt. When  the bbs prompt is

received,  the sequence  is completed,  and forward or unproto list

request may started.

The  bbs prompt,  is a  char >  at the end of a line. This may do a

problem  on some  bbs, since  many sysops  have set the char > into

the  welcome text  on their  bbs. In  the normal connection this is

not  a problem,  but with a remote protocol yes, because if tsthost

receive  this character,  it think  that it  is the bbs prompt, and

the sequence was not corrected.

For example, if the bbs welcome text have a line line this:

HALLO MARIO, FOR HELP TYPE THE COMMAND <?>

surely  you will  not be  able to start fwd or unproto, because the

character > at the end of the line confuse tsthost.

But,  this line,  HALLO MARIO, TYPE <?> TO HELP, do not do any type

of problem.

If  you have  this problem, you may ask to the sysop bbs to set you

like  a real  bbs. The  bbs nevar  send text  to real bbs, but when

connected send only the prompt (or password request).

NOTE:  If you cannot start fwd o unproto list, and cannot solve the

problem,  to obtain  help, please  switch on  channel  8,  manually

start  a fwd  or unproto  request, and  when disconnected,  do  the

command  WRITEBUF ABC.  Next send me a message whit the contents of

the ABC file, so i cam examine it.

TSTHOST v1.36 (c) IK1GKJ                                     17

FORWARD command

---------------

* FL

This   command  list all  messages that  must be forwarded but have

not yet been sent to the HomeBBS.

* FA nbr nbr nbr (up to 10 numbers)

Add the specified messages to the forward list.

* FR nbr nbr nbr (up to 10 numbers)

Remove the specified messages from the forward list.

* FBbmode ON OFF

Enable  (default) or  disable the fbb compressed forward mode. This

type   of  forward  require  about  32  kbyte  of  memory  for  the

compression  routine, if  the program do not found enough memory to

start  the compressed  forward, automatically switch to the MBL/RLI

forward type.

* FTIME 0..60 minutes

Every  FTIME minutes,  the PMS  will connect  the master BBS asking

for  the  forward. Any value  below  15 disables the function,  and

the forward is performed only if the HomeBBS calls your station.

* FWD

Manual start of the forward with the HomeBBS.

* HIeraddr TEXT | OFF

Is  a field  of max 20 characters, used to define your hierarchical

address, for example .IPIE.ITA.EU

OFF  disable the  function. This  field, if  present, is  used with

RLine command.

* RLine ON | OFF

If  ON, when your PMS forward a message, your R: line will be added

to  the message,  if OFF  no. WARNING:  with RLine  ON, you will be

considered  exactly like a regular bbs, so, if a user use a command

SR  on your  mail, the  return address  will  be  YOURCALL@YOURCALL

insted  of YOURCALL@YOURHOMEBBS. If @YOURCALL is not defined in the

homebbs  and/or adiacent  bbs inhis  forward file, mail sent to you

vill  nevar arrive  to you,  this because  you are not a bbs in the

regular  path. Whit  off this  is not  a problem, because your mail

will be returned to your homebbs, and homebbs is know in the path.

But,  some server have the capability to communicate one to another

trougth  the RLine,  and in  this case,  if you do not enable RLine

this type of server cannot work properly. So see you need.....

TSTHOST v1.36 (c) IK1GKJ                                     18

SERVER PG

---------

The  PG servers  are programs  written to  expand the capability of

the  software. They  MUST be  placed in the directory PG, one level

below  the  location  of  tsthost.exe. As  the  system  is inactive

while  executing   the  PG  servers, they must be small and use the

minimum  machine time. The displayed  image  is  saved  during  the

program  execution. The  server  must  NOT  access  the  video  RAM

directly.

The server receives the following command-line:

argv[0]     Drive\path\servername;

argv[1]     Callsign of the user, with ssid

argv[2]     Server level, 0 the first call, up to 255 the next.

argv[3...]  User-specified parameters, each word  is  a  separated

argument.

The  server data  output (with  printf() and putf()) is transmitted

to the user.

The server may return:

0 - The output of the server is  sent  to  the  user  and  returns

to the PMS prompt

1 - The output of the server is sent to the user and the server is

called again.

2 - The output of the server is executed like a PMS  command,  and

the user is reconnected to the PMS interface.

3 - The output of the server is executed like a PMS  command,  and

the server is called again.

NOTE   for  2   and 3:   The  output must be maximum 80 chars long,

and  must start  at the  line beginning;   the  line must NOT start

with   a  carriage   return or  line feed.  The server  must  NEVER

return another PG command.

TSTHOST v1.36 (c) IK1GKJ                                     19

For   instance, this is a very simple ECHO server  that  returns to

the user all he have typed, until the user types BYE:

#include <stdio.h>

#include <string.h>

main(int argc, char **argv)

{

int i,mode;

sscanf(argv[2],"%d",&mode);

if(!stricmp(argv[3],"BYE") && mode)

{

printf("\nBest 73\n");

return 0;

}

printf("\nHallo %s, you have send:\n",argv[1]);

for(i=3;i<argc;i++) printf("%s ",argv[i]);

printf("\nType BYE to end.\n");

return 1;

}

The user IK1AAA calls the server with PG ECHO AA BB.

The server returns HALLO IK1AAA, YOU HAVE SEND AA BB,

TYPE BYE TO END

the user types I AM MARIO

The server returns HALLO IK1AAA, YOU HAVE SEND I AM MARIO,

TYPE BYE TO END

the user types BYE

The   server returns BEST 73,  and the  user will return to the PMS

prompt.

TSTHOST v1.36 (c) IK1GKJ                                     20

AUTOMATIC PASSWORD

------------------

The  TSTHOST.PSW  file  contains  the  passwords to use with those

systems where it is requested for the access. The file format is:

STRING_TO_SEARCH PASSWORD

STRING_TO_SEARCH   is   the   system  prompt when  the password  is

being  requested, max  50  character long  with no spaces, PASSWORD

is  your  password,  up  to  255  alphanumeric  characters  without

spaces.

Example:

IK1MSL-8> abcdef

IW1BRX-8> 123456

IK1GKJ-8> ABCEDF

Warning:  Upper and  lower case are ignored  in  the  string being

searched, but are important in the password string.

Here  are some  example for  the BBS  prompt, that  can  solve  any

doubt,  and make you write the correct tsthost.psw. In the example,

my demo password is ABCDEF

IW1BRX-8> 11 22 33 44 55 [0123....           IW1BRX-8> ABCDEF

Password? IW1BRX-8> 11 22 33 ....            IW1BRX-8> ABCDEF

IW1BRX-8> Password? 11 22 33 .....           Password? ABCDEF

IW1BRX-8> Password ? 11 22 33 ....           ? ABCDEF

As  you have  seen, the  string being  searched is  ALWAYS the LAST

word or characters that came before the password code.

If the BBS prompt is:

Insert your password 11 22 33 44 55

and you insert in your tsthost.psw file:

Insert your password ABCDEF

TSTHost  will interpret  "Insert" as the string to be matched, and

"your"  as your  password. This  is an   error! The correct form in

tsthost.psw must be:

password ABCDEF

TSTHOST v1.36 (c) IK1GKJ                                     21

PMS INTERFACE

-------------

Every  channel of  the software is a PMS and  a  normal connection

channel at the same time. All non-connected  channels  are  set by

default in PMS mode. This means that a remote  user  that connects

your station will operate in PMS mode.

But,  if YOU  issue the  Connect command,  the  channel  becomes  a

normal conversation channel.

There  is   an exception  to this. When the software starts, if one

or  more   channel in   your  TNC   are already  connected for  any

reasons,  they are  conversation channels,  not PMS.  The reason is

that   it   is  impossible to  know if  those  channels  have  been

connected by  us  or by a remote user.

You   can  switch   from PMS   or  NORMAL  channel  mode  with  the

commands  TALK   (from PMS to NORMAL, for example after a chat with

the  remote user),   and   PMS   (from  NORMAL to  PMS). You cannot

switch  a session  if the  task is  not stand-by. The PMS cannot be

executed  if there is an active file transfer on the channel (YAPP,

send  or capture). A  TALK  command  has  no  effect  if the PMS on

that channel is processing a user command.

Moreover,  you cannot switch a window to conversation mode  that is

already  active in  PMS mode.  This is  correct because  you cannot

chat  with a  user, or  your  text  will  interfere  with  his  PMS

activity. You have to switch to TALK mode before chatting.

A note about the DOS  command:  when  the  user  performs  the DOS

command,   his  callsign   will be   searched  in   the TSTHOST.SYS

file,  and, if  found,  the  system  requires  a  password,  ALWAYS

CODIFIED  WITH THE   MD2  SYSTEM.  If the user enters the corrected

password,   the  system   will   run  COMMAND.COM  with  the  user-

specified   command.  NOTE   that there  are no  controls, the  DOS

access  to the  system  is complete without restrictions, so DO NOT

RUN programs that can lock the system.

The  password prompt is the PMS callsign without SSID,  with _PMS>

and the password request, like:

IK1GKJ_PMS> [0123456789]

TSTHOST v1.36 (c) IK1GKJ                                     22

THE SERVER

What  is the  server? The  server are programs com or exe that must

be  exist in  the tsthost.exe  directory. When  a personal  message

arrive  to your  pms interface,  and tsthost  detect a  server (see

SERVER  command), tsthost  copy the  contents of  this message to a

file,  next execute  the related  com or  exe file  and  pass  like

argument  the name  of this  file. This is a standard forward file,

so  the first  line will  contain SP SERVERNAME < IK1AAA, next line

will  be the title of the messages, next to end the contents of the

messages.  Server may  do anything, but any return message from the

server  to sender must go into the tsthost.in file. When the server

stop  the execution,  control return  to tsthost,  messaged  copied

will  be deleted,  original message  will  be  put  into  K  state.

Remember that will server are running, tsthost is stopped.

PMS command from KEYBOARD

-------------------------

* ENdmessage filename | OFF

This  command defines  a file  that will  be automatically added at

the  end   of any  message written  with the S?/SR functions of the

PMS.  If the   S?/SR  command already have the +FILENAME extension,

first  will be  added the  FILENAME contents,  then the  ENDMESSAGE

contents.

OFF Disables this function.

If ENDMESSAGE does not have a specified path, it will  be searched

in the program directory.

* R       to read messages

* K       to kill messages

* L       to list messages

Optional parameters for commands R, L, K:

(The example is for List command, but most of them also work for

Read and Kill)

L              List all messages  (only LIST, not READ/KILL)

LL 50          List last 50 messages (only LIST, not READ/KILL)

L 100          List message number 100

L 100 102 150  List messages 100 102 and 150 (max 10 numbers)

L 100-150      List message from 100 to 150

L -150         List message from begin to 150

L 150-         List message from 150 to end

TSTHOST v1.36 (c) IK1GKJ                                     23

L< CALL        List all messages sent by CALL

L> CALL        List all messages for CALL

L@ ROUTE       List all messages for that route.

LS STRING      List all messages that have STRING in the title.

LM             List all my messages

NOTE:  From keyboard, the sysop can access all  messages,  even if

the message is for someone else and if the message is Killed.

The exception is for the LM RM KM commands, that access  the sysop

messages and do not access the Killed messages.

* S  CALL [@ ROUTE] [+FILENAME]

* SB CALL [@ ROUTE] [+FILENAME]

* SP CALL [@ ROUTE] [+FILENAME]

* SR MESSAGE_NUMBER [+FILENAME]

* SC MESSAGE_NUMBER CALL [@ROUTE] [+FILENAME]

The 'SP' sets the message to Personal,  that  means  that only the

addressee can read the  message.  The  'SB'  sets  the message  to

Bulletin,  that means  that anybody  can read  the message.  If the

'SB'  or 'SP'  are not  expressly specified, (the 'S' command), the

program  assigns the 'P' mode for any valid CALL, otherwise the 'B'

mode.

SR  automatically replies  with a  P-type message  to the sender of

the  specified   message number.  The title  of the message will be

"Re:" and the title of the original message.

SC send a copy of the specified message number to CALL.

During   those   command,   you   cannot   switch the task, but you

must  terminate the  message editing  session. The  space for   the

internal  editor  is  150  lines.  If +FILENAME exists, the content

of  the specified  file is  appended to the end of your message. If

FILENAME  has a drive/path specification, the file is read from the

selected  location, otherwise  it will  be searched  in the WORKDIR

path.

* HDigi CALL-SSID | OFF

To  define a  digipeater to  use for  connect homebbs in UL Get and

FWD command. Pratically CONN HOME VIA DIGI

* HNode CALL-SSID | OFF

To  define a  node to  use for  connect homebbs  in UL  get and FWD

command.  OFF disabel  the NODE  usage. Pratically,  first will  be

connected the HNODE callsign, next will be connected the homebbs.

NOTE:  If is  defined HNode and also HDigi, program will attempt to

connect HNode via HDigi, next Homebbs.

TSTHOST v1.36 (c) IK1GKJ                                     24

* HOmebbs CALL-SSID [PASSWORD PROMPT]

Defines  the BBS  where the  PMS messages  have to  be transmitted,

those  with the  routing field  (@) set. SSID must be specified, as

IK1MSL is different from IK1MSL-8.

PASSWORD PROMPT  is  an optional  field used  ONLY if  the HomeBBS

requires a password for  the  system  access,  check  the relative

section in this manual.

* HAlias CALL-SSID

To  define an extra callsign that will be considered like a homebbs

when connected.

* Mailcopy message_number destination_filename [/A]

Similar  to READ  msg_nbr, the output will be written in a file. If

file  name does  not have a drive/path specification, the file will

be written in the workdir directory.

Note:  with /A  parameters,  the  message  will  be  appended,  not

overwrite destination_filename

* NBID bidnumer (1...65535)

To  assign a new bid start sequence. DO NOT USE THIS COMMAND IF YOU

DO NOT KNOW WHAT YOU ARE DOING!!. Read TSTHOST.BID section before.

If   you  do  this  command  without  parameters,  simple  will  be

displayed the next internal bid number.

* PMS

The current task is switched in PMS mode, for instance after a QSO

with the remote user that called you. The user returns to  the PMS

prompt. You cannot switch if you have any active file  transfer in

the channel (YAPP, record, send). See the TALK command.

* RIng

If ON (default), when the user does the 'T' command  the  Sysop is

called,  otherwise the  user receives  a message  that the Sysop is

not available.

* SERVER +/-SERVERNAME +/-SERVERNAME .... up to 10

To  define up  to 10  server program  that will  be  executed  when

arrive a personal message addressed to SERVERNAME@YOURPMS.

+  ADD server,  - REMOVE server from list. Do not specify extension

or  path to  the server  statement,  only  the  filename,  and  the

executable file must be located into the TSTHOST.EXE directory.

* TALK

The  current task is switched  to  the  normal  conversation mode,

for  example when  the remote  user TALKs the sysop (see PMS). You

cannot   switch  if   the  PMS   is   not   in   stand-by   status.

TSTHOST v1.36 (c) IK1GKJ                                     25

* Timeout minutes

Specify  the time after, without activity on  the channel,  i will

be automatically disconnected. Time is between 0 and 60 minutes; 0

(default) disables the timeout function.

* UPdatedms pday bday

When  a message  is killed, the file is not  deleted from the disk

but the status flag becomes 'K'. This command allows  the deletion

from the system of those messages that are killed, status 'K', and

the  PERSONAL   messages that   have   been   already  read, status

'Y', or forwarded, status 'F'.

PDAY  is the number  of  days (from creation) that a PERSONAL type

message  can exist  in  the  system,  BDAY is  for  BULLETIN  type

messages.  When a  message is  older than  this number  of days, it

will be deleted from the system, without checking the status flag.

Please  note that  you cannot run  this  command  if  any  user is

connected to the PMS interface.

For  example, to  erase  all  private messages after 10  days, and

bulletin messages after 4 days:

UP 10 4

* USerdir [path]

Sets the directory for the users connected to the PMS interface.

The default path is a subdirectory within  the  one  that contains

tsthost.exe, but you can change to a new path with this command.

Note that TSTHOST does not CREATE  the  directory,  the  path MUST

EXIST.

TSTHOST v1.36 (c) IK1GKJ                                     26

PMS commands for remote users

-----------------------------

* R       to read messages

* K       to kill messages

* L       to list messages

The valid parameters for the R, L, K commands are listed in the

following table (the examples are for the List command) :

L              List all messages (only LIST, not READ/KILL)

LL 50          List last 50 message (only LIST, not READ/KILL)

L 100          List message number 100

L 100 102 150  List messages 100 102 and 150 (max 10 number)

L 100-150      List message from 100 to 150

L -150         List message up to number 150

L 150-         List message from number 150 on

L< CALL        List all messages sent by CALL

L> CALL        List all messages for CALL

L@ ROUTE       List all messages for that route.

LS STRING      List all messages that have STRING in the title.

LM             List all my messages

NOTE:   Any  users have  access  to  all  bulletin    and  personal

messages  addressed to his callsign or that  he  have  sent. Killed

messages will never be listed.

* B

Disconnects the system.

* DOS DOS_COMMAND

If  enabled, TSTHost executes the  specified DOS command after the

password request.

* PG [[NAMESERVER] SERVER PARAMETERS]

If called without  arguments, the  program sends  to the  user the

contents   of  the   file TSTHOST.PGS, if exist, otherwise sends NO

PG  AVAILABLE. In  this way  it is  possible for  the sysop to have

some  PG servers  not declared. A user will not execute the servers

but  the sysop,   or  a  restricted  group  of  users,  can  easily

activate these extra services.

If  the server  name parameter  is specified,  the program executes

the  specified  server. The  output of  the server  is sent back to

the user.

TSTHOST v1.36 (c) IK1GKJ                                     27

* S  CALL [@ROUTE]   * SB CALL [@ROUTE]           * SR MSG_NBR

* SP CALL [@ROUTE]   * SC MSG_NBR CALL [@ROUTE]

Stores  a message  into the   PMS.  SB is for bulletin messages, SP

for  personal  mail. The  field @ROUTE  is optional.  If specified,

the  message will  be forwarded to the specified HomeBBS. If  the P

or  B types are not expressly specified,  the program automatically

assignes the P type for  any  valid  CALL, otherwise the B type.

SR replies a P-type message to the sender of a specific message.

SC send a copy of message number # to call.

To terminate the message editing, send CTRL-Z  or /EX in the first

text column.

* T

Calls  the sysop.  The program  displays a  video message to notify

the  sysop  the  user  request.  If CBELL is ON the programs sounds

the speaker, together with the message.

* U

List the users connected to the PMS interface.

NOTE:  Only the stations connected to the PMS are listed. Channels

not in PMS-mode are not listed.

* V FILENAME

Display to the users the contents of an ascii file.

* YD FILENAME

Sends  to the   user  the specified  file using  YAPP protocol. The

file must  be in  the  USERS  (USERDIR  command) directory or  its

subdirectories.

* YU FILENAME

TSTHost receives a file from the user, with the YAPP protocol.

The Resume option is always enabled, so if the transfer  stops for

any reason, the user can reconnect the system and restart the file

transfer from the  point where  he was  arrived. The file will  be

put  in the  USERDIR   directory, or  in a  subdirectory  specified

with the file name.

* W [PATH]

Display the contents of the USERDIR directory.

* ?

Sends to the  user the  contents of  the TSTHOST.HLP file. If this

file   do  not  exist,  the  user  receives  the  message  NO  HELP

AVAILABLE.

* @ CALL TEXT

Interrupts  the  user  CALL,  that  must  be  connected to the PMS

interface, by notifying the TEXT message.

TSTHOST v1.36 (c) IK1GKJ                                     28

UNPROTO LIST

------------

When a FBB BBS receives a new  message,  emits  a  special Unproto

frame  with the  message description.  Also, it  is possible to get

the BBS message list without connecting the system.

With  those  features,  a  user with the PC active all the day can

receive and maintain the list of the messages present on the BBS.

It is not needed to connect the BBS to get the message list, too.

To  use this feature  you must be enabled on the BBS system so, if

you need the Unproto list, contact the Sysop.

Even  if you are not enabled, the program  can  create  a  list by

intercepting  the Unproto  messages, but  cannot resyncronize  with

the BBS system if a packet is lost.

NOTE: to use the unproto list, monitor channel  must  be  open for

the  Unproto frames,  letter U.  AX MON  U,  AX MON US, AX MON UC,

AX MON USC are correct, some other modes without the 'U' aren't !

The Unproto list can save in memory up to 300 message headers. The

older messages are then discarded to leave the space for new ones.

The Unproto list feature requires 24 KBytes of RAM.

The  HOmebbs  command  sets the BBS from which the Unproto list is

accepted.  You must  specify  the callsign SSID, because IK1GKJ is

different from IK1GKJ-8.

You can set up  to 10 fields that are automatically  selected when

received. The list can be  obtained without user operations, or by

issuing   a  special command (please read the section about channel

8 on the manual), even if the BBS requires an access password.

The  message in  Unproto  list  is  transferred with the compressed

FBB  protocol. This  protocol requires  about 19K  of RAM  for  the

message decoding.

The  files are  then saved  in the  WORKDIR  directory,  where  the

filename  is the  TO field and with the .MSG extension, or put into

the PMS interface, see the UL INTERFACE command.

TSTHOST v1.36 (c) IK1GKJ                                     29

IMPORTANT NOTE

Some  programs allow the use of unproto frames with  the  poll bit

set, that is not a  problem  for  the  Unproto  list.  However, in

certain conditions, this can become a problem.

For  instance, you  are connected  to your  HomeBBS on  channel  1.

TSTHost  emits   a Sync   Unproto  frame  towards your HomeBBS, for

the  Unproto  list.  The  HomeBBS, when receives the Unproto  frame

with  the poll  bit set, returns you  a  DM  frame. This is correct

since your monitor channel is not connected.

But  your AX25 interface receives the DM frame ,  and  then  it is

redirected to the channel 1 where you are connected, to disconnect

the   link.  In this  case you  are disconnected by your TNC, while

for the BBS you are still connected.

To  avoid this  problem, TSTHost  starts by sending the @U0 command

to  disable the  poll bit  in unproto  frames. Maybe some Eproms or

Driver  do not recognise the command (as far as I know, only  TFPCR

v2.10)  and if  the Eprom/Driver  starts with  the poll bit set you

cannot  fix the  problem so  you'll be randomly disconnected by the

system.

TSTHOST v1.36 (c) IK1GKJ                                     30

UList Subcommands

-----------------

Use these commands after the ULIST prefix, for instance:

ULIST QUERY 122

* Autoselect +/-TO_Field

Automatic selection of the messages in the  Unproto list according

to  the desired  TO_Fields (up  to 10). For instance, to select the

ALL, DX and NEWS fields the command is:

UL A +ALL +DX +NEWS

Then, to add the KWD field and remove the ALL field:

UL A +KWD -ALL

UL A  without parameters displays the autoselect list.

* Disable

Disables the  Unproto list. The  Unproto frames are not decoded to

the list. The list is saved in the TSTHOST.ULS file in the TSTHost

default directory. The memory used for the list is released.

* Enable

Activates the Unproto list.  The previous list is loaded from disk

(TSTHOST.ULS), then any new message is added to the list.

Being  enabled by  the HomeBBS, a missing message activates a synch

request so to keep the list always updated.

* Get

Connects the HomeBBS to receive the selected messages.

* Interface File (default) | Pms

To  determine if  the received  unproto message  will be saved to a

disk file or conneted to the pms interface.

* List

To display the unproto messages selected and not yet received.

* Query msg_nbr

Manual  request to  the BBS  to send  a  list  from  the  specified

message  number. This  command should  not be  needed,   since  the

program is always automatically synchronised.

TSTHOST v1.36 (c) IK1GKJ                                     31

* Time 0..60 minutes

This  command    sets  the  time  interval,  in  minutes,  for  the

automatic  connection. If  there is   any message selected, TSTHost

connects  the HomeBBS  for the message request. Any value below  15

min.  disables this feature.

* View

Displays  in a  window the  message list.  Use the  standard cursor

keys  to scroll  the list,  INSert to  select/deselect   a message,

ESCape or RETURN to close the window.

TSTHOST v1.36 (c) IK1GKJ                                     32

TSTHost commands description

----------------------------

The uppercase letters are the minimum command abbreviation for the

command recognition. Facultative parameters are  shown  in  square

brackets, while mutually exclusive options are separated by |.

Send,Delete,Dir,Yput,View parameter window

------------------------------------------

If the parameters for the SEND, DELETE and DIR commands  have  not

been  specified,  the  program  activates  a request window on the

screen showing the files contained in the WORKDIR directory.

Select  the  desired  file  with  the  ENTER  key,  by  moving  the

selection   bar  with   the  cursor  keys. To log to another drive,

press  the TAB key and type the corresponding  letter.  The  ESCape

key  aborts the  request. The  ENTER key  has no  effect in the DIR

window.

* AUtoyapp [on|off]

When  activated, TSTHost  starts automatically  the  YAPP  protocol

when  the init  protocol code   is   received  from  the  connected

station,  otherwise you  have to  start the  file download with the

YREC command. The file is saved in the WORKDIR directory.

* AX25 subcommand command

TNC setup command, read the description further on.

* Bell [on|off]

If OFF, disables the PC-speaker beep when a CTRL-G is received.

* CBell [on|off]

When  activated,  the program will emit an acoustic signal for any

connection and disconnection.

* CLS

Clears the receive screen for the active channel.

* CLRbuf

Clears the receive buffer for the active channel.

TSTHOST v1.36 (c) IK1GKJ                                     33

* COLor process_number color_number

Selects the color for the screen display. Read  the  help  text by

issuing this command without parameters.

* Connect callsign [v digipeater digipeater]

Connect the specified  callsign. If the command is executed in the

monitor  windows or  in an  already    connected    channel,    the

program   will  attempt  to  connect  the  callsign  on  the  first

available channel

* COPy source destination

Copies the source file in the  destination name. If a path name is

not specified, the default directory is the WORKDIR. Wildcards are

not allowed, and you cannot  overwrite an existing file.

* CR filename | OFF

OFF  disable  the  function,  otherwise  every  hour  tsthost  will

execute  the specified file. This file for default will be searched

in  tsthost.exe directory,  if in  the filename  is not  present  a

specified  drive or  path. The file may be EXE, COM or BAT. Tsthost

always  attempt to  trapping the  output of the program , but for a

batch  file, is  possible trap  the output  of the  bat but not the

output  of the  program called  by the  batch file. To do this, you

must  redirect to a file or NULL device any program called from the

batch  file. This  is not  a problem, even the program write to the

screen  while executed,  when return  to tsthost  anything will  be

restored. Example of good .bat file:

@ECHO OFF

DOTHIS1 > NULL

DOTHIS2 /a /b >NULL

.....

* DElete [filename]

Deletes  the file.  If  no drive/path  is  specified,  the default

directory is the WORKDIR path. Wildcards are not  allowed.  If the

filename  is not  specified, TSTHost  provides a  screen window for

the selection.

* DIR

Directory  display  in the screen window. The directory start from

the WORKDIR directory.

TSTHOST v1.36 (c) IK1GKJ                                     34

* DISconnect

Disconnects the station.

* ECho [on|off]

If enable,  the  text entered from the keyboard is also written in

the receive window,  in the receive buffer and (if active)  in the

capture file.

* EXIT

Quit from program. No connected channel is disconnected.

* FUnction function_key [command.....]

It   is   possible   to  define ten  commands (up  to 80 characters

long)  that will be executed by depressing the keys  CTRL + F1/F10.

Issue  the   command  with  the  function key number to display the

current  assignment. For  instance,  FU 1 CON IK1MSL-8  assignes to

CTRL-F1  the connect command to IK1MSL-8

* Help or ?

Displays a command list.

* LOg [on|off]

Enables/disables the connection  record  log,  saved  in  the file

TSTHOST.LOG  in the  program directory.  Every  line  in  the  file

contains  the callsign,  time and  date   for any  connection   and

disconnection.   To avoid that this  file  grows  too much,  it  is

suggested   to   move   and   rename  the file in another directory

for further storage every month.

NOTE:   the  file   data  is   valid  only   if   connections   and

disconnections  happen   while   the programs are active (including

the  shell command  RUN). If  the program  starts while  the TNC is

already  connected, the  missing data  are logged  in the  file  as

question marks (??).

* PARam channel text

Use   this   command  with care,  only if  you know  very well  the

WA8DED  host mode software. This command set a specific value for a

channel  or   an    extended  command  not  included  in  the  AX25

subcommands  using the   host  mode commands,  and the  data is not

checked  by TSTHost.  It is  used for  particular settings  or non-

standard  Eproms. The  user is responsible  for  any  data  sent to

the  host interface.  The global communication  channel  is  number

TSTHOST v1.36 (c) IK1GKJ                                     35

0,   the   others   from   1   to  8 correspond  to  the eight task

windows.  Notice that  data written  on channel  0 is stored as the

default,   while   those   written  on  the other channels are used

until  the next  disconnection on  the channel  and return  to  the

channel 0 default value.

* PASsword

Automatic transmission  of  the  requested  password.  Support the

standard  request and  the MD2  algorithm. It  is also  possible to

send the password with the ALT_F1 key.

* RECord filename | off

Captures to a disk file the task activity. The file  logging stops

with  the OFF  filename or with the task  disconnection. The status

line  displays the  disk file  size  and  not  the  received  bytes

number  because   logging to  an existing  file appends data to the

end of the file without deleting the previous text.

To  send the  data to the printer, instead of the disk file, RECord

to a DOS device name (LPT1 PRN etc); REC OFF disables the printer.

It  is not  suggested to  print a  heavy and fast traffic on a slow

printer,  as this  can slow  down the whole system. This command is

not active in a PMS session.

* REName old_name new_name

Change  the name for the specified file. Wildcards are not allowed.

The   default  directory is  WORKDIR. It is possible to move a file

in  the same  disk to  a different  directory by specifying the new

path. For instance:

REN  pippo.txt pluto.txt

The file pippo.txt becomes pluto.txt

The file remains in the default directory.

REN  e:\bin\utils\vp.exe e:\bin\vp.exe

moves the file vp.exe from the path e:\bin\utils into

e:\bin

* RUn [filename]

Executes  the  Filename  DOS  program. If  the program name is not

specified, the command interpreter (COMMAND.COM) is run  until the

EXIT   command,   that   returns  to  TSTHost. The TSTHost activity

is suspended while executing DOS programs.

TSTHOST v1.36 (c) IK1GKJ                                     36

* SEnd [filename | OFF]

Sends the specified file to the connected station. OFF  closes the

file, or  it  is closed at the end of the file or when the task is

disconnected.  The file  must contain  ASCII text  and  not  binary

data,  and the  SEnd command cannot be issued in a PMS session. The

status  line   counter   displays the  number of  bytes sent to the

TNC,   so the file  can  be  closed when the TNC still have data in

the  buffer  to  be  transmitted.  If  the    filename    is    not

specified,  a  selection window is displayed on the screen.

* SOurce filename

Executes  a command  file. The  file contains  the commands  in the

same  form   as   for   the    keyboard.    Each  line  (up  to  80

characters)  must contain  a command.  The source  file is searched

ONLY in the  program directory.

* TNCDelay X Y

This   command   set  some program timings, that you shouldn't need

to  change. The  values are  expressed in  18th of  second and  the

range is from 3 to 255, that correspond to 2 tenth and 14 seconds.

The  X parameter  is   the maximum  delay for the TNC reply. If the

TNC  does   not   reply to  TSTHost   in  the  specific  time,  the

program  retries to synchronise the serial port. The Y parameter is

the speed for the serial port synchronisation.

If   your  Y parameter  is too  high, you  can go  to have a coffee

while  the program  resynchronises! You  must  increase  X  to  fix

frequent  timeouts, and  Y if  the program cannot resync the serial

port.

To   display   the   current  values  of  the  timings,  issue  the

command without parameters.

* UList subcommand command

Command  for FBB  Unproto list,  read   the   specific  section  in

this manual.

* View [filename]

Display  the contents of an ascii file, predefined path is WORKDIR.

If  you do  not provide  a filename,  a selection is made in window

mode.

TSTHOST v1.36 (c) IK1GKJ                                     37

* WOrkdir [path]

Set the default work path for TSTHost.  Usually, it corresponds to

the  directory where  the program  is stored.  When a filename does

not  have a  drive/directory path,  it will  be   searched  in  the

WOrkdir directory.

* WRitebuf filename

Writes  the contents  of the  receive buffer in an ASCII disk file.

If  the filename does not have a drive/directory path specified, it

is saved in the WOrkdir directory.

* YABort

Starts the procedure to interrupt the YAPP transfer protocol.

The  interruption is  not immediate  because the  terminal  station

must  empty   his   buffer   before   accepting   the   command. To

perform  an immediate abort, type this command TWICE.

* YAPpc [on|off]

Enables/disables  the YAPPC  protocol version.  You  cannot  change

this option if there is any YAPP file transfer active.

* YRec filename

YAPP  receive protocol  manual  start.  If  Autoyapp  is  on,  this

command  is not  requested. If  Autoyapp is  off, you  must execute

this  command just  after having  received by the connected station

the   Init  YAPP transmit  code. If  the filename  does not  have a

drive/directory  path specified,  it will  be saved  in the WOrkdir

directory.  This command is not active in a PMS session.

* YPut [filename]

Start  the   YAPP transmit  protocol. If  a drive/directory path is

not  specified, the  file is  taken  from  the  WOrkdir  directory.

This  command is   not  active   in a  PMS session. If the filename

parameter is not specified, the program activates a select window.

TSTHOST v1.36 (c) IK1GKJ                                     38

AX25 Subcommands

----------------

These   parameters   must   follow  the AX25  command, for instance

AX25  MONITOR N.  The TNC manual reports  more  informations  about

these commands.

* AX25l2v2 [On|Off]

Enables the level 2 of the Ax25 protocol.

* Beacon [0-60] [Beacon Text | <Beacon file]

Set   the   beacon   text.  The time value is expressed in minutes,

and  any value  below 5  disables   the   beacon.    If  the  first

character  of the   beacon   text   is '<', then the others are the

text  filename. If  no drive/directory  is specified, then the file

will be  searched in the TSTHOST.EXE directory.

Example for text beacon:   AX BE 30 ST. IK1GKJ * IJ45IK

Example for file beacon:   AX BE 30 <F:\UTILS\BEACON.TXT

* Check [0..65535]

After  this time,  if was  not detect  traffic on  the channel, the

program test the integrity of the link. In 10ms.

* CLock ON | OFF | SET

The SET parameter sets the current time/date on the TNC.

ON enables the time/date display in the monitor window and  in the

status  messages, OFF  disables this  time/date display.  NOTE: Not

all  TNC EPROMs  accept this  command; it  works fine  on TF24  and

TF26.

* Digipeat [On|Off]

Enables/disables the digipeating capability of the TNC.

* FRack [1..65535]

Pause  (if ACK  has not  been received)  for the TNC before sending

the  data packet  again. This  value can  be from  1 to 15 seconds.

Some  TNC EPROMs  accept valid  data from  16 to  65535,  expressed

in 1/33  of second.

* FUllduplex [On|Off]

Enables/disables the full duplex transmission of the TNC.

TSTHOST v1.36 (c) IK1GKJ                                     39

* MAxframe [1..7]

Max number of data packets to transmit before waiting for ACK.

* MHeard [ON | OFF | CLR 3..18]

Without parameters, displays the list of the received stations.

CLR clears the list.

ON  enables the list automatic update.

OFF disables the list automatic update.

The list size, set by CLR, must be a value between 3 and 18.

Warning: This command has been tested with EPROMs TF24 and TF26,

but not any EPROM accept this command.

* MYcall callsign

Set my callsign.

* MOnitor [NIUSC] [+- callsign callsign...]

N disable

I display I  Frames

U display UI frames

S display Supervisory frames

C display Connections frames

+ Include max. 8 callsign

- Exclude max. 8 callsign

* PAclen [80..250]

Byte size of the transmitted packet.

* PErsist [0..255]

Transmit  chance. When   the  TNC  must transmit data, it generates

a  random   number between 0 and 255.  If the number is higher than

the  PErsist value it can transmit, otherwise  the  TNC  must  wait

for SLottime time before generating a new number.

* POrtcall [channel 1..8] [call|OFF]

This  command  assigns  a  different  SSID  or  callsign  for every

channel.   The  command without  parameters displays  the  list  of

those  channels with  a callsign  different from  the  AX25  mycall

value.  OFF   sets  the  channel  to  use  the callsign-ssid set by

the AX25 mycall command.

TSTHOST v1.36 (c) IK1GKJ                                     40

To   activate   the  default callsign, use the OFF parameter. Note:

a  channel with  a particular  callsign will  not  change  even  by

changing  the global callsign. To disable the extended callsign use

OFF.  This command  has effect,  for a  connected  channel,    only

after  the disconnection

Example: AX25 MYCALL IK1GKJ     AX25 PORTCALL 2 IK1GKJ-2

* PTt [ON | OFF]

Enables/disables the TNC PTT line. The  software  works correctly,

but, while in the OFF state, the TNC  does  not  activate  the PTT

line.

* RESptime [0..65535]

Wait time before ACKing an incoming packet, in 10ms.

* RETry [0..127]

Retry  number before  failure and  station disconnection.  0 stands

for forever.

* Slottime [0..127]

See PErsist, in 10ms.

* Txdelay [0..127]

Delay   before   data   transmission after the PTT line activation,

in 10ms.

* UNproto [text]

Unproto text and route where the Unproto messages are addressed.

* USers [0..8]

Max number of incoming connections.

* Validcall [On|Off]

Enables/disables the valid callsign control.

TSTHOST v1.36 (c) IK1GKJ                                     41

THE STATUS LINE

---------------

From the bottom to the top of the screen:

The  last screen  line displays  the channel  status, the number of

bytes  transmitted for  send file,  the disk file size for incoming

text  capture. Only  the TNC  status and the number of free buffers

are displayed for Monitor and Unconnected channels.

* FR is the number of free buffers in the TNC (or TFPCX/R driver).

Every buffer contains 32 bytes. This values helps detecting if the

TNC   is   collapsing,   as  the Host software disables the monitor

if  this value  is under  256. The  TNC collapses  when the  FR  is

around  128. Among   the   various   reasons,  check the high speed

traffic  and slow    serial   connection  baud rate,   the  monitor

active   with  multiconnection, etc.  The free  buffers   value  is

global and not referred to a single channel.

*  BF is  the number  of packet  in the TNC queue to be transmitted

for  the channel.  TSTHost has a ten buffers queue. If the FR value

drops  below 300,  the queue  is reduced to 5, and if FR goes below

210 then the queue is reduced to 2.

This  function does  not  interfere with the MAXFRAME 7 parameter,

since the correct number of packets is BUF+UNACK.

* UN is the number of packets transmitted still to be confirmed.

This value depends on AX25 maxframe.

* RE is the time of the TNC attempts to  transmit  the  unack data

without   receiving   ACK   from   the   other  station. After AX25

retry value, the station will be disconnected.

* US is the amount of memory used by the system

* OK is the free memory in the system

* TX is displayed only by sending a file. This value is the number

of bytes read from the file and  transferred intothe TNC. When the

file transfer terminates on the PC side,  it  can  still  continue

because of the TNC buffer.

* RX is displayed only when the RECORD command is active, and this

value corresponds to the disk file size.

Next line up on the screen  displays the TSTHost status:  CONV for

conversation  mode   (typed data  is sent   to  the other station),

CMD   for  command   mode  (enter   commands  for   TSTHost  to  be

interpreted).  On the  right side  there  is  the  current  channel

number and the callsign and duration of the active connection.

TSTHOST v1.36 (c) IK1GKJ                                     42

Then there is  the  program  version.  REVIEW  indicates  that the

revision  buffer is  activated (PageUp  key);   PMS for the channel

set in a PMS session (default) or active in a PMS session.

Above  these lines,  there is  a line  containing the  callsign for

each  connected channel, from 1 to 8. If there is new data that you

have not yet read from a channel, TSTHost will flash the callsign.

Another line is for the YAPP transfer status line, active  only if

the channel is running a YAPP file transfer.

PROBLEMS

--------

Please   report  any   kind of   problem or  defect with  a message

to

IK1GKJ@IK1MSL, reporting:

- Defect type.

- Which operation you have done when you have noticed the defect.

- PC model and configuration

- Memory driver (EMM, QEMM etc.)

- PC Operating system version.

- (If possible) the TSR programs loaded in memory.

- TNC model and driver type

Thanks.

TSTHOST v1.36 (c) IK1GKJ                                     43

Features ........................................................1

Tsthost with TNC in KISS or BAYCOM modem ........................3

Tsthost with the standard HOST mode .............................4

Tsthost with 2 or more tnc ......................................5

Memory usage and EXPANDED memory ................................6

VGA mode ........................................................6

Short cut keys ..................................................7

Path of the files ...............................................8

PG - Server directory ........................................8

MAIL - Messagges directory ...................................8

USERS - Users directory ......................................9

WORKDIR - Standard work directory ............................9

TSTHOST.PSW - Automatic password file ........................9

TSTHOST.SYS - Password file for PMS DOS command. .............9

TSTHOST.CFG - Automatic configuration file ..................10

TSTHOST.SHD - Shutdown configuration file ...................10

TSTHOST.ULS - Unproto list file .............................10

TSTHOST.ENT - PMS connection text ...........................10

TSTHOST.HLP - PMS help file .................................10

TSTHOST.PGS - PMS PG server list ............................10

TSTHOST.BID - Bid file. .....................................11

TSTHOST.LOG - Connections log ...............................12

SOURCE files ................................................12

TSTHOST.IN - Mail import file ...............................12

TSTHOST.DMS - PMS messages database .........................13

FORWARD,  UNPROTO  list,  CHANNEL  8  and (optional) automatic

password request. .......................................14

FORWARD protocol ...............................................15

FORWARD command ................................................17

SERVER PG ......................................................18

AUTOMATIC PASSWORD .............................................20

PMS INTERFACE ..................................................21

THE SERVER ..................................................22

PMS command from KEYBOARD ...................................22

PMS commands for remote users ...............................26

UNPROTO LIST ...................................................28

IMPORTANT NOTE ..............................................29

UList Subcommands ...........................................30

TSTHost commands description ...................................32

Send,Delete,Dir,Yput,View parameter window ..................32

AX25 Subcommands ............................................38

THE STATUS LINE ................................................41