�� 7PNOU 2.7 ��

7PLUS sender for TSTHOST

Hello and thank you for uploading this server !

7PNOU is free, so you can use it as much as you want with TSTHOST.

It allows you to send binary files on the Packet network via the well known

7PLUS protocol.

The size of 7PLUS files can be set in using a variable located in 7PNOU.CFG

It also allows you to automatically send correction files, without calculating

for hours!

You just have to know 2 different syntaxes to use it:

Install:

-------

The 7PNOU.EXE file MUST be copied in your TSTHOST main directory (the one

where TSTHOST.EXE is located). The 7PLUS.EXE file must be on your computer in

ONLY ONE DIRECTORY accessible by the path !!!

Creation of 7PNOU.CFG:

---------------------

******************************************************************************

**  WARNING: You must delete any 7PNOU.CFG existing file if you use a prior **

**           version of 7PNOU !!!                                           **

******************************************************************************

Run 7PNOU.EXE once and answer the Questions...

Callsign:  Your of course!

Home BBS:  The BBS where you use to upload your messages.

Work Directory: It is the directory where the server looks for files to encode.

Max Size: It is the size of each 7PLUS file sent in bytes.

Please note that the size of generated files should not be too short or too

large, a good average is 3600 bytes.

TSTHOST must know that the 7PNOU server exists, so you must edit your

TSTHOST.CFG file and add his name on the SERVER line: SERVER +7PNOU...

Why is it better to use this program to send files ?

AUTO7P knows very well hows to do the same job, but it has been written to run

on BBS which are big stations used  for national and  international forwarding.

AUTO7P creates personnal messages adressed to AUTO7P @REMOTE_ADDRESS and a .ERR

file is  sent  to AUTO7P @YOUR_ADDRESS  if an  error is detected  by the remote

AUTO7P server.

But your PMS is not in the address book of BBS's and you will never receive

.ERR files addressed to you !

You will never encounter such a problem in using my program 7PNOU !

Moreover I feel that the files created by AUTO7P are a bit too big and so it

is not easy to forward them. Do not forget that it is almost impossible to

compress a 7PLUS file !

7PNOU also allows you to decode files sent with the 7PLUS protocol when you

receive them on your TSTHOST PMS. You just have to declare the line: 7PCOPY ON

in you TSTHOST.CFG file. Thus, any file containing a valid 7PLUS code will be

saved in a file named 7PDATA.FWD in the WORKDIR of TSTHOST (see TSTHOST.CFG).

Extraction of 7PLUS files:

-------------------------

TSTHOST has a file containing the commands which should be executed every

hour, TSTHOST.CRN

you just add the lines :

RUN 7PNOU EXTRACT [Full path]\7PDATA.FWD

RUN 7PNOU DELETIME 60

PS: the field [Full path] with \ should only be used if the file 7PDATA.FWD is

NOT in the main directory of TSTHOST.

The 7PNOU DELETIME 60 parameter tells the server that any 7PLUS file saved in

the C:\7PNOU directory which is older than 60 days must be deleted.

About the way that 7PLUS files are saved:

The server will create a directory named 7PNOU in the root of your hard drive

in order to save 7PLUS files. If all the parts of a file are collected, it will

be decoded and placed in the directory C:\7PNOU\CONVERT. Then a message will be

sent to the Sysop to advise him of the change. If decoding is not successfull,

a .ERR file will be adressed to the sender.

You absolutely do not have to create the C:\7PNOU directory, leave the program

alone... He knows very well how to do it by his own !!!

Sending a file with 7PLUS and 7PNOU:

-----------------------------------

IF YOU WANT TO SEND THE FILE MYPROG.ZIP:

Send a personnal message to 7PNOU:

SP 7PNOU

Title:

MYPROG.ZIP

Message:

7PNOU TO S[P/B] CALL @ADRESS

<end of message>

If your file MYPROG.ZIP is too big, you should use the option PART:

SP 7PNOU

Title:

MYPROG.ZIP

Message:

7PNOU TO S[P/B] CALL @ADRESS

PART 1-10

<end of message>

You can do two different things in using PART:

- Create a single file       : PART 2 will create MYPROG.P02

- Create a full set of files : PART 9-15 will create MYPROG.P09

to MYPROG.P0F

Warning: Your version of 7PLUS.EXE should be at least 2.15 if you

want to use the PART parameter !

I could have fixed this problem, but there would have been

many disk access.

If you can not find 7PLUS 2.15 or better, I will send you

mine... Just drop me a message!

Thus you can send your program by small part, a little bit every day...

Sending an INFO message :

-----------------------

After the "7PNOU TO..."   and "PART a-b" parameters, just add a line with INFO

eg:

SP 7PNOU

MYPROG.ZIP

7PNOU TO S[P/B] CALL @ADRESS

*  PART 1-10

INFO

Here is your info message. All the text you will write down here will

be sent in a message. It's title will be created by 7PNOU and will

look like "(INFO) MYPROG.ZIP".

*PART is optional

Sending an INF 7PLUS message :

----------------------------

eg:

SP 7PNOU

MYPROG.ZIP

7PNOU TO S[P/B] CALL @ADRESS

*  PART 1-10

INF7 Label of MYPROG.ZIP

*  INFO

Here is your info message. All the text you will write down here will

be sent in a message. It's title will be created by 7PNOU and will

look like "(INFO) MYPROG.ZIP".

* : PART and INFO are optional

The .INF file is append in the (INFO) message.

NOTE:  7PNOU TO  dest @ route  is the only request varible.

PART a-b

INF7 label

INFO                        (INFO MUST BE THE LAST !)

textinfo...

are optionals variables.

Creation of a correction file:

-----------------------------

This fonction is fully automatic IF YOU HAVE KEPT A COPY OF THE ORIGINAL FILE

IN THE WORK DIRECTORY OF 7PNOU (the directory where the server looks for files

to encode).

7PNOU will look for the sender of the message, create MYPROG.COR and send him

the file.

News on version 2.7:

-------------------

Bug 2.5 for send .COR files : fixed in 2.5a

A new directory \7PNOU\7PTEMP used for temp files by the program.

A new variable INF7 to send .INF file with the label of your peogram. It can

be used to give information to the BBS SYSOPS.

A new variable INFO is now available to send INFO messages. These messages allow

you to describe the file you want to send.

The program now speaks english in order to be usable by most of the ham's.

7PNOU needs at least roughly 140 Kbytes of memory to run. You should look at the

parameter Ok: xxxxxxxx on the right bottom corner of the screen. The value must

be above 154000 so you ave about 140Ko in shell dos.

7PNOU does exactly the same job AUTO7P does for BBS !

7PNOU sends messages to the user if there is a syntax error, if he can not find

the file to send...

If the server can not find the file, you will receive the contents of the Work

directory of 7PNOU.

A message will also be sent to the sysop to give him a Work Report.

Thank you so much for using this server !

Please send me your comments...

73 de Eric  F1NOU @F6CDD.FMLR.FRA.EU

Translated by Pascal F5JSD @F6CDD.FMLR.FRA.EU